3GPP TSG-SA5 Meeting #150 	S5-235767

Goteborg, Sweden, 21 - 25 August 2023

# Source:	MATRIXX Software

# Title:	pCR 28.204 Introduce applicable triggers

# Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This pCR is to introduce applicable triggers.

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.204 "Network slice-specific authentication and authorization charging in the 5G System (5GS)".

# 3	Rationale

This pCR is to introduce applicable triggers.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.204 [2]:

First change

#### 5.X.1.2	Applicable Triggers

##### 5.X.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. SUPI).

Each trigger condition (i.e. chargeable event) defined for the Network Slice-Specific Authentication and Authorization converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.X.1.2.1.1 summarizes the set of default trigger conditions and their category which shall be supported by the NSSAAF when charging is active for the corresponding Network Slice-Specific Authentication and Authorization functionality. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from NSSAAF towards the CHF.

Table 5.X.1.2.1.1: Default Trigger conditions in NSSAAF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Network Slice-Specific Authentication and Authorization request | - | Immediate | Not Applicable | Yes | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Network Slice-Specific Authentication and Authorization completed | - | Immediate | Not Applicable | Yes | PEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |

Table 5.X.1.2.1.2 summarizes the set of default trigger conditions and their category which shall be supported by the AMF when charging is active for the corresponding Network Slice-Specific Authentication and Authorization functionality. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from AMF towards the CHF.

Table 5.X.1.2.1.2: Extended Default Trigger conditions in AMF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Network Slice-Specific Authentication and Authorization completed | - | Immediate | Not Applicable | Yes | PEC: Charging Data Request [Event] |

NOTE: If the same triggers are enabled in the NSSAAF and the AMF, the respective CHF CDRs will be associated to the same procedure.

End of change