3GPP TSG-SA5 Meeting #150 	S5-235768

Goteborg, Sweden, 21 - 25 August 2023

# Source:	MATRIXX Software

# Title:	pCR 28.204 Introduce PEC message flow

# Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This pCR is to introduce PEC message flow

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.204 "Network slice-specific authentication and authorization charging in the 5G System (5GS)".

# 3	Rationale

This pCR is to introduce PEC message flow.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.204 [2]:

First change

### 5.X.2	Message flows

#### 5.X.2.1	General

The flows in the present document specify the interaction between the NSSAAF and the CHF and between the AMF and the CHF, for Network slice-specific authentication and authorization converged charging functionality, in different scenarios, based on 3GPP TS 23.501 [3] and 3GPP TS 23.502 [4] procedures and flows.

This interaction is based on Charging Data Request /Response specified in 3GPP TS 32.290 [6], exchanged between the NSSAAF embedding the CTF and the CHF, and between the AMF embedding the CTF and the CHF.

If both NSSAAF and AMF generate CHF CDRs, for the same procedure, they will be the same.

The following scenarios are supported by NSSAAF:

- 	PEC;

-	IEC;

-	ECUR.

The PEC scenario is supported by AMF.

As a general principle, the steps in the figures for the message flows below correspond to the steps of figures in 3GPP TS 23.502 [4], which is the reference. The present document specifies the charging specific extension part.

Next change

#### 5.X.2.2	Network slice-specific authentication and authorization charging

##### 5.X.2.2.1	General

The subclauses below describe Network slice-specific authentication and authorization charging message flows based on figure 4.2.9.2-1 of  3GPP TS 23.502 [4].

Next change

##### 5.X.2.2.x	Network slice-specific authentication and authorization – AMF - PEC

The following figure 5.X.2.2.x-1 describes a Network slice-specific authentication and authorization charging in PEC scenario for AMF:

Figure 5.X.2.2.x-1: Network slice-specific authentication and authorization – AMF - PEC

Steps 1 to 18: per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

18ch-a: NSSAA procedure in AMF completed:  AMF sends Charging Data Request [Event] to CHF with EAP-Success/Failure, GPSI and S-NSSAI.

18ch-b: The CHF creates a CDR.

18ch-c: CHF provides response to AMF.

Next change

##### 5.X.2.2.y	Network slice-specific authentication and authorization – NSSAAF - PEC

The following figure 5.X.2.2.y-1 describes a Network slice-specific authentication and authorization charging in PEC scenario for NSSAAF:

Figure 5.X.2.2.y-1: Network slice-specific authentication and authorization - PEC

Steps 1 to 17: per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

17ch-a: NSSAAF procedure with AAA-S for NSSAA is completed:  NSSAAF sends Charging Data Request [Event] to CHF with EAP-Success/Failure, GPSI and S-NSSAI.

17ch-b: The CHF creates a CDR.

17ch-c: CHF provides response to NSSAAF.

Step 18: per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

End of change