3GPP TSG-SA5 Meeting #150 	S5-235804

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0493 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Caller and Callee Information in Stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDC_CH |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Attributes for IMS volume-based charging added in TS 32.255 should also be added in TS 32.291 accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add callInfo as an attribute of PDUSessionChargingInformation. Add IDC_CH as a supported feature. Add call information in bindings for 5G data connectivity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging information for IMS volume-based charging is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2.6, 6.1.8, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

Table 6.1.6.1-2: Nchf_ConvergedCharging re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Supi |  | 3GPP TS 29.571 [371] |  | The identification of the user (i.e. IMSI, NAI, GLI, GCI). (NOTE 1) |  |  |  |
| Uint32 |  | 3GPP TS 29.571 [371] |  | Unsigned 32-bit integers |  |  |  |
| Uint64 |  | 3GPP TS 29.571 [371] |  | Unsigned 64-bit integers |  |  |  |
| PduSessionId |  | 3GPP TS 29.571 [371] |  | The identification of the PDU session. |  |  |  |
| PduSessionType |  | 3GPP TS 29.571 [371] |  | the type of a PDU session |  |  |  |
| Uri |  | 3GPP TS 29.571 [371] |  | String providing an URI |  |  |  |
| AccessType |  | 3GPP TS 29.571 [371] |  | The identification of the type of access network. |  |  |  |
| DateTime |  | 3GPP TS 29.571 [371] |  | The time. |  |  |  |
| ChargingId |  | 3GPP TS 29.571 [371] |  | Charging identifier allowing correlation of charging information |  |  |  |
| RatType |  | 3GPP TS 29.571 [371] |  | The identification of the RAT type. |  |  |  |
| RatingGroup |  | 3GPP TS 29.571 [371] |  | The identification of the rating group |  |  |  |
| IpAddr |  | 3GPP TS 29.571 [371] |  | Ipv4 address, Ipv6 address, or Ipv6Prefix |  |  |  |
| Ipv4Addr |  | 3GPP TS 29.571 [371] |  | Ipv4 address. |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [371] |  | The Ipv6 prefix allocated for the user. |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [371] |  | Ipv6 Address. |  |  |  |
| Pei |  | 3GPP TS 29.571 [371] |  | The Identification of a Permanent Equipment. |  |  |  |
| TimeZone |  | 3GPP TS 29.571 [371] |  | Time zone information |  |  |  |
| NfInstanceId |  | 3GPP TS 29.571 [371] |  | String uniquely identifying a NF instance. |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [371] |  | String identifying a Gpsi |  |  |  |
| DefaultQosInformation |  | 3GPP TS 29.571 [371] |  | Identifies the information of the default QoS. |  |  |  |
| SubscribedDefaultQos |  | 3GPP TS 29.571 [371] |  | subscribed default QoS. |  |  |  |
| AuthorizedDefaultQos |  | 3GPP TS 29.512 [302] |  | Authorized default QoS. |  |  |  |
| Ambr |  | 3GPP TS 29.571 [371] |  | Aggregate Maximum Bit rate |  |  |  |
| QosData |  | 3GPP TS 29.512 [302] |  | Contains QoS parameters |  |  |  |
| UserLocation |  | 3GPP TS 29.571 [371] |  | User location information |  |  |  |
| PlmnId |  | 3GPP TS 29.571 [371] |  | PLMN id |  |  |  |
| Guami |  | 3GPP TS 29.571 [371] |  | Globally Unique AMF Identifier |  |  |  |
| DurationSec |  | 3GPP TS 29.571 [371] |  | Identifies a period of time in units of seconds. |  |  |  |
| Snssai |  | 3GPP TS 29.571 [371] |  | SNSSAI |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [371] |  | additional details of the error |  |  |  |
| ServiceId |  | 3GPP TS 29.571 [371] |  | Identifier of service |  |  |  |
| SscMode |  | 3GPP TS 29.571 [371] |  | SSC Mode type |  |  |  |
| PresenceInfo |  | 3GPP TS 29.571 [371] |  | PRA information including PRAId, PRA element list and PRA status |  |  |  |
| Qfi |  | 3GPP TS 29.571 [371] |  | QoS flow identifier designated as "Qfi". |  |  |  |
| AmfId |  | 3GPP TS 29.571 [371] |  | AMF identifier |  |  |  |
| Dnn |  | 3GPP TS 29.571 [371] |  | Data Network Name |  |  |  |
| GroupId |  | 3GPP TS 29.571 [371] |  | Network internal Identifier for a group of IMSIs |  |  |  |
| ExternalGroupId |  | 3GPP TS 29.571 [371] |  | External Group Identifier for one or more subscriptions associated to a group of IMSIs |  |  |  |
| Bytes |  | 3GPP TS 29.571 [371] |  | String with format "byte" |  |  |  |
| Tai |  | 3GPP TS 29.571 [371] |  | Tracking Area Identifier |  |  |  |
| Area |  | 3GPP TS 29.571 [371] |  | List of TACs or Operator specific codes |  |  |  |
| CoreNetworkType |  | 3GPP TS 29.571 [371] |  | 5GC or EPC |  |  |  |
| ServiceAreaRestriction |  | 3GPP TS 29.571 [371] |  | Service Area restriction |  |  |  |
| GlobalRanNodeId |  | 3GPP TS 29.571 [371] |  | Global RAN Node Id |  |  |  |
| QosCharacteristics |  | 3GPP TS 29.512 [302] |  | Map of QoS characteristics for non standard 5QIs and non-preconfigured 5QIs. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [371] |  | See TS 29.500 [299] clause 6.6 |  |  |  |
| NsiLoadLevelInfo |  | 3GPP TS 29.520 [306] |  | Represents the load level information for an S-NSSAI and the associated network slice instance |  |  |  |
| ServiceExperienceInfo |  | 3GPP TS 29.520 [306] |  | ServiceExperience |  |  |  |
| ApplicationChargingId |  | 3GPP TS 29.571 [371] |  | Application provided charging identifier allowing correlation of charging information. |  | AF_Charging_Identifier |  |
| SharingLevel |  | 3GPP TS 28.541 [254] |  | Ressources sharing level |  |  |  |
| MobilityLevel |  | 3GPP TS 28.541 [254] |  | UE mobility Level |  |  |  |
| SsT |  | 3GPP TS 28.541 [254] |  | Slice Service type (SST) |  |  |  |
| Support |  | 3GPP TS 28.541 [254] |  | Supported, Not Supported indicator |  |  |  |
| Float |  | 3GPP TS 29.571 [371] |  | Number with format "float" |  |  |  |
| MaPduIndication |  | 3GPP TS 29.512 [302] |  | MA PDU session indication |  | ATSSS |  |
| AtsssCapability |  | 3GPP TS 29.571 [371] |  | ATSSS capabilities |  | ATSSS |  |
| SteeringFunctionality |  | 3GPP TS 29.571 [371] |  | Steering functionalities for MA PDU session |  | ATSSS |  |
| SteeringMode |  | 3GPP TS 29.512 [302] |  | Steering mode for MA PDU session |  | ATSSS |  |
| OperationalState |  | 3GPP TS 28.623 [257] |  | Operational state |  |  |  |
| AdministrativeState |  | 3GPP TS 28.623 [257] |  | Administrative state |  |  |  |
| RanNasRelCause |  | 3GPP TS 29.512 [302] |  | Indicates the RAN or NAS release cause code information. |  | EnhancedDiagnostics |  |
| Ecgi |  | 3GPP TS 29.571 [371] |  | E-UTRA Cell Id |  |  |  |
| Ncgi |  | 3GPP TS 29.571 [371] |  | NR Cell Id |  |  |  |
| ServingLocation |  | 3GPP TS 28.538 [310] |  | Serving location |  | Edge Computing |  |
| SoftwareImageInfo |  | 3GPP TS 28.538 [310] |  | Software image information |  | Edge Computing |  |
| AffinityAntiAffinity |  | 3GPP TS 28.538 [310] |  | Affinity and anti-requirements |  | Edge Computing |  |
| VirtualResource |  | 3GPP TS 28.538 [310] |  | Virtual resource requirements |  | Edge Computing |  |
| PlmnIdNid |  | 3GPP TS 29.571 [371] |  | PLMN Identity and, for SNPN, Network Identity |  | SNPN |  |
| CallInfo |  | 3GPP TS 29.512 [302] |  | Caller and callee information |  | IDC_CH |  |
| NOTE 1: A SUPI containing GLI or GCI is used to support 5G-RG and FN-RG in scenarios of wireline network. |  |  |  |  |  |  |  |

2nd Change

6.1.6.2.2.6	Type PDUSessionChargingInformation

Table 6.1.6.2.2.6-1: Definition of type PDUSessionChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| chargingId | ChargingId | OM | 0..1 | Charging identifier for correlation between different records of a single PDU session |  |
| sMFChargingId | string | OM | 0..1 | Used instead of ChargingId when feature is active. | SMF_Charging_Id |
| homeProvided ChargingId | ChargingId | Oc | 0..1 | Charging identifier for correlation between H-SMF and V-SMF. |  |
| sMFHomeProvidedChargingId | string | OM | 0..1 | Used instead of homeProvidedChargingId when feature is active. | SMF_Charging_Id |
| userInformation | UserInformation | OM | 0..1 | including information of user and user equipment, |  |
| userLocationinfo | UserLocation | OC | 0..1 | provides information on the location |  |
| IMS Session Information | CallInfo | OC | 0..1 | Indicates the IMS session related information | IDC_CH |
| mAPDUNon3GPPUserLocationInfo | UserLocation | OC | 0..1 | provides information on the location under the non-3GPP access for the MA PDU session | ATSSS |
| non3GPPUserLocationTime | DateTime | OC | 0..1 | represents the UTC time provided by the non-3GPP access, and is related to the userLocationTime. This filed is only present if the non-3GPP access provides a time. |  |
| mAPDUNon3GPPUserLocationTime | DateTime | OC | 0..1 | represents the UTC time provided by the non-3GPP access, and is related mAPDUNon3GPPUserLocationInfo. This filed is only present if the non-3GPP access for the MA PDU session provides a time. | ATSSS |
| presenceReportingAreaInformation | map(PresenceInfo) | OC | 0..N | When the data type is present in response message, it includes the PRA information provisioned by the CHF, in which case the "presenceState" attribute within the PresenceInfo data type shall not be supplied. When the data type is present in request message, it’s used to report user presence reporting area status. The "praId" attribute within the PresenceInfo data type shall be the key of the map. The location related attributes (i.e. "trackingAreaList", "ecgiList", "ncgiList") within the PresenceInfo data type are not required in the request message, and may be ignored by the CHF. |  |
| uetimeZone | TimeZone | OC | 0..1 | the UE Timezone the UE is currently located |  |
| pduSessionInformation | PDUSessionInformation | OC | 0..1 | PDU session level information, including PDU session ID, PDU type, SSC Mode, QoS, network slicing etc. It needs to be present in the request, but it is optional in the response. |  |
| unitCountInactivityTimer | DurationSec | OC | 0..1 | threshold for the time period resource idle Upon the initial interaction with the CHF, the SMF use this attribute to provide pre-configured threshold to CHF. when present in response message, it contains the threshold supplied by CHF in response of initial request to override existing threshold in SMF. It’s only present when unit count inactivity timer trigger is active. |  |
| rANSecondaryRATUsageReport | RANSecondaryRATUsageReport | OC | 0..1 | Secondary RAT usage reported from RAN. |  |

3rd Change

6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nchf_ConvergedCharging API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [299].

Table 6.1.8-1: Supported Features

| Feature number |  |  | Feature Name |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | CHFCQM |  |  | CHF-controlled quota management i.e. support for temporary offline |  |  |
| 2 |  |  | AF_Charging_Identifier |  |  | Indicates the support of long character strings as charging identifiers. |  |  |
| 3 |  |  | 5GIEPC_CH |  |  | 5GS interworking with EPC |  |  |
| 4 |  |  | ATSSS |  |  | This feature indicates support of Access Traffic Steering, Switching, Splitting (ATSSS). |  |  |
| 5 |  |  | ETSUN |  |  | This feature indicates support of Enhancing Topology of SMF and UPF in 5G Networks (ETSUN). |  |  |
| 6 |  |  | EnhancedDiagnostics |  |  | Support the enhanced diagnostics |  |  |
| 7 |  |  | AMF_subs_PRA |  |  | PRA(s) subscription by CHF in AMF |  |  |
| 8 |  |  | FilterRuleList |  |  | Support of multiple filter rules in the final unit indication |  |  |
| 9 |  |  | TEI17_NIESGU |  |  | This feature indicates support of GERAN/UTRAN access |  |  |
| 10 |  |  | IMS |  |  | This feature indicates support of IMS. |  |  |
| 11 |  |  | QoSMonitoring |  |  | This feature indicates support of QoS Monitoring |  |  |
| 12 |  |  | Announcement |  |  | This feature indicates support of announcements. |  |  |
| 13 |  |  | 5GLAN |  |  | This feature indicates support of 5G LAN-type services. |  |  |
| 14 |  |  | URLLC |  |  | This feature indicates support of URLLC. |  |  |
| 15 |  |  | NotifyInfoResponse |  |  | This feature indicates support of response with information for a notification. |  |  |
| 16 |  |  | ES4xx |  |  | Extended Support of HTTP 400, 403, 404 allowing use of either ChargingDataResponse or ProblemDetails in the response. |  |  |
| 17 |  |  | ES3xx |  |  | Extended Support of HTTP 307 and 308 redirections, an NF that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15 and 16. |  |  |
| 18 |  |  | EdgeComputing |  |  | This feature indicates support of edge computing domain charging. |  |  |
| 19 |  |  | 5GSCIoT |  |  | This featute indicates support of 5GS control plane CIoT optimization |  |  |
| 20 |  |  | SMF_Charging_Id |  |  | Indicates the support of strings as SMF charging identifiers. |  |  |
| 21 |  |  | SNPN |  |  | This feature indicates support of Stand-alone Non-Public Network. |  |  |
| 22 |  |  | IDC_CH |  |  | This feature indicates support of IMS Data Channel charging. |  |  |

4st Change

7.2	Bindings for 5G data connectivity

Table 7.2-1: Bindings of 5G data connectivity CDR field, Information Element and Resource Attribute

| Information Element | CDR Field | Resource Attribute |
| --- | --- | --- |
|  |  | ChargingDataRequest |
| Supported Features | - | /supportedFeatures |
| Multiple Unit Usage | List of Multiple Unit Usage | /multipleUnitUsage |
| UPF ID | UPF ID | /multipleUnitUsage/uPFID |
| Multi-homed PDU address | Multi-homed PDU address | /multipleUnitUsage/multihomedPDUAddress |
| Used Unit Container | Used Unit Container | /multipleUnitUsage/usedUnitContainer |
| PDU Container Information | PDU Container Information | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation |
| Time of First Usage | Time of First Usage | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/timeofFirstUsage |
| Time of Last Usage | Time of Last Usage | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/timeofLastUsage |
| QoS Information | QoS Information | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/qoSInformation |
| QoS Characteristics | QoS Characteristics | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/qoSCharacteristics |
| AF Charging Identifier | AF Charging Identifier | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/afChargingIdentifier |
| AF Charging Id String | AF Charging Id String | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/afChargingIdString |
| User Location Information | User Location Information | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/userLocationInformation |
| UE Time Zone | UE Time Zone | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/uetimeZone |
| RAT Type | RAT Type | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/rATType |
| Serving Network Function ID | Serving Network Function ID | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/servingNodeID |
| Presence Reporting Area Information | Presence Reporting Area Information | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/ presenceReportingAreaInformation |
| 3GPP PS Data Off Status | 3GPP PS Data Off Status | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/3gppPSDataOffStatus |
| MA PDU Steering functionality | MA PDU Steering functionality | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/mAPDUSteeringFunctionality |
| MA PDU Steering mode | MA PDU Steering mode | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/mAPDUSteeringMode |
| Sponsor Identity | Sponsor Identity | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/sponsorIdentity |
| Application Service Provider Identity | Application Service Provider Identity | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/applicationserviceProviderIdentity |
| Charging Rule Base Name | Charging Rule Base Name | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/chargingRuleBaseName |
| Traffic Forwarding Way | Traffic Forwarding Way | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/trafficForwardingWay |
| Qos Monitoring Report | Qos Monitoring Report | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/qosMonitoringReport |
| PDU Session Charging Information | PDU Session Charging Information | /pDUSessionChargingInformation |
| Charging Id | Charging Id | /pDUSessionChargingInformation/chargingId |
| SMF Charging Id | SMF Charging Id | /pDUSessionChargingInformation/sMFchargingId |
| Home Provided ChargingId | Home Provided ChargingId | /pDUSessionChargingInformation/ homeProvidedChargingId |
| SMF Home Provided ChargingId | SMF Home Provided ChargingId | /pDUSessionChargingInformation/ sMFHomeProvidedChargingId |
| User Information | User Information | /pDUSessionChargingInformation/ userInformation |
| User Identifier | User Identifier | /pDUSessionChargingInformation/userInformation/servedGPSI |
| User Equipment Info | User Equipment Info | /pDUSessionChargingInformation/userInformation/ servedPEI |
| Unauthenticated Flag | Unauthenticated Flag | /pDUSessionChargingInformation/userInformation/ unauthenticatedFlag |
| Roamer In Out | Roamer In Out | /pDUSessionChargingInformation/userInformation/ roamerInOut |
| User Location Info | User Location Info | /pDUSessionChargingInformation/ userLocationinfo |
| User Location Time |  |  |
| IMS Session Information | Call Information | /pDUSessionChargingInformation/iMSSessionInformation |
| Caller Information | Caller Information | /pDUSessionChargingInformation/iMSSessionInformation/callerInformation |
| Callee Information | Callee Information | /pDUSessionChargingInformation/ iMSSessionInformation/calleeInformation |
| MA PDU Non 3GPP User Location info | MA PDU Non 3GPP User Location info | /pDUSessionChargingInformation/mAPDUNon3GPPUserLocationInfo |
| Non 3GPP User Location Time | Non 3GPP User Location Time | /pDUSessionChargingInformation/non3GPPUserLocationTime |
| MA PDU Non 3GPP User Location Time | MA PDU Non 3GPP User Location Time | /pDUSessionChargingInformation/mAPDUNon3GPPUserLocationTime |
| UE Time Zone | UE Time Zone | /pDUSessionChargingInformation/uEtimeZone |
| Presence Reporting Area Information | Presence Reporting Area Information | /pDUSessionChargingInformation/ presenceReportingAreaInformation |
| PDU Session Information | PDU Session Information | /pDUSessionChargingInformation/pduSessionInformation |
| PDU Session ID | PDU Session ID | /pDUSessionChargingInformation/pduSessionInformation/pduSessionID |
| Network Slice Instance Identifier |  | /pDUSessionChargingInformation/pduSessionInformation/networkSlicingInfo |
| S NSSAI | Network Slice Instance Identifier | /pDUSessionChargingInformation/pduSessionInformation/networkSlicingInfo/sNSSAI |
| HPLMN S NSSAI | HPLMN S NSSAI | /pDUSessionChargingInformation/pduSessionInformation/networkSlicingInfo/hPlmnSNSSAI |
| PDU Type | PDU Type | /pDUSessionChargingInformation /pduSessionInformation/pduType |
| PDU Address | PDU Address | /pDUSessionChargingInformation /pduSessionInformation/pduAddress |
| PDU IPv4 Address | PDU IPv4 Address | /pDUSessionChargingInformation/pduSessionInformation/pduAddress/pduIPv4Address |
| PDU IPv6 Address with prefix | PDU IPv6 Address with prefix | /pDUSessionChargingInformation/pduSessionInformation/pduAddress/pduIPv6Addresswithprefix |
| PDU Address prefix length | PDU Address prefix length | /pDUSessionChargingInformation /pduSessionInformation/pduAddress/pduAddressprefixlength |
| IPv4 Dynamic Address Flag | IPv4 Dynamic Address Flag | /pDUSessionChargingInformation /pduSessionInformation/pduAddress/ iPv4dynamicAddressFlag |
| IPv6 Dynamic Address Flag | IPv6 Dynamic Prefix Flag | pDUSessionChargingInformation /pduSessionInformation/pduAddress/ iPv6dynamicPrefixFlag |
|  |  | pDUSessionChargingInformation /pduSessionInformation/pduAddress/ addIpv6AddrPrefixList |
| Additional PDU IPv6 Prefixes | Additional PDU IPv6 Prefixes | pDUSessionChargingInformation /pduSessionInformation/pduAddress/ addIpv6AddrPrefixes |
| SSC Mode | SSC Mode | /pDUSessionChargingInformation /pduSessionInformation/sscMode |
| MA PDU session information | MA PDU session information | /pDUSessionChargingInformation /pduSessionInformation/mAPDUSessionInformation |
| MA PDU session indicator | MA PDU session indicator | /pDUSessionChargingInformation /pduSessionInformation/mAPDUSessionInformation/mAPDUSessionIndicator |
| ATSSS capability | ATSSS capability | /pDUSessionChargingInformation /pduSessionInformation/mAPDUSessionInformation/aTSSSCapability |
| SUPI PLMN ID | SUPI PLMN ID | /pDUSessionChargingInformation /pduSessionInformation/hPlmnId |
| Serving Network Function ID | Serving Network Function ID | /pDUSessionChargingInformation /pduSessionInformation/ servingNetworkFunctionID |
| Serving CN PLMN ID | Serving CN PLMN ID | /pDUSessionChargingInformation/pduSessionInformation/servingCNPlmnId |
| RAT Type | RAT Type | /pDUSessionChargingInformation /pduSessionInformation/ratType |
| MA PDU Non 3GPP RAT Type | MA PDU Non 3GPP RAT Type | /pDUSessionChargingInformation /pduSessionInformation/mAPDUNon3GPPRATType |
| Data Network Name Identifier | Data Network Name Identifier | /pDUSessionChargingInformation /pduSessionInformation/dnnid |
| DNN Selection Mode | DNN Selection Mode | /pDUSessionChargingInformation /pduSessionInformation/dNNselectionMode |
| Authorized QoS information | Authorized Qos Information | /pDUSessionChargingInformation /pduSessionInformation/authorized qoSInformation |
| Subscribed QoS Information | Subscribed QoS Information | /pDUSessionChargingInformation /pduSessionInformation/subscribedQoSInformation |
| Authorized Session-AMBR | Authorized Session-AMBR | /pDUSessionChargingInformation /pduSessionInformation/authorizedSessionAMBR |
| Subscribed Session-AMBR | Subscribed Session-AMBR | /pDUSessionChargingInformation /pduSessionInformation/subscribedSessionAMBR |
| Charging Characteristics | Charging Characteristics | /pDUSessionChargingInformation /pduSessionInformation/ chargingCharacteristics |
| Charging Characteristics Selection Mode | Charging Characteristics Selection Mode | /pDUSessionChargingInformation /pduSessionInformation/chargingCharacteristicsSelectionMode |
| PDU session start Time | PDU session start Time | /pDUSessionChargingInformation /pduSessionInformation/startTime |
| PDU session stop Time | PDU session stop Time | /pDUSessionChargingInformation /pduSessionInformation/stopTime |
| Diagnostics | Diagnostics | /pDUSessionChargingInformation /pduSessionInformation/diagnostics |
| Enhanced Diagnostics | Enhanced Diagnostics | /pDUSessionChargingInformation /pduSessionInformation/enhancedDiagnostics |
| 3GPP PS Data Off Status | 3GPP PS Data Off Status | /pDUSessionChargingInformation /pduSessionInformation/3gppPSDataOffStatus |
| Session Stop Indicator | Session Stop Indicator | /pDUSessionChargingInformation /pduSessionInformation/sessionStopIndicator |
| Redundant Transmission Type | Redundant Transmission Type | /pDUSessionChargingInformation /pduSessionInformation/redundantTransmissionType |
| PDU Session Pair ID | PDU Session Pair ID | /pDUSessionChargingInformation /pduSessionInformation/pDUSessionPairID |
| 5G LAN Type Service | 5G LAN Type Service | /pDUSessionChargingInformation /pduSessionInformation/5GLANTypeService |
| Internal Group Identifier | Internal Group Identifier | /pDUSessionChargingInformation /pduSessionInformation/5GLANTypeService/internalGroupIdentifier |
| SNPN ID | SNPN ID | /pDUSessionChargingInformation /pduSessionInformation/sNPNID |
| Unit Count Inactivity Timer | - | /pDUSessionChargingInformation/unitCountInactivityTimer |
| RAN Secondary RAT Usage Report | RAN Secondary RAT Usage Report | /pDUSessionChargingInformation/rANSecondaryRATUsageReport |
| NG RAN Secondary RAT Type | NG RAN Secondary RAT Type | /pDUSessionChargingInformation/rANSecondaryRATUsageReport/rANSecondaryRATType |
| Qos Flows Usage Reports | Qos Flows Usage Reports | /pDUSessionChargingInformation/rANSecondaryRATUsageReport/qosFlowsUsageReports |
| Roaming QBC information | Roaming QBC information | /roamingQBCInformation |
| Multiple QFI container | Multiple QFI container | /roamingQBCInformation/multipleQFIcontainer |
| Triggers | Triggers | /roamingQBCInformation/multipleQFIcontainer/triggers |
| Trigger Timestamp | Trigger Timestamp | /roamingQBCInformation/multipleQFIcontainer/triggerTimestamp |
| Time | Time | /roamingQBCInformation/multipleQFIcontainer/time |
| Total Volume | Total Volume | /roamingQBCInformation/multipleQFIcontainer/totalVolume |
| Uplink Volume | Uplink Volume | /roamingQBCInformation/multipleQFIcontainer/uplinkVolume |
| Downlink Volume | Downlink Volume | /roamingQBCInformation/multipleQFIcontainer/downlinkVolume |
| Local Sequence Number | Local Sequence Number | /roamingQBCInformation/multipleQFIcontainer/localSequenceNumber |
| QFI Container information | QFI Container information | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation |
| QoS Flow Id | QoS Flow Id | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/qFI |
| Time of First Usage | Time of First Usage | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ timeofFirstUsage |
| Time of Last Usage | Time of Last Usage | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/timeofLastUsage |
| QoS Information | QoS Information | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/qoSInformation |
| QoS Characteristics | QoS Characteristics | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/qoSCharacteristics |
| User Location Information | User Location Information | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ userLocationInformation |
| UE Time Zone | UE Time Zone | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/uetimeZone |
| Presence Reporting Area Information | Presence Reporting Area Information | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/presenceReportingAreaInformation |
| RAT Type | RAT Type | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/rATType |
| Report Time | Report Time | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/reportTime |
| Serving Network Function ID | Serving Network Function ID | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ servingNetworkFunctionID |
| 3GPP PS Data Off Status | 3GPP PS Data Off Status | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/3gppPSDataOffStatus |
| EPS bearer Charging Id | EPS bearer Charging Id | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/3gppChargingId |
| Diagnostics | Diagnostics | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/diagnostics |
| Enhanced Diagnostics | Enhanced Diagnostics | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/enhancedDiagnostics |
| UPF ID | UPF ID | /roamingQBCInformation/uPFID |
| Roaming Charging Profile | Roaming Charging Profile | /roamingQBCInformation/roamingChargingProfile |
| Trigger | Trigger | /roamingQBCInformationroamingChargingProfile/trigger |
| Partial record method | Partial record method | /roamingQBCInformation/roamingChargingProfile/partialRecordMethod |
|  |  | ChargingDataResponse |
| Supported Features | - | /supportedFeatures |
| Multiple Unit information | - | /multipleUnitInformation |
| UPF ID | - | /multipleUnitInformation/uPFID |
| PDU Session Charging Information | - | /pDUSessionChargingInformation |
| Presence Reporting Area Information | - | /pDUSessionChargingInformation/ presenceReportingAreaInformation |
| Unit Count Inactivity Timer | - | /pDUSessionChargingInformation/unitCountInactivityTimer |
| Roaming QBC information | - | /roamingQBCInformation |
| Roaming Charging Profile | - | /roamingQBCInformation/roamingChargingProfile |

End of Change