3GPP TSG-SA5 Meeting #150	S5-235347

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft Cr | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 Use case and requirements for ML knowledge-based transfer Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of ML knowledge-based transfer Learning has been discussed and agreed (in TR 28.908). This CR is to normatively propose the use case and requirements for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use case “ML knowledge-based transfer Learning” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2. ML training phase |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 6.2	ML training phase

## 6.2.L.	ML-Knowledge-Based Transfer Learning

### 6.2.L.1	Description

An existing ML entity A can be leveraged to  produce or improve another ML entity B through transfer learning, a process which involves transferring an ML model from the source training function to the peer training function. However, ML entities may not be multi-vendor objects, i.e., it is not always possible to transfer an ML entity (or its portion) from one function to another. Instead, it should be only the knowledge contained in the ML entity that is transferred, in a process that may be termed as ML Knowledge-based Transfer Learning (MLKTL). Knowledge is the aggregate of experiences gathered by the source MLEntity through training, inference, updates, or testing. These experiences can be in the form of - but not limited to - statistical distributions or regression functions. So, as illustrated by Figure 6.2.l.1-1, the management system should support services for ML Knowledge-based Transfer Learning, which refers and includes all the means needed to realize transfer learning between any two ML entities based on ML knowledge in the ML entities. Through this means, the training functions (including multi-vendor functions) can exchange the knowledge e.g., statistics,  regression functions or decision matrices which avoids the need to exchange ML entities.

Figure 6.2.l.1-1: ML Knowledge-based Transfer Learning (MLKTL) flow between the source training function (the entity with the pre-trained ML entity), the target training function (the entity that shall train a new ML entity) and the MnS consumer (the operator or function that triggers or controls the MLKTL process)

### 6.2.L.2	Use cases

#### 6.2.L.2.1	Discovering sharable Knowledge

For the transfer learning, it is expected that the source MLEntity shares its knowledge with the target MLEntity, either simply as single knowledge transfer instance or through an interactive transfer learning process. The 3GPP management system should support means for an MnS consumer to discover this potentially shareable knowledge as well as means for the producer of MLKTL to share the knowledge with the MnS consumer.

#### 6.2.L.2.2	Knowledge sharing and transfer learning

The transfer learning may be triggered by a MnS consumer either to fulfil the learning for itself or for the learning to be accomplished through another ML Training function or MLEntity. MLKTL does not involve sharing the MLEntity or parts thereof but implies the means and services to enable the sharing of knowledge contained within the ML entity. The 3GPP management system should support the capabilities needed to realize the ML Knowledge transfer learning process.

Specifically, the 3GPP management system should support capabilities for an MnS consumer to request and receive sharable knowledge as well as means for the producer of MLKTL to share the knowledge with the MnS consumer or any stated target ML Training function (as specified by the MnS consumer). Similarly, the 3GPP management system should support capabilities for an MnS consumer to manage the MLKTL  process and the related requests associated with transfer learning between two ML entities or two ML training functions.

Note that, the use case and requirements here focus on the required management capabilities. The implementation of the knowledge transfer learning processes are implementation details and are out of the scope here.

### 6.2.L.3	Requirements for ML Knowledge Transfer Learning

Table 6.2.L.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MLKTL-1 | The 3GPP management system should have a capability enabling an MLEntity to register available knowledge (such as regression function or statistics) e.g. through a MLKnowledgeRegistration process. | Knowledge sharing and transfer learning (clause 6.2.L.2.2) |
| REQ-MLKTL-2 | the 3GPP management system should have a capability enabling an authorized MnS consumer to discover the available sharable knowledge from a given MLKTL MnS producer (e.g., a training function) according to a stated set of criteria. | Discovering sharable Knowledge (clause 6.2.L.2.1) |
| REQ-MLKTL-3 | the 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKTL MnS producer to provide some or all the knowledge available for sharing according to some stated criteria. | Discovering sharable Knowledge (clause 6.2.L.2.1) |
| REQ-MLKTL-4 | the 3GPP management system should have a capability enabling an authorized MnS consumer (e.g., an operator) to request a MLKTL MnS producer to trigger and execute on the MnS producer a knowledge-based transfer learning job instance to a specified ML entity. | Discovering sharable Knowledge (clause 6.2.L.2.1) |
| REQ-MLKTL-5 | The 3GPP management system should have a capability enabling an authorized MnS consumer (e.g., an operator or a managed function/entity) to manage the requests or jobs on MLKT e.g., to suspend, re-activate or cancel the ML Knowledge request; or to adjust the description of the desired knowledge, or to adjust the transfer learning conditions or characteristics | Knowledge sharing and transfer learning (clause 6.2.L.2.2) |

End of Changes