3GPP TSG-SA5 Meeting #150 	S5-235887

# Goteborg, Sweden, 21 Aug 2023 – 25 Aug 2023	Revision of S5-235257

# Source:	Amdocs, Nokia, Nokia Shanghai Bell

# Title:	Addition of a Key Issue

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.840: "Study on CHF Segmentation"

# 3	Rationale

Adding a key issue to enable segregation of Charging Function and its discovery by charging domains e.g.

- 5G connection and mobility domain charging

- IP Multimedia System (IMS) domain charging

- Edge Computing domain charging

- Proximity-based Services (ProSe) charging

- Exposure function Northbound Application Program Interfaces (APIs) charging.

# 4	Detailed proposal

First change

# 4	Overview

The current NRF Based Discovery can use the SUPI, GPSI PLMN and CHF Group ID for CHF Discovery and Selection according to TS 29.510 [2] clause 6.1.6.2.32, and TS 32.290[3] clause 6.1. The CHF(s) can be selected taking in account the business model in use by a Customer, for instance its possible to group the CHF instance(s) for enterprises customers and another one for non-enterprise customers, and taking in account the deployment model in place (e.g. central or local/edge).

The present document aims to study on how to enhance CHF Discovery and selection by the NF Consumers like SMF, AMF, SMSF , PCF and other NFs, as specified in various 3GPP Charging Specifications

Second change

# 5	Key Issues

## 5.1	Key Issue #1 CHF Discovery by NF Consumers

## 5.2	Key Issue #2 CHF Selection Extended Granularity

## 5.3	Key Issue #3 CHF Selection with Extended Subscription Data

## 5.4	Key Issue #4 CHF Discovery by Charging Domains

This key issue is for investigating how to support the discovery of a Charging Function segregated by one or more charging domains / subsystem / service. For example, there can be separate CHF NFs deployed e.g.

- 5G connection and mobility domain charging

- IP Multimedia System (IMS) domain charging

- Edge Computing domain charging

- Proximity-based Services (ProSe) charging

- Exposure function Northbound Application Program Interfaces (APIs) charging.

Sometimes a CHF NF instance could support a set of related domains: e.g.

- IMS and MMTel domains

- Network Slice Management, Network Slice Admission Control, and Network Slice Performance and Analytics domains.

The investigation covers the following:

- Definition of charging domain(s) / subsystem(s) or service(s) for distributed charging deployments.

- Determination of methods and procedures by which a CHF service producer can indicate the supported charging domain(s) / subsystem(s) or service(s)

- Determination of methods and procedures by which a CHF service consumer can indicate the charging domain(s), subsystem(s) or service(s) for which it requests the service(s)

- Determination of methods and procedures by which a CHF service consumer can discover a CHF service producer providing charging service for one or more charging domain(s), subsystem(s) or service(s)

End of changes