| 3GPP TR 28.909 V0.3.0 (2023-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on evaluation of autonomous network levels (Release 18 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope		7

2	References		7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms		7

3.2	Symbols		7

3.3	Abbreviations		7

4	Background and concepts	7

4.1 Concept for autonomous network level evaluation	8

4.1.1 Introduction	8

4.1.2 Autonomous network level qualitative evaluation	8

4.1.3 Autonomous network level quantitative evaluation	8

4.1.4 Key effectiveness indicator	8

5	Key Issues and potential solutions	9

5.1	Key Issue# 1: Generic methodology for autonomous network levels evaluation	9

5.1.1	Description	9

5.1.2.1	Potential solution#1	10

5.1.2.1.1	Process of evaluating the autonomous network level	10

5.1.2.1.2	Evaluation objects for autonomous network levels evaluation	10

5.2	Key Issue# 5.2: KEI for evaluating autonomy capability for radio network optimization	11

5.2.1	Description	11

5.2.2	Potential Solution	11

5.2.2.1	Potential solution#1	11

5.X	Key Issue# Num: KEI and process of autonomous network levels evaluation for <the use cases defined in Rel-17>		12

5.X.1	Description	12

6	Conclusion and recommendation	12

6.1	Analysis on the solution for generic methodology for autonomous network levels evaluation	12

Annex <X> (informative): Change history	13

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document studies on evaluation of autonomous network levels. It introduces the relevant studies in other SDOs, concepts of autonomous network levels evaluation and key effectiveness indicators (KEI). It identifies key issues related to autonomous network levels evaluation, documents potential solutions, and provides recommendations for the further normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]					3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.100 "Management and orchestration; Levels of autonomous network"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

KEI	Key Effectiveness Indicator

4	Background and concepts

Editor's note: this clause will contain background of relevant studies on autonomous network levels evaluation in other SDOs or industry parties, concepts of autonomous network levels evaluation and KEI.

4.1 Concept for autonomous network level evaluation

4.1.1 Introduction

Autonomous network level evaluation describes an approach for evaluating the autonomy capability of the autonomous network, which includes qualitative description evaluation and quantitative evaluation.

4.1.2 Autonomous network level qualitative evaluation

TS 28.100 [2] provides an approach for evaluating autonomous network levels based on the qualitative description of the autonomy capability (participation of the human and telecom system) for each task in the entire workflow, which is used for evaluating the autonomy capability of telecom system for individual scenario with certain management scope. Such evaluation approach is a qualitative evaluation approach for the autonomy capability of the telecom system, and the evaluation result can be Level 1, Level 2, Level 3, Level 4 and Level 5. For example, if RAN MnF implemented the following autonomy capability for radio coverage optimization, the qualitative evaluation result is level 2 (see the Figure 4.1.1-1) based on the generic classification of autonomous network level for network optimization in clause 7.1.2 in TS 28.100 [2].

-	Task C (Coverage related information collection), Task D (Coverage issues identification), Task F (Coverage issues demarcation) and Task G (Coverage issue root cause analysis) are accomplished by RAN MnF with human specified control information;

-	Task J (Coverage adjustment solutions execution) is fully accomplished by RAN MnF

-	Other Tasks are accomplished by Human.

Figure 4.1.1-1 Example of Autonomous network level qualitative evaluation result

4.1.3 Autonomous network level quantitative evaluation

In order to further differentiate different telecom systems with different autonomy capabilities but belong to same autonomous network level (result of qualitative evaluation), a quantitative evaluation approach needs to be introduced. The quantitative evaluation approach is used to derive the concrete autonomous network level score (ANLS) by considering more evaluation factors. The Autonomous network level score is derived based on the autonomous network level (derived from the qualitative evaluation) and further quantitative evaluation. The quantitative evaluation can be used to evaluate the autonomy capabilities for individual scenarios and/or management scope, as well as the whole telecom system for all scenarios.

4.1.4 Key effectiveness indicator

Key effectiveness indicator (KEI) describes the effective of introducing autonomy capability into telecom system. Existing KPIs and measurements could be used to evaluate the performance of the network, but it is not sufficient to reflect the effect from autonomous management and control perspective. Key effectiveness indicators could be used to help the NOPs to understand what benefits from autonomous management and control perspective they could get from upgrading their telecom system with more autonomy capabilities.

Figure 4.1.4-1 Introduce autonomy capabilities to 3GPP management system

As illustrated in Fig. 4.1.4-1, after introducing autonomy capabilities to the 3GPP management system, less human intervention is needed in a management workflow, which means more management tasks can be accomplished or partly accomplished by telecom system itself.

With the help of the autonomy capabilities, some technical effects can be acquired which are beneficial to the NOPs from business perspective. Some examples are as following.

-	Less human effort and higher automation ratio in the management workflow is beneficial to reduce the network operating expenditure (OPEX) for the NOPs.

-	Less human intervention is beneficial to speed up the whole management workflow, i.e. spend less time to achieve the management targets. Which means it is beneficial to increase income for the NOPs by optimizing the network or provisioning the service with less time, or to reduce business losses by recovering the fault with less time.

-	Less human effort and intervention is beneficial to achieve more refined network management and control to improve network and service performance with limited human resources.To measure the technical effects from autonomous management and control perspective, key effectiveness indicators are required to be defined, and the metrics for calculating them are required to be investigated.

5	Key Issues and potential solutions

Editor's note: this clause will contain the key issues and potential solutions for autonomous network levels evaluation .

5.1	Key Issue# 1: Generic methodology for autonomous network levels evaluation

Editor's note: this clause will contain the description and potential solutions of generic methodology for quantitatively evaluating the autonomous network levels (evaluation mechanisms for autonomous network levels).

5.1.1	Description

TS 28.100 [2] specifies the framework approach for evaluating autonomous network levels and concrete autonomous network level definition for network optimization, RAN NE deployment and fault management. The autonomous network level definition in TS 28.100[2] can be used to determine the ANL (L0-L5) for corresponding scenarios, however, there is no clear description on how to evaluating the autonomous network level based on the autonomous network level definition defined in TS 28.100 [2]. So, it is necessary to investigate the general process of evaluating the autonomous network level.

5.1.2		Potential solutions

5.1.2.1	Potential solution#1

5.1.2.1.1	Process of evaluating the autonomous network level

The general process of evaluating the autonomous network level is illustrated in Fig. 5.1.2-1:

Figure. 5.1.2-1 General process of evaluating the autonomous network level

Step 1: Determine the evaluation object

The evaluation object is the object to be evaluated based on the autonomous network level specified in TS 28.100 [2]. It is determined by the dimensions for autonomous network levels evaluation (i.e. scenario(s), management scope and entire workflow). For example, the evaluation object can be one specific scenario, e.g. [NR coverage optimization, RAN MnF, network optimization] or a group of scenarios like [NR optimization, RAN MnF, network optimization], which may include all NR optimization scenarios. Concrete description for evaluation sees clause 5.1.

Step 2: Mapping workflow of the evaluation object to corresponding standardized tasks of the generic workflow as defined in TS 28.100

In this step the workflow of the evaluation needs to be mapped to the standardized tasks of the generic workflow defined in TS 28.100 [2]. For example, the standardized tasks for generic network optimization is defined in clause 7.1.1 in TS 28.100 [2].

Step 3: Determine the evaluation result for the evaluation object

In case the evaluation object represents an individual scenario, following evaluation process are used.

-	Determine the autonomous network level based on the generic classification of autonomous network level for the evaluation object. For each task of the workflow, corresponding autonomy capability of the evaluation object needs to be analysed. The autonomous network level for the evaluation object can be determined based on the comparison of autonomy capability for each task and generic classification of autonomous network level defined in TS 28.100 [2].

In case the evaluation object represents a group of individual scenarios, the evaluation result of the evaluation object can be derived based on the evaluation result of each individual scenario (see the case of evaluation object represent an individual scenario). For example, the evaluation result for radio network optimization can be determined by the evaluation result of each individual radio network scenario (e.g. NR coverage optimization, NR throughput optimization and NR capacity optimization).

Note: process of evaluating the autonomous network level is for information, no need to be normalized.

5.1.2.1.2	Evaluation objects for autonomous network levels evaluation

Evaluation objects should be identified and determined before making an evaluation. The autonomous network levels can be evaluated by using the framework approach for evaluating autonomous network levels specified in TS 28.100 [2] by evaluating the autonomy capability of the specified workflow in each individual scenario and/or each individual management scope. Based on the autonomous network levels evaluation results of each individual scenarios and/or management scope, the autonomous network levels of groups of scenarios and/or management scope, or even the whole telecom system can be then evaluated with the generic evaluation mechanisms. So, it is necessary to define a common description for evaluation object. Based on the definition for evaluation, 3GPP Management system can have the capability to obtain the autonomous network level evaluation result (i.e. ANL) for corresponding evaluation object.

The dimensions for evaluating autonomous network levels i.e. scenarios, management scope and workflow described in TS 28.100[2] are reused as input for the evaluation objects for autonomous network levels evaluation and the evaluation objects are further elaborated in present document.

-	Scenarios: based on the scenario type defined in TS 28.100 [2], aspects which could identify specific network capabilities are used to derive a specific scenario. For example, for radio network, following aspects (non-exhaustive list) can be used to derive a specific scenario:

-		RAT:  e.g. UTRAN, eUTRAN, NR, and combination of them.

-		Network performance: e.g. coverage, RAN UE throughput, capacity, energy efficiency, latency, and combination of them.

-		Network environment: Indoor, Outdoor (e.g. urban, rural, high-speed rail), and combination of them.

-	Management scope: the management scope described in TS 28.100 is reused for evaluation purpose.

-	Workflow: the workflow described in TS 28.100 is reused for evaluation purpose.

Editor’s Note：How to model the evaluation object based on above input is FFS.

5.2	Key Issue# 5.2: KEI for evaluating autonomy capability for radio network optimization

5.2.1	Description

Key effectiveness indicator (KEI) describes the effective of introducing autonomy capability into telecom system. Regarding the radio network optimization related scenarios (e.g. radio network coverage optimization), following aspects can be considered as evaluation effect for autonomy capability for radio network optimization.

-	Network performance improvement by introducing autonomy capability for radio network optimization. For example, telecom system A can improve the 30% coverage performance by introducing certain autonomy capability.

-	Automation effect for corresponding radio network optimization tasks by introducing autonomy capability. For example, telecom system A without autonomy capability can analyse the root cause for 10% coverage issue cells, which after introducing certain autonomy capability, telecom system A can analyse the root cause for 90% coverage issue cells.

-	Optimization effect for radio network optimization. For example, telecom system A without autonomy capability needs one week to optimize the radio network, while, after introducing certain autonomy capability, telecom system A only needs one day to optimization the same radio network.

5.2.2	Potential Solution

5.2.2.1	Potential solution#1

Based on the description in clause 5.2.1, following are three dimensions used to evaluate the autonomy capability for radio network optimization:

-	Network performance gain, this is used to measure the network performance improvement ratio by introducing autonomy capability for radio network optimization. The network performance can be coverage performance, capacity performance, throughput performance and other performance, which depends on the concrete radio network optimization scenario. For example, following coverage performance gain example can be used for the coverage optimization use case. For example, the coverage performance gain can be proportion of the reduced number of weak coverage cells (e.g. RSRP < -110dB) by introducing the autonomy capability for network optimization to the total number of weak coverage cells before introducing the autonomy capability for network optimization.

-	Automation ratio of optimization, including the automation ratio for corresponding network optimization tasks (including task of network issue demarcation analysis, task of network issue root cause analysis, task of network adjustment solution analysis, task of network adjustment solution evaluation and determination, etc.). For example, coverage issue root cause analysis automation ratio represents the proportion of the number of the coverage issue cells whose root cause analysed by the telecom system automatically to the total number of coverage issue cells.

-	Reduction ratio of optimization period, which means the reduced ratio for the time period that the telecom system taken for the network optimization. For example, the reduction ratio of optimization period can be proportion of reduced optimization period to the original optimization before introducing the autonomy capability for network optimization.

5.X	Key Issue# Num: KEI and process of autonomous network levels evaluation for <the use cases defined in Rel-17>

Editor's note: this clause will contain the description and potential solutions of KEI and process of autonomous network levels evaluation for each use case defined in Rel-17.

5.X.1	Description

5.X.2		Potential solutions for KEI of ANL evaluation

5.X.3		Potential solutions for process of ANL evaluation

6	Conclusion and recommendation

Editor's note: this clause will be used to document the conclusions and recommendation of the study, including potential autonomy requirements for corresponding management services with evaluation of autonomous network levels.

6.1	Analysis on the solution for generic methodology for autonomous network levels evaluation

The potential solution for general process of autonomous network levels evaluation is described in clause 5.1.2, the 3 steps general process is investigated to identify the potential requirements and solutions to implement the evaluation, but the process itself is informative and there could be refined process which depends on concrete implementation. According to the general process, conclusions and recommendations are as following:

-	Existing TS 28.100 [2] can be used for mapping workflow of the evaluation object to corresponding standardized tasks.

-	To determine the evaluation result for the evaluation object,

-	Existing TS 28.100 [2] can be used for the qualitative evaluation of autonomous network level defined in clause 4.1.2.

-	Quantitative evaluation of autonomous network level to derive the concrete ANLS defined in clause 4.1.3 is depends on the evaluator's specific evaluation purpose and concrete implementation of the evaluation process, thus ANLS does not need to be normalized in 3GPP.

Editor's note: the conclusion and recommendation for evaluation object is FFS.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222121 | - | - | - | Initial skeleton | 0.0.0 |
| 2022-04 | SA5#142e | S5-222617 S5-222618 S5-222619 | - | - | - | 1) Add TR structure and abbreviation 2) Add scope 3) Add concept description for autonomous network level evaluation | 0.1.0 |
| 2023-03 | SA5#147 | S5-232888 | - | - | - | 1) Add generic methodology for autonomous network levels evaluation. 2) Add KEI for evaluating autonomy capability for radio network optimization. 3) Align TR front page title with SA WG5 official title. | 0.2.0 |
| 2023-03 | SA5#147 | - | - | - | - | Uploading the right version of the specification | 0.2.1 |
| 2023-08 | SA5#150 | S5-235594 S5-236106 | - | - | - | 1. Update concept for key effectiveness indicator. 2. Add conclusion and recommendation for generic methodology for autonomous network levels evaluation. | 0.3.0 |