3GPP TSG-SA5 Meeting #150 	S5-235876

Goteborg, Sweden, 21 - 25 August 2023

# Source:	Ericsson

# Title:	New solution for IoT Charging information Optimization

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: "Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Shortening the table for solution #1.15 and adding a new solution covering the same key issue.

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 32.255: "5G data connectivity domain charging; stage 2".

[3]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[6]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[7]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[8]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[9]	3GPP TS 23.501: "3GPP TS 23.501:"System Architecture for the 5G System".

[10]	3GPP TS 23.203: "Policy and charging control architecture".

[11]	3GPP TS 29.594: "5G System; Spending Limit Control Service; Stage 3".

[12]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[13]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[14]	IETF RFC 8506 (2019): "Diameter Credit-Control Application".

[15]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[16]	3GPP TS 29.594: "5G System; Spending Limit Control Service; Stage 3".

[17]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[18]	3GPP TS 32.256: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

[19]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[x]	3GPP TS 29.591: "5G System; Network Exposure Function Southbound Services; Stage 3".

[y]	3GPP TS 29.541: "5G System; Network Exposure (NE) function services for Non-IP Data Delivery (NIDD) and Short Message Services (SMS); Stage 3".

[z]	3GPP TS 29.542: "5G System; Session management services for Non-IP Data Delivery (NIDD); Stage 3".

Second change

#### 5.1.5.x	Solution #1.x: IoT NEF charging information optimization

A possible solution for key issues #1f and #1g covering requirements REQ-CH_INFO-03, optimization of charging information.

Currently, charging for data transfer is based on volume based measurement of the connections in down and uplink directions which has been enhanced to cover big data transfers.

This solution would bring the following advantages for IoT Charging:

- Charging for loT subscriptions (which can be devices, group of devices, loT applications, external Identifiers) is simplified.

- Overload of charging system due to many CDRs, one CDR per payload, may be avoided, by reducing the number of requests to Charging, limit down the Charging Data to IoT related one.

- A new option to control loT devices and/or their environment, on this case its considered that each IoT device corresponds to a PDU Session (one UE and one subscriber).

The solution proposes the use of the current NIDD (Non-IP Data Delivery) handling as specified in 3GPP TS 32.254 [17] and adding new trigger for number of API invocations/notifications.

The NIDD delivery uses the following, according to 3GPP TS 29.591 [x]:

-	MO NIDD, the interaction between the SMF and the NEF uses Nnef_SMContext service, as specified in 3GPP TS 29.541 [y];

-	MT NIDD, the interaction between the SMF and the NEF uses Nsmf_NIDD service as specified in 3GPP TS 29.542 [z].

When delivering the message to/from the AF the following can be used:

-	NiddDownlinkDataTransfer service, and it can be updated by the NiddDownlinkDataTransferPatch service

-	NiddUplinkDataNotification service

To have a trigger each time an uplink message is delivered to the AF or received from the AF may cause a lot of interaction and adding a new limit triggering after a number of events to default trigger conditions in NEF, 3GPP TS 32.254 [17] table 5.4.1.2.1.1

Table 5.1.5.x-1: Extension to default trigger conditions in NEF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| API Invocation/Notification limit | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |

Table 5.1.5.x-2: Charging Data Request message content

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [12] |
| Subscriber Identifier | OM | Described in TS 32.290 [12], and holds the identifier of the AF |
| NF Consumer Identification | M | Described in TS 32.290 [12] |
| Invocation Timestamp | M | Described in TS 32.290 [12] |
| Invocation Sequence Number | M | Described in TS 32.290 [12] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | This field indicates, if included, that this is a one-time event and that there will be no update or termination. |
| One-time Event Type | OC | Described in TS 32.290 [12]. |
| Notify URI | - | This field is only applicable for the notification of abort charging. |
| Triggers | OC | This field is described in TS 32.290 [12] and holds the NEF specific triggers described in clause 5.4.1.x |
| Multiple Unit Usage | OC | This field contains the parameters for the quota management request and/or usage reporting. |
| Rating Group | M | Described in TS 32.290 [12] |
| Requested Unit | OC | Described in TS 32.290 [12] |
| Used Unit Container | OC | Described in TS 32.290 [12] |
| NEP API Container Information | OC | Used instead of NEF API Charging Information if more than one NEF API Charging Information needs to be reported described in table 5.1.5.x-3. |
| NEF API Charging Information | OM | This field holds the NEF API specific information described in TS 32.254 [17] clause 6.3.1.4 |

Table 5.1.5.x-3: Structure of the NEF API Container Information

| Information Element | Description |
| --- | --- |
| NEP API Container Information | This parameter holds the same information as the NEF API Charging Information. |

Extension of the Triggers information element in the Charging Data Request message contents in TS 32.290 [12] table 6.1.1.2.1:

Table 5.1.5.x-4: Extension of Triggers information element

| Information Element |  | Converged Charging Category |  | Offline Only Charging Category |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Triggers |  | OC |  | OC |  | This field holds the triggers supplied from the CHF for the charging session that are independent of rating group for quota management and without quota management. |  |
| Trigger Type |  | OC |  | OC |  | This field holds the events whose occurrence led to charging event is issued towards the CHF |  |
| Trigger Category |  | M |  | M |  | This field indicates whether the charging data generated by the NF consumer for the trigger lead to a Charging Event towards the CHF immediately or not. |  |
| Time Limit |  | OC |  | OC |  | This field holds the time limit if trigger type is "Expiry of data time limit" |  |
| Volume Limit |  | OC |  | OC |  | This field holds the total (uplink and downlink) volume limit if trigger type is "Expiry of data volume limit". |  |
| Event Limit |  | OC |  | OC |  | This field holds the maximum number of events if trigger type is "Expiry of data event limit". |  |
| Max Numbers Of Charging Condition Changes |  | OC |  | OC |  | This field holds the maximum number if trigger type is "Max number of charging condition changes". |  |
| Max Numbers Of Invocations/Notifications |  | OC |  | OC |  | This field indicates the maximum number of invocations accepted and notifications delivered |  |
| Tariff Time Change |  | OC |  | OC |  | This field contains UTC time indicating the switch time when the tariff will be changed. |  |

End of changes