3GPP TSG-SA5 Meeting #151	S5-236331

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0222 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.531 Clarify initiation of Procedure of Network Slice Subnet Instance Modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure of Network Slice Subnet Instance Modification may be initated with DN of NetworkSliceSubnet MOI in the modifyMOIAttributes operation (when the network slice subnet allocation was initiated using AllocateNssi operation) or DN of NetworkSliceController MOI in the modifyMOIAttributes operation (when the network slice subnet allocation was initiated using createMOI operation with NetworkSliceSubnetController IOC). This is not mentioned in the description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description enhanced to clarify that Procedure of Network Slice Subnet Instance Modification may be initated with DN of NetworkSliceSubnet MOI in the modifyMOIAttributes operation (when the network slice subnet allocation was initiated using AllocateNssi operation) or DN of NetworkSliceSubnetController MOI in the modifyMOIAttributes operation (when the network slice subnet allocation was initiated using createMOI operation with NetworkSliceSubnetController IOC). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent/incomplete specification leads to confusion and incorrect implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.7	Procedure of Network Slice Subnet Instance Modification

The Figure 7.7-1 illustrates the procedure of modifying an existing NSSI.

Figure 7.7-1: Network Slice Subnet Instance Modification Request procedure

1a)	Network Slice Subnet Management Service Provider (NSSMS_P) receives a modifyMOIAttributes request (defined in TS 28.532 [8]) from Network Slice Subnet Management Service Consumer (NSSMS_C) with the DN of NetworkSliceSubnet MOI and the new network slice subnet related requirements (see SliceProfile defined in clause 6.3.3 in TS 28.541[6]).

1b-1)	Network Slice Subnet Management Service Provider (NSSMS_P) receives a modifyMOIAttributes request (defined in TS 28.532 [8]) from Network Slice Subnet Management Service Consumer (NSSMS_C) with the DN of NetworkSliceSubnetController MOI (see NetworkSliceSubnetController IOC and its attribute defined in TS 28.541[6]) and the new network slice subnet related requirements in attribute inputSliceProfile. The NSSMS_C may check the status and completion of the network slice instance modification procedure any time by monitoring the values of NetworkSliceSubnetController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor by querying the values or by subscribing to notifications.

1b-2)	The NSSMS_P updates NetworkSliceSubnetController MOI attributes inputSliceProfile, and may update attributes operationalState, availabilityStatus and processMonitor to indicate progress.

1b-3)	The NSSMS_P sends the response to NSSMS_C for the modifyMOIAttributes request.

NOTE: Network subnet slice modification may be initiated using DN of NetworkSliceSubnet MOI in the modifyMOIAttributes operation (when the network slice subnet allocation was initiated using AllocateNssi operation) as described in steps 1a or using DN of NetworkSliceSubnetController MOI in the modifyMOIAttributes operation (when the network slice subnet allocation was initiated using createMOI operation with NetworkSliceSubnetController IOC) as described in steps 1b.

2)	Based on the new network slice subnet related requirements, NSSMS_P invokes the procedure of feasibility check and reservation of network slice subnet as described in clause 7.14. If the modification requirements can be satisfied, go to step 3), else go to step 6).

3) NSSMS_P decomposes the NetworkSliceSubnet MOI modification request into modification requests for each constituent MOIs.

4a)	NSSMS_P invokes procedure of network slice subnet instance modification for each constituent NetworkSliceSubnet MOI(s) that is configured in attribute networkSliceSubnetRef of the NetworkSliceSubnet MOI.

4b)	For each constituent NF instance (ManagedFunction MOI DN that is configured in attribute of managedFunctionRef of NetworkSliceSubnet MOI), NSSMS_P invokes procedure of NF instance creation as described in clause 7.10 or procedure of NF instance modification as described in clause 7.11.

4c)	If the NetworkSliceSubnet MOI contains the virtualized part, NSSMS_P invokes the NS instance scaling and/or NS instance updating and/or NS instance instantiation procedure as described in TS 28.526 [7].

4d) If the NetworkSliceSubnet MOI contains the TN part, NSSMS_P invokes the corresponding procedure of coordination with relevant TN Manager to handle the TN part as described in clause 7.9.

5) NSSMS_P updates NetworkSliceSubnet MOI attributes.

6a)	If NSSMS_P sent new network slice subnet related requirements in NetworkSliceSubnet MOI (in step 1a), then NSMS_P sends response for modifyMOIAttributes.

6b)	If NSSMS_C sent the new network slice subnet related requirements in NetworkSliceSubnetController MOI (in step 1b-1), then NSSMS_P updates NetworkSliceSubnetController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor to indicate completion of the procedure.

End of change