3GPP TSG-SA5 Meeting #151	S5-236410

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | DraftCR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Input to draft CR TS 28.105 Use case and requirements for ML Training activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, NTT DOCOMO, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of ML Training activation has been discussed and agreed (in TR 28.908). This CR is to normatively propose the use case and requirements for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use case “ML Training activation” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.B(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 6.2	ML training phase

### 6.2.B		ML training activation and deactivation

#### 6.2.B.1	Description

The ML training function and the associated ML entity may need to be configured to undertake training in the 5G system to align with the consumer´s expectation, e.g., to enable the ML training function to perform training for the right ML entity using the right data. The ML training MnS producer should provide a capability to configure the ML training function or its associated ML entities (e.g., with policies, target, conditions where applicable), including:

- MnS consumer-initiated activation and deactivation of training functions.

- MnS consumer-initiated activation and deactivation of training on ML entities.

- MnS producer-initiated activation and deactivation of ML training on ML entities

#### 6.2.B.2		Use cases

##### 6.2.B.2.1	MnS consumer-initiated activation and deactivation of ML training functions

The MnS Consumer monitors the network performance or the performance of ML training and determines whether to configure or re-configure or when to activate or deactivate an ML training function. The activation of the ML training function does not imply that all its ML entities shall be activated. The training shall only be executed for those ML entities that are activated.

The MnS Consumer may adjust any of the parameters of the ML training function, e.g., to add or remove a network parameter that must be monitored by the ML training. The ML training should provide feedback on the configured parameters to any authorized MnS consumer.

##### 6.2.B.2.3	MnS consumer-initiated activation and deactivation of training on ML entities

The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate the training on or for a specific ML entity. If one ML entity does not reach the expected performance at training, the consumer may decide to deactivate this ML entity so that no further training can be executed for that ML entity.  For Energy Saving (ES) purpose for instance, the ML entity for deciding the loads at which energy saving is triggered may already be active. But after collecting information of the coverage and capacity booster cells in an area, the MnS Consumer may decide that it is necessary to estimate the future load in the cells and so may decide to also activate the ML entity for predicting the future cell loads. The ML training MnS should include a capability to activate or deactivate training on one or more ML entities associated to a given training function.

##### 6.2.B.2.4	MnS producer-initiated activation and deactivation of ML training on ML entities

The MnS producer monitors the network performance and determines on whether to, and when to activate or deactivate the ML training. For Energy Saving (ES) purpose for instance, based on the observed energy efficiency, the MnS producer may decide to activate or deactivate training for the ML entity or decide to use another ML entity. The MnS producer should inform the authorized MnS consumer about the activation, deactivation and related configurations. The configuration actions conducted by the MnS producer including activation and deactivation may also be triggered by a predefined policy provided by the consumer.

#### 6.2.B.3	Requirements for ML training Configuration management

Table 6.2.B.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_ TRAIN_ACT-1 | The MnS producer for ML training should have a capability allowing an authorized MnS consumer to activate and deactivate a training function and inform an authorized MnS consumer about the activation or deactivation of the ML training function. | MnS consumer-initiated activation and deactivation of ML training (clause 6.B.2.1) |
| REQ-AIML_ TRAIN_ACT-2 | The MnS producer for ML training should have a capability allowing an authorized MnS consumer to activate or deactivate one or more ML entities associated with an ML training function. | MnS consumer-initiated activation and deactivation of ML training (clause 6.2.B.2.2) |
| REQ-AIML_ TRAIN_ACT-3 | The MnS producer for AI/ML inference management should have a capability to allow an authorized MnS consumer to provide the policy for activating and deactivating the ML entities used by an AI/ML training function. | MnS producer-initiated activation and deactivation of ML inference on ML entities (clause 6.5.B.2.3) |
| REQ-AIML_TRAIN_ACT-4 | The ML training MnS producer should have a capability to inform an authorized MnS consumer about the activation and deactivation of an ML entity triggered by the ML training MnS producer. | MnS producer-initiated activation and deactivation of ML training on ML entities (clause 6.2.B.2.2) |

End of Changes