3GPP TSG-SA5 Meeting #151 	S5-236625

Xiamen, China, 9 - 13 October 2023

# Source:	Huawei

# Title:	Update architecture considerations

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the updated architecture considerations of TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Charging aspects of time sensitive networks".

3	Rationale

This contribution proposes updated architecture considerations of TS 32.282.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

4	Architecture considerations

4.1	High-level time sensitive networking architecture

As per the requirements described in the TS 22.261 [102] and TS 22.104 [101] about clock synchronization, time-sensitive communication, the 5G system architecture references to support of integration with TSN and enablers for TSC and TS service are specified TS 23.501 [200] clause 5.27 and clause 5.28.

-	5GS acts as a Layer 2 Ethernet Bridge. When integrated with IEEE TSN network, 5GS functions acts as one or more TSN Bridges of the TSN network. The configuration of the 5GS Bridge can be provided by TSN AF and CNC.

-	5G System features that can be used independently or in combination to enable time-sensitive communication and time synchronization.

The Figure 4.1.1 describes the architecture for 5GS appearing as TSN bridge, specified in clause 4.4.8.2 in TS 23.501 [200] and the Figure 4.1.2 describes the architecture for AF requested support of TSC and TS, specified in clause 4.4.8.3 in TS 23.501 [200].

Figure 4.1.1: System architecture view with 5GS appearing as TSN bridge

Figure 4.1.2: Architecture to enable Time Sensitive Communication and Time Synchronization services

Next change

4.2	Time sensitive networking converged charging architecture

The TSN converged charging architectures includes the following scenarios, derived from the high-level TSN architecture.

-	TSN 5GS bridge configuration and management converged charging: The TSN AF embedding the CTF that generates charging events towards the CHF, shown in Figure 4.2.1 and Figure 4.2.2.

-	Enablers for TSC and TS converged charging: the charging architecture depends on whether AF is trusted by operator.

-	When AF is trusted by operator, the TSCTSF embedding the CTF that generates charging events towards the CHF, shown in Figure 4.2.3 and Figure 4.2.4.

-	When AF is not trusted by operator, the NEF embedding the CTF that generates charging events towards the CHF, as specified in TS 32.254 [15].

-	TSC traffic converged charging: The SMF embedding the CTF that generates charging events towards the TSN CHF via UE CHF, as specified in TS 32.255 [16]. The interaction between UE CHF and TSN CHF is specified in the Annex X in TS 32.240 [1].

Architectural options of figure 4.2.1 apply to the TSN 5GS bridge configuration and management converged charging architectures in this clause.

Figure 4.2.2 depicts the TSN 5GS bridge configuration and management converged charging architecture in reference point representation for non-roaming. The N102 reference point is defined for the interactions between TSN AF and CHF.

Figure 4.2.1: TSN 5GS bridge configuration and management converged charging architecture

Figure 4.2.2: TSN 5GS bridge configuration and management converged charging architecture non-roaming reference point representation

Architectural options of figure 4.2.3 apply to the enablers for TSC and TS converged charging architectures in this clause.

Figure 4.2.4 depicts the enablers for TSC and TS converged charging architecture in reference point representation for non-roaming. The N103 reference point is defined for the interactions between TSCTSF and CHF.

Figure 4.2.3: Enablers for TSC and TS converged charging architecture

Figure 4.2.4: Enablers for TSC and TS converged charging architecture non-roaming reference point representation

Ga is described in clause 5.2.4 and Btsn in clause 5.2.5 of the present document and Nchf is described in TS 32.290 [50].

End of change