# 3GPP TSG SA WG5 Meeting #151                 	           S5-236977

# Xiamen, China, 9-13 October 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.240 | CR | 0474 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add annex for B2B charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | B2B_CH |  |  |  |  |  | Date: |  |  | 2023-10-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To provide a clear documentation of the business to business (B2B) charging aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new annex to describe the document structure for B2B charging. Specify the charging architectures to B2B charging, including the reference to the charging specification for each charging scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The business to business charging aspects is not clearly documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.240 … CR 0475 |  |  |
| Other comments: |  | CR 0474 and CR 0475 refer to the same new Annex in TS 32.240. CR 0474 should be implemented before CR 0475. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Annex X (Informative):

Business to Business (B2B) charging architecture and principles

X.1	General

This annex contains the business to business (B2B) charging principles and architectures.

The B2B describes a type of business relationship in which businesses provide goods or services to other businesses.

X.2	Charging architecture and references

X.2.1	General

In the B2B scenario, the charging architecture may involve two modes of interaction:

-	Business CHF interaction: the NF (CTF) collects and reports the charging information per business, alternatively it collects and reports the charging information per consumer but for business charging purposes, to the business CHF (i.e. B-CHF).

-	Business CHF via consumer CHF interaction: the NF (CTF) collects and reports the charging information per consumer to a consumer CHF (i.e. C-CHF), and the C-CHF reports the charging information per business to the business CHF (i.e. B-CHF).

X.2.2	B2B charging architecture and references with business CHF only

In the B2B charging architecture with business CHF only interaction, the charging information collected by the NF (CTF) is per business.

The B-CHF performs charging functionalities for the business.

The related NF(s) and the corresponding references for B2B charging with business CHF only, but not limited to:

-	Network exposure domain charging: NEF charging per API invocation/notification, as specified in TS 32.254 [14].

-	Edge charging: CEF based charging and EAS deployment charging, as specified in TS 32.257 [17].

-	Short message charging: SMSF charging for IoT, as specified in TS 32.274 [34], in this case the charging information is collected per consumer but used in a business contest.

-	Prose charging: ProSe Function or DDNMF for prose discovery and prose communication charging, as specified in TS 32.277 [37], in this case the charging information is collected per consumer but used in a business contest.

-	5G LAN charging: NEF charging/CEF based charging for 5G LAN management and configuration, as specified in TS 32.254 [14].

-	NPN charging: SMF and AMF charging for the inter-provider charging of NPN, as specified in TS 32.255 [15] and TS 32.256 [16], in this case the charging information is collected per consumer but used in a business contest.

-	Network slice charging: NSPA, NSM, NSACF and NSSAAF charging per S-NSSAI for network slice.

-	TSN charging: TSN AF charging for TSN Bridge management and configuration, NEF and TSCTSF charging for the enabler of TSC and TS.

X.2.3	B2B charging architecture and references with business CHF via consumer CHF

In the B2B charging architecture with business CHF via consumer CHF interaction, the charging information collected by the NF (CTF) is per consumer.

The C-CHF performs charging functionalities for the consumer.

The B-CHF performs charging functionalities for the business.

The related NF(s) and the corresponding references for the B2B charging with business CHF via consumer CHF include, but not limited to:

-	Network slice charging: multiple CCSs to support converged charging for tenant and UEs belonging to the tenant at same time, as described in Annex D in TS 32.255 [71].

-	TSN charging: SMF charging for TSC traffic in TSN bridge.

End of change