# 3GPP TSG-SA5 Meeting #151	S5-237064

# Xiamen, China, October 09-13, 2023                                             	was S5-236342

# Source:	Nokia

# Title:	Rel-18 pCR TS 28.318 Availability KPI definition

# Document for:	Approval

Agenda Item:	6.6.4.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[Y]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[Z]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[A]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[B]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

# 3	Rationale

This pCR builds on the Samsung document (tDoc number S5-235714) which was noted in the SA5#150 meeting that provided some definition of the availability KPI.

# 4	Detailed proposal

It is proposed to agree to the following change to TS 28.318.

Begin Change

# 7	Informational model definitions

## 7.Y	Additional gNB Performance Measurements and KPI for NSOEU

### 7.Y.1   Performance Measurements

#### 7.Y.1.1 Total cell In-Service duration

a)	This measurement provides the total time duration for which an active cell in a gNB-DU remains In-Service. The gNB-DU reports the Service Status of the Active cells to the gNB-CU. If the Service Status value is “In-Service” it means the active cell is operational, and it is able to serve UEs. The active cells are considered as In-Service until the gNB-DU reports the Service Status as “Out-Of-Service” to the gNB-CU. The gNB-DU reports the Service Status to gNB-CU using the GNB DU CONFIGURATION UPDATE messages.

b)	CC.

c)	This measurement is obtained in two steps. First step involves determining the time interval between the time stamps Tin and Tout. Where Tin is the time stamp when the Service Status is reported as “In-Service” for a cell by gNB-DU to gNB-CU and Tout is the time stamp when Service Status is reported as “Out-Of-Service” by gNB-DU to gNB-CU for the same cell in the GNB DU CONFIGURATION UPDATE messages. Second step involves summing up all these time intervals of a cell which fall within the desired observation time frame Tobv. This gives total in-service duration of a cell within the observation time frame Tobv. Where, Tobv is monitorGranularityPeriod  as defined in TS 28.622[X] clause 4.3.16.

d)	Each measurement is an integer value representing the number of minutes for which the cell was in-Service.

e)	SO.CellInServiceTotal.NCGI, where NCGI is NR Cell Global Identifier as defined in TS 28.541[A]. The number of measurements is equal to the number of NCGIs (cells).

f)	GNBCUCPFunction.

g)	Valid for packet switching.

h)	5GS.

End Change