3GPP TSG SA WG5 Meeting #151                 	           									S5-236293

Xiamen, China, 9-13 October 2023

# Source:	MATRIXX Software

# Title:	pCR 28.204 Introduction of AAA-S triggered Network Slice-Specific Authorization Revocation flows

# Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This pCR is to introduce message flows for AAA-S triggered Network Slice-Specific Authorization Revocation

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.204 "Network slice-specific authentication and authorization charging in the 5G System (5GS)".

# 3	Rationale

This pCR is to add message flows for  "AAA-S triggered Network Slice-Specific Authorization Revocation" procedure.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.204 [2]:

First change

##### 5.2.2.y	AAA Server triggered Network Slice-Specific Authorization Revocation charging

##### 5.2.2.y.1	General

The subclauses below describe "AAA Server triggered Network Slice-Specific Authorization Revocation" charging message flows based on figure 4.2.9.4-1 of  3GPP TS 23.502 [4].

##### 5.2.2.y.1	AAA Server triggered Network Slice-Specific Authorization Revocation - AMF - PEC

The following figure 5.2.2.y.1-1 describes a "AAA Server triggered Network Slice-Specific Authorization Revocation" charging in PEC scenario for AMF:

Figure 5.2.2.y.1-1: AAA Server triggered Network Slice-Specific Authorization Revocation procedure - AMF - PEC

Steps 1 to 4: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

4ch-a: AAA-S Network Slice-Specific Authentication revocation Notification received in AMF :  AMF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

4ch-b: The CHF creates a CDR.

4ch-c: CHF provides response to AMF.

##### 5.2.2.y.2	AAA Server triggered Network Slice-Specific Authorization Revocation- NSSAAF - PEC

The following figure 5.2.2.y.2-1 describes a "AAA Server triggered Network Slice-Specific Authentication revocation" charging in PEC scenario for NSSAAF:

Figure 5.2.2.y.2-1: AAA Server triggered Network Slice-Specific Authorization Revocation procedure - NSSAAF - PEC

Steps 1 to 3c: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

3cch-a: AAA-S Network Slice-Specific Authorization Revocation completed in NSSAAF :  NSSAAF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

3cch-b: The CHF creates a CDR.

3cch-c: CHF provides response to NSSAAF.

##### 5.2.2.y.3	AAA Server triggered Network Slice-Specific Authorization Revocation - NSSAAF - IEC

The following figure 5.2.2.y.3-1 describes a "AAA Server triggered Network Slice-Specific Authorization Revocation" charging in IEC scenario for NSSAAF:

Figure 5.2.2.y.3-1: AAA Server triggered Network Slice-Specific Authorization Revocation procedure - NSSAAF - IEC

Steps 1 to 2: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

2ch-a: AAA-S Network Slice-Specific Authorization Revocation request received in NSSAAF :  NSSAAF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

2ch-b: Account, Rating, control  by the CHF.

2ch-c: The CHF creates a CDR.

2ch-d: CHF provides response to NSSAAF.

Steps 3 to 5: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

##### 5.2.2.y.4	AAA Server triggered Network Slice-Specific Authorization Revocation - NSSAAF - ECUR

The following figure 5.2.2.y.4-1 describes a "AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization" charging in ECUR scenario for NSSAAF:

Figure 5.2.2.y.4-1: AAA Server triggered Network Slice-Specific Authorization Revocation procedure - NSSAAF - ECUR

Steps 1 to 2: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

2ch-a: AAA-S Network Slice-Specific Authorization Revocation request received in NSSAAF :  NSSAAF sends Charging Data Request [Initial] to CHF with GPSI and S-NSSAI.

2ch-b: Account, Rating, control  by the CHF.

2ch-c: The CHF opens a CDR.

2ch-d: CHF provides response to NSSAAF.

3cch-a: AAA Server triggered Network Slice-Specific Authorization Revocation procedure is completed:  NSSAAF sends Charging Data Request [Termination] to CHF with GPSI and S-NSSAI.

3cch-b: Account, Rating, control by the CHF.

3cch-c: The CHF closes the CDR.

3cch-d: CHF provides response to NSSAAF.

End of change