3GPP TSG-SA5 Meeting #151	S5-236655

Xiamen, China, 9 - 13 October 2023 									revision of S5-236459

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | DraftCR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 Use case and requirements for ML training policy-based activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of policy-based activation of ML has been discussed and agreed (in TR 28.908). This CR is to normatively propose the use case and requirements for policy-based activation of ML training |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use case “ML training policy-based activation” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.V(new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 6.2	ML training phase

### 6.2.B		ML training activation and deactivation

### 6.2.B.2		Use cases

#### 6.2.B.2.W	Policy-based ML activation

An ML Entity is generally responsible for a defined scope (e.g., a geographical region) of the radio coverage area, as either of a recommendation/decision-making capability or an analysis capability. However, it is not always obvious or desired to train the ML entity for the full scope. So, it is necessary to ensure that ML training of an ML entity is activated in a gradual way, through “phased activation steps”. E.g., for a sub scope of the ML Entity which is gradually expanded as confidence is gained by the ML entity.

If the activation procedure is entirely relying on the MnS consumer to micro-manage every activation step, it would require extensive interaction between the MnS consumer and MnS producer. Instead, the producer needs to be instructed by the MnS consumer on the ways to perform the right activations through activation policies.

The policy-based activation may be instructed via one or more ML activation policies, where an ML activation policy is a sequence of tuples of conditions and activation settings that may be executed by the ML producer. Conditions may define specific outcomes on performance metrics for which a particular activation may be executed while activation settings define specific attributes of the ML capability activation scope (e.g., object or object type, network context, activation time window) for which ML capability should be activated.

The ML training MnS should include a capability to use policies to activate training for ML entities for a sub scope and gradually progressively increase the scope.

### 6.2.B.3	Requirements for ML training Configuration management

Table 6.2.B.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_POL_ACT-1: | The ML training MnS producer should have a capability enabling an authorized MnS consumer to provide policies for activation of ML capabilities, the policies instructing the ML MnS producer on how and when to activate ML training for a particular scope. | Policy-based ML activation (clause 6.2.B.2.W) |
| REQ-ML_POL_ACT-2: | the ML training MnS producer should have a capability to activate ML training for an ML entity on given scope based on the policies specified by the ML training MnS consumer | Policy-based ML activation (clause 6.2.B.2.W |
| REQ-AIML_ INF_ACT-4: | The ML training MnS producer should have a capability to allow an authorized MnS consumer to partially and progressively activate or deactivate the ML training for an ML entity. | Policy-based ML activation (clause 6.2.B.2.W |

End of Changes