3GPP TSG-SA5 Meeting #152 	S5-237808

Chicago,US, 13-17 November 2023

# Source:	China Mobile, Huawei

# Title:	Add Bmbs interface information

# Document for:	Approval

Agenda Item:	7.4.5

1	Decision/action requested

Agree the Bmbs interface information of TS 32.27x.

2	References

[1] 		3GPP TS 32.27x: "Charging management; 5G Multicast-broadcast Services charging".

3	Rationale

This contribution proposes Bmbs interface information in TS 32.27x.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.27x [1].

First change

5.2.5	Bmbs CDR file transfer

Details of the Bmbs protocol application are specified in TS 32.297 [6].

Next change

6.1.3	CDR description on the Bmbs interface

6.1.3.1	General

This clause describes the CDR content and format generated for 5G data connectivity - MBS session charging.

The following tables provide a brief description of each CDR parameter. The category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1]. Full definitions of the CDR parameters, sorted by the name in alphabetical order, are provided in TS 32.298 [7].

6.1.3.2	MBS session charging CHF CDR data

If enabled, CHF CDRs for MBS session charging shall be produced for each MBS session.

The fields of MBS session charging CHF CDR are specified in table 6.1.3.2.1.

Table 6.1.3.2.1: MBS session charging CHF record data

| Field |  | Category | Description |
| --- | --- | --- | --- |
| Record Type |  | M | CHF record. |
| Recording Network Function ID |  | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| NF Consumer Information |  | M | This field holds the information of the MB-SMF that used the charging service. |
| NF Functionality |  | M | This field contains the function of the node (i.e. MB-SMF) |
| NF Name |  | OC | This field holds the name of the MB-SMF used. |
| NF Address |  | OC | This fields holds the IP Address of the MB-SMF used. |
| NF PLMN ID |  | OC | This field holds the PLMN identifier (MCC MNC) of the MB-SMF. |
| List of Multiple Unit Usage |  | OM | This field holds a list of changes in charging conditions for all service data flows within this PDU session.This list is categorized per rating group or per combination of rating group and service id or per combination of rating group, sponsor identity and application service provider identity. In addition, usage is differentiated between with and without quota management. Each change is time stamped. Charging conditions are used to categorize traffic volumes, elapsed time and number of events, such as per tariff period. |
| Rating Group |  | OM | This filed holds the rating group. |
| Used Unit Container |  | OC | This field holds the used units and information connected to the reported units. |
| Triggers |  | OC | This field holds the reason for closing the used unit container. |
| Trigger Timestamp |  | OC | This field holds the timestamp of the trigger. |
| Time | OC | This field holds the amount of used time. |
| Downlink Volume |  | OC | This field holds the amount of used volume in downlink direction. |
| Rating Indicator |  | OC | This field indicates if the units have been rated or not. |
| Local Sequence Number |  | M | This field holds the container sequence number. |
| MBS Container Information |  | OC | This field holds the 5G MBS session data connectivity specific information defined in clause 6.2.1.3. |
| Record Opening Time |  | M | Time stamp when the MBS session is activated in the MB-SMF or record opening time on subsequent partial records. |
| Duration |  | M | This field holds the duration of this record. |
| Record Sequence Number |  | C | Partial record sequence number, only present in case of partial records. |
| Cause for Record Closing |  | M | The reason for the release of the record. |
| Diagnostics |  | OM | This field holds a more detailed reason for the release of the MBS session, when a single cause is applicable. |
| Local Record Sequence Number |  | OM | Consecutive record number created by the CDF. The number is allocated sequentially including all CDR types. |
| Record Extensions |  | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| MBS Session Charging Information |  | OM | This field holds the 5G MBS session data connectivity specific information defined in clause 6.2.1.2. |

End of change