3GPP TSG-SA5 Meeting #152	S5-237938

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0302 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.532 Remove-Update FM related parts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the definition of the fault management Mns is moved to the new TS 28.111, FM related parts should be removed from this specification as specfied in the approved S5-237045 Rel-18 dCR 28.532 Remove-Update FM related parts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove - 2 References (FM related items, duplicate reference to X.733) - 3.1 Definitions (Matching-Criteria-Attributes) - 3.2 Abbreviations (FS) - 11.2 Generic fault supervision management service (stage 2) - 12.2 Generic fault supervision management service (stage 3) - A.2 Generic fault supervision management service (OpenAPI) OpenAPI/TS28532_FileData‎ReportingMnS.yaml ‎– change reference to Fault.yaml |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicate and slightly conflicting definition of fault management in multiple specification would lead to confusion and interoperability problems. Incorrect YAML references. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 3.1, 3.2, 11.2, 12.2, A.2, A.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... TS 28.111 |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/861 The CR based should be agreed/approved together with the TS 28.111. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	 Void

[3]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	3GPP TS 28.531: "Management and orchestration ; Provisioning; ".

[6]	3GPP TS 28.554: "Management and orchestration ; 5G end to end Key Performance Indicators (KPI)".

[7]	 Void

[8]	 Void

[9]	 Void

[10]	 Void

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	 Void

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	 Void

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[16]	 Void

[17]	Void

[18]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[19]	3GPP TS 32.401: "Telecommunication management; Perfomance Measurement (PM); Concept and requirements".

[20]	ISO 8601:2004: "Data elements and interchange formats – Information interchange – Representation of dates and times".

[21]	Void.

[22]	Void.

[23]	Void.

[24]	Void.

[25]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects ".

[26]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[27]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[28]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[29]	W3C REC-xml-names-19990114: "Namespaces in XML".

[30]	Void.

[31]	Void

[32]	IETF RFC 6241 "Network Configuration Protocol (NETCONF)".

[33]	3GPP TS 32.160 " Management and orchestration; Management service template ".

[34]	IETF RFC 7950 "The YANG 1.1 Data Modeling Language".

[35]	Void

[36]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[37]	IETF RFC 7396: "JSON Merge Patch".

[38]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[39]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[40]	IETF RFC 6455: "The WebSocket Protocol".

[41]	IETF RFC 793: "Transmission Control Protocol".

[42]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[43]	Void

[44]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[45]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://github.com/onap/vnfrqts-requirements/blob/05f26fac2b941513a7d0e856b99fd8c61d688299/docs/Chapter8/ves7_1spec.rst#resource-structure.

[46]	3GPP SA5 FORGE OpenAPI definitions: https://forge.3gpp.org/rep/sa5/MnS/tree/Rel-16/OpenAPI.

[47]	3GPP TS 32.404: "Performance Management (PM); Performance measurements; Definitions and template".

[48]	Void

[49]	IETF RFC 8040: "RESTCONF protocol".

[50]	IETF RFC 7951: " JSON Encoding of Data Modeled with YANG".

[51]	IETF RFC 6243: "With-defaults Capability for NETCONF".

[52]	IETF RFC 3339: " Date and Time on the Internet: Timestamps".

[53]	3GPP SA5 FORGE OpenAPI definitions: https://forge.3gpp.org/rep/sa5

Next change

# 3	Definitions and abbreviations

## 3.1	Definitions

Void.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

MnS	Management Service

Next change

## 11.2	Void

Next change

## 12.2	Void

Next change

## A.2	Void

Next change

## A.7.2	OpenAPI document "TS 28532_FileDataReportingMnS.yaml"

openapi: 3.0.1

info:

title: File Data Reporting MnS

version: 17.1.0

description: >-

OAS 3.0.1 definition of the File Data Reporting MnS

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/fileDataReportingMnS/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/files:

get:

summary: Read information about available files

description: >-

Information about available files is read with HTTP GET. The files for

which information shall be returned are identified with the path

component (base resource) and the query component (fileDataType, beginTime,

endTime) of the URI.

parameters:

- name: fileDataType

in: query

description: >-

This parameter selects files based on the file data type.

required: true

schema:

$ref: '#/components/schemas/FileDataType'

- name: beginTime

in: query

description: >-

This parameter selects files based on the earliest time they

became available

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

- name: endTime

in: query

description: >-

This parameter selects files based on the latest time they

became available

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

responses:

'200':

description: >-

'Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body.'

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/FileInfo'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions:

post:

summary: Create a subscription

description: >-

To create a subscription the representation of the subscription is

POSTed on the /subscriptions collection resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS28111_Fault.yaml#/components/schemas/Subscription'

responses:

'201':

description: >-

Success case ("201 Created").

The representation of the newly created subscription resource shall

be returned.

content:

application/json:

schema:

$ref: 'TS28111_Fault.yaml#/components/schemas/Subscription'

headers:

Location:

description: URI of the newly created subscription resource

required: true

schema:

type: string

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

callbacks:

notifyFileReady:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyFileReady'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyFilePreparationError:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyFilePreparationError'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions/{subscriptionId}:

delete:

summary: Delete a subscription

description: >-

The subscription is deleted by deleting the corresponding subscription

resource. The resource to be deleted is identified with the path

component of the URI.

parameters:

- name: subscriptionId

in: path

description: Identifies the subscription to be deleted.

required: true

schema:

type: string

responses:

'204':

description: >-

Success case ("204 No Content").

The subscription resource has been deleted. The response message body

is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

FileDataType:

type: string

enum:

- Performance

- Trace

- Analytics

- Proprietary

FileNotificationTypes:

type: string

enum:

- notifyFileReady

- notifyFilePreparationError

FileInfo:

type: object

properties:

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

fileSize:

type: integer

fileReadyTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

fileExpirationTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

fileCompression:

type: string

fileFormat:

type: string

fileDataType:

$ref: '#/components/schemas/FileDataType'

NotifyFileReady:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

fileInfoList:

type: array

items:

$ref: '#/components/schemas/FileInfo'

additionalText:

type: string

NotifyFilePreparationError:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

fileInfoList:

type: array

items:

$ref: '#/components/schemas/FileInfo'

reason:

type: string

additionalText:

type: string

End of  changes