# 3GPP TSG-SA5 Meeting #152	S5-238224

# Chicago, USA,  13-17, November 2023                                                      was S5-237950

# Source:	Samsung, DTAG

# Title:	Rel-18 pCR TS 28.318 NRM Definitions for coordinated rapid recovery

# Document for:	Approval

Agenda Item:	6.6.4.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 3	Rationale

This pCR proposes the NRM definitions required to support the procedure defined in clause 6.1 of TS 28.318.

# 4	Detailed proposal

It is proposed to agree to the following text to clause 7 in TS 28.318.

Begin Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	DIRECTIVE (EU) 2019/ 944 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL - of 5 June 2019 - on common rules for the internal market for electricity and amending Directive 2012/ 27/ EU (europa.eu)
https://eur-lex.europa.eu/legal-content/EN/TXT/PDF/?uri=CELEX:32019L0944&from=EN

[3]	IEC TC 57 https://www.iec.ch/ords/f?p=103:7:511571509228708::::FSP_ORG_ID,FSP_LANG_ID:1273,25

[4]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements"

[5]	IEC "Bringing intelligence to the grid", International Electrotechnical Commission, Geneva, Switzerland, 2018.  https://www.iec.ch/basecamp/bringing-intelligence-grid <accessed: 12.7.23>

[6]	IEEE SMARTGRID, "Standards", IEEE, 2023. https://smartgrid.ieee.org/about-ieee-smart-grid/standards <accessed: 12.7.23>

[7]		Sendin, A., Stafford, J., Grilli, A., "Utilities and Telecommunications in a Nutshell", EUTC, Ediciones Experiencia, 2022.

[8]	3GPP TS 22.104, "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[9]	3GPP TS 22.261, "Service requirements for the 5G system; Stage 1"

[10]	IEC TR 61000-3-6 Electromagnetic compatibility (EMC) - Part 3-6: Limits - Assessment of emission limits for the connection of distorting installations to MV, HV and EHV power systems.

[11]	IETF RFC 1628, "UPS Management Information Base", May 1994.

[12]	ETSI ES 202 336-3: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks); Part 3: AC UPS power system control and monitoring information model".

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

Next Change

## 7.Y	Model

### 7.Y.2       Class diagrams

#### 7.Y.2.1      Relationships

This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document

Figure 7.Y.2.1-1: DSO Rapid Recovery NRM fragment

Editor's Note: All datatypes present in the document will be represented in the UML relationship diagram.

#### 7.Y.2.2	Inheritance

This clause depicts the inheritance relationships.

<<InformationObjectClass>>

Top

<<InformationObjectClass>>

DsoRapidRecovery

<<InformationObjectClass>>

UPSInfo

Figure 7.Y.2.2-1: DSO Rapid Recovery NRM fragment

### 7.Y.3       Class definitions

#### 7.Y.3.1	DsoRapidRecovery

##### 7.Y.3.1.1 Definition

This IOC represents the mutual information exchange between DSO and MNO to achieve Rapid Recovery of DSO’s energy service outage for all DSO sites which require service restoration. DSO uses DsoRapidRecovery IOC to configure the recovery related information like energy service outage start time, affected region, expected restoration time, expected coordination from MNO, feasible coordination that can be provided by MNO, DSO’s service restoration information. MNO would use this IOC to configure information related to intervention time, duration and location that can be feasible from MNO. The sharing of DSO’s energy service outage and recovery information to MNO has mutual benefits. MNO can plan and prepare well its UPS power supply backup on its sites for communication services. The MNO can know which MNO sites are affected, the estimate for how long, and when the energy outage ends for those sites. The DSO is able to identify when they can perform remote restoration procedures and the MNO can arrange to provide communication services at that time. Overall, MNO can understand DSO’s requirement of MNO’s Rapid Intervention and can support DSO for its rapid recovery of distribution services

The energySupplyId, a data element used extensively in this class definition is known to both the MNO and DSO and is the means by which information is organized, essentially the 'key identifier' used to exchange information about specific sites, which is essential for most Rapid Recovery actions (see Annex C.2) and Rapid Intervention actions (see Annex C.2).

##### 7.Y.3.1.2 Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dSORapidRecoveryInfo | M | T | F | F | T |

##### 7.Y.3.1.3	Attribute constraints

None.

#### 7.Y.3.1.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 7.Y.3.2 DSORapidRecoveryInfo  <<dataType>>

##### 7.Y.3.2.1 Definition

This <<dataType>>  defines the DSO Rapid Recovery related information for a particular location/region.

##### 7.Y.3.2.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoServiceOutageInfo | M | T | T | F | T |
| dsoRapidInterventionInfo | M | T | T | F | T |
| mnoRapidInterventionInfo | M | T | T | F | T |
| dsoServiceRestoreInfo | M | T | T | F | T |

##### 7.Y.3.2.3	Attribute constraints

None.

#### 7.Y.3.3 ServiceOutageInfo  <<dataType>>

##### 7.Y.3.3.1 Definition

This <<dataType>> defines several information shared by DSO to MNO related to its energy service outage including the starting time of DSO energy service outage, outage substation locations etc.

The attribute dsoOutStartTime defines the time stamp at which the DSO energy service outage occurs. Along with outage locations and expected recovery time this could be beneficial for MNO to plan and prepare (e.g when and where to activate and de-activate) its UPS power backup at different site locations for its crucial communication services and also for assisting DSO for MRI (MNO Rapid Intervention). In case of MRI assistance MNO may reserve its UPS resources to be used when required.

The attribute dsoOutRegion defines the regions where DSO energy service outage occurs. These are usually the locations where DSO subsystems are situated. This is usually defined as latitude-longitude pair and energySupplyId. See the definition of Energy Supply ID in clause 3. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. The energySupplyIdidentifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation.

The attribute dsoRestoreTimeExpected defines the time by which DSO expects its energy distribution service to be restored through its manual dispatch and processes. This is usually a much longer time duration that may not be acceptable for some crucial public services like hospitals.

##### 7.Y.3.3.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoOutStartTime | M | T | T | F | T |
| dsoOutRegion | M | T | T | F | T |
| dsoRestoreTimeExpected | M | T | T | F | T |

##### 7.Y.3.3.3	Attribute constraints

None.

#### 7.Y.3.4 DsoInterventionInfo  <<dataType>>

##### 7.Y.3.4.1 Definition

This <<dataType>> defines several information shared by DSO to MNO related to its requirement for MNO’s Rapid Intervention (MRI).

The attribute dsoTransmissionRestoreTime defines the time at which DSO restores its Energy “transmission” service. For DSO’s distribution service to restore first its transmission line/service needs to be restored. From this time a DSO can use its automated smart energy services e.g SCADA to recover its distribution services much quicker in substations instead of manual processes but for this MNO’s communication services via MRI is required to DSO. Hence this is also the time from when the DSO expects MRI to start.The attribute dsoRequiredInterventionDuration defines time “duration” for which the DSO requires MRI in order to restore its energy distribution services rapidly. This time duration is usually defined in minutes.

The attribute dsoPriorityRecoveryLocation defines the locations of DSO substations that need recovery on priority by using smart energy services because some of the substations locations might be providing electrical energy for crucial services like hospitals, mission critical service avenues, government offices etc. This is used by MNO to ascertain the gNBs at which the UPS power needs to be reserved as part of MRI because the gNBs are servicing these substations. This is usually defined as latitude-longitude pair.  The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region containing substation. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation.

##### 7.Y.3.4.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoTransmissionRestoreTime | M | T | T | F | T |
| dsoRequiredInterventionDuration | M | T | T | F | T |
| dsoPriorityRecoveryLocation | M | T | T | F | T |

##### 7.Y.3.4.3	Attribute constraints

None.

#### 7.Y.3.5 MnoInterventionInfo  <<dataType>>

##### 7.Y.3.5.1 Definition

This <<dataType>> defines several information shared by MNO to DSO for feasible fulfillment of DSO’s rapid recovery requirements by MNO. It may be possible that instead of full, some part of  requested Rapid Recovery requirements MNO is actually able to cater in terms of intervention start time, location etc.

The attribute mnoInterventionTime defines the time at which a MNO will actually be able to provide dedicated communication services to DSO (as MRI) depending on its site’s UPS backup status , engagement in other critical service avenues like hospitals etc. DSO needs this to use its smart energy services like SCADA for recovering its energy distribution services.

The attribute mnoInterventionDuration defines the time duration for which a MNO will actually be able to provide dedicated communication services to DSO (as MRI) depending on its UPS backup status, engagement in other critical service avenues like hospitals etc. This time duration is usually defined in minutes.

The attribute mnoInterventionLocation defines the locations having DSO’s substations where its actually feasible for MNO to provide its dedicated communication services (as MRI) for DSO’s smart energy services usage. The criteria for location feasibility may depend on MNO’s UPS backup status per site, or other prior crucial service commitments in other locations like hospital, government offices etc. This is usually defined as latitude-longitude pair and energySupplyId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region containing substation. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation.

##### 7.Y.3.5.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnoInterventionTime | M | T | T | F | T |
| mnoInterventionDuration | M | T | T | F | T |
| mnoInterventionLocation | M | T | T | F | T |

##### 7.Y.3.5.3	Attribute constraints

None.

#### 7.Y.3.6 ServiceRestoreInfo  <<dataType>>

##### 7.Y.3.6.1 Definition

This <<dataType>> defines information shared by DSO to MNO regarding actual final restoration of DSO’s distribution services. From this MNO gets to know at what time it can stop its Rapid Intervention for the DSO and at which locations. MNO also gets to know when and where (gNB sites) it does not need UPS backup anymore for its communication services/operations. DSO will update this information once the energy distribution service has been restored.

The attribute dsoRestoreTimeActual defines the time at which DSO finally restores its energy distribution services rapidly  by using MRI. This time is much smaller than dsoRestoreTimeExpected.

The attribute dsoRestoreLocationActual defines the Substation locations where DSO is able to finally restore its energy distribution services. These may be same or fewer than dsoPriorityRecoveryLocation or mnoInterventionLocation. This is usually defined as latitude-longitude pair and energySupplyId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation.

##### 7.Y.3.6.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoRestoreTimeActual | M | T | T | F | T |
| dsoRestoreLocationActual | M | T | T | F | T |

##### 7.Y.3.6.3	Attribute constraints

None.

#### 7.Y.3.7 ServiceLocationIdentifier  <<datatype>>

##### 7.Y.3.7.1 Definition

This <<datatype>> defines information shared by DSO to MNO regarding location of DSO distribution facility in terms of latitude, longitude and energySupplyId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is known between both the parties using out-of-band mechanisms hence not subject to standardisation.

##### 7.Y.3.7.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| latitude | M | T | T | F | T |
| longitude | M | T | T | F | T |
| energySupplyId | O | T | T | F | T |

##### 7.Y.3.7.3	Attribute constraints

None.

#### 7.Y.3.8	UPSInfo

##### 7.Y.3.8.1 Definition

This IOC represents the UPS related information of MNO site or a base station. It is name-contained by DsoRapidRecovery IOC. This is required by DSO from MNO to achieve coordinated intelligent outage planning by DSO during its energy outage recovery. If the information like MNO sites’ UPS backup status, base station ids mapped to energy supply ids, battery draining status at MNO site is known to DSO then DSO may work more intelligently on its energy outage recovery, taking into account which base station Ids of MNO their infrastructure relies on as well as the UPS backup remaining life in that MNO site.

##### 7.Y.3.8.2 Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| isUpsAvailable | M | T | F | F | T |
| totalUpsBackupDuration | M | T | F | F | T |
| remainingUpsBackupDuration | M | T | F | F | T |
| energySupplyId | M | T | F | F | T |
| baseStationId | M | T | F | F | T |

##### Editor's Note: An explanation about how the association between the UPS and the corresponding baseStationId and energySupplyId is done is missing and must be added.

##### 7.Y.3.8.3	Attribute constraints

None.

##### 7.Y.3.8.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

### 7.Y.4	Attribute definitions

#### 7.Y.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document in addition to those existing in clause 4.4.1 of [X].

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| dSORapidRecoveryInfo | This defines the DSO and MNO coordinated recovery related information for a particular location/region. | type: DSORapidRecoveryInfo multiplicity: 1.* isOrdered: False isUnique: Yes defaultValue: None isNullable: False |
| dsoServiceOutageInfo | Contains list of attributes that define several information shared by DSO to MNO related to its energy service outage like starting time of DSO energy service outage, outage locations, expected restore time of outage. | type: ServiceOutageInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRapidInterventionInfo | Contains list of attributes that define several information shared by DSO to MNO related to its requirement for MNO’s Rapid Intervention (MRI) like expected time to start MRI, expected duration of MRI, expected locations that require MRI on priority. | type: DsoInterventionInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoRapidInterventionInfo | Contains list of attributes that define several information shared by MNO to DSO for feasible fulfillment of DSO’s rapid recovery requirements by MNO. It may be possible that instead of full, some part of requested MRI requirements MNO is actually able to cater in terms of intervention start time, location, intervention duration. | type: MnoInterventionInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoServiceRestoreInfo | Contains list of attributes that define information shared by DSO to MNO regarding actual final restoration of DSO’s distribution services. From this information, MNO gets to know at what time it can stop its MRI services for DSO and at which locations. MNO also gets to know when and where (gNB sites) it does not need UPS backup anymore for its communication services/operations. | type: ServiceRestoreInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoOutStartTime | Time stamp at which the DSO energy service outage occurs. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoOutRegion | Regions where DSO energy service outage occurs. These are the locations where DSO subsystems are situated. This is defined as latitude -longitude pair and energySupplyId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. | Type: ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRestoreTimeExpected | Time by which DSO expects its energy distribution service to get restored through its manual dispatch and processes. This is usually a longer time period that may not be acceptable for some crucial public services like hospitals. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoTransmissionRestoreTime | Time at which DSO restores its Energy “transmission” service. This is the time from when the DSO expects MRI to start. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRequiredInterventionDuration | Time duration for which the DSO requires MRI in order to restore its energy distribution services rapidly. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoPriorityRecoveryLocation | Locations of DSO substations that need recovery on priority by using smart energy services. This is defined as latitude-longitude pair and energySupplyId. This can be used by MNO to ascertain the gNBs at which the UPS power needs to be reserved as part of MRI. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. | Type:ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoInterventionTime | Time at which a MNO will actually be able to provide dedicated communication services to DSO (as MRI) depending on its UPS backup status , other critical service avenues etc. DSO needs this to use its smart energy services like SCADA for recovering its energy distribution services. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoInterventionDuration | Time duration for which a MNO will actually be able to provide dedicated communication services to DSO (as MRI) depending on its UPS backup status , other critical service avenues etc. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoInterventionLocation | Locations having DSO’s substations where its actually feasible for MNO to provide its dedicated communication services (as MRI) for DSO’s smart energy services usage. The criteria for location feasibility may depend on MNO’s UPS backup status per site, or other prior crucial service commitments in other locations like hospital, government offices etc. This is defined as latitude-longitude pair and energySupplyId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. | Type: ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRestoreTimeActual | Time at which DSO finally restores its energy distribution services rapidly by using MRI. This time is much smaller than dsoRestoreTimeExpected | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRestoreLocationActual | Substation locations where DSO is able to finally restore its energy distribution services. These may be same or fewer than dsoPriorityRecoveryLocation or mnoInterventionLocation. This is defined as latitude-longitude pair and meterId. The point defined by the Lat/Log pair serves as one point of an area polygon defining the region. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. | Type: ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| latitude | This defines the latitude | type: float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | This defines the longitude | type: float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySupplyId | energySupplyId identifies the MNO facility (e.g. base station) using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. A meter with a unique energySupplyId is installed by DSO in the MNO site. | type: String multiplicity: 1.* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| isUpsAvailable | It tells if the MNO site has UPS installed/available or not. There might be some sites where UPS is not installed yet or is under progress etc. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| totalUpsBackupDuration | Total time duration for which MNO site can operate on its UPS backup i.e. capacity of UPS backup in terms of time duration (e.g. minutes) for which it can provide power supply to MNO site from onset of DSO outage. It is defined in minutes. allowedValues: Integer with a minimum value of 1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remainingUpsBackupDuration | Remaining Time duration for which MNO site can operate on its UPS backup at a given time i.e. capacity of UPS backup in terms of time duration (e.g. minutes) for which it can provide power supply to MNO site from a given time. It is defined in minutes. allowedValues: Integer with a minimum value of 1 [NOTE1] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| baseStationId | Base station identity as defined by MNO for its base stations in a given site. This is like a user label for identification of the site/MNO facility. | type: String multiplicity: 1 isOrdered: N/A isUnique:True defaultValue: None isNullable: False |
| NOTE 1: The value of remainingUpsBackupDuration can be taken from IETF RFC 1628 [11], upsEstimatedMinutesRemaining. |  |  |

### 7.Y.5	Common notifications

Refer to clause 4.5.2 of [X] for Configuration notifications.

End of Change