3GPP TSG-SA5 Meeting #152 	S5-237478

# Chicago, USA, 13 Nov 2023 – 17 Nov 2023

# Source:	Amdocs, Nokia, Nokia Shanghai Bell

# Title:	Solution for Topic 4 Key Issue 4a

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.840

# 2	References

[1]		3GPP TR 28.840: "Study on CHF Segmentation"

# Rationale

This pCR is to propose a solution for the study topic for CHF selection by Charging Domain.

# 4	Detailed proposal

First change

### 5.4.4	Possible Solutions

#### 5.4.4.x	Solution #4.x:  Use enumeration to identify charging domain or subsystem

The solution addresses key issue #4a (Definition of charging domain(s) or subsystem(s) for distributed charging deployments) of topic #4 on CHF Discovery by Charging Domains.

TS 32.240 defines a ‘domain’ as a part of communication network that provides resources using a certain bearer technology.

The TS 21.905 defines a ‘3GPP System’ as ‘A telecommunication system conforming to 3GPP specifications, consisting of one or more 3GPP core networks, one or more 3GPP access networks (providing GSM/EDGE, UTRA, E-UTRA, or NR radio access), and/or non-3GPP access networks (such as WLAN), and User Equipment.’

Furthermore, TS 32.240 defines the list of Charging TS that specify the charging functionality for a domain (CS, PS, 5GS), subsystem (IMS) and service levels (e.g. MMS, LCS, PoC, MBMS, SMS, MMTel etc). These are in number ranges TS 32.25x, TS 32.26x and TS 32.27x. Network Slicing is covered under TS 28.201 and TS 28.202.

The TS 32.240 clause 1 also specifies that the set of TSs in the TS 32.28x range covers common services, such as the Advice of Charge service, and the set of in the TS 32.29x range covers common aspects, such as CDR parameter and syntax descriptions etc.

Therefore, it follows that the set of middle tier TS in range 32.25x. 32.26x, 32.27x as well as TS 28.20x are sufficient to identify a list of charging domains and subsystems, that could be supported by a CHF instance in a distributed charging deployment.

TS 32.240 clause 1 shows the list of charging domains, subsystems and services and their document structure as follows:

A few subsystems do not have converged charging defined at present. Therefore, the list of charging domains and subsystems, that can be supported by a CHF instance in a distributed charging deployment, can be defined in an enumerated list as per the table below.

The list can be extended in future as new subsystems are defined or converged charging specifications are added for more subsystems.

| Charging Domain or Subsystem Identification Enumeration | Middle Tier Specification | NF Consumer |
| --- | --- | --- |
| CS_Domain | TS 32.250 |  |
| PS_Domain | TS 32.251 |  |
| NEF_NB_API | TS 32.254 | SCEF for Ro/Rf charging NEF for CHF converged charging |
| Data_Connectivity | TS 32.255 | SMF |
| 5G_Connection_And_Mobility | TS 32.256 | AMF |
| Short_Message_Service | TS 32.274 | SMSF |
| IP_Multimedia_Subsystem | TS 32.260 | IMS Node e.g. MRFC, IMS-GWF (connected to S-CSCF using ISC) and SIP AS for converged charging |
| Multimedia_Messaging_Service | TS 32.270 | MMS Node |
| MultiMedia_Telephony | TS 32.275 | MMTel AS for converged charging |
| Proximity_Based_Services | TS 32.277 | CEF or 5G-DDNMF (5G Direct Discovery Name Management Function) for CHF converged charging |
| Multimedia_Broadcast_And_Multicast_Service | TS 32.273 | BM-SC (Broadcast Multicast - Service Centre) |
| Network_Slice_Performance_And_Analytics | TS 28.201 | CEF (MnS producer or NWDAF) |
| Network_Slice_Management | TS 28.202 | MnS producer or CEF |
| Network_Slice_Admission_Control | TS 28.203 | NSACF (Network Slice Admission Control Function) |
| Network_Slice_AA | TS 28.204 | NSAAF (Network Slice Authorization and Authentication) |
| Edge_Computing | TS 32.257 | CEF (MnS producer) and EES (Edge Enabler Server) |

The list of subsystems that do not have converged charging specifications defined, and therefore excluded from the above list are given below for reference:

- Control Plane Data Transfer Domain Charging; TS 32.253

- Voice Call Service Charging; TS 32.276

- Monitoring Event Domain Charging; TS 32.278

- Location Services Charging; TS 32.271

- Push to Talk Over Cellular Domain Charging; TS 32.272

End of changes