# 3GPP TSG-SA5 Meeting #152	S5-237500

# 13 - 17 November 2023, Chicago, IL, USA

# Source:	Nokia, Nokia Shanghai Bell

# Title:		Input to DraftCR 28.105 ML inference policy-based activation

# Document for:	Approval

# Agenda Item:	6.4.3.1 - AIML_MGMT_WoP#1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	S5-237244 DraftCR : “Draft CR AIML_MGMT - TS 28.105; Enhancements for AI-ML management”.

# 3	Rationale

The use case of policy-based activation of ML has been discussed and agreed (in TR 28.908). This CR is to normatively propose the use case and requirements for ML inference.

Alternatively, ML Inference activation and deactivation can also be subclause of 6.5.5 Executing AI/ML Inference.

# 4	Detailed proposal

Start of First change

## 6.5	AI/ML Inference Phase

### 6.5.B		ML inference  activation and deactivation

### 6.5.B.2		Use cases

#### 6.5.B.2.W	Policy-based ML activation

The activation procedure should not entirely rely on the MnS consumer micro-managing every activation step. Yet, it can also not be left fully to the MnS producer, as the producer may not have a “full picture” on other ML entities/capabilities that are currently in operation or activated on different MnS producers according to the MnS consumer's interests. The producer needs to be instructed by the MnS consumer on the ways to perform the right activations of ML capabilities through activation policies.

The policy-based activation may be instructed via one or more ML activation policies, where an ML activation policy is a sequence of tuples of conditions and activation settings that may be executed by the ML producer. Conditions may define specific outcomes on performance metrics for which a particular activation may be executed while activation settings define specific attributes of the ML capability activation scope (e.g., object or object type, network context, activation time window) for which ML capability should be activated.

One use of policy-based activation is the managed (e.g., gradual) activation of the ML capabilities. An ML Entity is generally responsible for a scope (e.g., a specific list of NR cells) of the radio coverage area, as either of a decision-making capability or an analysis capability. However, it is difficult to “predict” or quantify the benefits of using an ML entity in each context, before using it. So, it is necessary to ensure that ML training capabilities of an ML entity are activated in a gradual way, through “phased activation steps”. E.g., for a sub scope of the ML Entity which is gradually expanded as confidence is gained that the ML entity performs well.

For example, managed (e.g., gradual) activation may activate the ML entity only for a limited or specific number of cells covering part of a geographical coverage area and not the whole city or only for a limited period (say between 18:00 and 06:00) rather than for the entire operation time. The ML training MnS should include a capability to activate ML entities for a sub scope and progressively increase the scope.

### 6.5.B.3	Requirements for ML inference activation and deactivation

Table 6.5.B.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_POL_ACT-1: | The ML inference MnS producer should have a capability to activate the ML capabilities based on the policies provided by the ML inference MnS consumer, the policies instructing the ML MnS producer on how or/and when to activate which ML capabilities. | Policy-based ML activation (clause 6.5.B.2.W) |
| REQ-AIML_POL_ACT-2: | The ML inference MnS producer should have a capability to allow an authorized MnS consumer to partially and progressively activate or deactivate the ML inference capabilities for an ML entity. | Policy-based ML activation (clause 6.5.B.2.W |

End of Changes