3GPP TSG-SA5 Meeting #152 	S5-237784

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0474 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.255 Addition of CHF selection by H-CHF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHRACHF |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of CHF selection for the CHF-to-CHF connection in LBO roaming scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of CHF selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CHF selection will be undefined for CHF-to-CHF in LBO roaming scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.1.9.2	CHF selection

In home routed scenario, at PDU session establishment, the CHF selection mechanism specified in clause 5.1.8 applies to:

-	The V-SMF for CHF selection in VPLMN, with the following differences:

-	CHF address(es) selection mechanisms based on PCF and UDM are not applicable.

-	When charging characteristics is used it will be based on local configuration .

-	When NRF is used, the V-CHF can be selected based on UE identified as in-bound roamer and the PLMN Id of the H-PLMN.

-	The H-SMF for CHF selection in HPLMN can be done as for non-roaming case.

In roaming home routed, PDU session upon V-SMF change:

-	Intra-PLMN V-SMF change: CHF address supplied by the old V-SMF shall be used.

-	Inter-PLMN V-SMF change: CHF selection mechanism as per V-SMF CHF selection in VPLMN at PDU session establishment.

In local breakout scenario, at PDU session establishment, the CHF selection mechanism specified in clause 5.1.8 applies to:

-	The V-SMF for CHF selection in VPLMN, is the same as in the home routed scenario.

-	The V-SMF for CHF selection in HPLMN, with the following differences.

-	CHF address(es) selection mechanisms based on PCF, UDM, and local configuration are not applicable.

-	 NRF based discovery, the H-CHF can be selected based on the H-PLMN of the UE.

H-CHF selection by the V-CHF, local breakout scenario, is based on the following and with this priority order (highest to lowest):

-	NRF based discovery, can be based on HPLMN or UE.

-	V-CHF locally provisioned charging characteristics.

End of changes