3GPP TSG-SA5 Meeting #152 	S5-237792

Chicago, US, 13-17 November 2023

# Source:	Ericsson

# Title:	Addition of solution for CHF Selection based on SUPI or Group ID

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.840.

# 2	References

[1]		3GPP TR 28.840: "Study on CHF segmentation"

# 3	Rationale

New solution for key issue #2.1 reusing existing solution.

# 4	Detailed proposal

First change

### 5.1.4.x	Solution 1.x: SCP retrieved CHF Group Id

A possible solution for key issue #2.1 covers the requirement REQ-CH_ SELCHF_LA-01.

According to TS 23.501 [5] clause 3.1 the CHF group id refers to one or more CHF instances managing a specific set of SUPIs. The SCP can optionally, TS 23.501 [5] clause 6.2.19, interact with UDR, to resolve the CHF Group ID based on UE identity e.g., SUPI or IMPI/IMPU. This means that by using an SCP the CHF Group Id can be used for further selecting CHF instances that can handle the request.

End of changes