3GPP TSG-SA5 Meeting #152	S5-238360

# Chicago, US, 13-17 November 2023  										     Revision of S5-237244

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | - | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements for AI-ML management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC, Nokia, Nokia Shanghai Bell, HUAWEI, CATT, Ericsson, Deutsche Telekom, TELUS, China Mobile, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This DraftCR incorporates the following agreed contributions under AIML_MGMT work item: 1. Input to draft CRs approved in SA5#149: S5-234827 S5-234597 S5-234828 S5-234649 S5-234653 S5-234822 S5-234829 S5-234658 S5-234820 S5-234821 S5-234833 S5-234807 S5-234819 S5-234659 S5-234350 2. Input to draft CRs approved in SA5#150: S5-235827 S5-235828 S5-236075 S5-235837 S5-235838 S5-235839 S5-235840 S5-235999 3. Input to draft CRs approved in SA5#151: S5-236941 S5-236938 S5-236939 S5-237162 S5-236940 S5-236935 S5-237114 S5-237036 4. Input to draft CRs approved in SA5#152: S5‑238132 S5‑238111 S5‑238112 S5‑238341 S5‑238350 S5‑237611 S5‑237691 S5‑238273 S5‑238123 S5‑238347 The detailed reasons for change can be found in the relevant contributions listed above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add terms related to ML training - Correction to the Scope - Add AI/ML workflow - Add AI/ML management capabilities - Add UC, requirements and NRMs for ML entity joint training - Add UC, requirements, and NRMs for ML validation - Enhance the ML training to include re-training - Add NRM for producer initiated training - Add use case and requirements for training data effectiveness reporting - Add use case and requirements for managing the capabilities of ML Entities - Add use cases, requirements and NRMs for AI/ML inference activation - Add use case and requirements for AI/ML inference performance evaluation - Add use case and requirements for AI/ML inference performance measurements selection - Add use case, requirements and NRMs for ML training and testing performance management - Add use case, requirements and NRMs for update of ML Entities - Add use case, requirements, and NRMs for ML Testing - Add use case, requirements and NRMs for joint testing of multiple ML entities - Add use case, requirements and NRMs for ML entity loading - Add use case, requirements and NRMs for AI/ML function configuration management - Add ML Inference History use case and requirements - Harmonize usage of ML model and entity - Add use case and requirements for AI/ML inference emulation - Add AI/ML functionalities management scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements of AI/ML management mentioned in the “summary of change” cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3.1, 4A, 6, 7, Annex A. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1. S5-234847 Initial version of the DraftCR 2. S5-235139 Revision based on the updated baseline 3. S5-235988 Update based on the outcome of SA5#150 4. S5-237244 Update based on the latest TS baseline and outcome of SA5#151 5. S5-238360 Update based on the latest TS baseline and outcome of SA5#152 |  |  |  |  |  |  |  |  |

First modified section

# 1	Scope

The present document specifies the Artificial Intelligence / Machine Learning (AI/ML) management capabilities and services for 5GS where AI/ML is used, including management and orchestration (e.g., MDA, see 3GPP TS 28.104 [2]) 5G networks (e.g., NWDAF, see 3GPP TS 23.288 [3]) and NG-RAN (e.g., RAN intelligence defined in TS 38.300 [16] and TS 38.401 [17]).

Next modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[8]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[9]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[10]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model repertoire".

[14]	3GPP TS 32.160: "Management and orchestration; Management service template".

[15]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[16]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[17]	3GPP TS 38.401: "NG-RAN; Architecture description".

[18]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Next modification

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ML entity: a manageable artifact of an ML model.

NOTE 1: 	An ML entity may contain metadata related to the model. Metadata may include e.g. the applicable runtime context for the ML model.

AI decision entity: an entity that applies a non-ML based logic for making decisions that can be managed as a single composite entity.

ML model: mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

NOTE 2: The ML models are proprietary and not in scope for standardization.

ML model training: process performed by an ML training function to take training data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss.

ML initial training: the ML model training that generates the initial version of an ML entity..

ML re-training: the ML model training that generates a new version of a trained ML entity using the same type, but different values or distributions of training data used to train the model associated to the previous version of the ML entity.

NOTE 3: 	A new version of a trained ML entity supports the same type of inference as the previous version of the ML entity, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML entity.

ML training: refers to end-to-end processes to enable an ML training function to perform ML model initial training or re-training (as defined above).

NOTE 4: 	ML training may include interaction with other parties to collect and format the data required for ML model training.

ML training function: a logical function with ML modeling training capabilities; also referred to as MLT function.

AI/ML inference function: a logical function that employs an ML model and/or AI decision entity to conduct inference.

Next modification

# 4a	AI/ML management functionality and service framework

## 4a.0	AI/ML operational workflow

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN, and management system), the generic AI/ML operational workflow in the lifecycle of an ML entity, is depicted in Figure 4a.0-1.

Figure 4a.0-1: AI/ML operational workflow

The workflow involves 4 main phases; training, emulation, deployment, and inference phase. The main tasks for each phase are briefly described below:

Training phase:

-	ML model training: training, including initial training and re-training, of an ML model or a group of ML models. It also includes validation of the ML entity to evaluate the performance when the ML entity performs on the training data and validation data. If the validation result does not meet the expectation (e.g., the variance is not acceptable), the ML model associated with that entity needs to be re-trained. The ML model training is the initial phase of the workflow.

-	ML testing: testing of the validated ML entity to evaluate the performance of the trained ML model when it performs on testing data. If the testing result meets the expectation, the ML entity may proceed to the next phase, otherwise the ML model associated with that entity may need to be re-trained.

Emulation phase:

-	ML emulation: running an ML entity for inference in an emulation environment. The purpose is to evaluate the inference performance of the ML entity in the emulation environment prior to applying it to the target network or system.

NOTE: 	The emulation phase is considered optional and can be skipped in the AI/ML operational workflow.

Deployment phase:

-	ML entity loading: the process (a.k.a. a sequence of atomic actions) of making a trained ML entity available for use at the target AI/ML inference function.

The deployment phase may not be needed in some cases, for example when the training function and inference function are co-located.

Editor’s note:

1.	The relationship between SA5 loading processe and RAN, SA1 (model transfer) and SA2 (NWDAF) processes may be investigated later.

2.	The term “loading” may be revisited.

Inference phase:

-	AI/ML inference: performing inference using the ML entity by the AI/ML inference function.

## 4a.1	Functionality and service framework for ML training

An ML training Function playing the role of ML training MnS producer, may consume various data for ML training purpose.

As illustrated in Figure 4a.1-1 the ML training capability is provided via ML training MnS in the context of SBMA to the authorized consumer(s) by ML training MnS producer.

Figure 4a.1-1: Functional overview and service framework for ML training

The internal business logic of ML training leverages the current and historical relevant data, including those listed below to monitor the networks and/or services where relevant to the ML model, prepare the data, trigger and conduct the training:

-	Performance Measurements (PM) as per 3GPP TS 28.552 [4], 3GPP TS 32.425 [5] and Key Performance Indicators (KPIs) as per 3GPP TS 28.554 [6].

-	Trace/MDT/RLF/RCEF data, as per 3GPP TS 32.422 [7] and 3GPP TS 32.423 [8].

-	QoE and service experience data as per 3GPP TS 28.405 [9] and 3GPP TS 28.406 [10].

-	Analytics data offered by NWDAF as per 3GPP TS 23.288 [3].

-	Alarm information and notifications as per 3GPP TS 28.532 [11].

-	CM information and notifications.

-	MDA reports from MDA MnS producers as per 3GPP TS 28.104 [2].

-	Management data from non-3GPP systems.

-	Other data that can be used for training.

## 4a.2	AI/ML functionalities management scenarios

The ML training function and/or AI/ML inference function can be located in the RAN domain MnS consumer (e.g. cross-domain management system) or the domain-specific management system (i.e. a management function for RAN or CN), or Network Function.

For MDA, the ML training function can be located inside or outside of MDAF. The AI/ML inference function is in the MDAF.

For NWDAF, the ML training function can be located in NWDAF or management system, the AI/ML inference function is in the NWDAF.

For RAN intelligence, the ML training function and AI/ML inference function can both be located in the gNB, or the ML training function can be located in the management system and AI/ML inference function is located in the gNB.

Therefore, there might exist several location scenarios for ML training function and AI/ML inference function.

Scenario 1:

The ML training function and AI/ML inference function are both located in the 3GPP management system (e.g. RAN domain management function). For instance, for RAN domain-specific MDA, the ML training function and AI/ML inference functions for MDA can be located in the RAN domain-specific MDAF. As depicted in figure 4a.2-1.

Figure 4a.2-1: Management for RAN domain analytics

Similarly, for CN domain-specific MDA the ML training function and AI/ML inference function can be located in CN domain-specific MDAF or in the cross-domain MDAF.

Scenario 2:

The ML training function is located in the 3GPP RAN domain-specific management function while the AI/ML inference function is located in gNB. See figure 4a.2-2.

Figure 4a.2-2: Management for RAN intelligence where the ML training is located in RAN domain management function and AI/ML inference is located in gNB

Scenario 3:

The ML training function and AI/ML inference function are both located in the gNB. See figure 4a.2-3.

Figure 4a.2-3: Management for RAN intellgence where the ML training and AI/ML inference are both located in gNB

# 6	AI/ML management use cases and requirements

## 6.1	General

Each operational step in the workflow (see clause 5.0) is supported by one or more AI/ML management capabilities as depicted below for each of the operational phases.

Management capabilities for ML training

-	ML training management: allowing the MnS consumer to request the ML entity training, consume and control the producer-initiated training, and manage the ML entity training/retraining process. The training management capability may include training performance management and setting a policy for the producer-initiated ML entity training.

-	ML validation:  ML training capability also includes validation to evaluate the performance of the ML entity when performing on the validation data, and to identify the variance of the performance on the training and validation data. If the variance is not acceptable, the ML entity would need to be tuned (re-trained) before being made available for the next step in the operational workflow (e.g., ML entity testing).

-	ML testing management:  allowing the MnS consumer to request the ML entity testing, and to receive the testing results for a trained ML entity. It may also include capabilities for selecting the specific performance metrics to be used or reported by the ML testing function. MnS consumer may also be allowed to trigger ML entity re-training based on the ML entity testing performance requirements.

Management capabilities for ML emulation phase:

Editor’s note: description of Management capabilities for ML emulation phase is to be added later.

Management capabilities for ML entity deployment phase:

-	ML entity loading management: allowing the MnS consumer to trigger, control  and/or monitor the ML entity loading process.

Management capabilities for AI/ML inference phase:

-	AI/ML inference management

Editor’s note: description of Management capabilities for AI/ML inference phase is to be added later.

The use cases and corresponding requirements for AI/ML management capabilities are specified in the following clauses for each phase of the operational workflow.

## 6.2	Void

-

## 6.2a	ML training phase

### 6.2a.1	ML model training

#### 6.2a.1.1		Description

In operational environment before the ML entity is deployed to conduct inference, the ML model associated with the ML entity needs to be trained (e.g. by ML training function which may be a separate or an external entity to the AI/ML inference function). The ML model training can be initial training or re-training of an already trained ML entity.

The ML model is trained by the ML training (MLT) MnS producer, and the training can be triggered by request(s) from one or more MLT MnS consumer(s), or initiated by the MLT MnS producer (e.g., as a result of model performance evaluation).

#### 6.2a.1.2	Use cases

##### 6.2a.1.2.1	ML model training requested by consumer

The ML training capabilities are provided by an MLT MnS producer to one or more consumer(s).

Figure 6.2a.1.2.1-1: ML training requested by MLT MnS consumer

The ML training may be triggered by the request(s) from one or more MLT MnS consumer(s). The consumer may be for example a network function, a management function, an operator, or another functional differentiation.

To trigger an initial ML training, the MnS consumer needs to specify in the ML training request the inference type which indicates the function or purpose of the ML entity, e.g. CoverageProblemAnalysis. The MLT MnS producer can perform the initial training according to the designated inference type. To trigger an ML re-training, the MnS consumer needs to specify in the ML training request the identifier of the ML entity to be re-trained.

The consumer may provide the data source(s) that contain(s) the training data which are considered as inputs candidates for training. To obtain the valid training outcomes, consumers may also designate their requirements for model performance (e.g. accuracy, etc) in the training request.

The performance of the ML entity depends on the degree of commonality between the distribution of the data used for training and the distribution of the data used for inference. As time progresses, the distribution of the input data used for inference might change as compared to the distribution of the data used for training. In such a scenario, the performance of the ML entity degrades over time. The MLT MnS producer may re-train the ML model associated to the entity if the inference performance of the ML entity falls below a certain threshold, which needs to be configurable by the MnS consumer.

Following the ML training request by the MLT MnS consumer, the MLT MnS producer provides a response to the consumer indicating whether the request was accepted.

If the request is accepted, the MLT MnS producer decides when to start the ML training with consideration of the request(s) from the consumer(s). Once the training is decided, the producer performs the followings:

-	selects the training data, with consideration of the consumer provided candidate training data. Since the training data directly influences the algorithm and performance of the trained ML Entity, the MLT MnS producer may examine the consumer's provided training data and decide to select none, some or all of them. In addition, the MLT MnS producer may select some other training data that are available;

-	trains the ML model using the selected training data;

-	provides the training results (including the identifier of the ML entity generated from the initially trained ML model or the version number of the ML entity associated with the re-trained model, training performance results, etc.) to the MLT MnS consumer(s).

##### 6.2a.1.2.2	ML training initiated by producer

The ML training or re-training may be initiated by the MLT MnS producer, for instance as a result of performance evaluation of the ML entity, or based on feedback or new training data received from the consumer, or when new training data which are not from the consumer describing the new network status/events become available.

Therefore, there is a need to monitor the performance and/or the KPIs of the ML entity and use the thresholds that the MLT MnS consumer configured for the MLT MnS producer to trigger the training or re-training.

When the MLT MnS producer decides to start the ML training, the producer performs the followings:

-	selects the training data;

-	trains the ML model using the selected training data;

-	provides the training results (including the identifier of the ML entity generated from the initially trained ML model or the version number of the ML entity associated with the re-trained model, training performance, etc.) to the MLT MnS consumer(s) who have subscribed to receive the ML training results.

##### 6.2a.1.2.3	ML model and ML entity selection

For a given machine learning-based use case, different entities that apply the respective ML model or AI/ML inference function may have different inference requirements and capabilities. For example, one consumer with specific responsibility and wish to have an AI/ML inference function supported by an ML model or  entity trained for city central business district where mobile users move at speeds not exceeding 30 km/hr. On the other hand, another consumer, for the same use case may support a rural environment and as such wishes to have an ML model and AI/ML inference function fitting that type of environment. The different consumers need to know the available versions of ML entities, with the variants of trained ML models or entities and to select the appropriate one for their respective conditions.

Besides, there is no guarantee that the available ML models/entities have been trained according to the characteristics that the consumers expect. As such the consumers need to know the conditions for which the ML models or ML entities have been trained to then enable them to select the models that are best fit to their conditions and needs.

The models that have been trained may differ in terms of complexity and performance. For example, a generic comprehensive and complex model may have been trained in a cloud-like environment but such a model cannot be used in the gNB and instead, a less complex model, trained as a derivative of this generic model, could be a better candidate. Moreover, multiple less complex models could be trained with different levels of complexity and performance which would then allow different relevant models to be delivered to different network functions depending on operating conditions and performance requirements. The network functions need to know the alternative models available and interactively request and replace them when needed and depending on the observed inference-related constraints and performance requirements.

##### 6.2a.1.2.4	Managing ML training processes

This machine learning capability relates to means for managing and controlling ML model training processes.

To achieve the desired outcomes of any machine learning relevant use-case, the ML model applied for such analytics, needs to be trained with the appropriate data. The training may be undertaken in a managed function or in a management function.

In either case, the network (or the OAM system thereof) not only needs to have the required training capabilities but needs to also have the means to manage the training of the ML models. The consumers need to be able to interact with the training process, e.g., to suspend or restart the process; and also need to manage and control the requests related to any such training process.

##### 6.2a.1.2.5	Handling errors in data and ML decisions

Traditionally, the ML models/entities (e.g., ML entity1 and ML entity2 in figure 6.2a.1.2.5-1) are trained on good quality data, i.e. data that were collected correctly and reflected the real network status to represent the expected context in which the ML entity is meant to operate. Good quality data is void of errors, such as:

-	Imprecise measurements, with added noise (such as RSRP, SINR, or QoE estimations).

-	Missing values or entire records, e.g., because of communication link failures.

-	Records which are communicated with a significant delay (in case of online measurements).

Without errors, an ML entity can depend on a few precise inputs, and does not need to exploit the redundancy present in the training data. However, during inference, the ML entity is very likely to come across these inconsistencies. When this happens, the ML entity shows high error in the inference outputs, even if redundant and uncorrupted data are available from other sources.

Figure 6.2a.1.2.5-1: The propagation of erroneous information

As such the system needs to account for errors and inconsistencies in the input data and the consumers should deal with decisions that are made based on such erroneous and inconsistent data. The system should:

1)	enable functions to undertake the training in a way that prepares the ML entities to deal with the errors in the training data, i.e., to identify the errors in the data during training;

2)	enable the MLT MnS consumers to be aware of the possibility of erroneous input data that are used by the ML entity.

##### 6.2a.1.2.6	ML entity joint training

Each ML entity supports a specific type of inference. An AI/ML inference function may use one or more ML entities to perform the inference(s). When multiple ML entities are employed, these ML entities may operate together in a coordinated way, such as in a sequence, or even a more complicated structure. In this case, any change in the performance of one ML entity may impact another, and consequently impact the overall performance of the whole AI/ML inference function.

There are different ways in which the group of ML entities may coordinate. An example is the case where the output of one ML entity can be used as input to another ML entity forming a sequence of interlinked ML entities. Another example is the case where multiple ML entities provide the output in parallel (either the same output type where outputs may be merged (e.g., using weights), or their outputs are needed in parallel as input to another ML entity. The group of ML entities needs to be employed in a coordinated way to support an AI/ML inference function.

Therefore, it is desirable that the ML models associated with these coordinated ML entities can be trained or re-trained jointly, so that the group of these ML entities can complete a more complex task jointly with better performance.

The ML entity joint training may be initiated by the MLT MnS producer or the MLT MnS consumer, with the grouping of the ML entities shared by the MLT MnS producer with the MLT MnS consumer.

##### 6.2a.1.2.7	ML entity validation performance reporting

During the ML training process, the generated ML entity needs to be validated. The purpose of ML validation is to evaluate the performance of the ML entity when performing on the validation data, and to identify the variance of the performance on the training data and the validation data. The training data and validation data are of the same pattern as they normally split from the same data set with a certain ratio in terms of quantity of the data samples.

In the ML training, the ML entity is generated based on the learning from the training data, and validated using the validation data. The performance of the ML entity has tight dependency on the data (i.e., training data) from which the ML entity is generated. Therefore, an ML entity performing well on the training data may not necessarily perform well on other data e.g., while conducting inference. If the performance of ML entity is not good enough according to the result of ML validation, the ML entity will be tuned (the model associated with it be re-trained) and validated again. The process of ML entity tuning and validation is repeated by the ML training function, until the performance of the ML entity meets the expectation on both training data and validation data. The MnS producer subsequently selects one or more ML entities with the best level of performance on both training data and validation data as the result of the ML training, and reports accordingly to the consumer. The performance of each selected ML entity on both training data and validation data also needs to be reported.

The performance result of the validation may also be impacted by the ratio of the training data and the validation data. MnS consumer needs to be aware of the ratio of training data and the validation data, coupled with the performance score on each data set, in order to be confident about the performance of ML entity.

##### 6.2a.1.2.8	Training data effectiveness reporting

Training data effectiveness refers to the process of evaluating the contribution of a single data instance or a type of input training data (e.g., one measurement type among all types of input training data) to ML model training process.

To train a ML model, high quality and large volume of training data instances are mandatory. The open use of all available data can be costly, both in terms of data collection process and from a computational resources perspective since the data also contains the unnecessary data samples that are computed through the ML model. It is better that the training function evaluates the usefulness of different data samples and indicates that usefulness to the consumer so that the data used for re-training can be further optimized.

The 3GPP management system needs to support means to report the extent of effectiveness of the different training data samples used in ML training based on insight of how the different portion of data contribute differently to the model accuracy.

#### 6.2a.1.3	Requirements for ML training

Table 6.2a.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to request ML training. | ML training requested by consumer (clause 6.2a.1.2.1) |
| REQ- ML_TRAIN-FUN-02 | The MLT MnS producer shall have a capability allowing the authorized MLT MnS consumer to specify the data sources containing the candidate training data for ML training. | ML training requested by consumer (clause 6.2a.1.2.1) |
| REQ- ML_TRAIN-FUN-03 | The MLT MnS producer shall have a capability allowing the authorized MLT MnS consumer to specify the inference type of the ML entity to be trained. | ML training requested by consumer (clause 6.2a.1.2.1) |
| REQ- ML_TRAIN-FUN-04 | The MLT MnS producer shall have a capability to provide the training result to the MLT MnS consumer. | ML training requested by consumer (clause 6.2a.1.2.1), ML training initiated by producer (clause 6.2a.1.2.2) |
| REQ- ML_TRAIN-FUN-05 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML entity. (See Note) | ML training initiated by producer (clause 6.2a.1.2.2) |
| REQ- ML_TRAIN-FUN-06 | The MLT MnS producer shall have a capability to provide the version number of the ML entity and the time when it is generated by ML re-training to the authorized MLT MnS consumer. | ML training requested by consumer (clause 6.2a.1.2.1), /ML training initiated by producer (clause 6.2a.1.2.2) |
| REQ- ML_TRAIN-FUN-07 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML training. | ML training requested by consumer (clause 6.2a.1.2.1), ML training initiated by producer (clause 6.2a.1.2.2), ML entity joint training (clause 6.2a.1.2.6) |
| REQ- ML_TRAIN-FUN-08 | The MLT MnS producer should have a capability to provide the grouping of ML entities to an authorized MLT MnS consumer to enable coordinated inference. | ML entity joint training (clause 6.2a.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The MLT MnS producer should have a capability to allow an authorized MLT MnS consumer to request joint training of a group of ML entities. | ML entity joint training (clause 6.2a.1.2.6) |
| REQ- ML_TRAIN-FUN-10 | The MLT MnS producer should have a capability to jointly train a group of ML entities and provide the training results to an authorized consumer. | ML entity joint training (clause 6.2a.1.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to discover the characteristics of available ML entitiesincluding the contexts under which each of the models associated with the entities were trained. | ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to select an ML entity. | ML models and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-03 | The MLT MnS producer shall have a capability to enable an authorized MLT MnS consumer to request for a model to be trained to satisfy the consumer's expectations. | ML training requested by consumer (clause 6.2a.2.1), ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-04 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to request for information and be informed about the available alternative ML entities of differing complexity and performance. | ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-05 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to request one of the known or available alternative models of differing complexity and performance to be used for inference. | ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-06 | The 3GPP management system shall have a capability to provide a selected ML entity to the authorized MLT MnS consumer. | ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_TRAIN- MGT-01 | The MLT MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML training, e.g. to modify the characteristics of the request or to delete a request. | ML training requested by consumer (clause 6.2a.2.1),Managing ML Training Processes (clause 6.2a.2.4) |
| REQ-ML_TRAIN- MGT-02 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training; or to adjust the training conditions and/or characteristics. | ML training requested by consumer (clause 6.2a.2.1), Managing ML training processes (clause 6.2a.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer (e.g. the function/entity different from the function that generated a request for ML training) to request for a report on the outcomes of a specific training instance. | Managing ML training processes (clause 6.2a.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML training processes (clause 6.2a.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the MLT function to report to any authorized MLT MnS consumer about specific ML training process and/or report about the outcomes of any such ML training process. | Managing ML training processes (clause 6.2a.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an MLT function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2a.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML-inference function. | Handling errors in data and ML decisions (clause 6.2a.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall enable a producer of data services (e.g. a gNB) to provide to an authorized consumer (e.g. an MLT function) a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2a.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the inference function. | Handling errors in data and ML decisions (clause 6.2a.2.5) |
| REQ-ML_VLD-01 | The MLT MnS producer should have a capability to validate the ML entities during the ML training process and report the performance of the ML entities on both the training data and validation data to the authorized consumer. | ML entity validation performance reporting (clause 6.2a.1.2.7) |
| REQ-ML_VLD-02 | The MLT MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML training and validation process. | ML entity validation performance reporting (clause 6.2a.1.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2a.1.2.8) |
| NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML entity supports) of ML entity. |  |  |

### 6.2a.2	Performance management for ML training

#### 6.2a.2.1	Description

In the ML model training phase (including training and validation), the performance of ML entity needs to be evaluated. The performance is the degree to which the ML entities fulfils the objectives for which it was trained and can be evaluated for training data as training performance or for testing data as testing performance. The related performance indicators need to be collected and analyzed.

#### 6.2a.2.2	Use cases

##### 6.2a.2.2.1	Performance indicator selection for ML training and testing

The ML model training function may support training for single or different kinds of ML models and may support the capability to evaluate each kind of ML entity by one or more performance indicators.

The MnS consumer may prefer to use some performance indicator(s) over others to evaluate one kind of ML entity. The performance indicators for training mainly include the following aspects:

-	ML training process monitor performance indicators: the performance indicators of the system that trains the ML entity, including training duration indicator.

-	ML training model performance indicators: performance indicators of the ML entity itself, including

- Accuracy indicator

- Precision indicator

- Recall indicator

- F1 score indicator

- MSE(Mean Squared Error) indicator

- MAE(Mean Absolute Error) indicator

The MnS consumer may prefer to use some performance indicator(s) over others to evaluate one kind of ML entity. The performance indicators for testing mainly include the following aspects:

-	ML testing model performance indicators: performance indicators of the ML entity itself, including:

- Accuracy indicator

- Precision indicator

- Recall indicator

- F1 score indicator

- MSE(Mean Squared Error) indicator

- MAE(Mean Absolute Error) indicator

Therefore, the MnS producer for ML training and testing needs to provide the name(s) of supported performance indicator(s) for the MnS consumer to query and select for ML entity performance evaluation. The MnS consumer may also need to provide the performance requirements of the ML entity using the selected performance indicators.

The MnS producer for ML training and testing uses the selected performance indicators for evaluating ML training and testing, and reports with the corresponding performance score in the ML training report or ML testing report when the training or testing is completed.

##### 6.2a.2.2.2	ML entity performance indicators query and selection for ML training and testing

The ML entity performance evaluation and management is needed during training and testing phase. The related performance indicators need to be collected and analyzed. The MnS producer of ML training or testing should determine which indicators are needed, i.e., select some indicators based on the use case and use these indicators for performance evaluation.

The ML MnS consumer or testing may have different requests on AI/ML performance, depending on its use case and requirements, which may imply that different performance indicators may be relevant for performance evaluation. The MnS producer for ML training/testing can be queried to provide the information on supported performance indicators referring to ML entity training/testing phase. Such performance indicators in training phase may be for example accuracy/precision/recall/F1-score/MSE/MAE, and in test phase may be data drift in data statistics.  Based on supported performance indicators in different phase as well as based on consumer’s requirements, the MnS consumer for ML training or ML testing may request a sub-set of supported performance indicators to be monitored and used for performance evaluation. Management capabilities are needed to enable the MnS consumer for ML training or ML testing to query the supported performance indicators and select a sub-set of performance indicators in training  phase to be used for performance evaluation.

##### 6.2a.2.2.3	MnS consumer policy-based selection of ML entity performance indicators for ML training and testing

ML entity performance evaluation and management is needed during ML training  phase. The related performance indicators need to be collected and analysed. The MnS producer for ML training  should determine which indicators are needed or may be reported, i.e., select some indicators based on the service and use these indicators for performance evaluation.

The MnS consumer for ML training or testing may have differentiated levels of interest in the different performance dimensions or metrics. Thus, depending on its use case, the AI/ML MnS consumer may indicate the preferred behaviour and performance requirement that needs to be considered during training or testing of/from the ML entity by the ML MnS producer for ML training or testing. These performance requirements need not indicate the technical performance indicators used for ML training, testing or inference, such as "accuracy" or "precision" or "recall" or "MSE" or "MAE" or “F1 score" etc. The ML MnS consumer for ML training or testing may not be capable enough to indicate the performance metrics to be used for training or testing.

#### 6.2a.2.3	Requirements for ML training and testing performance management

Table 6.2a.2.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN_PM-1 | The ML Training or Testing MnS producer of the 3GPP management system shall have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the ML training function or ML testing function is able to train or test. | Performance indicator selection for ML training (clause 6.2a.2.2.1) |
| REQ-ML_TRAIN_PM-2 | The ML Training or Testing MnS producer of the 3GPP management system shall have a capability to allow an authorized consumer to query what performance indicators are supported by the ML training function or ML testing function for each kind of ML entity. | Performance indicator selection for ML training (clause 6.2a.2.2.1) |
| REQ-ML_TRAIN_PM-3 | The ML Training or Testing MnS producer of the 3GPP management system shall have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function or ML testing function for reporting the training or testing performance for each kind of ML entity. | Performance indicator selection for ML training (clause 6.2a.2.2.1) |
| REQ-ML_TRAIN_PM-4 | The ML Training MnS producer of the 3GPP management system shall have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function. | Performance indicator selection for ML training (clause 6.2a.2.2.1) |

### 6.2a.3	ML testing

#### 6.2a.3.1	Description

During ML entity training phase, after the training and validation, the ML entity needs to be tested to evaluate the performance of the ML entity when it conducts inference using the testing data. Testing may involve interaction with third parties (besides the developer of the ML training function), e.g., the operators may use the ML training function or third-party systems/functions that may rely on the inference results computed by the ML entity for testing.

If the testing performance is not acceptable or does not meet the pre-defined requirements, the consumer may request the ML training producer to re-train the ML model with specific training data and/or performance requirements.

#### 6.2a.3.2	Use cases

##### 6.2a.3.2.1	Consumer-requested ML entity testing

After receiving an ML training report about a trained ML entity from the ML training MnS producer, the consumer may request the ML testing MnS producer to test the ML entity before applying it to the target inference function.

The ML testing is to conduct inference on the tested ML entity using the testing data as inference inputs and produce the inference output for each testing dataset example.

The ML testing MnS producer may be the same as or different from the ML training MnS producer.

After completing the ML testing, the ML testing MnS producer provides the testing report indicating the success or failure of the ML testing to the consumer. For a successful ML testing, the testing report contains the testing results, i.e., the inference output for each testing dataset example.

The ML testing MnS producer needs to have the capabilities to provide the services needed to enable the consumer to request testing and receive results on the testing of an ML entity.

##### 6.2a.3.2.2	Producer-initiated ML entity testing

The ML entity testing may also be initiated by the MnS producer, after the ML entity is trained and validated. A consumer (e.g., an operator) may still need to define the policies (e.g., allowed time window, maximum number of testing iterations, etc.) for the testing of a given ML entity. The consumer may pre-define performance requirements for the ML entity testing and allow the MnS producer to decide on whether re-training/validation need to be triggered. Re-training may be triggered by the testing MnS producer itself based on the performance requirements supplied by the MnS consumer.

##### 6.2a.3.2.3	Joint testing of multiple ML entities

A group of ML entities may work in a coordinated manner for complex use cases. In such cases an ML entity is just one step of the inference processes of an AI/ML inference function, with the inference outputs of an ML entity as the inputs to the next ML entity.

The group of ML entities is generated by the ML training function. The group, including all contained ML entities, needs to be tested. After the ML testing of the group, the MnS producer provides the testing results to the consumer.

NOTE:	This use case is about the ML entities testing during the training phase and is irrelevant to the testing cases that the ML entities have been deployed.

#### 6.2a.3.3	Requirements for ML testing

Table 6.2a.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TEST-1 | The ML testing MnS producer shall have a capability to allow an authorized consumer to request the testing of a specific ML entity. | Consumer-requested ML entity testing (clause 6.2a.3.2.1) |
| REQ-ML_TEST-2 | The ML testing MnS producer shall have a capability to trigger the testing of an ML entity and allow the MnS consumer to set the policy for the testing. | Producer-initiated ML entity testing (6.2a.3.2.2) |
| REQ-ML_TEST-3 | The ML testing MnS producer shall have a capability to report the performance of the ML entity when it performs inference on the testing data. | Consumer-requested ML entity testing (clause 6.2a.3.2.1), and producer-triggered ML entity testing (clause 6.2a.3.2.2) |
| REQ-ML_TEST-4 | The ML testing MnS producer shall have a capability allowing an authorized consumer to request the testing of a group of ML entities. | Joint testing of multiple ML entities (clause 6.2a.3.2.3) |

## 6.3	AI/ML emulation phase

### 6.3.1	Description

Before the ML entity is applied in the production network, the MnS inference consumer may want to receive results of inference in one or more environments that emulate (to different extents) the expected inference characteristics, in a process that may be termed as Inference emulation. The Inference emulation phase enables this.

### 6.3.2	Use cases

#### 6.3.2.1	AI/ML Inference emulation

After the ML entity is validated and tested during development, the MnS consumer may wish to receive information from an inference emulation process that indicates if the ML entity or the associated ML inference function is working correctly under certain runtime context.

The management system should have the capabilities enabling an MnS consumer:

-	to receive the results from running inference through an AI/ML inference emulation environment available at the emulation MnS producer

NOTE: the requirements shall be removed if the solution is not agreed in R18.

### 6.3.3	Requirements for Managing AI/ML Inference emulation

Table 6.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AI/ML_EMUL-1: | The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to receive reporting about the ML inference emulation. | AI/ML Inference emulation (clause 6.3.2.1) |

## 6.4	ML entity deployment phase

### 6.4.1	ML entity loading

#### 6.4.1.1	Description

ML entity loading refers to the process of making an ML entity available for use in the inference function . After a trained ML entity meets the performance criteria per the ML entity testing and optionally ML emulation, the ML entity could be loaded into the target inference function(s) in the system. The way for loading the ML entity is not in scope of the present document.

#### 6.4.1.2	Use cases

##### 6.4.1.2.1	Consumer requested ML entity loading

After a trained ML entity is tested and optionally emulated, if the performance of the ML entity meets the MnS consumer’s requirements, the MnS consumer may request to load the ML entity to one or more target inference function(s) where the ML entity will be used for conducting inference.Once the ML entity loading request is accepted, the MnS consumer (e.g., operator) needs to know the progress of the loading and needs to be able to control (e.g., cancel, suspend, resume) the loading process. For a completed ML entity loading,  the ML entity instance loaded to each target inference function needs to be manageable individually, for instance, to be activated/deactivated individually or concurrently.

##### 6.4.1.2.2	Control of producer-initiated ML entity loading

To enable more autonomous AI/ML operations, the MnS producer is allowed to load the ML entity without the consumer’s specific request.

In this case, the consumer needs to be able to set the policy for the ML loading, to make sure that ML entities loaded by the MnS producer meet the performance target. The policy could be, for example, the threshold of the testing performance of the ML entity, the threshold of the inference performance of the existing ML model, the time schedule allowed for ML entity loading, etc.

ML models are typically trained and tested to meet specific requirements for inference, addressing a specific use case or task. The network conditions may change regularly, for example, the gNB providing coverage for a specific location is scheduled to accommodate different load levels and/or patterns of services at different times of the day, or on different days in a week. A dedicated ML entity may be loaded per the policy to adapt to a specific load/traffic pattern.

##### 6.4.1.2.3	ML entity registration

After multiple iterations, there could be a large number of ML entities with different versions, deployment environments, performance levels, and functionalities. ML entity registration refers to the process of recording, tracking, controlling those trained ML entities enabling future retrieval, reproducibility, sharing and loading in the target inference functions across different environments. For example, the inference MnS consumer could recall the most applicable version dealing with a sudden changed deployment environment of the target inference function by tracking the registration information.

The MLT MnS producer should register the ML entity along with its loading information, e.g., ML entity metadata and relevant information (e.g., description, version, version date, target inference function, deployment environment, etc.).

#### 6.4.1.3	Requirements for ML entity loading

Table 6.4.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- ML_LOAD-FUN-01 | The MnS producer for ML entity loading shall have a capability allowing an authorized consumer to request to trigger the ML entity loading. | Consumer requested ML entity loading (clause 6.4.1.2.1) |
| REQ- ML_LOAD-FUN-02 | The MnS producer for ML entity loading shall have a capability allowing an authorized consumer to provide a policy for the MnS producer to trigger the ML entity loading. | Producer-initiated ML entity loading (clause 6.4.1.2.2) |
| REQ- ML_LOAD-FUN-03 | The MnS producer for ML entity loading shall be able to inform an authorized consumer about the progress of ML entity loading. | Consumer requested ML entity loading (clause 6.4.1.2.1) and Producer-initiated ML entity loading (clause 6.4.1.2.2) |
| REQ- ML_LOAD-FUN-04 | The MnS producer for ML entity loading shall have a capability allowing an authorized consumer to control the process of ML entity loading. | Consumer requested ML entity loading (clause 6.4.1.2.1) and Producer-initiated ML entity loading (clause 6.4.1.2.2) |
| REQ- ML_REG-01 | The MLT MnS producer should have a capability to register an ML entity to record the relevant information that may be used for loading. | ML entity registration (Clause 6.4.1.2.3) |
| REQ- ML_REG-02 | The MLT MnS producer should have a capability to allow an authorized consumer (e.g., an AI/ML inference function) to acquire the registration information of ML entities. | ML entity registration (Clause 6.4.1.2.3) |

## 6.5	AI/ML inference phase

### 6.5.1	AI/ML inference performance management

#### 6.5.1.1	Description

In the AI/ML inference phase, the performance of the AI/ML inference function and ML entity need to be evaluated against the MnS consumer's provided performance expectations/targets, to identify and timely fix any problem. Actions to fix any problem would be e.g., to trigger the ML re-training, ML testing, or re-deployment.

#### 6.5.1.2	Use cases

##### 6.5.1.2.1	AI/ML inference performance evaluation

In the AI/ML inference phase, the AI/ML inference function (including e.g., MDAF, NWDAF or RAN intelligence functions) uses one or more ML entities for inference to generate the AI/ML inference output. The performance of a running ML entity may degrade over time due to changes in network state, which will affect the related network performance and service. Thus, it is necessary to evaluate performance of the ML entity during the AI/ML inference process. If the inference output is executed, the network performance related to each AI/ML inference function also needs to be evaluated.

The consumer (e.g., a Network or Management function) may take some actions according to the AI/ML inference output provided by the AI/ML inference function. If the actions are taken accordingly, the network performance is expected to be optimized. Each AI/ML inference function has its specific focus and will impact the network performance from different perspectives.

The consumer may choose to not take any actions for various reasons, e.g., lacking confidence in the inference output, avoiding potential conflict with other actions or when no actions are needed or recommended at all according to the inference output.

For evaluating the performance of the AI/ML inference function and ML entity, the MnS producer responsible for ML inference performance management needs to be able to get the inference output generated by each AI/ML inference function. Then, the MnS producer can evaluate the performance based on the inference output and related network measurements (i.e., the actual output).

Depending on the performance evaluation results, some actions (e.g., deactivate the running entity, start retraining, change the running entity with a new one, etc) can be taken to avoid generating the inaccurate inference output.

To monitor the performance in the AI/ML inference phase, the MnS producer responsible for AI/ML inference performance management can perform evaluation periodically. The performance evaluation period may be determined based on the network change speed. Besides, a consumer (e.g., an operator) may wish to control and manage the performance evaluation capability. For example, the operator may configure the performance evaluation period of a specified ML entity.

##### 6.5.1.2.2	AI/ML performance measurements selection based on MnS consumer policy

Evaluation and management of the performance of an ML entity is needed during inference phase. The related performance measurements need to be collected and analysed. The MnS producer for inference should determine which measurements are needed or may be reported, i.e., select some measurements based on the service and use these measurements for performance evaluation.

The MnS consumer for inference may have differentiated levels of interest in the different performance dimensions or metrics. Thus, depending on its use case, the MnS consumer may indicate the preferred behaviour and performance requirement that needs to be considered during inference from the ML entity by the AI/ML inference MnS Producer. The AI/ML inference MnS consumer may not be capable enough to indicate the performance metrics. Instead, the AI/ML MnS consumer may indicate the requirement using a policy or guidance that reflects the preferred performance characteristics of the ML entity. Based on the indicated policy/guidance, the AI/ML MnS producer may then deduce and apply the appropriate performance indicators for inference. Management capabilities are needed to enable the MnS consumer to indicate the behavioural and performance policy/guidance that may be translated by the MnS producer into one or more technical performance measurements during inference.

#### 6.5.1.3	Requirements for AI/ML inference performance management

Table 6.5.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF_PE-01 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to get the inference output provided by an AI/ML inference function (e.g., MDAF, NWDAF or RAN intelligence function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-02 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to get the performance evaluation of an AI/ML inference output as measured by a defined set of performance metrics | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-03 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to provide feedback about an AI/ML inference output expressing the degree to which the inference output meets the consumer's expectations. | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-04 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to be informed about the executed actions that were triggered based on the inference output provided by an AI/ML inference function (e.g., MDAF, NWDAF or RAN intelligence function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-05 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to obtain the performance data related to an ML entity or an AI/ML inference function (e.g., MDAF, NWDAF or RAN intelligence function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ-AI/ML_PERF-SEL-1 | The MLT MnS producer shall have a capability allowing an authorized MnS consumer to discover supported AI/ML performance measurements related to AI/ML inference and select some of the desired measurements based on the MnS consumer’s requirements. | AI/ML performance measurements selection based on MnS consumer policy (clause 6.5.1.2.2) |
| REQ-AI/ML_PERF-POL-1 | The AI/ML MnS producer shall have a capability allowing the authorized MnS consumer to indicate a performance policy related to AI/ML inference phase. | AI/ML performance measurements selection based on MnS consumer policy (clause 6.5.1.2.2) |

### 6.5.2	AI/ML update control

#### 6.5.2.1	Description

In many cases, network conditions change makes the capabilities of the ML entity/entities decay, or at least become inappropriate for the changed conditions. In such cases, the MnS consumer should still be enabled to trigger updates, e.g., when the consumer realizes that the insight or decisions generated by the function are no longer appropriate for the observed network states, when the consumer observes the inference performance of ML entity/entities is decreasing.

The MnS consumer may request the AI/ML inference MnS producer to use an updated ML entity/entities for the inference with some specific performance requirements. This gives flexibility to the AI/ML inference MnS producer on how to address the requirements by for example getting ML entity/entities updated, which may be loading the already trained ML entity/entities or may lead to requesting to train/re-train the ML entity/entities by utilizing the ML training MnS.

#### 6.5.2.2	Use cases

##### 6.5.2.2.1	Availability of new capabilities or ML entities

Depending on their configurations, AI/ML inference functions may learn new characteristics during their utilization, e.g., if they are configured to learn through reinforcement learning or if they are configured to download new versions of their constituent ML entities. In such cases, the authorized consumer of AI/ML may wish to be informed by the AI/ML Inference MnS producer (e.g., the operator, a management function, or a network function) about their new capabilities.

##### 6.5.2.2.2	Triggering ML entity update

When the inference capabilities of AI/ML inference functions degenerate, the typical action may be to trigger re-training of the constituent ML entities. It is possible, however, that the AI/ML inference MnS producer only offers inference capabilities and is not equipped with capabilities to update, train/re-train its constituent ML entities. Nevertheless, the authorized MnS consumer may still need to request for improvements in the capabilities of the AI/ML inference function. In such cases, the authorized MnS consumer may still wish to request for an improvement and may specify in its request e.g., a new version of the ML entities, i.e., to have the ML entities updated or re-trained. The corresponding internal actions taken by the AI/ML MnS inference producer may not be necessarily known by the consumer.

The AI/ML inference MnS consumer needs to request the AI/ML inference MnS producer to update its capabilities or its constituent ML entities and the AI/ML MnS producer should respond accordingly. For example, the AI/ML inference MnS producer may download new software that supports the required updates, download from a remote server a file containing configurations and parameters to update one or more of its constituent ML entities, or it may trigger one or more remote or local AI/ML-related processes (including training/re-training, testing, etc.) needed to generate the required updates. Related notifications for update can be sent to the AI/ML inference MnS consumer to indicate the information of the update process, e.g., the update is finished successfully, the maximum time taken to complete the update is reached but the performance does not achieve the requirements, etc.

Besides, an AI/ML inference MnS consumer may wish to manage the update process(es), e.g., to define policies on how often the update may occur, suspend or restart the update or adjust the update conditions or characteristics, the requirements could include, e.g., the times when the update may be executed, the expected achievable performance for updating, the expected time taken to complete the update, etc.

#### 6.5.2.3	Requirements for AIML update control

Table 6.5.2.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_UPDATE-1 | The AI/ML Inference MnS producer should have a capability to inform an authorized MnS consumer of the availability of AI/ML capabilities or ML entities or versions thereof (e.g., as learned through a training process or as provided via a software update) and the readiness to update the AI/ML capabilities of the respective network function when triggered | Availability of new capabilities or ML entities (clause 6.5.2.2.1) |
| REQ-AIML_UPDATE-2 | The AI/ML Inference MnS producer should have a capability to inform an authorized MnS consumer of the expected performance gain if/when the AI/ML capabilities or ML entities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities | Availability of new capabilities or ML entities (clause 6.5.2.2.1) |
| REQ-AIML_UPDATE-3 | The AI/ML Inference MnS producer should have a capability to allow an authorized MnS consumer to request the AI/ML MnS producer to update its ML entities using a specific version of newly available AI/ML capabilities or ML entities or using AI/ML capabilities or ML entities with requirements (e.g., the minimum achievable performance after updating, the maximum time taken to complete the update, etc).. | Triggering ML entity update (clause 6.5.2.2.2) |
| REQ-AIML_UPDATE-4 | The AI/ML Inference MnS producer should have a capability for the AI/ML MnS producer to inform an authorized MnS consumer about of the process or outcomes related to any request for updating the AI/ML capabilities or ML entities | Triggering ML entity update (clause 6.5.2.2.2) |
| REQ-AIML_UPDATE-5 | The AI/ML Inference MnS producer should have a capability for the AI/ML MnS producer to inform an authorized MnS consumer about of the achieved performance gain following the update of the AI/ML capabilities of a network function with/to the specific newly available ML entities or set of AI/ML capabilities | Triggering ML entity update (clause 6.5.2.2.2) |
| REQ-AIML_UPDATE-6 | The AI/ML Inference MnS producer should have a capability for an authorized MnS consumer (e.g., an operator or the function/entity that generated the request for updating the AI/ML capabilities) to manage the request and subsequent process, e.g. to suspend, re-activate or cancel the request or process; or to adjust the characteristics of the capability update; or to define how often the update may occur, suspend, restart or cancel the request or to further adjust the requirements of the update. | Triggering ML entity update (clause 6.5.2.2.2) |

### 6.5.3	AI/ML inference capabilities management

#### 6.5.3.1	Description

A network or management function that applies AI/ML to accomplish specific tasks may be considered to have one or more ML entities, each having specific capabilities.

Different network functions, e.g., MDA Functions, may need to rely on existing AI/ML capabilities to accomplish the desired inference. However, the details of such ML-based solutions (i.e., which ML entities are applied and how) for accomplishing those inference functionalities is not obvious. The management services are required to identify the capabilities of the involved ML entities and to map those capabilities to the desired logic.

#### 6.5.3.2	Use cases

##### 6.5.3.2.1	Identifying capabilities of ML entities

Network functions, especially network automation functions, may need to rely on capabilities of ML entities that are not internal to those network functions to accomplish the desired automation (inference). For example, as stated in TS 28.104 [2], “An MDA Function may optionally be deployed as one or more AI/ML inference function(s) in which the relevant ML entities are used for inference per the corresponding MDA.” Similarly, owing to the differences in the kinds and complexity of intents that need to be fulfilled, an intent fulfillment solution may need to employ the capabilities of existing AI/ML inference functions to fulfill the intents. In any such case, management services are required to identify the capabilities of those existing ML entities that are employed by AI/ML inference functions.

Figure 6.5.3.2.1-1: Request and reporting on AI/ML inference capabilities

Figure 6.5.3.2.1-1 shows that the consumer may wish to obtain information about the available AI/ML inference capabilities to determine how to use them for the consumer's needs, e.g., for fulfillment of intent targets or other automation targets.

##### 6.5.3.2.2	Mapping of the capabilities of ML entities

Besides the discovery of the capabilities of ML entities, services are needed for mapping the ML entities and capabilities. In other words, instead of the consumer discovering specific capabilities, the consumer may want to know the ML entities that can be used to achieve a certain outcome. For this, the producer should be able to inform the consumer of the set of available ML entities that together achieve the consumer's automation needs.

In the case of intents for example, the complexity of the stated intents may significantly vary - from simple intents which may be fulfilled with a call to a single ML entity to complex intents that may require an intricate orchestration of multiple ML entities. For simple intents, it may be easy to map the execution logic to one or multiple ML entities. For complex intents, it may be required to employ multiple ML entities along with a corresponding functionality that manages their interrelated execution. The usage of the ML entities requires the awareness of their capabilities and interrelations.

Moreover, given the complexity of the required mapping to the multiple ML entities, services should be supported to provide the mapping of ML entities and capabilities.

#### 6.5.3.3	Requirements for AI/ML inference capabilities management

Table 6.5.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_CAP-01 | The AI/ML inference MnS Producer shall have a capability allowing an authorized MnS consumer to request the capabilities of existing ML entities available within the AI/ML inference producer. | Identifying capabilities of ML entities (clause 6.5.3.2.1) |
| REQ- ML_CAP-02 | The AI/ML inference MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML entity as a decision described as a triplet <object(s), parameters, metrics> with the entries respectively indicating: the object or object types for which the ML entity can undertake optimization or control; the configuration parameters on the stated object or object types, which the ML entity optimizes or controls to achieve the desired outcomes; and the network metrics which the ML entity optimizes through its actions. | Identifying capabilities of ML entities (clause 6.5.3.2.1) |
| REQ-ML_CAP-03 | The AI/ML inference MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML entity as an analysis described as a tuple <object(s), characteristics> with the entries respectively indicating: the object or object types for which the ML entity can undertake analysis; and the network characteristics (related to the stated object or object types) for which the ML entity produces analysis | Identifying capabilities of ML entities (clause 6.5.3.2.1) |
| REQ-ML_CAP-04 | The AI/ML inference MnS Producer shall have a capability allowing an authorized MnS consumer to request a mapping of the consumer's inference targets to the capabilities of one or more ML entities. | Mapping of the capabilities of ML entities (clause 6.5.3.2.2) |

### 6.5.4	AI/ML inference capability configuration management

#### 6.5.4.1	Description

The AI/ML inference function and the associated ML entity may need to be managed and configured to conduct inference in the 5G system to align with the consumer´s expectation, e.g., to enable the AI/ML inference function to perform inference.

The MnS producer for AI/ML inference management needs to provide a capability for configuration of the AI/ML inference function.

#### 6.5.4.2	Use cases

#### 6.5.4.2	Use cases

##### 6.5.4.2.1	Managing NG-RAN AI/ML-based distributed Network Energy Saving

An NG-RAN AI/ML-based distributed Network Energy Saving capability may use one or more ML entities to derive energy saving recommendations.

This NG-RAN AI/ML-based distributed Network Energy Saving capability needs to be managed. The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML-based Distributed Network Energy Saving function. The activation and deactivation actions for AI/ML-based Distributed Network Energy Saving conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

##### 6.5.4.2.2	Managing NG-RAN AI/ML-based distributed Mobility Optimization

An AI/ML-based distributed Mobility Optimization capability may use one or more ML entities to derive handover recommendations.

This NG-RAN AI/ML-based distributed Mobility Optimization capability needs to be managed. The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML-based Distributed Mobility Optimization function. The activation and deactivation actions for AI/ML-based Distributed Mobility Optimization conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

##### 6.5.4.2.3	Managing NG-RAN AI/ML-based distributed Load Balancing

An NG-RAN AI/ML-based distributed Load Balancing capability may use one or more ML entities to derive load balancing recommendations.

This NG-RAN AI/ML-based distributed Load Balancing capability needs to be managed. The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML-based Distributed Load balancing function. The activation and deactivation actions for AI/ML-based Distributed Load balancing conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

#### 6.5.4.3	Requirements for AI/ML inference management

Table 6.5.4.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF-01 | The MnS producer of NG-RAN AI/ML-based distributed Network Energy Saving should enable an authorized MnS consumer to request to manage the Network Energy Saving inference capability | Managing AI/ML-based for NG-RAN distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- AI/ML_INF-02 | The MnS producer of NG-RAN AI/ML-based distributed Mobility Optimization should enable an authorized MnS consumer to request to manage the Mobility Optimization inference capability | Managing AI/ML-based for NG-RAN distributed Mobility Optimization (clause 6.5.4.2.2) |
| REQ- AI/ML_INF-03 | The MnS producer of NG-RAN AI/ML-based distributed Load Balancing should enable an authorized MnS consumer to request to manage the Load Balancing inference capability | Managing AI/ML-based for NG-RAN distributed Load Balancing (clause 6.5.4.2.3) |
| REQ-AIML_ INF_ACT-1 | The MnS producer for AI/ML inference management should have a capability allowing an authorized MnS consumer to activate and deactivate an ML inference function. | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) Managing AI/ML-enabled for distributed Mobility Optimization (clause 6.5.4.2.2) Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |
| REQ-AIML_ INF_ACT-2 | The MnS producer for AI/ML inference management should have a capability to allow an authorized MnS consumer to provide the policy for activating and deactivating inference function. Note: The policies instructing the ML MnS producer on how or/and when to activate which ML capabilities. | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) Managing AI/ML-enabled for distributed Mobility Optimization (clause 6.5.4.2.2) Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |

### 6.5.5		Executing AI/ML Inference

#### 6.5.5.1	Description

TBD

#### 6.5.5.2		Use cases

##### 6.5.5.2.1		AI/ML Inference History - tracking inferences and context

For different automation requirements in specific network domain  management/automation functions (e.g., MDAS, SON) may apply ML functionality to make the appropriate inferences in different contexts. The context is the set of appropriate conditions under which the inference was made including network conditions, traffic characteristics, time of day, weather, and climate, etc. And depending on the contexts, the different inferences may have different outcomes. The inference history, which is the history of such inferences and the contexts within which they are taken, may be of interest to different consumers. The AI/ML inference history includes recommendations and insights derived by the ML Entity and the contexts, e.g., network resources, time periods, traffic conditions, etc. under which those recommendations and insights were derived.

ML Inference History Control

MLEntity

Request ML Inference History

Report on ML Inference History

ML  MnS Producer (provides Inference History )

MnS Consumer

The inferences (need to be tracked for future reference, e.g., to evaluate the appropriateness/effectiveness of the decisions for those contexts or to evaluate degradations in the ML entity's decision-making capability. For this, the network not only needs to have the required inference capabilities but needs also to have the means to track and enable usage of the history of the inferences made by the ML entity. The MnS producer, i.e., a specific AI/ML inference function should also provide the capability for AI/ML inference history Control, the means to control the process of compiling and reporting on AI/ML inference history.

Figure 6.5.5.2.1-1: Example use and control of AI/ML inference history request and reporting.

#### 6.5.5.3	Requirements for Executing AI/ML Inference

Table 6.5.5.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AI/ML_INF-HIST-01 | The MnS producer for AI/ML inference management should have a capability allowing an authorized consumer to request the inference history of a specific ML entity. | AI/ML Inference History - tracking inferences and context (clause 6.5.5.2.1) |
| REQ-AI/ML-INF-HIST-02 | The MnS producer for AI/ML inference management should have a capability enabling an authorized consumer to define the reporting characteristics (e.g., reporting period) related to a specific instance of ML inference history or the reporting thereof. | AI/ML Inference History - tracking inferences and context (clause 6.5.5.2.1) |

Next modified section

# 7	Information model definitions for AI/ML management

## 7.1	Imported and associated information entities

### 7.1.1	Imported information entities and local labels

Table 7.1.1-1

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.622 [12], IOC, Top | Top |
| 3GPP TS 28.622 [12], IOC, SubNetwork | SubNetwork |
| 3GPP TS 28.622 [12], IOC, ManagedElement | ManagedElement |
| 3GPP TS 28.622 [12], IOC, ManagedFunction | ManagedFunction |
| 3GPP TS 28.622 [12], IOC, ThresholdMonitor | ThresholdMonitor |
| 3GPP TS 28.541 [18], IOC, GNBCUCPFunction | GNBCUCPFunction |
| 3GPP TS 28.104 [2], IOC, MDAFunction | MDAFunction |
| 3GPP TS 28.622 [12], dataType, TimeWindow | TimeWindow |
| 3GPP TS 28.622 [12], dataType, GeoArea | GeoArea |
| 3GPP TS 28.622 [12], dataType, ThresholdInfo | ThresholdInfo |

### 7.1.2	Associated information entities and local labels

Table 7.1.2-1

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.104 [2], IOC, MDAFunction | MDAFunction |
| 3GPP TS 28.541 [18], IOC, NWDAFFunction | NWDAFFunction |

## 7.2a	Common information model definitions for AI/ML management

### 7.2a.1	Class diagram

#### 7.2a.1.1	Relationships

Figure 7.2a.1.1-1: Relations for common information models for AI/ML management

#### 7.2a.1.2	Inheritance

Figure 7.2a.1.2-1: Inheritance Hierarchy for common information models for AI/ML management

### 7.2a.2	Class definitions

#### 7.2a.2.1	MLEntity

##### 7.2a.2.1.1	Definition

This IOC represents the ML entity. ML model or ML entity are not subjects for standardization.

The MLEntity may contain 3 types of contexts - TrainingContext which is the context under which the MLEntity has been trained, the ExpectedRunTimeContext which is the context where an MLEntity is expected to be applied or/and the RunTimeContext which is the context where the ML entity is being applied. . It also contains a reference named retrainingEventsMonitorRef which is a pointer to ThresholdMnonitor MOI. This indicates the list of performance measurements and the corresponding thresholds that are monitored and used to identify the need for re-training by the MnS Producer. After the MLEntity MOI has been instantiated, the MnS Consumer can request MnS producer to instantiate a ThresholdMonitor MOI and update the reference in the MLEntity MOI that can be used by the MnS producer to decide on the re-training of the MLEntity. The MnS producer can be ML Training MnS producer or ML Inference MnS Producer.

##### 7.2a.2.1.2	Attributes

Table 7.2a.2.1.2 -1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- |
| mLEntityId | M | T | F | F | T |  |
| inferenceType | M | T | F | F | T |  |
| mLEntityVersion | M | T | F | F | T |  |
| expectedRunTimeContext | O | T | T | F | T |  |
| trainingContext | CM | T | F | F | T |  |
| runTimeContext | O | T | F | F | T |  |
| supportedPerformanceIndicators | O | T | F | F | T |  |
| Attribute related to role |  |  |  |  |  |
| retrainingEventsMonitorRef | O | T | T | F | T |
| sourceTrainedMLEntityRef | CM | T | F | F | T |

##### 7.2a.2.2.3	Attribute constraints

Table 7.2a.2.2.3-1

| Name | Definition |
| --- | --- |
| trainingContext Support Qualifier | Condition: The trainingContext represents the status and conditions related to training and should be added when training is completed. |
| sourceTrainedMLEntityRef Support Qualifier | Condition: The MLEntity MOI containing this attribute represents an ML entity loaded to an inference function. |

##### 7.2a.2.2.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

#### 7.2a.2.2	MLEntityRepository

##### 7.2a.2.2.1	Definition

The IOC MLEntityRepository represents the repository that contains the ML entities .

The MLEntityRepository MOI may contain one or more MLEntity(s).

##### 7.2a.2.2.2	Attributes

Table 7.a.2.2.2 -1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLRepositoryId | M | T | F | F | F |

##### 7.2a.2.2.3	Attribute constraints

None.

##### 7.2a.2.2.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

#### 7.2a.2.3	MLEntityCoordinationGroup

##### 7.2a.2.3.1	Definition

This IOC represents the group of ML entities, which can be trained and tested jointly and used to perform inference in a coordinated way.

One ML entity may have dependencies on one or more of the other ML entities of the same group.

One group is associated with at least two ML entities.

##### 7.2a.2.3.2	Attributes

Table 7.2a.2.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| memberMLEntityRefList | M | T | F | F | T |

##### 7.2a.2.3.3	Attribute constraints

None.

##### 7.2a.2.3.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

## 7.3a	Information model definitions for AI/ML operational phases

### 7.3a.1	Information model definitions for ML Training

#### 7.3a.1.1	Class diagram

##### 7.3a1.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant to ML model training. For the UML semantics, see  TS 32.156 [13].

Figure 7.3a.1.1.1-1: NRM fragment for ML training

Figure 7.3a.1.1.1-2: NRM fragment for ML testing

##### 7.3a.1.1.2	Inheritance

Figure 7.3a.1.1.2-1: Inheritance Hierarchy for ML training related NRMs

Figure 7.3a.1.1.2-2: Inheritance Hierarchy for ML testing related NRMs

#### 7.3a.1.2	Class definitions

##### 7.3a.1.2.1	MLTrainingFunction

###### 7.3a.1.2.1.1	Definition

The IOC MLTrainingFunction represents the entity that undertakes ML training and is also the container of the MLTrainingRequest IOC(s).

The entity represented by MLTrainingFunction MOI supports training of one or more MLEntity(s).

###### 7.3a.1.2.1.2	Attributes

Table 7.3a.1.2.1.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| mLEntityRepositoryRef | CM | T | F | F | T |

###### 7.3a.1.2.1.3	Attribute constraints

None.

###### 7.3a.1.2.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.1.2.2	MLTrainingRequest

###### 7.3a.1.2.2.1	Definition

The IOC MLTrainingRequest represents the ML model training request that is created by the ML training MnS consumer.

The MLTrainingRequest MOI is contained under one MLTrainingFunction MOI.

The MLTrainingRequest MOI may represent the request for initial ML training or re-training. For ML re-training,  the  MLTrainingRequest is associated to one MLEntity for re-training a single ML entity, or associated to one MLEntityCoordinationGroup for re-training a group of coordinated ML entities.

The MLTrainingRequest may have a source to identify where it is coming from, and which may be used to prioritize the training resources for different sources. The sources may be for example the network functions, operator roles, or other functional differentiations.

Each MLTrainingRequest may indicate the expectedRunTimeContext that describes the specific conditions for which the MLEntity should be trained.

In case the request is accepted, the ML training MnS producer decides when to start the ML training. Once the MnS producer decides to start the training based on the request, the ML training MnS producer instantiates one or more MLTrainingProcess MOI(s) that are responsible to perform the followings:

-	collects (more) data for training, if the training data are not available or the data are available but not sufficient for the training;

-	prepares and selects the required training data, with consideration of the consumer’s request provided candidate training data if any. The ML training MnS producer may examine the consumer's provided candidate training data and select none, some or all of them for training. In addition, the ML training MnS producer may select some other training data that are available in order to meet the consumer’s requirements for the MLentity training;

-	trains the MLEntity using the selected and prepared training data.

The MLTrainingRequest may have a requestStatus field to represent the status of the specific MLTrainingRequest:

-	The attribute values are "NOT_STARTED", "TRAINING_IN_PROGRESS", "SUSPENDED", "FINISHED", and "CANCELLED".

-	When value turns to "TRAINING_IN_PROGRESS", the ML training MnS producer instantiates one or more MLTrainingProcess MOI(s) representing the training process(es) being performed per the request and notifies the MLT MnS consumer(s) who subscribed to the notification.

When all of the training process associated to this request are completed, the value turns to "FINISHED".

###### 7.3a.1.2.2.2	Attributes

Table 7.3a.1.2.2.1-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceType | CM | T | F | F | T |
| candidateTrainingDataSource | O | T | T | F | T |
| trainingDataQualityScore | O | T | T | F | T |
| trainingRequestSource | M | T | T | F | T |
| requestStatus | M | T | F | F | T |
| expectedRuntimeContext | O | T | T | F | T |
| performanceRequirements | M | T | T | F | T |
| cancelRequest | O | T | T | F | T |
| suspendRequest | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLEntityToTrainRef | CM | T | F | F | T |
| mLEntityCoordinationGroupToTrainRef | CM | T | F | F | T |

###### 7.3a.1.2.2.3	Attribute constraints

Table 7.3a.1.2.2.3-1

| Name | Definition |
| --- | --- |
| inferenceType Support Qualifier | Condition: MLTrainingRequest MOI represents the request for initial ML training. |
| mLEntityToTrainRef Support Qualifier | Condition: MLTrainingRequest MOI represents the request for ML re-training. |
| mLEntityCoordinationGroupToTrainRef Support Qualifier | Condition: MLTrainingRequest MOI represents the request for joint training of a group of ML entities. |

###### 7.3a.1.2.2.4	Notifications

The common notifications defined in clause 7.e are valid for this IOC, without exceptions or additions.

##### 7.3a.1.2.3	MLTrainingReport

###### 7.3a.1.2.3.1	Definition

The IOC MLTrainingReport represents the ML model training report that is provided by the training MnS producer.

The MLTrainingReport MOI is contained under one MLTrainingFunction MOI.

###### 7.3.1.2.3.2	Attributes

Table 7.3a.1.2.3.2-1

| Attribute name |  | Support Qualifier |  |  | isReadable |  | isWritable |  |  |  | isInvariant |  |  | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mLEntityId |  | M |  |  | T |  | F |  |  |  | F |  |  | T |  |
| areConsumerTrainingDataUsed |  | M |  |  | T |  | F |  |  |  | F |  |  | T |  |
| usedConsumerTrainingData |  | CM |  |  | T |  | F |  |  |  | F |  |  | T |  |
| confidenceIndication |  | O |  |  | T |  | F |  |  |  | F |  |  | T |  |
| modelPerformanceTraining |  | M |  |  | T |  | F |  |  |  | F |  |  | T |  |
| modelPerformanceValidation |  | O |  | T |  |  |  | F |  | F |  |  | T |  |  |  |
| dataRatioTrainingAndValidation |  | O |  | T |  |  |  | F |  | F |  |  | T |  |  |  |
| areNewTrainingDataUsed |  | M |  |  | T |  | F |  |  |  | F |  |  | T |  |
| Attribute related to role |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| trainingRequestRef |  | CM |  |  | T |  | F |  |  |  | F |  |  | T |  |
| trainingProcessRef |  | M |  |  | T |  | F |  |  |  | F |  |  | T |  |
| lastTrainingRef |  | CM |  |  | T |  | F |  |  |  | F |  |  | T |  |
| mLEnityGeneratedRef |  | M |  |  | T |  | F |  |  |  | F |  |  | T |  |
| mLEnityCoordinationGroupGeneratedRef | CM |  | T |  |  | F |  |  | F |  |  | T |  |  |

###### 7.3a.1.2.3.3	Attribute constraints

Table 7.3a.1.2.3.3-1

| Name | Definition |
| --- | --- |
| usedConsumerTrainingData Support Qualifier | Condition: The value of areConsumerTrainingDataUsed attribute is ALL or PARTIALLY. |
| trainingRequestRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for the ML model training that was requested by the MnS consumer (via MLTrainingRequest MOI). |
| lastTrainingRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for the ML model training that was not initial training (i.e. the model has been trained before). |
| mLEnityCoordinationGroupGeneratedRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for a joint training of a group of ML entities. |

###### 7.3a.1.2.3.4	Notifications

The common notifications defined in clause 7.e are valid for this IOC, without exceptions or additions.

##### 7.3a.1.2.4	MLTrainingProcess

###### 7.3a.1.2.4.1	Definition

The IOC MLTrainingProcess represents the ML training process.

One MLTrainingProcess MOI may be instantiated for each MLTrainingRequest MOI or a set of MLTrainingRequest MOIs.

For each MLEntity under training, a MLTrainingProcess is instantiated, i.e. an MLTrainingProcess is associated with exactly one MLEntity. The MLTrainingProcess may be associated with one or more MLTrainingRequest MOI.

The MLTrainingProcess does not have to correspond to a specific MLTrainingRequest, i.e. a MLTrainingRequest does not have to be associated to a specific MLTrainingProcess. The MLTrainingProcess may be managed separately from the MLTrainingRequest MOIs, e.g. the MLTrainingRequest MOI may come from consumers which are network functions while the operator may wish to manage the MLTrainingProcess that is instantiated following the requests. Thus, the MLTrainingProcess may be associated to either one or more MLTrainingRequest MOI.

Each MLTrainingProcess instance needs to be managed differently from the related MLEntity, although the MLTrainingProcess may be associated to only one MLEntity. For example, the MLTrainingProcess may be triggered to start with a specific version of the MLEntity and multiple MLTrainingProcess instances may be triggered for different versions of the MLEntity. In either case the MLTrainingProcesse instances are still associated with the same MLEntity but are managed separately from the MLEntity.

Each MLTrainingProcess has a priority that may be used to prioritize the execution of different MLTrainingProcesse instances. By default, the priority of the MLTrainingProcess may be related in a 1:1 manner with the priority of the MLTrainingRequest for which the MLTrainingProcess is instantiated.

Each MLTrainingProcess may have one or more termination conditions used to define the points at which the MLTrainingProcess may terminate.

The "progressStatus" attribute represents the status of the ML model training and includes information the ML training MnS consumer can use to monitor the progress and results. The data type of this attribute is "ProcessMonitor" (see 3GPP TS 28.622 [12]). The following specializations are provided for this data type for the ML training process:

-	The "status" attribute values are "RUNNING", "CANCELLING", "SUSPENDED", "FINISHED", and "CANCELLED". The other values are not used.

-	The "timer" attribute is not used.

-	When the "status" is equal to "RUNNING" the "progressStateInfo" attribute shall indicate one of the following states: "COLLECTING_DATA", "PREPARING_TRAINING_DATA", "TRAINING".

-	No specifications are provided for the "resultStateInfo" attribute. Vendor specific information may be provided though.

When the training is completed with "status" equal to "FINISHED", the MLT MnS producer provides the training report, by creating an MLTrainingReport MOI, to the MLT MnS consumer.

###### 7.3a.1.2.4.2	Attributes

Table 7.3a.1.2.4.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLTrainingProcessId | M | T | T | F | T |
| priority | M | T | T | F | T |
| terminationConditions | M | T | T | F | T |
| progressStatus | M | T | F | F | T |
| cancelProcess | O | T | T | F | T |
| suspendProcess | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingReportRef | M | T | F | F | T |

###### 7.3a.1.2.4.3	Attribute constraints

Table 7.3a.1.2.4.3-1

| Name | Definition |
| --- | --- |
| trainingRequestRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for the ML model training that was requested by the training MnS consumer (via MLTrainingRequest MOI). |

###### 7.3a.1.2.4.4	Notifications

The common notifications defined in clause 7.e are valid for this IOC, without exceptions or additions.

##### 7.3a.1.2.5	MLTestingFunction

###### 7.3a.1.2.5.1	Definition

The ML entity testing may be conducted by the ML training function, or by a separate function.

In case the ML entity testing is conducted by a function separate from the ML training function, the IOC MLTestingFunction is instantiated and represents the logical function that undertakes ML entity testing.

The entity represented by MLTestingFunction MOI supports testing of one or more MLEntity(s).

###### 7.3a.1.2.5.2	Attributes

Table 7.3a.1.2.5.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLEntityIdList | M | T | F | F | F |

###### 7.3a.1.2.5.3	Attribute constraints

None.

###### 7.3a.1.2.5.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.1.2.6	MLTestingRequest

###### 7.3a.1.2.6.1	Definition

The IOC MLTestingRequest represents the ML entity testing request that is created by the ML testing MnS consumer.

The MLTestingRequest MOI is contained under one MLTestingFunction MOI or MLTrainingFunction MOI which represents the logical function that conducts the ML entity testing. Each MLTestingRequest is associated to at least one MLEntity.

In case the request is accepted, the ML testing MnS producer decides when to start the ML testing. Once the MnS producer decides to start the testing based on the request, the ML testing MnS producer:

-	collects (more) data for testing, if the testing data are not available or the data are available but not sufficient for the testing;

-	prepares and selects the required testing data;

-	tests the MLEntity by performing inference using the selected testing data, and

-	reports the performance of the MLEntity when it performs on the selected testing data.

The MLTestingRequest may have a requestStatus field to represent the status of the request:

-	The attribute values are "NOT_STARTED", "TESTING_IN_PROGRESS", "SUSPENDED", "FINISHED", and "CANCELLED".

###### 7.3a.1.2.6.2	Attributes

Table 7.3a.1.2.6.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| requestStatus | M | T | F | F | T |
| cancelRequest | O | T | T | F | T |
| suspendRequest | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLEntityToTestRef | CM | T | F | F | T |
| mLEntityCoordinationGroupToTestRef | CM | T | F | F | T |

###### 7.3a.1.2.6.3	Attribute constraints

Table 7.3a.1.2.6.3-1

| Name | Definition |
| --- | --- |
| mLEntityToTestRef Support Qualifier | Condition: The MLTestingRequest MOI represents the request for testing of a single ML entity. |
| mLEntityCoordinationGroupToTestRef Support Qualifier | Condition: The MLTestingRequest MOI represents the request for joint testing of a group of ML entities. |

###### 7.3a.1.2.6.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.1.2.7	MLTestingReport

###### 7.3a.1.2.7.1	Definition

The IOC MLTestingReport represents the ML testing report that is provided by the ML testing MnS producer.

The MLTestingReport MOI is contained under one MLTestingFunction MOI or MLTrainingFunction MOI which represents the logical function that conducts the ML entity testing.

For the joint testing of a group of ML entities, the ML testing report contains the testing results for every ML entity in the group.

###### 7.3a.1.2.7.2	Attributes

Table 7.3a.1.2.7.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| modelPerformanceTesting | M | T | F | F | T |
| mLTestingResult | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| testingRequestRef | CM | T | F | F | T |

###### 7.3a.1.2.7.3	Attribute constraints

Table 7.3a.1.2.7.3-1

| Name | Definition |
| --- | --- |
| testingRequestRef Support Qualifier | Condition: The MLTestingReport MOI represents the report for the ML model testing that was requested by the MnS consumer (via MLTestingRequest MOI). |

###### 7.3a.1.2.7.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

### 7.3a.2	Information model definitions for ML emulation Phase

#### 7.3a.2.1	Class diagram

##### 7.3a.2.1.1	Relationships

##### 7.3a.2.1.2	Inheritance

#### 7.3a.2.2	Class definitions

### 7.3a.3	Information model definitions for ML deployment phase

#### 7.3a.3.1	Class diagram

##### 7.3a.3.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant to ML deployment phase. For the UML semantics, see  TS 32.156 [13].

Figure 7.3a.3.1.1-1: NRM fragment for ML entity loading

##### 7.3a.3.1.2	Inheritance

Figure 7.3a.3.1.2-1: : Inheritance Hierarchy for ML entity loading related NRMs

#### 7.3a.3.2	Class definitions

##### 7.3a.3.2.1	MLEntityLoadingRequest

###### 7.3a.3.2.1.1	Definition

This IOC represents the ML entity loading request that is created by the MnS consumer. Using this IOC, the MnS consumer requests the MnS producer to load an ML entity to the target inference function.

This IOC has a requestStatus field to represent the status of the request:

-	The attribute value is one of  "NOT_STARTED", "LOADING_IN_PROGRESS", "SUSPENDED", "FINISHED_SUCCESS ",  FINISHED_FAILED" and "CANCELLED".

-	When value turns to "LOADING_IN_PROGRESS", the MnS producer instantiates one or more MLEntityLoadingProcess MOI(s) representing the loading process(es) being performed per the request and notifies the MnS consumer(s) who subscribed to the notification.

###### 7.3a.3.2.1.2	Attributes

Table 7.3a.3.2.1.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| requestStatus | M | T | T | F | T |
| cancelRequest | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLEntityToLoadRef | M | T | F | F | T |

###### 7.3a.3.2.1.3	Attribute constraints

None.

###### 7.3a.3.2.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.3.2.2	MLEntityLoadingPolicy

###### 7.3a.3.2.2.1	Definition

This IOC represents the ML entity loading policy set by the MnS consumer to the producer for loading an ML entity to the target inference function(s).

This IOC is used for the MnS consumer to set the conditions for the producer-initated ML entity loading. The MnS producer is only allowed to load the ML entity when all of the conditions are met.

###### 7.3a.3.2.2.2	Attributes

Table 7.3a.3.2.2.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceType | CM | T | T | F | T |
| mLEntityId | CM | T | T | F | T |
| policyForLoading | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |

###### 7.3a.3.2.2.3	Attribute constraints

Table 7.3a.3.2.2.3-1

| Name | Definition |
| --- | --- |
| inferenceType Support Qualifier | Condition: The ML entity loading policy is related to an initially trained ML entity. |
| mLEntityId Support Qualifier | Condition: The ML entity loading policy is related to a re-trained ML entity. |

###### 7.3a.3.2.2.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.3.3.3	MLEntityLoadingProcess

###### 7.3a.3.3.3.1	Definition

This IOC represents the ML entity loading process.

For the consumer requested ML entity loading, one or more  MLEntityLoadingProcess MOI(s) may be instantiated for each ML entity loading request presented by the MLEntityLoadingRequest MOI.

For the producer-initiated ML entity loading, one or more  MLEntityLoadingProcess MOI(s) may be instantiated and associated with each MLEntityLoadingPolicy MOI.

One MLEntityLoadingProcess MOI represent the ML entity loading process(es) corresponding to one or more target inference function(s).

The "progressStatus" attribute represents the status of the ML entity loading process and includes information the MnS consumer can use to monitor the progress and results. The data type of this attribute is "ProcessMonitor" (see 3GPP TS 28.622 [12]). The following specializations are provided for this data type for the ML entity loading process:

-	The "status" attribute values are "RUNNING", "CANCELLING", "SUSPENDED", "FINISHED", and "CANCELLED". The other values are not used.

-	The "timer" attribute is not used.

-	When the "status" is equal to "RUNNING" the "progressStateInfo" attribute shall indicate one of the following state: "LOADING".

-	No specifications are provided for the "resultStateInfo" attribute. Vendor specific information may be provided though.

When the loading is completed with "status" equal to "FINISHED", the MnS producer creates the MOI(s) of loaded MLEntity under each MOI of the target inference function(s).

###### 7.3a.3.3.3.2	Attributes

Table 7.3a.3.3.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| progressStatus | M | T | F | F | T |
| cancelProcess | O | T | T | F | T |
| suspendProcess | O | T | T | F | T |
| resumeProcess | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| MLEntityLoadingRequestRef | CM | T | F | F | T |
| MLEntityLoadingPolicyRef | CM | T | F | F | T |
| LoadedMLEntityRef | M | T | F | F | T |

###### 7.3a.3.3.3.3	Attribute constraints

Table 7.3a.3.3.3.3-1

| Name | Definition |
| --- | --- |
| MLEntityLoadingRequestRef Support Qualifier | Condition: The MLEntityLoadingProcess MOI is corresponding to the ML entity loading requested by the MnS consumer. |
| MLEntityLoadingPolicyRef Support Qualifier | Condition: The MLEntityLoadingProcess MOI is corresponding to the ML entity loading initiated by the MnS producer. |

###### 7.3a.3.3.3.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

### 7.3a.4	Information model definitions for ML inference phase

#### 7.3a.4.1	Class diagram

##### 7.3a.4.1.1	Relationships

Figure 7.3a.4.1.1-1: NRM fragment for ML update

Figure 7.3a.4.1.1-2: NRM fragment for intelligence capabilities

##### 7.3a.4.1.2	Inheritance

Figure 7.3a.4.1.2-1: Inheritance Hierarchy for ML update related NRMs

Figure 7.3a.4.1.2-2: Inheritance Hierarchy for intelligence capabilities

Editor note 1: the relation between AiMlESCapability, AiMlMOCapability, AiMlLBCapability and DESManagementFunction[x], DMROFunction[x], DLBOFunction[x] are FFS.

Editor note 2: the relations between AiMlInferenceCapability and MDAFunction, AnLFFunction are FFS.

#### 7.3a.4.2	Class definitions

##### 7.3a.4.2.1 	MLUpdateFunction

###### 7.3a.4.2.1.1	Definition

This IOC represents the function responsible for ML update.

The MOI of MLUpdateFunction is name-contained in an MOI of either a subnetwork, a managedFunction or a managementFunction.

The MLUpdateFunction is be associated with one or more ML entities.

The MLUpdateFunction contains one or more MLUpdateRequest(s)as well as one or more MLUpdateProcess(s), where an MLUpdateProcess is instantiated corresponding to one received MLUpdateRequest.

###### 7.3a.4.2.1.2	Attributes

The MLUpdateFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622 [12]) and the following attributes:

Table 7.3a.4.2.1.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| availMLCapabilityReport | M | T | F | F | F |
| Attributes related to Role |  |  |  |  |  |
| mLEntityRef | M | T | F | F | F |

###### 7.3a.4.2.1.3	Attribute constraints

None.

###### 7.3a.4.2.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.4.2.2 	MLUpdateRequest

###### 7.3a.4.2.2.1	Definition

This IOC represents the properties of MLUpdateRequest.

For each request to update the ML capabilities, a consumer creates a new MOI of MLUpdateRequest on the MLUpdateFunction, i.e.,  MLUpdateRequest is instantiated for each request for updating ML capabilities:

-	Each MLUpdateRequest is associated to at least one MLEntity

-	Each MLUpdateRequest may have a RequestStatus field that is used to track the status of the specific MLUpdateRequest or the associated MLUpdateProcess. The RequestStatus is updated by MnS producer when there is a change in status of the update progress. The RequestStatus is an enumeration with the values: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED

-	Each MLUpdateRequest may contain specific reporting requirements including an mLUpdateReportingPeriod that defines the time duration upon which the MnS consumer expects the ML update is reported. The reporting requirements contained in the MLUpdateRequest are mapped to an existing MLUpdateProcess instance.

-	The MLUpdateRequest may specify a performanceGainThreshold which defines the minimum performance gain that shall be achieved with the capability update. This implies that the difference in the performances between the existing capabilities and the new capabilities needs to be at least performanceGainThreshold, otherwise the new capabilities shall not be applied. A threshold of performanceGainThreshold=0% implies that the capabilities should be applied even if there is no noticeable performance gain.

-	The MLUpdateRequest may indicates the maximum time that should be taken to complete the update.

###### 7.3a.4.2.2.2	Attributes

The MLUpdateRequest IOC includes attributes inherited from Top IOC (defined in TS 28.622 [30]) and the following attributes:

Table 7.3a.4.2.2.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceGainThreshold | O | T | T | T | F |
| newCapabilityVersionId | O | T | T | T | F |
| updateTimeDeadline | O | T | T | T | F |
| requestStatus | M | T | T | F | T |
| mLUpdateReportingPeriod | O | T | T | F | T |
| Attributes related to Role |  |  |  |  |  |
| mLUpdateProcessRef | M | T | F | F | F |
| mLEntityRef | M | T | F | F | F |

###### 7.3a.4.2.2.3	Attribute constraints

None.

###### 7.3a.4.2.2.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.4.2.3	 MLUpdateProcess

###### 7.3a.4.2.3.1	Definition

This IOC represents the ML update process.

For each MLUpdateRequest to update the ML capabilities, the MLUpdateProcess is instantiated for the MLUpdateRequest unless the MLUpdateRequest is associated with an ongoing MLUpdateProcess if the MLUpdateProcess is updating the same MLEntity(s) as stated in the MLUpdateRequest i.e., the MLUpdateProcess is associated with at least one MLUpdateRequest. Relatedly, the MLUpdateProcess is associated with at least one MLEntity.

-	Each MLUpdateProcess may have a status attribute (i.e., progressStatus) used to indicate progress status of theupdate process.

-	The MLUpdateProcess has the capability of compiling and delivering reports and notifications relating to the ML update request or process.

-	Each MLUpdateProcess may have attributes specifying the ML capability update reporting characteristics (e.g. periodically, after completion, etc.).

###### 7.3a.4.2.3.2	Attributes

The MLUpdateProcess IOC includes attributes inherited from Top IOC (defined in TS 28.622 [30]) and the following attributes:

Table 7.3a.4.2.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| progressStatus | M | T | T | F | T |
| Attributes related to Role |  |  |  |  |  |
| mLEntityRef | M | T | F | F | F |
| MLUpdateRequestRef | M | T | F | F | F |
| MLUpdateReportRef | M | T | F | F | F |

###### 7.3a.4.2.3.3	Attribute constraints

None.

###### 7.3a.4.2.3.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.4.2.4	 MLUpdateReport

###### 7.3a.4.2.4.1	Definition

This IOC represents the properties of ML update report.

-	The ML update process may generate one or more MLUpdateReport(s),

-	Each MLUpdateReport is associated to one or more MLEntity(s) to indicate ML entities that have been updated.

-	The MLUpdateReport may indicate the achieved performance gain for the specific ML capability update, which is the gain in performance of the new capabilities compared with the original capabilities.

-	MLUpdateReport provides reports about MLEntity(s) or MLUpdateProcess(s) that themselves are associated with MLEntity(s) for which update is requested and/or executed. Correspondingly, both the MLUpdateRequest(s)and the MLUpdateProcess(s) are conditionally mandatory in that at least one of them must be associated with an instance of MLUpdateReport.

###### 7.3a.4.2.4.2	Attributes

Table 7.3a.4.2.4.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| UpdatedMLCapability | M | T | F | F | F |
| Attributes related to Role |  |  |  |  |  |
| mLEntityRef | M | T | F | F | F |
| mLUpdateProcessRef | M | T | F | F | F |

###### 7.3a.4.2.4.3	Attribute constraints

None.

###### 7.3a.4.2.4.4	Notifications

The notifications specified for the IOC using this <<datatype>> for its attribute(s), shall be applicable.

##### 7.3a.4.2.5 	AiMlInferenceCapability

###### 7.3a.4.2.5.1	Definition

This IOC represents the common properties of the AI/ML inference capability.

The AiMlInferenceCapability MOI may be associated with one or more MOIs that represent the functions/functionalities (Note) that use the AI/ML inference capability represented by the subject AiMlInferenceCapability MOI.

The MOI of AiMlInferenceCapability or the MOI of the IOC inheriting from the AiMlInferenceCapability IOC contains one or more MOI(s) of MLEntity.

NOTE: 	The IOCs representing the functions/functionalities (Note) that use the AI/ML inference capability include, but are not limited to MDAFunction, AnLFFunction, DMROFunction, DLBOFunction, and DESManagementFunction.

###### 7.3a.4.2.5.2	Attributes

Table 7.3a.4.2.5.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| activationStatus | M | T | T | F | T |
| managedActivationScope | O | T | T | F | T |
| Attributes related to role |  |  |  |  |  |

###### 7.3a.4.2.5.3	Attribute constraints

None.

###### 7.3a.4.2.5.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.4.2.6	AiMlESCapability

###### 7.3a.4.2.6.1	Definition

In the present document, this IOC represents the inference capability supporting the NG-RAN AI/ML-based distributed Network Energy Saving (see 3GPP TS 38.300 [16]).

###### 7.3a.4.2.6.2	Attributes

Table 7.3a.4.2.6.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |

###### 7.3a.4.2.6.3	Attribute constraints

None.

###### 7.3a.4.2.6.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.4.2.7	AiMlMOCapability

###### 7.3a.4.2.7.1	Definition

In the present document, this IOC represents the inference capability supporting the NG-RAN AI/ML-based distributed Mobility Optimization (see 3GPP TS 38.300 [16]).

###### 7.3a.4.2.7.2	Attributes

Table 7.3a.4.2.7.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |

###### 7.3a.4.2.7.3	Attribute constraints

None.

###### 7.3a.4.2.7.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.4.2.8	AiMlLBCapability

###### 7.3a.4.2.8.1	Definition

In the present document, this IOC represents the inference function supporting the NG-RAN AI/ML-based distributed Load Balancing (see 3GPP TS 38.300 [16]).

###### 7.3a.4.2.8.2	Attributes

Table 7.3a.4.2.8.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |

###### 7.3a.4.2.8.3	Attribute constraints

None.

###### 7.3a.4.2.8.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

## 7.2	Void

## 7.3	Void

## 7.4	Data type definitions

### 7.4.1	ModelPerformance <<dataType>>

#### 7.4.1.1	Definition

This data type specifies the performance of an ML entity when performing inference. The performance score is provided for each inference output.

#### 7.4.1.2	Attributes

Table 7.4.1.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceOutputName | M | T | F | F | T |
| performanceScore | M | T | F | F | T |
| performanceMetric | M | T | F | F | T |
| decisionConfidenceScore | O | T | F | F | T |

#### 7.4.1.3	Attribute constraints

None.

#### 7.4.1.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

### 7.4.2	Void

### 7.4.3	MLContext <<dataType>>

#### 7.4.3.1	Definition

The MLContext represents the status and conditions related to the MLEntity. Specially it may be one of three types of context - the ExpectedRunTimeContext, the TrainingContext and  the RunTimeContext.

#### 7.4.3.2	Attributes

Table 7.4.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceEntityRef | CM | T | F | F | F |
| dataProviderRef | M | T | F | F | F |

#### 7.4.3.3	Attribute constraints

Table 7.4.3.3-1

| Name | Definition |
| --- | --- |
| inferenceEntityRef Support Qualifier | Condition: The MLContext is used for ExpectedRunTimeContext or RunTimeContext. |

#### 7.4.3.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

### 7.4.4	SupportedPerfIndicator <<dataType>>

#### 7.4.4.1	Definition

This data type specifies a Performance indicator of an ML entity. The data type may be used to  indicate which performance indicators shall be applicable to either of training, testing or inference.

#### 7.4.4.2	Attributes

Table 7.4.4.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceIndicatorName | M | T | F | F | T |
| isSupportedForTraining | CM | T | F | F | T |
| isSupportedForTesting | CM | T | F | F | T |

#### 7.4.4.3	Attribute constraints

Table 7.4.4.3-1

| Name | Definition |
| --- | --- |
| isSupportedForTraining Support Qualifier | Condition: if the performance indicator named performanceIndicatorName is applicable for training, the isSupportedforTraining must be stated |
| isSupportedForTesting Support Qualifier | Condition: if the performance indicator named performanceIndicatorName is applicable for testing, the isSupportedForTesting must be stated |

#### 7.4.4.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

### 7.4.5	AvailMLCapabilityReport <<dataType>>

#### 7.4.5.1	Definition

This dataType represents the the report of available  ML capabilities following the update for specific ML capability(es).

-	The ML update process may generate one or more AvailMLCapabilityReport(s), which indicate to the consumer that new ML capability(es) is/are available and can be applied.

-	Each AvailMLCapabilityReport is associated to one or more MLEntity(s) and may indicate the one or more MLEntity(s) to which it applies.

-	The AvailMLCapabilityReport may include CapabilityVersions which indicate that there are multiple candidate sets of available ML capabilities with a different version number for each set.

-	The AvailMLCapabilityReport may include the expectedPerformanceGains, which provides  information on the expected performance gain if/when the ML capabilities of the respective network function are updated with/to the specific set of newly available ML capabilities.

-	associated to one or more MLEntity(s) and may indicate the one or more MLEntity(s) to which it applies.

#### 7.4.5.2	Attributes

The AvailMLCapabilityReport includes the following attributes:

Table 7.4.5.2-1

| Attribute name | Support Qualifier | isReadable |  |  | isWritable |  | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| availMLCapabilityReportID | M | T |  |  | F |  | F | F |
| mLCapabilityVersionId | M |  | T | F |  | F |  | F |
| expectedPerformanceGains | O |  | T | F |  | F |  | F |
| Attributes related to Role |  |  |  |  |  |  |  |  |
| mLEntityRef | M |  | T | F |  | F |  | F |

#### 7.4.5.3	Attribute constraints

None.

#### 7.4.5.4	Notifications

The notifications specified for the IOC using this <<datatype>> for its attribute(s), shall be applicable.

### 7.4.6	AiMlManagementPolicy <<dataType>>

#### 7.4.6.1	Definition

This data type represents the properties of a policy for AI/ML management.

#### 7.4.6.2	Attributes

Table 7.4.6.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| thresholdList | M | T | T | F | T |

#### 7.4.6.3	Attribute constraints

None.

#### 7.4.6.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

### 7.4.7	ManagedActivationScope <<choice>>

#### 7.4.7.1	Definition

This <<choice>> defines the scopes for activating or deactivating the ML Inference function. It is a choice between the scopes  parameter required for the activation or deactivation.

#### 7.4.7.2	Attributes

Table 7.4.7.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 dNList | CM | T | T | F | T |
| CHOICE_1.2 timeWindow | CM | T | T | F | T |
| CHOICE_1.3 geoPolygon | CM | T | T | F | T |

#### 7.4.7.3	Attribute constraints

Table 7.4.7.3-1

| Name | Definition |
| --- | --- |
| dNList Support Qualifier CM | Condition: if the sub scope is per list of managed elements (e.g., DN list) |
| timeWindow Support Qualifier CM | Condition: if the sub scope is per list of time window. |
| geoPolygon Support Qualifier CM | Condition: if the sub scope is per list of GeoArea. |

#### 7.4.7.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLEntityId | It identifies the ML entity. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTrainingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the ML model supports. allowedValues: the values of the MDA type (see 3GPP TS 28.104 [2]), Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related MLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingProcessRef | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| trainingReportRef | It is the DN of the MLTrainingReport MOI that represents the reports of the ML training. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication | It indicates the confidence (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityList | It describes the list of MLEntity. | type: MLEntity multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestSource | It describes the entity that requested to instantiate the MLTrainingRequest MOI. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.requestStatus | It describes the status of a particular ML training request. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | It indicates the priority of the training process. The priority may be used by the ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the MLtraining MnS producer to terminate a specific training process. allowedValues: MODEL UPDATED_IN_INFERENCE_FUNCTION, INFERENCE FUNCTION_TERMINATED, INFERENCE FUNCTION_UPGRADED, INFERENCE_CONTEXT_CHANGED. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityVersion | It indicates the version number of the ML entity. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTraining | It indicates the performance score of the ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLTrainingProcess". When the ML training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - “COLLECTING_DATA” - “PREPARING_TRAINING_DATA” - “TRAINING ” + DN of the MLEntity being trained The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an ML entity. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor's specific extensions. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an ML entity, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | Type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.cancelRequest | It indicates whether the ML training MnS consumer cancels the ML training request. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingRequest.suspendRequest | It indicates whether the ML training MnS consumer suspends the /ML training request. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.cancelProcess | It indicates whether the ML training MnS consumer cancels the ML training process. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.suspendProcess | It indicates whether the ML training MnS consumer suspends the ML training process. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef | It describes the target entities that will use the ML entity for inference. | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the ML entity e.g. for training or inference | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext | This describes the context where an MLEntity is expected to be applied or/and the RunTimeContext which is the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext | This specify the context under which the MLEntity has been trained. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext | This specifies the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTrainRef | It identifies the DN of the MLEntity requested to be trained. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEnityGeneratedRef | It identifies the DN of the MLEntity generated by the ML training. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEntityRepositoryRef | It idenfifies the DN of the MLEntityRepository. | Type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLRepositoryId | It indicates the unique ID of the ML repository. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceValidation | It indicates the performance score of the ML entity when performing on the validation data. allowedValues: N/A | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dataRatioTrainingAndValidation | It indicates the ratio (in terms of quantity of data samples) of the training data and validation data used during the training and validation process. It is represented by the percentage of the validation data samples in the total training data set (including both training data samples and validation data samples). The value is an integer reflecting the rounded number of percent * 100. allowedValues: { 0 .. 100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityIdList | It identifies a list of ML entities. allowedValues: N/A. | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| MLTestingRequest.requestStatus | It describes the status of a particular ML testing request. allowedValues: NOT_STARTED, TESTING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTestRef | It identifies the DN of the MLEntity requested to be tested. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTesting | It indicates the performance score of the ML entity when performing on the testing data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTestingResult | It provides the address where the testing result (including the inference result for each testing data example) is provided. The detailed testing result format is vendor specific. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| testingRequestRef | It identifies the DN of the MLTestingRequest MOI. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| supportedPerformanceIndicators | It identifies a specific PerformanceIndicator of an ML entity. allowedValues: N/A. | type: SupportedPerfIndicator multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| performanceIndicatorName | It indicates the identifier of the specific performance indicator. allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isSupportedForTraining | It indicates whether the specific performance indicator is supported a performance metric of ML training for the ML entity Default value is set to "FALSE". allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: False isUnique: True defaultValue: FALSE isNullable: False |
| isSupportedForTesting | It indicates whether the specific performance indicator is supported a performance metric of ML testing for the ML entity. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: False isUnique: True defaultValue: FALSE isNullable: False |
| mLUpdateProcessRef | It is the DN of the mLUpdateProcess MOI that represents the process of updating an ML entity. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateRequestRef | It is the DN of the MLUpdateRequest MOI that represents an ML update request. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportRef | It is the DN of the MLUpdateReport MOI that represents an ML update report. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportingPeriod | It specifies the time duration upon which the MnS consumer expects the ML update is reported. | Type: TimeWindow (see TS 28.622 [12]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availMLCapabilityReport | It represents the available ML capabilities. allowedValues: N/A. | Type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| UpdatedMLCapability | It represents the updated ML capabilities. allowedValues: N/A. | Type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| newCapabilityVersionId | It indicates the specific version of AI/ML capabilities to be applied for the update. It is typically the one indicated by the MLCapabilityVersionID in a newCapabilityVersion | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mlCapabilityVersionId | It indicates the version of ML capabilities that is available for the update. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceGainThreshold | It defines the minimum performance gain as a percentage that shall be achieved with the capability update, i.e., the difference in the performances between the existing capabilities and the new capabilities should be at least performanceGainThreshold otherwise the new capabilities should not be applied. Allowed value: float between 0.0 and 100.0 | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availableMLCapability | It indicates an available version of alternative ML capabilities that can be used for update. Note: MLCapability is FFS. | type: MLCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectedPerformanceGains | It indicates the expected performance gain if/when the AI/ML capabilities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities. | Type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| achievedPerformanceGain | It indicates the performance gain that is achieved by updating the AI/ML entity or network function. | Type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| updateTimeDeadline | It indicates the maximum as stated in the MLUpdate request that should be taken to complete the update | Type: TimeWindow (see TS 28.622 [12]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEntityRef | It indicates the list of references to MLEntity instances that can be updated. | Type: DN multiplicity: 1 .. * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLUpdateRequest.requestStatus | It describes the status of a particular ML update request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | Type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| memberMLEntityRefList | It identifies the list of member ML entities within a level of an ML entity coordination group. allowedValues: DN list | Type: DN (see TS 32.156 [13]) multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| mLEntityCoordinationGroupToTrainRef | It identifies the DN of the MlEntityCoordinationGroup requested to be trained. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEnityCoordinationGroupGeneratedRef | It identifies the DN of the MlEntityCoordinationGroup generated by the ML training. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEntityCoordinationGroupToTestRef | It identifies the DN of the MlEntityCoordinationGroup requested to be tested. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| retrainingEventsMonitorRef | It indicates the DN of the ThresholdMonitor MOI that indicates the performance measurements and its corresponding thresholds to be used by MnS producer to initiate the re-training of the MLEntity. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sourceTrainedMLEntityRef | It identifies the DN of the source trained MLEntity whose copy has been loaded from the ML entity repository to the inference function. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingRequest.requestStatus | It describes the status of a particular ML entity loading request. allowedValues: NOT_STARTED, LOADING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLEntityLoadingRequest.cancelRequest | It indicates whether the MnS consumer cancels the ML entity loading request. Setting this attribute to "TRUE" cancels the ML entity training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " LOADING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLEntityToLoadRef | It identifies the DN of a trained MLEntity requested to be loaded to the target inference function(s). | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| policyForLoading | It provides the policy for controlling ML entity loading triggered by the MnS producer. This policy contains two thresholds in the thresholdList attribute. The first threshold is related to the ML entity to be loaded, and the second threshold is related to the existing ML entity being used for inference. | Type: AiMlManagementPolicy multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thresholdList | It provides the list of threshold. | Type: ThresholdInfo (see TS 28.622 [12]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingProcess.cancelProcess | It indicates whether the MnS consumer cancels the ML entity loading process. Setting this attribute to "TRUE" cancels the process. Cancellation is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingProcess.suspendProcess | It indicates whether the MnS consumer suspends the ML entity loading process. Setting this attribute to "TRUE" suspends the process. Suspension is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingProcess.resumeProcess | It indicates whether the MnS consumer resumes the ML entity loading process. Setting this attribute to "TRUE" resumes the process. Suspension is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "SUSPENDED". Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingRequestRef | It identifies the DN of the associated MLEntityLoadingRequest. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingPolicyRef | It identifies the DN of the associated MLEntityLoadingPolicyRef. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| performanceMetricName | It identifies the name of the performance metric or measurement. AllowedValues: the name of the performance metrics specified for the ML training phase and AI/ML inference phase. | Type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thresholdDirection | It indicates the direction of a threshold indicating the direction for which a threshold crossing triggers a threshold. When the threshold direction is configured to "UP", the associated threshold is triggered only when the subject performance metric value is equal to or greater than the threshold value. The threshold is not triggered, when the performance metric value is smaller than the threshold value. Vice versa, When the threshold direction is configured to "DOWN", the associated threshold is triggered only when the subject performance metric value is equal or smaller than the threshold value. The threshold is not triggered, when the performance metric value s greater than the threshold value. allowedValues: - UP - DOWN | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdValue | It specifies the value against which the monitored performance metric is compared. allowedValues: float or integer | Type: Union multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| LoadedMLEntityRef | It identifies the DN of the MLEntity that has been loaded to the inference function. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| activationStatus | It describes the activation status. allowedValues: ACTIVATED, DEACTIVATED. | Type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | Type: ManagedActivationScope multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.dNList | It indicates the list of DN, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: DN multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.timeWindow | It indicates the list of time window; the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: TimeWindow (see TS 28.622 [12]) multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.geoPolygon | It indicates the list of GeoArea, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: GeoArea(see TS 28.622 [12]) multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| NOTE: When the performanceScore is to indicate the performance score for ML training, the data set is the training data set. When the performanceScore is to indicate the performance score for ML validation, the data set is the validation data set. When the performanceScore is to indicate the performance score for ML testing, the data set is the testing data set. |  |  |

### 7.5.2	Constraints

None.

## 7.6	Common notifications

### 7.6.1	Configuration notifications

This clause presents a list of notifications, defined in 3GPP TS 28.532 [11], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

Table 7.6.1-1

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

Next modified section

Annex A (informative):
PlantUML source code for NRM class diagrams

# A.1	General

This annex contains the PlantUML source code for the NRM diagrams defined in clause 7.2 of the present document.

# A.2	PlantUML code for Figure 7.3a.1.1.1-1: NRM fragment for ML model training

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

skinparam nodesep 60

class ManagedEntity <<ProxyClass>>

class MLEntity <<InformationObjectClass>>

class MLEntityCoordinationGroup <<InformationObjectClass>>

class MLTrainingFunction <<InformationObjectClass>>

class MLTrainingRequest <<InformationObjectClass>>

class MLTrainingReport <<InformationObjectClass>>

class MLTrainingProcess <<InformationObjectClass>>

class MLEntityRepository <<InformationObjectClass>>

class ThresholdMonitor <<InformationObjectClass>>

ManagedEntity "1" *-- "*" MLTrainingFunction: <<names>>

MLTrainingFunction "1" *-- "*" MLTrainingProcess: <<names>>

MLTrainingFunction "1" *-- "*" MLTrainingRequest: <<names>>

MLTrainingFunction "1" *-- "*" MLTrainingReport: <<names>>

'SubNetwork "1" *-- "*" MLEntityRepository: <<names>>

MLEntityRepository "1" *-- "*" MLEntity : <<names>>

MLTrainingFunction "1" *-- "*" ThresholdMonitor : <<names>>

MLTrainingFunction "*" -l-> "*" MLEntityRepository

MLTrainingProcess "1" <-r-> "1" MLTrainingReport

MLTrainingReport "1" --> "1" MLTrainingReport

MLTrainingProcess "*" -l-> "*" MLTrainingRequest

MLTrainingRequest "*" --> "0..1" MLEntity

MLTrainingRequest "*" --> "0..1" MLEntityCoordinationGroup

MLTrainingReport "*" --> "1" MLEntity

MLTrainingReport "*" --> "1" MLEntityCoordinationGroup

MLEntity"*" -u-> "1" ThresholdMonitor

(MLTrainingReport, MLEntity) ... (MLTrainingReport, MLEntityCoordinationGroup) : {xor}

(MLTrainingRequest, MLEntity) ... (MLTrainingRequest, MLEntityCoordinationGroup) : {xor}

note left of ManagedEntity

This represents the following IOCs:

SubNetwork or

ManagedFunction or

ManagedElement

end note

@enduml

# A.3	PlantUML code for Figure 7.3a.1.1.2-1: Inheritance Hierarchy for ML model training related NRMs

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class ManagedFunction <<InformationObjectClass>>

class MLTrainingFunction <<InformationObjectClass>>

class MLTrainingRequest <<InformationObjectClass>>

class MLTrainingProcess <<InformationObjectClass>>

class MLTrainingReport <<InformationObjectClass>>

ManagedFunction <|-- MLTrainingFunction

Top <|-- MLTrainingRequest

Top <|-- MLTrainingProcess

Top <|-- MLTrainingReport

@enduml

# A.4	PlantUML code for Figure 7.2a.1.2-1: Inheritance Hierarchy for common information models for AI/ML management

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class MLEntityRepository <<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

class MLEntityCoordinationGroup <<InformationObjectClass>>

Top <|-- MLEntityRepository

Top <|-- MLEntity

Top <|-- MLEntityCoordinationGroup

@enduml

# A.5	PlantUML code for Figure 7.2a.1.1-1: Relationships for common information models for AI/ML management

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

skinparam nodesep 60

class MLEntityRepository <<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

class MLEntityCoordinationGroup <<InformationObjectClass>>

class SubNetwork <<InformationObjectClass>>

MLEntityRepository "1" *-- "*" MLEntity: <<names>>

MLEntityRepository "1" *-- "*" MLEntityCoordinationGroup: <<names>>

SubNetwork"1" *-- "*" MLEntityRepository : <<names>>

MLEntityCoordinationGroup "*" -r-> "2..*" MLEntity

@enduml

# A.6	PlantUML code for Figure 7.3a.1.1.1-2: NRM fragment for ML entity testing

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class MLTestingEntity <<ProxyClass>>

class TestingFunction <<ProxyClass>>

class MLEntity <<InformationObjectClass>>

class MLEntityCoordinationGroup <<InformationObjectClass>>

class MLTestingFunction <<InformationObjectClass>>

class MLTestingRequest <<InformationObjectClass>>

class MLTestingReport <<InformationObjectClass>>

MLTestingEntity "1" *-- "*" MLTestingFunction: <<names>>

TestingFunction "1" *-- "*" MLTestingRequest : <<names>>

TestingFunction "1" *-- "*" MLTestingReport : <<names>>

MLTestingRequest "*" --> "0..1" MLEntity

MLTestingRequest "*" --> "0..1" MLEntityCoordinationGroup

MLTestingReport "*" -l-> "1" MLTestingRequest

(MLTestingRequest, MLEntity) ... (MLTestingRequest, MLEntityCoordinationGroup) : {xor}

note left of MLTestingEntity

Represents the following IOCs:

Subnetwork or

ManagedFunction or

ManagedElement

end note

note left of TestingFunction

Represents the following IOCs:

MLTestingFunction or

MLTrainingFunction

end note

@enduml

# A.7	PlantUML code for Figure 7.3a.1.1.2-2: Inheritance Hierarchy for ML entity testing related NRMs

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class ManagedFunction <<InformationObjectClass>>

class MLTestingFunction <<InformationObjectClass>>

class MLTestingRequest <<InformationObjectClass>>

class MLTestingReport <<InformationObjectClass>>

ManagedFunction <|-- MLTestingFunction

Top <|-- MLTestingRequest

Top <|-- MLTestingReport

@enduml

# A.8	PlantUML code for Figure 7.3a.4.1.1-1: NRM fragment for ML update

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

skinparam nodesep 60

class MLUpdateEntity <<ProxyClass>>

class MLUpdateFunction <<InformationObjectClass>>

class MLUpdateRequest <<InformationObjectClass>>

class MLUpdateProcess <<InformationObjectClass>>

class MLUpdateReport <<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

MLUpdateEntity "1" *-- "*" MLUpdateFunction:<<names>>

MLUpdateFunction "1" *-- "*" MLUpdateRequest:<<names>>

MLUpdateFunction "1" *-- "*" MLUpdateProcess:<<names>>

MLUpdateFunction "1" *-- "*" MLUpdateReport:<<names>>

MLUpdateFunction "1" --> "*" "MLEntity"

MLUpdateRequest "*" <-r-> "1" "MLUpdateProcess"

MLUpdateProcess "1" <-r-> "1" "MLUpdateReport"

MLUpdateProcess "*" --> "*" "MLEntity"

MLUpdateReport "*" --> "*" "MLEntity"

MLUpdateRequest "*" --> "*" "MLEntity"

note left of MLUpdateEntity

Represents the IOCs:

SubNetwork or

ManagedFunction or

ManagementFunction

end note

@enduml

# A.9	PlantUML code for Figure 7.3a.4.1.2-1: Inheritance Hierarchy for ML update related NRMs

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class ManagedFunction <<InformationObjectClass>>

class MLUpdateFunction <<InformationObjectClass>>

class MLUpdateRequest <<InformationObjectClass>>

class MLUpdateProcess <<InformationObjectClass>>

class MLUpdateReport <<InformationObjectClass>>

ManagedFunction <|-- MLUpdateFunction

Top <|-- MLUpdateRequest

Top <|-- MLUpdateProcess

Top <|-- MLUpdateReport

@enduml

# A.10	PlantUML code for Figure 7.3a.3.1.1-1: NRM fragment for ML entity loading

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class AiMlInferenceFunction <<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

class MLEntityLoadingRequest <<InformationObjectClass>>

class MLEntityLoadingPolicy <<InformationObjectClass>>

class MLEntityLoadingProcess <<InformationObjectClass>>

AiMlInferenceFunction "1" *-- "*" MLEntityLoadingRequest : <<names>>

AiMlInferenceFunction "1" *-- "*" MLEntityLoadingPolicy : <<names>>

AiMlInferenceFunction "1" *-- "*" MLEntityLoadingProcess : <<names>>

MLEntityLoadingRequest  "1" <-r- "*" MLEntityLoadingProcess

MLEntityLoadingProcess  "*" -r-> "1" MLEntityLoadingPolicy

MLEntityLoadingRequest "1" --> "1" MLEntity

MLEntityLoadingProcess "1" --> "1" MLEntity

AiMlInferenceFunction "1" *-- "*" MLEntity : <<names>>

(MLEntityLoadingProcess, MLEntityLoadingRequest) ... (MLEntityLoadingProcess, MLEntityLoadingPolicy) : {xor}

@enduml

# A.11	PlantUML code for Figure 7.3a.3.1.2-1: Inheritance Hierarchy for ML entity loading related NRMs

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class MLEntityLoadingRequest <<InformationObjectClass>>

class MLEntityLoadingPolicy <<InformationObjectClass>>

class MLEntityLoadingProcess <<InformationObjectClass>>

Top <|-- MLEntityLoadingRequest

Top <|-- MLEntityLoadingPolicy

Top <|-- MLEntityLoadingProcess

@enduml

# A.12	PlantUML code for Figure 7.3a.4.1.1-2: NRM fragment for intelligence capabilities

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

skinparam nodesep 60

class AiMlInferenceCapability  <<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

class ManagedEntity <<ProxyClass>>

AiMlInferenceCapability  "1" *-- "*" MLEntity : <<names>>

ManagedEntity  "1" *-- "*" AiMlInferenceCapability : <<names>>

note left of ManagedEntity #white

Represents the IOCs:

ManagedElement or

SubNetwork or

ManagedFunction

end note

@enduml

# A.13	PlantUML code for Figure 7.3a.4.1.2-2: Inheritance Hierarchy for intelligence functions

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class AiMlInferenceCapability << InformationObjectClass >>

class AiMlMOCapability <<InformationObjectClass>>

class AiMlLBCapability <<InformationObjectClass>>

class AiMlESCapability <<InformationObjectClass>>

ManagedFunction <|-- AiMlInferenceCapability

AiMlInferenceCapability <|-- AiMlMOCapability

AiMlInferenceCapability <|-- AiMlLBCapability

AiMlInferenceCapability <|-- AiMlESCapability

@enduml

Next modified section

Annex B (normative):
OpenAPI definition of the AI/ML NRM

# B.1	General

This annex contains the OpenAPI definition of the AI/ML NRM in YAML format.

The information models of the AI/ML NRM are defined in clause 7.

Mapping rules to produce the OpenAPI definition based on the information model are defined in 3GPP TS 32.160 [14].

# B.2	Solution Set (SS) definitions

## B.2.1	OpenAPI document "TS28105_AiMlNrm.yaml"

openapi: 3.0.1

info:

title: AI/ML NRM

version: 18.1.0

description: >-

OAS 3.0.1 specification of the AI/ML NRM

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.105; AI/ML Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.105/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MLContext:

type: object

properties:

inferenceEntityRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

dataProviderRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

RequestStatus:

type: string

enum:

- NOT_STARTED

- TRAINING_IN_PROGRESS

- SUSPENDED

- FINISHED

- CANCELLED

PerformanceRequirements:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

ModelPerformance:

type: object

properties:

inferenceOutputName:

type: string

performanceMetric:

type: string

performanceScore:

type: number

format: float

decisionConfidenceScore:

type: number

format: float

TrainingProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type defined in “genericNrm.yaml” with specialisations for usage in the "MLTrainingProcess".

type: object

properties:

mLTrainingProcessId:

type: string

status:

type: string

enum:

- RUNNING

- CANCELLING

- CANCELLED

- SUSPENDED

- FINSHED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

enum:

- COLLECTING_DATA

- PREPARING_TRAINING_DATA

- TRAINING

resultStateInfo:

type: string

#-------- Definition of abstract IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

MLTrainingFunction:

$ref: '#/components/schemas/MLTrainingFunction-Multiple'

MLEntityRepository:

$ref: '#/components/schemas/MLEntityRepository-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

MLTrainingFunction:

$ref: '#/components/schemas/MLTrainingFunction-Multiple'

MLEntityRepository:

$ref: '#/components/schemas/MLEntityRepository-Multiple'

MLTrainingFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

mLEntityRepositoryRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

MLTrainingRequest:

$ref: '#/components/schemas/MLTrainingRequest-Multiple'

MLTrainingProcess:

$ref: '#/components/schemas/MLTrainingProcess-Multiple'

MLTrainingReport:

$ref: '#/components/schemas/MLTrainingReport-Multiple'

MLTrainingRequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mLEntityId:

type: string

inferenceType:

type: string

candidateTrainingDataSource:

type: array

items:

type: string

trainingDataQualityScore:

type: number

format: float

trainingRequestSource:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

requestStatus:

$ref: '#/components/schemas/RequestStatus'

expectedRuntimeContext:

$ref: '#/components/schemas/MLContext'

performanceRequirements:

$ref: '#/components/schemas/PerformanceRequirements'

cancelRequest:

type: boolean

suspendRequest:

type: boolean

mLEntityToTrainRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLTrainingProcess-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mLTrainingProcessId:

type: string

priority:

type: integer

terminationConditions:

type: string

progressStatus:

$ref: '#/components/schemas/TrainingProcessMonitor'

cancelProcess:

type: boolean

suspendProcess:

type: boolean

trainingRequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

trainingReportRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLTrainingReport-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mLEntityId:

type: string

areConsumerTrainingDataUsed:

type: boolean

usedConsumerTrainingData:

type: array

items:

type: string

confidenceIndication:

type: integer

modelPerformanceTraining:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

areNewTrainingDataUsed:

type: boolean

trainingRequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

trainingProcessRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trainingReportRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

lastTrainingRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLEnityGeneratedRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLEntity-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

mLEntityId:

type: string

inferenceType:

type: string

mLEntityVersion:

type: string

expectedRunTimeContext:

$ref: '#/components/schemas/MLContext'

trainingContext:

$ref: '#/components/schemas/MLContext'

runTimeContext:

$ref: '#/components/schemas/MLContext'

retrainingEventsMonitorRef:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ThresholdMonitor-Single'

MLEntityRepository-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

mLRepositoryId:

type: string

MLEntity:

$ref: '#/components/schemas/MLEntity-Multiple'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

MLTrainingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingFunction-Single'

MLTrainingRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingRequest-Single'

MLTrainingProcess-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingProcess-Single'

MLTrainingReport-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingReport-Single'

MLEntity-Multiple:

type: array

items:

$ref: '#/components/schemas/MLEntity-Single'

MLEntityRepository-Multiple:

type: array

items:

$ref: '#/components/schemas/MLEntityRepository-Single'

#-------- Definitions in TS 28.104 for TS 28.532 ---------------------------------

resources-AiMlNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/MLTrainingFunction-Single'

- $ref: '#/components/schemas/MLTrainingRequest-Single'

- $ref: '#/components/schemas/MLTrainingProcess-Single'

- $ref: '#/components/schemas/MLTrainingReport-Single'

- $ref: '#/components/schemas/MLEntity-Single'

- $ref: '#/components/schemas/MLEntityRepository-Single'

End of modifications