3GPP TSG-SA5 Meeting #153	S5-240042

# Sevilla, Spain, 29th Jan 2024 - 2nd Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0308 | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.623 Add annex with example for advertising NRM properties |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2024-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The attribute that provides the NRM properties of the MnS producer provides addressing information. The information that MnS consumer can expect in the OpenAPI SS and YANG-Netconf SS would be different, and hence it is useful to defined this in an informative annex using an example. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces an informative Annex to describe the difference in the usage of attribute between the OpenAPI SS and YANG-Netconf SS for advertising NRM properties of the MnS producer with example. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Usage of attribute “mnsAddressForNrmProperties” of MnSInfo IOC, and the difference in OpenAPI SS and YANG SS is not described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.622 CR 0269 TS 28.623 CR 0254 TS 28.623 CR 0307 |  |  |
| Other comments: |  | CR 0269 for TS 28.622 Introduces stage 2 changes for attribute “mnsAddressForNrmProperties” introdcued to MnSInfo IOC CR 0254 for TS 28.623 Introduces stage 2 changes for attribute “mnsAddressForNrmProperties” introdcued to MnSInfo IOC CR 0307 for TS 28.623 Introduces correction to Schema definition Issues for SubNetwork of OpenAPI SS, and enhancement to “SubNetwork-Single” |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 28.622: “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	Void

[7]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[8]	W3C REC-xml11-20060816: "Extensible Markup Language (XML) 1.1 (Second Edition)".

[9]	Void.

[10]	W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures.

[11]	W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes.

[12]	W3C REC-xml-names-20060816: "Namespaces in XML 1.1 (Second Edition)".

[13]	Void

[14]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[15]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[16]	IETF RFC 8528: "YANG Schema Mount".

[17]	Management and Orchestration APIs Stage 3 Repository https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA102/

[X]	RFC8525: YANG Library

Next Change

Annex X (informative):
Examples

# X.1	Description

This annex describes additional information and examples related to the usage of object classes (IOC) and attributes. The examples are grouped by NRM fragments.

# X.2	RESTful HTTP-based solution set

## X.2.1	Description

This describes additional information and examples for RESTful HTTP-based solution set.

## X.2.2	MnS Registry NRM fragment

Attribute mnsAddressForNrmProperties of MnSInfo IOC (defined in clause 4.3.42 of TS 28.622 [4]) is used to point to the Generic NRM YAML file in the RESTful-HTTP based SS. The schema file name may be the standardized YAML file name, i.e., "TS28623_GenericNrm.yaml"or may contain vendor name along with the 3GPP standardized YAML file name, for e.g., "TS28623_GenericNrm_vendorA.yaml". MnS consumer may obtain the information related to all the managed resources supported by the MnS producer from SubNetwork-Single IOC of the Generic NRM (i.e., the file obtained from mnsAddressForNrmProperties). SubNetwork-Single IOC contains references to resources defined the same file (e.g., '#/components/schemas/SubNetwork-ncO') and references to resources defined in other files (e.g., 'TS28541_NrNrm.yaml#/components/schemas/SubNetwork-ncO-NrNrm'). The reference to resource may contain relative path to the file or an absolute path to the file. The resource defined in other files may futher refer to resources defined in the same file or other files. The MnS consumer may iteratively obtain information related to the managed resources defined in each of these files.

For example:

mnsAddressForNrmProperties = http://example.com/folder/TS28623_GenericNrm.yaml

or

mnsAddressForNrmProperties = http://example.com/folder/TS28623_GenericNrm_VendorA.yaml

# X.3	YANG/Netconf-based solution set

## X.3.1	Description

This describes additional information and examples for YANG/Netconf-based solution set.

## X.3.2	MnS Registry NRM fragment

Attribute mnsAddressForNrmProperties of MnSInfo IOC is used to point to the YANG Library (ietf-yang-library defined in RFC8525 [X]) in the YANG/Netconf-based solution set. YANG/Netconf-based solution set uses YANG Library to advertise supported IOCs, attributes and constrains or deviations, from the 3GPP defined standard NRM schema to the MnS consumers.

For example:

mnsAddressForNrmProperties = http://example.com/folder/ietf-yang-library.yang

End of change