3GPP TSG-SA5 Meeting #153	S5-240288

# Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.421 | CR | 0131 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 32.421 Add missing requirement for N41 and N42 in AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.501 defines the reference point N41 between AMF and the CHF in HPLMN and N42 between AMF and the CHF in VPLMN. But the requirement to support the interface N41 and N42 for tracing are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding N41 and N42 interfaces for tracing in AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | N41, N42 interfaces cannot be traced by 3GPP trace functionality. Misalignment with TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.422 CR 0437 TS 32.423 CR 0172 TS 28.623 CR 0324 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 5.2	Requirements for Trace data

The high level requirements for Trace data, common to both Management activation/deactivation and Signalling Based Activation/Deactivation, are as follows:

-	The Trace records have to contain Information Elements or signalling messages from control signalling and/or the characteristics of the user data. The following list contains the Network Elements and the Traceable interfaces in the NEs where tracing is needed:

-	MSC Server: A, Iu-CS, Mc and MAP (G, B, E, F, D, C) interfaces; CAP

-	MGW: Mc, Nb-UP, Iu-UP

-	HSS: MAP (C, D, Gc, Gr) Cx, S6a, S6d, Sh, N70, N71 and NU1interfaces and location and subscription information

-	EIR: MAP(F), S13, S13’, MAP (Gf)

-	SGSN: Gb, Iu-PS, Gn, MAP (Gr, Gd, Gf), CAP (Ge) Gs, S6d, S4, S3 and S13' interfaces

-	GGSN: Gn, Gi and Gmb interfaces

-	S-CSCF: Mw, Mg, Mr and Mi interfaces

-	P-CSCF: Gm and Go interfaces

-	RNC: Iu-CS, Iu-PS, Iur, Iub and Uu interfaces

-	BM-SC: Gmb interface

-	eNB/en-gNB: S1-MME, X2, Uu, F1-C, E1

-	MME: S1-MME, S3, S6a, S10, S11, S13, N26

-	Serving Gateway: S4, S5, S8, S11

-	PDN Gateway: S2a, S2b, S2c, S5, S6b, Gx, S8, SGi

-	AUSF: N12, N13

-	AMF: N1, N2, N8, N11, N12, N14, N15, N20, N22, N26, N41, N42

-	NEF: N29, N30, N33

-	NRF: N27

-	NSSF: N22, N31

-	PCF: N5, N7, N15

-	SMF: N4, N7, N10, N11, N16, N16a, N38, S5-C

-	SMSF: N20, N21

-	UDM: N8, N10, N13, N21, NU1

-	UPF: N4

-	AF: N5, N33

ng-eNB: NG-C, Xn-C, Uu

-	gNB-CU-CP: NG-C, Xn-C, Uu, F1-C, E1, X2-C

-	gNB-CU-UP: E1

-	gNB-DU: F1-C

-	A unique ID within a Trace Session shall be generated for each Trace Recording Session. This is called the Trace Recording Session Reference.

The high level requirements for Trace data applicable for Signalling Based Activation/Deactivation for Service Level Tracing for IMS, are as follows:

-	In the context of Service Level Tracing for IMS Trace records have to contain Information Elements or signalling messages or end user visible event from control signalling and/or the characteristics of the user data. The following list contains the IMS NEs and the Traceable interfaces in the NEs where tracing is needed:

-	HSS: Sh and Cx interfaces and location and subscription information;

-	S-CSCF: ISC, Mw, Mg, Mr and Mi interfaces;

-	P-CSCF: Gm and Go interfaces;

-	I-CSCF: Mw and Cx interfaces;

-	SIP AS: ISC interface;

-	MRF: Mr interfaces;

-	MGCF: Mg interfaces;

-	BGCF: Mi interfaces;

- A unique ID within a Trace Session shall be generated for each Trace Recording Session. This is called the Trace Recording Session Reference. In the context of Service Level Tracing for IMS the Trace Recording Session shall be unique across all IMS NEs within a PLMN.

Changes to existing NEs and interfaces above may be required. These changes would be dependent upon various 3GPP working groups and possibly other non-3GPP industry groups for completion of Trace Session Activation/Deactivation.

For a detailed description of NEs and interfaces above see 3GPP TS 23.002 [4].

End of Changes