3GPP TSG-SA5 Meeting #152	S5-240416

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	pCR 28.830 Add solution of performance degradation

# Document for:	Approval

Agenda Item:	6.7.3.1

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	TR 28.830 "Fault supervision evolution"

[3]	TS 28.104: "Management Data Analytics (MDA)"

[4]	TS 28.809: "Study on enhancement of Management Data Analytics (MDA)"

[5]	TS 28.532: "Generic management services"

3	Rationale

This tdoc discusses performance degradation analysis in FSEV study [1, 2].

WoP#2 of the FS_FSEV study item is as follows: “2. How fault supervision evolution supports 5G use cases, such as 5G SLS deterioration, risk prediction”. The issue of 5G SLS deterioration may be generalized into performance degradation issue.

In TS 28.532 [5], “Notification notifyThresholdCrossing” is supported for performance assurance. A MnS producer sends this notification to subscribed MnS consumers when a "ThresholdMonitor" (3GPP TS 28.622) on that MnS producer detects the threshold crossing of a monitored performance metric.

The threshold configuration is challenging considering the complex situations of different service types, capacity, radio conditions, observed user service experiences etc. MDA capability may be used to report the recommended threshold settings flexibly according to changing conditions, or learn the trend of potential errors or failures according to correlation analysis of real time performance measurements, historical data and configuration data etc. The performance degradation may be caused by many factors such as equipment error, configuration error, protocol error, congestion, radio interference, weak coverage etc.

MDA may be used for performance degradation analysis to report the threshold settings and trendIndication of the performance metrics which encounter degradation issues. The fault supervision management service producer may consider MDA analytics report for threshold configurations.

Proposal 1: It is proposed to introduce the capability of MDA assisted performance degradation analysis.

4	Detailed proposal

1st Change

5.2.2.4	Potential solution 4:  Performance degradation analysis

5.2.2.4.1	Introduction

The MDA assisted fault management may provide performance degradation analysis.

5.2.2.4.2	Description

In TS 28.532 [5], “Notification notifyThresholdCrossing” is supported for performance assurance. A MnS producer sends this notification to subscribed MnS consumers when a "ThresholdMonitor" (3GPP TS 28.622) on that MnS producer detects the threshold crossing of a monitored performance metric.

MDA may be used for performance degradation analysis to report the threshold settings and trendIndication of the performance metrics which encounter degradation issues. The fault supervision management service producer may consider MDA analytics report for threshold configurations.

A new MDA type of performance degradation analysis will be added to include these attributes. The following table shows the analytics information of the MDA performance degradation analysis.

Table 5.2.2.4.2-1: MDA analytics output for performance degradation

| Attribute Name | Description |
| --- | --- |
| perMeasIdentifier | The identifier of the performance or KPI metric; |
| perMeasName | The name of the performance measurement or KPI metric which is identified or predicted as degrade, the name refers to definitions in TS 28.552 [8] and 28.554 [9]; |
| perMeasInfo | Statistics or predictions of the concerned performance measurements or KPIs. |
| trendIndication | Provide the analytics report of the trend of the degraded performance measurements or KPIs. The fault supervision management service producer may consider the MDA analytics report for the inclusion of trendIndication. The fault supervision management service producer may include the trendIndication to its consumer as defined in TS 28.532 [5]. |
| managedObject | The object instance where the performance measurements or KPIs are applicable. |

End of change