3GPP TSG-SA5 Meeting #153	S5-240625

# Sevilla, Spain, 29th Jan 2024 - 2nd Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0517 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Addition of cell availability performance measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSOEU |  |  |  |  |  | Date: |  |  | 2024-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of a new performance measurement, i.e., cell avalilability |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the cell availability performance measurement definition to the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The lack of performance measurements definitions in one specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Begin Change

#### 5.1.3.X Cell AvailabilityNRCellDU

a) This measurement provides the amount of time that the NRCellDU part of the cell is available to provide services to UEs within a given granularity period T(mon)(as defined in 28.622 [13]).  It is computed as the amount of time that the NRCellDU is unlocked and enabled to serve UEs in each granularity period Tmon.

b) SI.

c) This measurement is obtained as: CellAvailNRCellDU =  TOSTNRCellDU=Enabled) && (ASTNRCellDU=Unlocked, where CellAvailNRCellDU is the availability of the NRCellDU, Tmon is the granularity period, TOSTNRCellDU=Enabled) && (ASTNRCellDU=Unlockedis the total amount of time that the operational state attribute(OST) and the administrative state attribute(AST) of the NRCellDU are enabled and unlocked, respectively, in a given Tmon. For each granularity period, the counter is reset to 0, and for every second that the value of the OST attribute is equal to “enabled” and the AST attribute value is equal to “unlocked”, the counter is increased by 1, otherwise the counter is not incremented. At the end of the granularity period, the CellAvailNRCellDU will be represented as the current value of the counter in a given granularity period.

d) A single value in seconds. The CellAvailNRCellDU measurement is computed as the ratio of this counter value over the corresponding Tmon.

e) CellAvailNRCellDU

f) NRCellDU

g) Valid for packet switched traffic

h) 5GS

End Change