3GPP TSG-SA5 Meeting #153 	S5-240745

# Sevilla, ES, 29 Jan 2024 – 02 Feb 2024	Revision of S5-240226

# Source:	Amdocs, Nokia, Nokia Shanghai Bell, Ericsson

# Title:	Use Supported Features

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.840

# 2	References

[1]		3GPP TR 28.840: "Study on CHF Segmentation"

# Rationale

This pCR is to propose a solution for the study topic for CHF selection by Charging Domain.

# 4	Detailed proposal

First change

### 5.4.4	Possible Solutions

#### 5.4.4.x	Solution #4.x: Use Supported Features

One possible solution for Key Issue #4b, #4c and #4d is to use the list of Supported Features and the feature negotiation mechanism described in TS 32.291 and TS 29.500.

The supportedFeatures is a string of hexadecimal characters (TS 29.571 [10] table 5.2.2-1), this means that there isn’t a real limit to the number of features that can be indicated. The current list of features is a mix of domains like IMS and edge computing, together with small additions like support of SMF charging id as a string (TS 32.291 [11] clause 6.1.8). Adding the domains as feature would make it possible to also describe the attributes that are connected to that domain.

This solution proposal is to extend the list of supported features to include additional charging domains, as indicated by solution for Key Issue #4a.

The NF Consumer can use this list of features during NRF Lookup to identify the required NF Producer (CHF).

The NF Consumer and NF Producer (CHF) can also use the feature negotiation mechanism described in TS 29.500 clause 6.6.2 to request and indicate support for a specific Charging domain.

End of changes