3GPP TSG-SA5 Meeting #153 	S5-240923d3

Seville, Spain, 29 January - 2 February 2024                                             was S5-240560

# Source:	Samsung, EUTC, EDF, Deutsche Telekom

# Title:	Rel-18 pCR 28.318 Configuration, scope adjustment

# Document for:	Approval

Agenda Item:	6.6.4.1 (NSOEU WoP#1)

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

None.

# 3	Rationale

In Rel-18, NSOEU will not support CAPIF as there remain open issues and questions. In Rel-19, FS_MExpo will consider and resolve these issues and add this support to TS 28.318. It is intended that the change to the scope and the new annex proposed by this pCR will be removed in Rel-19, replaced by standardized mechanisms.

For Rel-18, additional text is added to TS 28.318 to clarify

- exposure of management services is done directly, by means of an MNO providing a MnS producer accessible over an IP network to an authorized third party. The authorized third party communicates by means of a MnS consumer, according to the interactions specified in TS 28.318.

- authorization of the MNO by the DSO and the DSO by the MNO is necessary, but the means by which it is achieved are not standardized in Rel-18. An example is given how this could be achieved.

- security of the communication between the DSO and MNO is necessary (including integrity protection and confidentiality), but the means by which it is achieved are not standardized in Rel-18. An example is given how this could be achieved.

- configuration information is needed by the DSO in order to make use of the management services defined in TS 28.318. This information is listed, but the form of the configuration provided by the MNO to the DSO is not specified. An example is given how this could be achieved.

# 4	Detailed proposal

Changes are proposed be made to TS 28.318 v0.2.0.

changes in d1

- remove the ABNFs

- add a table for the configuration elements

- including a reference to cell ID and DN (both in the text and add to the references section)

changes in d2

- minor rewording of the text in X.4

changes in d3

- replace the word 'must be supported' with 'has to be supported

Begin Change

# 1	Scope

The present document provides normative specifications of Stage 1, stage 2, and stage 3 to realize network and service operations to support energy utility use cases by 5G networks.

The present document does not specify all aspects required to support network service operations for energy utilities. Annex X identifies unspecified configuration and functionality that are needed in order to complement the normative specification. This configuration and functionality that is not standardized has to be supported in order to deploy and operate the management services specified in the present document.

Next Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	DIRECTIVE (EU) 2019/ 944 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL - of 5 June 2019 - on common rules for the internal market for electricity and amending Directive 2012/ 27/ EU (europa.eu)
https://eur-lex.europa.eu/legal-content/EN/TXT/PDF/?uri=CELEX:32019L0944&from=EN

[3]	IEC TC 57 https://www.iec.ch/ords/f?p=103:7:511571509228708::::FSP_ORG_ID,FSP_LANG_ID:1273,25

[4]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements"

[5]	IEC "Bringing intelligence to the grid", International Electrotechnical Commission, Geneva, Switzerland, 2018.  https://www.iec.ch/basecamp/bringing-intelligence-grid <accessed: 12.7.23>

[6]	IEEE SMARTGRID, "Standards", IEEE, 2023. https://smartgrid.ieee.org/about-ieee-smart-grid/standards <accessed: 12.7.23>

[7]		Sendin, A., Stafford, J., Grilli, A., "Utilities and Telecommunications in a Nutshell", EUTC, Ediciones Experiencia, 2022.

[8]	3GPP TS 22.104, "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[9]	3GPP TS 22.261, "Service requirements for the 5G system; Stage 1"

[10]	IEC TR 61000-3-6 Electromagnetic compatibility (EMC) - Part 3-6: Limits - Assessment of emission limits for the connection of distorting installations to MV, HV and EHV power systems.

[11]	IETF RFC 1628, "UPS Management Information Base", May 1994.

[12]	ETSI ES 202 336-3: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks); Part 3: AC UPS power system control and monitoring information model".

[13]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[15]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[16]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[17]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[x]	3GPP TS 23.003: "Numbering, addressing and identification".

[y]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

Next Change

Annex X (informative):
Unspecified needed configuration and functionality

# X.1	General

The present document provides a specification for management services to support the use cases corresponding to requirements in clause 5.

Some aspects needed to support a DSO deploying a MnS consumer for functionality defined in clause 6 and annex A are not defined in the present document. These aspects are described in this annex.

# X.2	Service reachability configuration

In order for a MnS consumer deployed by a DSO to communicate with a MnS producer deployed by an MNO, the DSO needs configuration information. Specifically the network address and port number to access the MnS producer must be known.

For example, the MNO could provide the DSO with a DNS name to obtain the MnS producer service address, as well as a port number.

# X.3	Service security management

While authentication and authorization of the MNO by the DSO and the DSO by the MNO are needed, the means by which this is achieved is not specified in the present document.

Further, the communication between the MNO and DSO needs to be secure, including protecting confidentiality and the integrity of the data communicated. The means by which this is achieved is not specified in the present document.

This could be achieved for example by means of an exchange (by some secure means) of credentials between trusted parties and use of a secure communication layer such as TLS.

# X.4	Service configuration

The procedures defined in annex A require configuration information before they can be used.

For performance monitoring aspects, the MNO must provide the full DN (corresponding to each cell id) to the DSO to obtain the associated performance metrics. The correspondence between DN [y] and Cell ID [x] is, in the present document, required configuration. This format of this configuration is not specified, nor is the method by which the MNO provides it to the DSO. Not all cell ids are relevant to DSO, so the mapping will be restricted to only those that serve the DSO equipment.

Minimum configuration elements for performance monitoring are as follows:

-	DN; as specified in 3GPP TS 32.300 [y].

-	Cell ID; as specified in 3GPP TS 23.003 [x].

For coordinated energy service recovery, the DSO needs to know the list of Energy Supply IDs that correspond to critical sites of the MNO. The location is also needed, so that it is possible to identify the critical sites affected when there are energy outages that affect a particular region. The present document does not specify the form of the list of Energy Supply IDs and associated locations, or the means by which it is provided by the MNO to the DSO.

Typical configuration elements for coordinated recovery are as follows:

-	Energy Supply ID; As defined by the DSO and known to the MNO.

-	Cell ID; as specified in 3GPP TS 23.003 [x].

-	Latitude; The latitude of the site.

-	Longitude; The longitude of the site.

End Change