3GPP TSG-SA5 Meeting #154 	S5-241440

Changsha, China, 15 - 19 April 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Add Concepts and background for Exposure of Management Services in Clause 4

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1]          3GPP TR 28.879: “Study on OAM for service management and exposure to external consumers”.

# 3	Rationale

This pCR proposes to add the following changes to clause 2 and add clause 4.X to clause 4 of TR 28.879[1].

# 4	Detailed proposal

It is proposed to make the following changes in TR 28.879.

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[Y]           3GPP TS 28.537: “Management and orchestration; Management capabilities”.

Second Change

# 4 Concepts and Background

## 4.X Exposure of management services

### 4.X.1 Overview

The present document studies a generic approach to expose management services to external MnS consumers. Depending on the service-level agreements between the NOP and the external MnS consumers, the exposed management service may differ as shown in Figure 4.X.1-1. From Figure 4.X.1-1, the MnF A produces MnS A that is directly consumed by the internal MnS consumer and MnF A'. The MnF A' in turn produces two exposed MnSs, i.e., MnS B and MnS C that are consumed by the external MnS consumers 1 and 2 respectively.

Accordingly, the exposed MnSs B and C will be a subset of MnS A and might differ depending on:

- The managed entities represented by the components of the NRM that the different external MnS consumers should have access to, i.e., what parts of the MnS component type B of MnS A should each external MnS consumer have access to.

- The operations (i.e., CRUD) or notifications that the different external MnS consumers can perform on the accessible managed entities, i.e., the allowed MnS component type A of MnS A for each external MnS consumer.

- Whether or not external MnS consumers are allowed to access the performance and fault information associated with the accessible managed entities, i.e., the allowed MnS component type C of MnS A for each external MnS consumer.

Figure 4.X.1-1: Example of Exposed Management Services concept.

To provide an exposed MnS to be consumed, it first needs to be published to the MnS discovery producer entity and then the exposed MnS will be available to be discovered by the external MnS consumers. Moreover, authentication and authorization mechanisms need to be applied to ensure that the external MnS consumers only have access to the allowed MnS component type A, B, or C for the exposed MnS.

### 4.X.2 Background

A MnS can be available (i.e., instantiated, and ready to be exposed) and hence discoverable by internal and external MnS consumers. An available MnS is described by the MnsInfo IOC (see clause 4.3.42 of TS 28.622 [X]). The available MnSs in the management system can be found in the MnSRegistry IOC (see clause 4.3.41 of TS 28.622 [X]) or in the MnS discovery producer (see clause 5 in TS 28.537 [Y]) which could be the MnS registry or any other authorized management entity.

End of Changes