3GPP TSG-SA5 Meeting #154 	S5-241482

Changsha, China, 15 - 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.204 | CR | 0001 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.204 Align message content description with TR 28.826 conclusion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MATRIXX Software |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 28.286 concluded on Solution #6.10: Only Applicable Common IEs should be reflected in common part description compared to TS 32.290. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove non applicable IEs from Table 6.1.1.2-1 and Table 6.1.1.3-1 Correct clause numbering in the description for NSSAA Charging Information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear when IEs are applicable or not |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2, 6.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the NSSAAF and AMF, as used for network slice-specific authentication and authorization.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in 3GPP TS 32.290 [6] |
| Subscriber Identifier | OM | This field contains the identification of the individual subscriber in the PLMN i.e. SUPI. |
| NF Consumer Identification | M | Described in 3GPP TS 32.290 [6] and holds the identifier of the NSACF |
| NF Functionality | M | Described in 3GPP TS 32.290 [6]. |
| NF Name | OC | Described in 3GPP TS 32.290 [6]. |
| NF Address | OC | Described in 3GPP TS 32.290 [6]. |
| NF PLMN ID | OC | Described in 3GPP TS 32.290 [6]. |
| Charging Identifier | OM | Described in 3GPP TS 32.290 [6]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [6]. |
| Invocation Sequence Number | M | Described in 3GPP TS 32.290 [6]. |
| One-time Event | OC | Described in 3GPP TS 32.290 [6]. |
| One-time Event Type | OC | Described in 3GPP TS 32.290 [6]. |
| Supported Features | OC | Described in 3GPP TS 32.290 [6]. |
| Service Specification Information | OC | Described in 3GPP TS 32.290 [6]. |
| Triggers | OC | Described in 3GPP TS 32.290 [6] and holds the Network slice-specific authentication and authorization specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OM | Described in 3GPP TS 32.290 [6]. |
| Rating Group | M | Described in 3GPP TS 32.290 [6]. |
| Requested Unit | OC | Described in 3GPP TS 32.290 [6]. |
| NSSAA Charging Information | C | This field holds NSSAA specific information described in clause 6.2.1.2 |

Next change

#### 6.1.1.3	Charging data response message

Table 6.1.1.3-1 illustrates the basic structure of a Charging Data Response message from the CHF to the NSSAAF and AMF as used for network slice-specific authentication and authorization.

Table 6.1.1.3-1: Charging Data Response message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in 3GPP TS 32.290 [6]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [6]. |
| Invocation Result | OC | Described in 3GPP TS 32.290 [6]. |
| Invocation Sequence Number | OM | Described in 3GPP TS 32.290 [6]. |
| Session Failover | OC | Described in 3GPP TS 32.290 [6]. |
| Supported Features | OC | Described in 3GPP TS 32.290 [6]. |
| Multiple Unit Information | OC | Described in 3GPP TS 32.290 [6]. |
| Result Code | OC | Described in 3GPP TS 32.290 [6]. |
| Rating Group | OM | Described in 3GPP TS 32.290 [6]. |
| Validity Time | OC | Described in 3GPP TS 32.290 [6]. |

End of changes