3GPP TSG-SA5 Meeting #154	S5-241627

# Changsha, CHINA, 15 Apr - 19 Apr 2024											Revision of S5-23xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.204 | CR | 0002 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28204 Correction on trigger for NSSAA message content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSSAA_CH |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the conclusion of triggers mechanism, the triggers for IEC, PEC and ECUR is not required to be reportted to CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the triggers in NSSAA message content. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The triggers for NSSAA charging is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2,6.1.1.3,6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the NSSAAF and AMF, as used for network slice-specific authentication and authorization.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in 3GPP TS 32.290 [6] |
| Subscriber Identifier | OM | This field contains the identification of the individual subscriber in the PLMN i.e. SUPI. |
| NF Consumer Identification | M | Described in 3GPP TS 32.290 [6] and holds the identifier of the NSACF |
| NF Functionality | M | Described in 3GPP TS 32.290 [6]. |
| NF Name | OC | Described in 3GPP TS 32.290 [6]. |
| NF Address | OC | Described in 3GPP TS 32.290 [6]. |
| NF PLMN ID | OC | Described in 3GPP TS 32.290 [6]. |
| Charging Identifier | OM | Described in 3GPP TS 32.290 [6]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [6]. |
| Invocation Sequence Number | M | Described in 3GPP TS 32.290 [6]. |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in 3GPP TS 32.290 [6]. |
| One-time Event Type | OC | Described in 3GPP TS 32.290 [6]. |
| Notify URI | - | This field is not applicable. |
| Supported Features | OC | Described in 3GPP TS 32.290 [6]. |
| Service Specification Information | OC | Described in 3GPP TS 32.290 [6]. |
| Triggers | OC | This field is not applicable. |
| Multiple Unit Usage | OM | Described in 3GPP TS 32.290 [6]. |
| Rating Group | M | Described in 3GPP TS 32.290 [6]. |
| Requested Unit | OC | Described in 3GPP TS 32.290 [6]. |
| Used Unit Container | - | This field is not applicable. |
| NSSAA Charging Information | C | This field holds NSSAA specific information described in clause 6.2.1.2 |

Next change

6.1.1.3	Charging data response message

Table 6.1.1.3-1 illustrates the basic structure of a Charging Data Response message from the CHF to the NSSAAF and AMF as used for network slice-specific authentication and authorization.

Table 6.1.1.3-1: Charging Data Response message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in 3GPP TS 32.290 [6]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [6]. |
| Invocation Result | OC | Described in 3GPP TS 32.290 [6]. |
| Invocation Sequence Number | OM | Described in 3GPP TS 32.290 [6]. |
| Session Failover | OC | Described in 3GPP TS 32.290 [6]. |
| Supported Features | OC | Described in 3GPP TS 32.290 [6]. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Information | OC | Described in 3GPP TS 32.290 [6]. |
| Result Code | OC | Described in 3GPP TS 32.290 [6]. |
| Rating Group | OM | Described in 3GPP TS 32.290 [6]. |
| Granted Unit | OC | Described in 3GPP TS 32.290 [6]. |
| Validity Time | OC | Described in 3GPP TS 32.290 [6]. |
| Final Unit Indication | - | This field is not applicable. |
| Time Quota Threshold | - | This field is not applicable. |
| Volume Quota Threshold | - | This field is not applicable. |
| Unit Quota Threshold | - | This field is not applicable. |
| Quota Holding Time | - | This field is not applicable. |
| Triggers | - | This field is not applicable. |

Next change

6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by NSSAAF and AMF for Network slice-specific authentication and authorization converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.2-1 defines the basic structure of the supported fields in the Charging Data Request message for Network slice-specific authentication and authorization converged charging.

Table 6.2.2-1: Supported fields in Charging Data Request message

| Information Element | NSSAA NF | NSSAAF | AMF |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/T/E | E |
| Session Identifier |  | ITE | E |
| Subscriber Identifier |  | ITE | E |
| NF Consumer Identification |  | ITE | E |
| Charging Identifier |  | ITE | E |
| Invocation Timestamp |  | ITE | E |
| Invocation Sequence Number |  | ITE | E |
| Retransmission Indicator |  | - | - |
| One-time Event |  | --E | E |
| One-time Event Type |  | --E | E |
| Notify URI |  | - | - |
| Supported Features |  | ITE | E |
| Service Specification Information |  | ITE | E |
| Triggers |  | - | - |
| Multiple Unit Usage |  | ITE | - |
| Rating Group |  | I-E | E |
| Requested Unit |  | I-E | E |
| Used Unit Container |  | - | - |
| NSSAA Charging Information |  |  |  |
| NSSAA message type |  | ITE | E |
| User identification |  | ITE | E |
| S NSSAI |  | ITE | E |
| AAA P Address |  | ITE | E |
| AAA S Address |  | ITE | E |
| EAP ID Response |  | ITE | E |
| EAP auth status |  | ITE | E |
| AMF Identifier |  | - | E |

Table 6.2.2-2 defines the basic structure of the supported fields in the Charging Data Response message for Network slice-specific authentication and authorization converged charging.

Table 6.2.2-2: Supported fields in Charging Data Response message

| Information Element | NSSAA NF | NSSAAF | AMF |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/T/E | E |
| Session Identifier |  | ITE | E |
| Invocation Timestamp |  | ITE | E |
| Invocation Result |  | ITE | E |
| Invocation Sequence Number |  | ITE | E |
| Session Failover |  | I-- | - |
| Supported Features |  | I-E | E |
| Triggers |  | - | - |
| Multiple Unit Information |  | I-E | E |
| Result Code |  | I-E | E |
| Rating Group |  | I-E | E |
| Granted Unit |  | I-E | E |
| Validity Time |  | I-E | E |
| Final Unit Indication |  | - | - |
| Time Quota Threshold |  | - | - |
| Volume Quota Threshold |  | - | - |
| Unit Quota Threshold |  | - | - |
| Quota Holding Time |  | - | - |
| Triggers |  | - | - |

End of change