3GPP TSG-SA5 Meeting #154	S5-241771

# Changsha, China, 15 - 19 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0557 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case of the measurement “PDCCH CCE Usage” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, China Unicom, Huawei, ZTE, CATT, Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - TS28.552 has defined the evaluation of PRB Usage (see subclause 5.1.1.2), but not the usage of PDCCH. Recently, the CR to TS38.314 “Introduction of PDCCH CCE Usage for gNB Layer 2 measurement [L2M_PDCCH_Usage]” has been adopted at TSG-RAN WG2 Meeting #125, and it is necessary to introduce this measurement to TS 28.552 as an important scheme to evaluate the CCE usage for operators. - In real-life deployment, it is observed that the number of users is large while PDSCH/PUSCH PRB usage is relatively low. In such case, the PDCCH is the bottleneck channel. Introducing the usage of PDCCH is essential for both OAM performance observability and inter-cell load balancing. - Similar to PDSCH/PUSCH usage metric, multiple MIMO layers of PDCCH can be scheduled to the UE. Therefore, the introduction of PDCCH CCE Usage need to take the space division multiplexing into consideration to reflect the usage of resources of PDCCH precisely. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use case of the new measurement "PDCCH CCE Usage" to evaluate the usage of resources of PDCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is impossible to precisely reflect the usage of PDCCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

A.A	Use case of measurement for PDCCH CCE Usage

The PDCCH CCE Usage measurement could provide operators the load information on PDCCH of radio network. Under the circumstance in which the usage of PDCCH is high and the performance of the whole cell is affected, this measurement can help operators identify problems and perform network maintenance to improve user experience and cell performance.

The PDCCH CCE Usage also considers the space division multiplexing and MIMO layer. Therefore, this measurement is suitable to evaluate the usage of cell capacity with or without MIMO deployment and needs to be monitored.

<< End of change >>