3GPP TSG-SA5 Meeting #154 	S5-241772

Changsha, China, 15 - 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 1003 | rev | 1 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 32.298 Correcting generic CDR syntax |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The import of bound parameters is incorrect as well as references to corresponding versions in release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting the import of bound parameters ad updating versions for release 17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASN.1 will fail validation in a release 17 context. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.2.1, 5.2.2.2, 5.2.2.4, 5.2.4.1, 5.2.4.2, 5.2.4.4, 5.2.4.6, 5.2.4.7, and 5.2.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-241754 |  |  |  |  |  |  |  |  |

First change

### 5.2.1	Generic ASN.1 definitions

This subclause contains generic CDR syntax definitions, where the term "generic" implies that these constructs are applicable for more than one domain/service/subsystem. Examples of this are syntax definitions that are imported from non-charging 3GPP TSs, e.g. TS 29.002 [214].

.$GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AddressString,

ISDN-AddressString,

LCSClientExternalID,

LCSClientInternalID

FROM MAP-CommonDataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version20 (20)}

-- from TS 29.002 [214]

PositionMethodFailure-Diagnostic,

UnauthorizedLCSClient-Diagnostic

FROM MAP-ER-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-ER-DataTypes (17) version20 (20)}

-- from TS 29.002 [214]

ObjectInstance

FROM CMIP-1 {joint-iso-itu-t ms (9) cmip (1) modules (0) protocol (3)}

-- from Rec. X.711 [304]

ManagementExtension

FROM Attribute-ASN1Module {joint-iso-itu-t ms (9) smi (3) part2 (2) asn1Module (2) 1}

-- from Rec. X.721 [305]

AE-title

FROM ACSE-1 {joint-iso-itu-t association-control (2) modules (0) apdus (0) version1 (1) };

-- Note that the syntax of AE-title to be used is from

-- ITU-T Rec. X.227[306) / ISO 8650 corrigendum and not "ANY"

--

--  Generic Data Types

--

--

#### -- B

--

BCDDirectoryNumber		::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- a directory number e.g. calling/called/connected/translated number.

-- The encoding of the octet string is in accordance with the

-- the elements "Calling party BCD number", "Called party BCD number"

-- and "Connected number" defined in TS 24.008 [208].

-- This encoding includes type of number and number plan information

-- together with a BCD encoded digit string.

-- It may also contain both a presentation and screening indicator

-- (octet 3a).

-- For the avoidance of doubt, this field does not include

-- octets 1 and 2, the element name and length, as this would be

-- redundant.

--

--

#### -- C

--

CallDuration 			::= INTEGER

--

-- The call duration is counted in seconds.

-- For successful calls /sessions / PDP contexts, this is the chargeable duration.

-- For call attempts this is the call holding time.

--

CalledNumber			::= BCDDirectoryNumber

CallingNumber	::= BCDDirectoryNumber

CellId	::= OCTET STRING (SIZE(2))

--

-- Coded according to TS 24.008 [208]

--

ChargeIndicator			::= INTEGER

{

noCharge			(0),

charge				(1)

}

CauseForRecClosing	::= INTEGER

--

-- Cause codes 0 to 15 are defined 'CauseForTerm' (cause for termination)

-- There is no direct correlation between these two types.

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214]

--

-- In PGW-CDR and SGW-CDR the value servingNodeChange is used for partial record

-- generation due to Serving Node Address list Overflow

-- In SGSN servingNodeChange indicates the SGSN change

--

-- sWGChange value is used in both the S-GW, TWAG and ePDG for inter serving node change

--

{

normalRelease					(0),

partialRecord					(1),

abnormalRelease					(4),

cAMELInitCallRelease			(5),

volumeLimit						(16),

timeLimit						(17),

servingNodeChange				(18),

maxChangeCond					(19),

managementIntervention			(20),

intraSGSNIntersystemChange		(21),

rATChange						(22),

mSTimeZoneChange				(23),

sGSNPLMNIDChange 				(24),

sGWChange						(25),

aPNAMBRChange					(26),

mOExceptionDataCounterReceipt	(27),

unauthorizedRequestingNetwork	(52),

unauthorizedLCSClient			(53),

positionMethodFailure			(54),

unknownOrUnreachableLCSClient	(58),

listofDownstreamNodeChange		(59)

}

CauseForTerm			::= INTEGER

--

-- Cause codes from 16 up to 31 are defined as 'CauseForRecClosing'

-- (cause for record closing).

-- There is no direct correlation between these two types.

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214].

--

{

normalRelease						(0),

partialRecord						(1),

partialRecordCallReestablishment	(2),

unsuccessfulCallAttempt				(3),

abnormalRelease						(4),

cAMELInitCallRelease				(5),

unauthorizedRequestingNetwork		(52),

unauthorizedLCSClient				(53),

positionMethodFailure				(54),

unknownOrUnreachableLCSClient		(58)

}

ChargingID	::= INTEGER (0..4294967295)

--

-- Generated in P-GW, part of IP-CAN bearer

-- 0..4294967295 is equivalent to 0..2**32-1

--

CivicAddressInformation		::= OCTET STRING

--

-- as defined in subclause 3.1 of IETF RFC 4776 [409] excluding the first 3 octets.

--

CNIPMulticastDistribution		::= ENUMERATED

{

nO-IP-MULTICAST				(0),

iP-MULTICAST				(1)

}

--

#### -- D

--

DataVolumeOctets		::= INTEGER

--

-- The volume of data transferred in octets.

--

DynamicAddressFlag	::= BOOLEAN

Diagnostics						::= CHOICE

{

gsm0408Cause								[0] INTEGER,

-- See TS 24.008 [208]

gsm0902MapErrorValue						[1] INTEGER,

--

-- Note: The value to be stored here corresponds to the local values defined in the MAP-Errors

-- and MAP-DialogueInformation modules, for full details see TS 29.002 [214].

--

itu-tQ767Cause							[2] INTEGER,

-- See Q.767 [309]

networkSpecificCause						[3] ManagementExtension,

-- To be defined by network operator

manufacturerSpecificCause				[4] ManagementExtension,

-- To be defined by manufacturer

-- May be used for CHF generated diagnostics

positionMethodFailureCause				[5] PositionMethodFailure-Diagnostic,

-- See TS 29.002 [214]

unauthorizedLCSClientCause				[6] UnauthorizedLCSClient-Diagnostic,

-- See TS 29.002 [214]

diameterResultCodeAndExperimentalResult	[7] INTEGER

-- See TS 29.338 [230], TS 29.337 [231], TS 29.128 [244]

-- May be used for Nchf received diagnostics

}

DiameterIdentity		::= OCTET STRING

--

#### -- E

--

Ecgi	::= SEQUENCE

{

plmnId					[0] PLMN-Id,

eutraCellId			[1] EutraCellId,

nid						[2] Nid OPTIONAL

}

EnhancedDiagnostics					::= SEQUENCE

{

rANNASCause						[0] SEQUENCE OF RANNASCause

}

EutraCellId		::= UTF8String

--

-- See 3GPP TS 29.571 [249] for details

--

--

#### -- G

--

GSNAddress	::= IPAddress

--

#### -- I

--

InvolvedParty ::= CHOICE

{

sIP-URI		[0] GraphicString, -- refer to rfc3261 [401]

tEL-URI		[1] GraphicString,	-- refer to rfc3966 [402]

uRN			[2] GraphicString,	-- refer to rfc5031 [407]

iSDN-E164 	[3] GraphicString,	-- refer to ITU-T Recommendation E.164[308]

externalId 	[4] UTF8String		-- refer to clause 19.7.2 TS 23.003 [200]

}

IPAddress	::= CHOICE

{

iPBinaryAddress				IPBinaryAddress,

iPTextRepresentedAddress	IPTextRepresentedAddress

}

IPBinaryAddress	::= CHOICE

{

iPBinV4Address			[0] IPBinV4Address,

iPBinV6Address			    IPBinV6AddressWithOrWithoutPrefixLength

}

IPBinV4Address	::= OCTET STRING (SIZE(4))

IPBinV6Address	::= OCTET STRING (SIZE(16))

IPBinV6AddressWithOrWithoutPrefixLength ::= CHOICE

{

iPBinV6Address				[1] IPBinV6Address,

iPBinV6AddressWithPrefix	[4] IPBinV6AddressWithPrefixLength

}

IPBinV6AddressWithPrefixLength ::= SEQUENCE

{

iPBinV6Address				IPBinV6Address,

pDPAddressPrefixLength		PDPAddressPrefixLength DEFAULT 64

}

IPTextRepresentedAddress	::= CHOICE

{	--

-- IPv4 address are formatted in the "dotted decimal" notation according to IETF RFC 1166 [411].

-- IPv6 address are formatted according to clause 4 of IETF RFC 5952 [412]. The mixed IPv4 IPv6

-- notation according to clause 5 of IETF RFC 5952 [412] is not used.

-- IPv6 address prefix are formatted in the "/" notation and according to clause 4 of

-- IETF RFC 5952 [412].

--

iPTextV4Address		[2] IA5String (SIZE(7..15)),

iPTextV6Address		[3] IA5String (SIZE(2..45))

}

--

#### -- L

--

LCSCause				::= OCTET STRING (SIZE(1))

--

-- See LCS Cause Value, TS 49.031 [227]

--

LCSClientIdentity 		::= SEQUENCE

{

lcsClientExternalID	[0] LCSClientExternalID OPTIONAL,

lcsClientDialedByMS	[1] AddressString OPTIONAL,

lcsClientInternalID	[2] LCSClientInternalID OPTIONAL

}

LCSQoSInfo				::= OCTET STRING (SIZE(4))

--

-- See LCS QoS IE, TS 49.031 [227]

--

LevelOfCAMELService		::= BIT STRING

{

basic						(0),

callDurationSupervision		(1),

onlineCharging				(2)

}

LocalSequenceNumber ::= INTEGER (0..4294967295)

--

-- Sequence number of the record in this node

-- 0.. 4294967295 is equivalent to 0..2**32-1, unsigned integer in four octets

--

LocationAreaAndCell		::= SEQUENCE

{

locationAreaCode	[0] LocationAreaCode,

cellId				[1] CellId,

mCC-MNC				[2] MCC-MNC OPTIONAL

}

LocationAreaCode		::= OCTET STRING (SIZE(2))

--

-- See TS 24.008 [208]

--

--

#### -- M

--

ManagementExtensions	::= SET OF ManagementExtension

MBMS2G3GIndicator		::= ENUMERATED

{

twoG				(0),	-- For GERAN access only

threeG			(1),	-- For UTRAN access only

twoG-AND-threeG	(2) 	-- For both UTRAN and GERAN access

}

MBMSInformation			::= SET

{

tMGI							[1] TMGI OPTIONAL,

mBMSSessionIdentity			[2] MBMSSessionIdentity OPTIONAL,

mBMSServiceType				[3] MBMSServiceType OPTIONAL,

mBMSUserServiceType			[4] MBMSUserServiceType OPTIONAL, -- only supported in the BM-SC

mBMS2G3GIndicator			[5] MBMS2G3GIndicator OPTIONAL,

fileRepairSupported			[6] BOOLEAN OPTIONAL,			  -- only supported in the BM-SC

rAI							[7] RoutingAreaCode OPTIONAL,	  -- only supported in the BM-SC

mBMSServiceArea				[8] MBMSServiceArea OPTIONAL,

requiredMBMSBearerCaps		[9] RequiredMBMSBearerCapabilities OPTIONAL,

mBMSGWAddress				[10] GSNAddress OPTIONAL,

cNIPMulticastDistribution	[11] CNIPMulticastDistribution OPTIONAL,

mBMSDataTransferStart		[12] MBMSTime OPTIONAL,

mBMSDataTransferStop			[13] MBMSTime OPTIONAL

}

MBMSServiceArea		::= OCTET STRING

MBMSServiceType		::= ENUMERATED

{

mULTICAST		(0),

bROADCAST		(1)

}

MBMSSessionIdentity		::= OCTET STRING (SIZE (1))

--

-- This octet string is a 1:1 copy of the contents of the MBMS-Session-Identity

-- AVP specified in TS 29.061 [82]

--

MBMSTime	::= OCTET STRING (SIZE (8))

--

-- This value indicates the time in seconds relative to 00:00:00 on 1 January 1900 (calculated as

-- continuous time without leap seconds and traceable to a common time reference) where binary

-- encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in

-- the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second as

-- specified in TS 29.061 [82].

--

MBMSUserServiceType		::= ENUMERATED

{

dOWNLOAD		(0),

sTREAMING	(1)

}

MCC-MNC		::= OCTET STRING (SIZE(3))

--

-- See TS 24.008 [208]

--

MessageClass		::= ENUMERATED

{

personal			(0),

advertisement		(1),

information-service	(2),

auto				(3)

}

MessageReference		::= OCTET STRING

--

-- The default value shall be one octet set to 0

--

MSCAddress		::= AddressString

MscNo					::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

MSISDN					::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

MSTimeZone	::= OCTET STRING (SIZE (2))

--

-- 1. Octet: Time Zone and 2. Octet: Daylight saving time, see TS 29.060 [215]

--

--

#### -- N

--

Ncgi	::= SEQUENCE

{

plmnId					[0] PLMN-Id,

nrCellId			[1] NrCellId,

nid						[2] Nid OPTIONAL

}

Nid		::= UTF8String--

-- See 3GPP TS 29.571 [249] for details.

--

NodeID		::= IA5String (SIZE(1..20))

NodeAddress ::= CHOICE

{

iPAddress 	[0] IPAddress,

domainName	[1] GraphicString

}

NrCellId		::= UTF8String

--

-- See 3GPP TS 29.571 [249] for details.

--

--

#### -- P

--

PDPAddressPrefixLength		::=INTEGER (1..64)

--

-- This is an integer indicating the length of the PDP/PDN IPv6 address prefix

-- and the default value is 64 bits.

--

PDPAddress		::= CHOICE

{

iPAddress				[0] IPAddress

--	eTSIAddress				[1] ETSIAddress

--								has only been used in earlier releases for X.121 format

}

PLMN-Id		::= OCTET STRING (SIZE (3))

--

-- This is in the same format as octets 2, 3 and 4 of the Routing Area Identity (RAI) IE specified

-- in TS 29.060 [215]

--

PositioningData	::= OCTET STRING (SIZE(1..33))

--

-- See Positioning Data IE (octet 3..n), TS 49.031 [227]

--

PriorityType	::= ENUMERATED

{

low			(0),

normal		(1),

high		(2)

}

PSCellInformation	::= SEQUENCE

{

nRcgi					[0] Ncgi OPTIONAL,

ecgi					[1] Ecgi OPTIONAL

}

--

#### -- R

--

RANNASCause				::=	OCTET STRING

-- This octet string is a 1:1 copy of the contents (i.e. starting with octet 5)

-- of the "RAN/NAS Cause" information element specified in TS 29.274 [223].

RATType		::= INTEGER (0..255)

--

--This integer is 1:1 copy of the RAT type value as defined in TS 29.061 [215].

--

RecordingEntity 		::= AddressString

RecordType 	::= INTEGER

--

--	Record values 0..17 and 87,89  are CS specific. The contents are defined in TS 32.250 [10]

--

{

moCallRecord			(0),

mtCallRecord			(1),

roamingRecord			(2),

incGatewayRecord		(3),

outGatewayRecord		(4),

transitCallRecord		(5),

moSMSRecord				(6),

mtSMSRecord				(7),

moSMSIWRecord			(8),

mtSMSGWRecord			(9),

ssActionRecord			(10),

hlrIntRecord			(11),

locUpdateHLRRecord		(12),

locUpdateVLRRecord		(13),

commonEquipRecord		(14),

moTraceRecord			(15),	-- used in earlier releases

mtTraceRecord			(16),	-- used in earlier releases

termCAMELRecord			(17),

--

--	Record values 18..22 are GPRS specific. The contents are defined in TS 32.251 [11]

--

sgsnPDPRecord			(18),

sgsnMMRecord			(20),

sgsnSMORecord			(21),	-- also MME UE originated SMS record

sgsnSMTRecord			(22),	-- also MME UE terminated SMS record

--

--  Record values 23..25 are CS-LCS specific. The contents are defined in TS 32.250 [10]

--

mtLCSRecord				(23),

moLCSRecord				(24),

niLCSRecord				(25),

--

--  Record values 26..28 are GPRS-LCS specific. The contents are defined in TS 32.251 [11]

--

sgsnMTLCSRecord			(26),

sgsnMOLCSRecord			(27),

sgsnNILCSRecord			(28),

--

--  Record values 30..62 are MMS specific. The contents are defined in TS 32.270 [30]

--

mMO1SRecord				(30),

mMO4FRqRecord			(31),

mMO4FRsRecord			(32),

mMO4DRecord				(33),

mMO1DRecord				(34),

mMO4RRecord				(35),

mMO1RRecord				(36),

mMOMDRecord				(37),

mMR4FRecord				(38),

mMR1NRqRecord			(39),

mMR1NRsRecord			(40),

mMR1RtRecord			(41),

mMR1AFRecord			(42),

mMR4DRqRecord			(43),

mMR4DRsRecord			(44),

mMR1RRRecord			(45),

mMR4RRqRecord			(46),

mMR4RRsRecord			(47),

mMRMDRecord				(48),

mMFRecord				(49),

mMBx1SRecord			(50),

mMBx1VRecord			(51),

mMBx1URecord			(52),

mMBx1DRecord			(53),

mM7SRecord				(54),

mM7DRqRecord			(55),

mM7DRsRecord			(56),

mM7CRecord				(57),

mM7RRecord				(58),

mM7DRRqRecord			(59),

mM7DRRsRecord			(60),

mM7RRqRecord			(61),

mM7RRsRecord			(62),

--

--  Record values 63..70, 82, 89..91 are IMS specific.

--  The contents are defined in TS 32.260 [20]

--

sCSCFRecord				(63),

pCSCFRecord				(64),

iCSCFRecord				(65),

mRFCRecord				(66),

mGCFRecord				(67),

bGCFRecord				(68),

aSRecord				(69),

eCSCFRecord				(70),

iBCFRecord				(82),

tRFRecord				(89),

tFRecord				(90),

aTCFRecord				(91),

--

--  Record values 71..75 are LCS specific. The contents are defined in TS 32.271 [31]

--

lCSGMORecord			(71),

lCSRGMTRecord			(72),

lCSHGMTRecord			(73),

lCSVGMTRecord			(74),

lCSGNIRecord			(75),

--

--  Record values 76..79,86 are MBMS specific.

--  The contents are defined in TS 32.251 [11] and TS 32.273 [33]

--

--  Record values 76,77 and 86 are MBMS bearer context specific

--

sgsnMBMSRecord			(76),

ggsnMBMSRecord			(77),

gwMBMSRecord			(86),

--

--  Record values 78 and 79 are MBMS service specific and defined in TS 32.273 [33]

--

sUBBMSCRecord			(78),

cONTENTBMSCRecord		(79),

--

--  Record Values 80..81 are PoC specific. The contents are defined in TS 32.272 [32]

--

pPFRecord				(80),

cPFRecord				(81),

--

--	Record values 84,85 and 92,95,96, 97 are EPC specific.

--  The contents are defined in TS 32.251 [11]

--

sGWRecord				(84),

pGWRecord				(85),

tDFRecord				(92),

iPERecord				(95),

ePDGRecord				(96),

tWAGRecord				(97),

--

--  Record Value 83 is MMTel specific. The contents are defined in TS 32.275 [35]

--

mMTelRecord				(83),

--

--  Record value  87,88 and 89 are CS specific. The contents are defined in TS 32.250 [10]

--

mSCsRVCCRecord			(87),

mMTRFRecord				(88),

iCSRegisterRecord		(99),

--

--  Record values 93 and 94 are SMS specific. The contents are defined in TS 32.274 [34]

--

sCSMORecord				(93),

sCSMTRecord				(94),

--

--  Record values 100, 101 and 102 are ProSe specific. The contents are defined in TS 32.277 [36]

--

pFDDRecord				(100),

pFEDRecord				(101),

pFDCRecord				(102),

--

--  Record values103 and 104 are Monitoring Event specific. The contents are defined in TS

--	32.278 [38]

--

mECORecord				(103),

mERERecord				(104),

--

--  Record values 105 to 106 are CP data transfer specific. The contents are defined in TS

--	32.253 [13]

--

cPDTSCERecord			(105),

cPDTSNNRecord			(106), --

--  Record values 110 to 113 are SMS specific. The contents are defined in TS

--	32.274 [34]

--

sCDVTT4Record			(110),

sCSMOT4Record			(111),

iSMSMORecord			(112),

iSMSMTRecord			(113),

--

--  Record values 120 are Exposure Function API specific. The contents are defined in TS

--	32.254 [14]

--

eASCERecord				(120),

--

--  Record values from 200 are specific to Charging Function domain

--

chargingFunctionRecord		(200)

--

}

RequiredMBMSBearerCapabilities		::= OCTET STRING (SIZE (3..14))

--

-- This octet string is a 1:1 copy of the contents (i.e. starting with octet 5) of the

-- "Quality of service Profile" information element specified in TS 29.060 [75].

--

RoutingAreaCode	::= OCTET STRING (SIZE(1))

--

-- See TS 24.008 [208]

--

--

#### -- S

--

SCSASAddress		::= SET

--

--

--

{

sCSAddress		[1] IPAddress,

sCSRealm		[2] DiameterIdentity

}

Session-Id	::= GraphicString

--

-- rfc3261 [401]: example for SIP CALL-ID: f81d4fae-7dec-11d0-a765-00a0c91e6bf6@foo.bar.com

--

ServiceContextID		::= UTF8String

ServiceSpecificInfo  ::=  SEQUENCE

{

serviceSpecificData		[0] GraphicString OPTIONAL, 
	serviceSpecificType		[1] INTEGER OPTIONAL

}

SMSResult					::= Diagnostics

SmsTpDestinationNumber ::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- the SMS address field the encoding of the octet string is in

-- accordance with the definition of address fields in TS 23.040 [201].

-- This encoding includes type of number and numbering plan indication

-- together with the address value range.

--

SubscriberEquipmentNumber	::= SET

--

-- If SubscriberEquipmentType is set to IMEISV and IMEI is received, the number of digits is 15.

--

{

subscriberEquipmentNumberType	[0]	SubscriberEquipmentType,

subscriberEquipmentNumberData	[1]	OCTET STRING

}

SubscriberEquipmentType	::= ENUMERATED

--

-- It should be noted that depending on the services, not all equipment types are applicable.

-- For IMS equipment types 0 and 3 are applicable.

-- In 5GS, for PEI defined as:

--        - IMEI or IMEISV, iMEISV type is used and the data is per TS 23.003 [200] format.

--        - MAC address, mAC type is used, and the data is converted from JSON format of the PEI

--          described in TS 29.571 [249].

--        - EUI-64, uEI64 type is used, and the data is converted from JSON format of the PEI

--          described in TS 29.571 [249].

{

iMEISV			(0),

mAC				(1),

eUI64			(2),

modifiedEUI64	(3)

}

SubscriptionID	::= SET

--

-- See TS 23.003 [200] and TS 29.571 [249]

--

{

subscriptionIDType	[0]	SubscriptionIDType,

subscriptionIDData	[1]	UTF8String

}

SubscriptionIDType	::= ENUMERATED

{

eND-USER-E164		(0),

eND-USER-IMSI		(1),

eND-USER-SIP-URI		(2),

eND-USER-NAI			(3),

eND-USER-PRIVATE		(4)

--

-- eND-USER-NAI can be used for externalIdentifier.

-- eND-USER-IMSI can be used for 5G BRG or 5G CRG.

-- eND-USER-NAI can be used for GLI or GCI for wireline access network scenarios

-- NAI format for GCI and GLI is specified in 28.15.5 and 28.15.6 of TS 23.003 [200].

--

}

SystemType	::= ENUMERATED

--

--  "unknown" is not to be used in PS domain.

--

{

unknown				(0),

iuUTRAN				(1),

gERAN				(2)

}

--

#### -- T

--

ThreeGPPPSDataOffStatus		::= ENUMERATED

{

active		    (0),

inactive		(1)

}

TimeStamp	::= OCTET STRING (SIZE(9))

--

-- The contents of this field are a compact form of the UTCTime format

-- containing local time plus an offset to universal time. Binary coded

-- decimal encoding is employed for the digits to reduce the storage and

-- transmission overhead

-- e.g. YYMMDDhhmmssShhmm

-- where

-- YY 	= 	Year 00 to 99		BCD encoded

-- MM 	= 	Month 01 to 12 		BCD encoded

-- DD	=	Day 01 to 31		BCD encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

-- ss	=	second 00 to 59		BCD encoded

-- S	=	Sign 0 = "+", "-"	ASCII encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

--

TMGI		::= OCTET STRING

--

-- This  octet string is a 1:1 copy of the contents (i.e. starting with octet 4)

-- of the "TMGI" information element specified in TS 29.060 [75].

--

.#END

Second change

#### 5.2.2.1	CS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.250 [10].

.$CSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) csChargingDataTypes (1) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AE-title,

BCDDirectoryNumber,

CallDuration,

CalledNumber,

CallingNumber,

CauseForTerm,

CellId,

ChargeIndicator,

Diagnostics,

LCSCause,

LCSClientIdentity,

LCSQoSInfo,

LevelOfCAMELService,

LocationAreaAndCell,

LocationAreaCode,

ManagementExtensions,

MCC-MNC,

MessageReference,

MSCAddress,

MscNo,

MSISDN,

NodeAddress,

ObjectInstance,

PositioningData,

RecordingEntity,

RecordType,

SMSResult,

SmsTpDestinationNumber,

SystemType,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

BearerServiceCode

FROM MAP-BS-Code {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-BS-Code (20) version20 (20)}

-- from TS 29.002 [214]

TeleserviceCode

FROM MAP-TS-Code {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-TS-Code (19) version20 (20)}

-- from TS 29.002 [214]

SS-Code

FROM MAP-SS-Code { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-SS-Code (15) version20 (20)}

-- from TS 29.002 [214]

MOLR-Type

FROM SS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version16 (16)}

-- from TS 24.080 [209]

DefaultCallHandling,

DefaultSMS-Handling,

NotificationToMSUser,

ServiceKey

FROM MAP-MS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version20 (20)}

-- from TS 29.002 [214]

CallReferenceNumber,

NumberOfForwarding

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CH-DataTypes (13) version20 (20)}

-- from TS 29.002 [214]

AddressString,

BasicServiceCode,

IMEI,

IMSI,

ISDN-AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version18 (18) }

-- from TS 29.002 [214]

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25)  version18 (18) }

-- from TS 29.002 [214]

IMS-Charging-Identifier,

InterOperatorIdentifierList,

TransitIOILists

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version2 (1)}

BasicService

FROM Basic-Service-Elements	{itu-t(0) identified-organization (4) etsi (0) 196 basic-service-elements (8) }

-- from "Digital Subscriber Signalling System No. one (DSS1) protocol" ETS 300 196 [310]

EXTENSION,

PARAMETERS-BOUND,

SupportedExtensions

FROM CAP-classes {itu-t(0) identified-organization(4) etsi(0) mobileDomain(0) umts-network(1) modules(3) cap-classes(54) version8(7)}

DestinationRoutingAddress

FROM CAP-datatypes { itu-t(0) identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) cap-datatypes (52) version8 (7) }

-- from TS 29.078 [217]

;

--

--  CS CALL AND EVENT RECORDS

--

CSRecord {PARAMETERS-BOUND : bound}	::= CHOICE

--

-- Record values 0..21 are circuit switch specific

--

{

moCallRecord				[0] MOCallRecord {bound},

mtCallRecord				[1] MTCallRecord,

roamingRecord			[2] RoamingRecord,

incGatewayRecord			[3] IncGatewayRecord,

outGatewayRecord			[4] OutGatewayRecord,

transitRecord			[5] TransitCallRecord,

moSMSRecord				[6] MOSMSRecord,

mtSMSRecord				[7] MTSMSRecord,

moSMSIWRecord			[8] MOSMSIWRecord,

mtSMSGWRecord			[9] MTSMSGWRecord,

ssActionRecord			[10] SSActionRecord,

hlrIntRecord				[11] HLRIntRecord,

locUpdateHLRRecord		[12] LocUpdateHLRRecord,

locUpdateVLRRecord		[13] LocUpdateVLRRecord,

commonEquipRecord		[14] CommonEquipRecord,

recTypeExtensions		[15] ManagementExtensions,

termCAMELRecord			[16] TermCAMELRecord {bound},

mtLCSRecord				[17] MTLCSRecord,

moLCSRecord				[18] MOLCSRecord,

niLCSRecord				[19] NILCSRecord,

mSCsRVCCRecord			[20] MSCsRVCCRecord,

mMTRFRecord				[21] MTRFRecord,

iCSRegisterRecord		[22] ICSregisterRecord

}

MOCallRecord {PARAMETERS-BOUND : bound}	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI OPTIONAL,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

callingNumber				[4] CallingNumber OPTIONAL,

calledNumber					[5] CalledNumber OPTIONAL,

translatedNumber				[6] TranslatedNumber OPTIONAL,

connectedNumber				[7] ConnectedNumber OPTIONAL,

roamingNumber				[8] RoamingNumber OPTIONAL,

recordingEntity				[9] RecordingEntity,

mscIncomingTKGP				[10] TrunkGroup OPTIONAL,

mscOutgoingTKGP				[11] TrunkGroup OPTIONAL,

location						[12] LocationAreaAndCell OPTIONAL,

changeOfLocation				[13] SEQUENCE OF LocationChange OPTIONAL,

basicService					[14] BasicServiceCode OPTIONAL,

transparencyIndicator		[15] TransparencyInd OPTIONAL,

changeOfService				[16] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed			[17] SEQUENCE OF SuppServiceUsed OPTIONAL,

aocParameters				[18] AOCParameters OPTIONAL,

changeOfAOCParms				[19] SEQUENCE OF AOCParmChange OPTIONAL,

msClassmark					[20] Classmark OPTIONAL,

changeOfClassmark			[21] ChangeOfClassmark OPTIONAL,

seizureTime					[22] TimeStamp OPTIONAL,

answerTime					[23] TimeStamp OPTIONAL,

releaseTime					[24] TimeStamp OPTIONAL,

callDuration					[25] CallDuration,

dataVolume					[26] DataVolume OPTIONAL,

radioChanRequested			[27] RadioChanRequested OPTIONAL,

radioChanUsed				[28] TrafficChannel OPTIONAL,

changeOfRadioChan			[29] ChangeOfRadioChannel OPTIONAL,

causeForTerm					[30] CauseForTerm,

diagnostics					[31] Diagnostics OPTIONAL,

callReference				[32] CallReferenceNumber,

sequenceNumber				[33] INTEGER OPTIONAL,

additionalChgInfo			[34] AdditionalChgInfo OPTIONAL,

recordExtensions				[35] ManagementExtensions OPTIONAL,

gsm-SCFAddress				[36] Gsm-SCFAddress OPTIONAL,

serviceKey					[37] ServiceKey OPTIONAL,

networkCallReference			[38] NetworkCallReference OPTIONAL,

mSCAddress					[39] MSCAddress OPTIONAL,

cAMELInitCFIndicator			[40] CAMELInitCFIndicator OPTIONAL,

defaultCallHandling			[41] DefaultCallHandling OPTIONAL,

hSCSDChanRequested			[42] NumOfHSCSDChanRequested OPTIONAL,

hSCSDChanAllocated			[43] NumOfHSCSDChanAllocated OPTIONAL,

changeOfHSCSDParms			[44] SEQUENCE OF HSCSDParmsChange OPTIONAL,

fnur							[45] Fnur OPTIONAL,

aiurRequested				[46] AiurRequested OPTIONAL,

chanCodingsAcceptable		[47] SEQUENCE OF ChannelCoding OPTIONAL,

chanCodingUsed				[48] ChannelCoding OPTIONAL,

speechVersionSupported		[49] SpeechVersionIdentifier OPTIONAL,

speechVersionUsed			[50] SpeechVersionIdentifier OPTIONAL,

numberOfDPEncountered		[51] INTEGER OPTIONAL,

levelOfCAMELService			[52] LevelOfCAMELService OPTIONAL,

freeFormatData				[53] FreeFormatData OPTIONAL,

cAMELCallLegInformation		[54] SEQUENCE OF CAMELInformation {bound} OPTIONAL,

freeFormatDataAppend			[55] BOOLEAN OPTIONAL,

defaultCallHandling-2		[56] DefaultCallHandling OPTIONAL,

gsm-SCFAddress-2				[57] Gsm-SCFAddress OPTIONAL,

serviceKey-2					[58] ServiceKey OPTIONAL,

freeFormatData-2				[59] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[60] BOOLEAN OPTIONAL,

systemType					[61] SystemType OPTIONAL,

rateIndication				[62] RateIndication OPTIONAL,

locationRoutNum				[63] LocationRoutingNumber OPTIONAL,

lrnSoInd						[64] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus				[65] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara						[66] JurisdictionInformationParameter OPTIONAL,

jIPSoInd						[67] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus				[68] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType			[69] PartialRecordType OPTIONAL,

guaranteedBitRate			[70] GuaranteedBitRate OPTIONAL,

maximumBitRate				[71] MaximumBitRate OPTIONAL,

redial						[72] BOOLEAN OPTIONAL,	-- set indicates redial attempt

reasonForServiceChange		[73] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator		[74] BOOLEAN OPTIONAL,

iCSI2ActiveFlag				[75] NULL OPTIONAL,

iMS-Charging-Identifier		[76] IMS-Charging-Identifier OPTIONAL,

privateUserID				[77] GraphicString OPTIONAL

}

MTCallRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] CalledNumber OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

connectedNumber			[5] ConnectedNumber OPTIONAL,

recordingEntity			[6] RecordingEntity,

mscIncomingTKGP			[7] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[8] TrunkGroup OPTIONAL,

location					[9] LocationAreaAndCell OPTIONAL,

changeOfLocation			[10] SEQUENCE OF LocationChange OPTIONAL,

basicService				[11] BasicServiceCode OPTIONAL,

transparencyIndicator	[12] TransparencyInd OPTIONAL,

changeOfService			[13] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[14] SEQUENCE OF SuppServiceUsed OPTIONAL,

aocParameters			[15] AOCParameters OPTIONAL,

changeOfAOCParms			[16] SEQUENCE OF AOCParmChange OPTIONAL,

msClassmark				[17] Classmark OPTIONAL,

changeOfClassmark		[18] ChangeOfClassmark OPTIONAL,

seizureTime				[19] TimeStamp OPTIONAL,

answerTime				[20] TimeStamp OPTIONAL,

releaseTime				[21] TimeStamp OPTIONAL,

callDuration				[22] CallDuration,

dataVolume				[23] DataVolume OPTIONAL,

radioChanRequested		[24] RadioChanRequested OPTIONAL,

radioChanUsed			[25] TrafficChannel OPTIONAL,

changeOfRadioChan		[26] ChangeOfRadioChannel OPTIONAL,

causeForTerm				[27] CauseForTerm,

diagnostics				[28] Diagnostics OPTIONAL,

callReference			[29] CallReferenceNumber,

sequenceNumber			[30] INTEGER OPTIONAL,

additionalChgInfo		[31] AdditionalChgInfo OPTIONAL,

recordExtensions			[32] ManagementExtensions OPTIONAL,

networkCallReference		[33] NetworkCallReference OPTIONAL,

mSCAddress				[34] MSCAddress OPTIONAL,

hSCSDChanRequested		[35] NumOfHSCSDChanRequested OPTIONAL,

hSCSDChanAllocated		[36] NumOfHSCSDChanAllocated OPTIONAL,

changeOfHSCSDParms		[37] SEQUENCE OF HSCSDParmsChange OPTIONAL,

fnur						[38] Fnur OPTIONAL,

aiurRequested			[39] AiurRequested OPTIONAL,

chanCodingsAcceptable	[40] SEQUENCE OF ChannelCoding OPTIONAL,

chanCodingUsed			[41] ChannelCoding OPTIONAL,

speechVersionSupported	[42] SpeechVersionIdentifier OPTIONAL,

speechVersionUsed		[43] SpeechVersionIdentifier OPTIONAL,

gsm-SCFAddress			[44] Gsm-SCFAddress OPTIONAL,

serviceKey				[45] ServiceKey OPTIONAL,

systemType				[46] SystemType OPTIONAL,

rateIndication			[47] RateIndication OPTIONAL,

locationRoutNum			[48] LocationRoutingNumber OPTIONAL,

lrnSoInd					[49] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[50] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[51] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[52] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[53] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType		[54] PartialRecordType OPTIONAL,

guaranteedBitRate		[55] GuaranteedBitRate OPTIONAL,

maximumBitRate			[56] MaximumBitRate OPTIONAL,

reasonForServiceChange	[57] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[58] BOOLEAN OPTIONAL,

iCSI2ActiveFlag			[59] NULL OPTIONAL,

iMS-Charging-Identifier	[60] IMS-Charging-Identifier OPTIONAL,

privateUserID			[61] GraphicString OPTIONAL

}

RoamingRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN OPTIONAL,

callingNumber			[3] CallingNumber OPTIONAL,

roamingNumber			[4] RoamingNumber OPTIONAL,

recordingEntity			[5] RecordingEntity,

mscIncomingTKGP			[6] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[7] TrunkGroup OPTIONAL,

basicService				[8] BasicServiceCode OPTIONAL,

transparencyIndicator	[9] TransparencyInd OPTIONAL,

changeOfService			[10] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[11] SEQUENCE OF  SuppServiceUsed OPTIONAL,

seizureTime				[12] TimeStamp OPTIONAL,

answerTime				[13] TimeStamp OPTIONAL,

releaseTime				[14] TimeStamp OPTIONAL,

callDuration				[15] CallDuration,

dataVolume				[16] DataVolume OPTIONAL,

causeForTerm				[17] CauseForTerm,

diagnostics				[18] Diagnostics OPTIONAL,

callReference			[19] CallReferenceNumber,

sequenceNumber			[20] INTEGER OPTIONAL,

recordExtensions			[21] ManagementExtensions OPTIONAL,

networkCallReference		[22] NetworkCallReference OPTIONAL,

mSCAddress				[23] MSCAddress OPTIONAL,

locationRoutNum			[24] LocationRoutingNumber OPTIONAL,

lrnSoInd					[25] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[26] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[27] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[28] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[29] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType		[30] PartialRecordType OPTIONAL

}

TermCAMELRecord {PARAMETERS-BOUND : bound}	::= SET

{

recordtype					[0] RecordType,

servedIMSI					[1] IMSI,

servedMSISDN					[2] MSISDN OPTIONAL,

recordingEntity				[3] RecordingEntity,

interrogationTime			[4] TimeStamp,

destinationRoutingAddress	[5] DestinationRoutingAddress {bound},

gsm-SCFAddress				[6] Gsm-SCFAddress,

serviceKey					[7] ServiceKey,

networkCallReference			[8] NetworkCallReference OPTIONAL,

mSCAddress					[9] MSCAddress OPTIONAL,

defaultCallHandling			[10] DefaultCallHandling OPTIONAL,

recordExtensions				[11] ManagementExtensions OPTIONAL,

calledNumber					[12] CalledNumber,

callingNumber				[13] CallingNumber OPTIONAL,

mscIncomingTKGP				[14] TrunkGroup OPTIONAL,

mscOutgoingTKGP				[15] TrunkGroup OPTIONAL,

seizureTime					[16] TimeStamp OPTIONAL,

answerTime					[17] TimeStamp OPTIONAL,

releaseTime					[18] TimeStamp OPTIONAL,

callDuration					[19] CallDuration,

dataVolume					[20] DataVolume OPTIONAL,

causeForTerm					[21] CauseForTerm,

diagnostics					[22] Diagnostics OPTIONAL,

callReference				[23] CallReferenceNumber,

sequenceNumber				[24] INTEGER OPTIONAL,

numberOfDPEncountered		[25] INTEGER OPTIONAL,

levelOfCAMELService			[26] LevelOfCAMELService OPTIONAL,

freeFormatData				[27] FreeFormatData OPTIONAL,

cAMELCallLegInformation		[28] SEQUENCE OF CAMELInformation {bound} OPTIONAL,

freeFormatDataAppend			[29] BOOLEAN OPTIONAL,

defaultCallHandling-2		[30] DefaultCallHandling OPTIONAL,

gsm-SCFAddress-2				[31] Gsm-SCFAddress OPTIONAL,

serviceKey-2					[32] ServiceKey OPTIONAL,

freeFormatData-2				[33] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[34] BOOLEAN OPTIONAL,

mscServerIndication			[35] BOOLEAN OPTIONAL,

locationRoutNum				[36] LocationRoutingNumber OPTIONAL,

lrnSoInd						[37] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus				[38] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara						[39] JurisdictionInformationParameter OPTIONAL,

jIPSoInd						[40] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus				[41] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType			[42] PartialRecordType OPTIONAL

}

IncGatewayRecord		::= SET

{

recordType				[0] RecordType,

callingNumber			[1] CallingNumber OPTIONAL,

calledNumber				[2] CalledNumber,

recordingEntity			[3] RecordingEntity,

mscIncomingTKGP			[4] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[5] TrunkGroup OPTIONAL,

seizureTime				[6] TimeStamp OPTIONAL,

answerTime				[7] TimeStamp OPTIONAL,

releaseTime				[8] TimeStamp OPTIONAL,

callDuration				[9] CallDuration,

dataVolume				[10] DataVolume OPTIONAL,

causeForTerm				[11] CauseForTerm,

diagnostics				[12] Diagnostics OPTIONAL,

callReference			[13] CallReferenceNumber,

sequenceNumber			[14] INTEGER OPTIONAL,

recordExtensions			[15] ManagementExtensions OPTIONAL,

locationRoutNum			[16] LocationRoutingNumber OPTIONAL,

lrnSoInd					[17] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[18] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[19] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[20] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[21] JurisdictionInformationParameterQueryStatus OPTIONAL,

reasonForServiceChange	[22] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[23] BOOLEAN OPTIONAL

}

OutGatewayRecord		::= SET

{

recordType				[0] RecordType,

callingNumber			[1] CallingNumber OPTIONAL,

calledNumber				[2] CalledNumber,

recordingEntity			[3] RecordingEntity,

mscIncomingTKGP			[4] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[5] TrunkGroup OPTIONAL,

seizureTime				[6] TimeStamp OPTIONAL,

answerTime				[7] TimeStamp OPTIONAL,

releaseTime				[8] TimeStamp OPTIONAL,

callDuration				[9] CallDuration,

dataVolume				[10] DataVolume OPTIONAL,

causeForTerm				[11] CauseForTerm,

diagnostics				[12] Diagnostics OPTIONAL,

callReference			[13] CallReferenceNumber,

sequenceNumber			[14] INTEGER OPTIONAL,

recordExtensions			[15] ManagementExtensions OPTIONAL,

locationRoutNum			[16] LocationRoutingNumber OPTIONAL,

lrnSoInd					[17] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[18] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[19] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[20] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[21] JurisdictionInformationParameterQueryStatus OPTIONAL,

reasonForServiceChange	[22] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[23] BOOLEAN OPTIONAL
}

TransitCallRecord		::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

mscIncomingTKGP			[2] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[3] TrunkGroup OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

calledNumber				[5] CalledNumber,

isdnBasicService			[6] BasicService OPTIONAL,

seizureTimestamp			[7] TimeStamp OPTIONAL,

answerTimestamp			[8] TimeStamp OPTIONAL,

releaseTimestamp			[9] TimeStamp OPTIONAL,

callDuration				[10] CallDuration,

dataVolume				[11] DataVolume OPTIONAL,

causeForTerm				[12] CauseForTerm,

diagnostics				[13] Diagnostics OPTIONAL,

callReference			[14] CallReferenceNumber,

sequenceNumber			[15] INTEGER OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

locationRoutNum			[17] LocationRoutingNumber OPTIONAL,

lrnSoInd					[18] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[19] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[20] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[21] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[22] JurisdictionInformationParameterQueryStatus OPTIONAL

}

MOSMSRecord				::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msClassmark				[4] Classmark,

serviceCentre			[5] AddressString,

recordingEntity			[6] RecordingEntity,

location					[7] LocationAreaAndCell OPTIONAL,

messageReference			[8] MessageReference,

originationTime			[9] TimeStamp,

smsResult				[10] SMSResult OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

destinationNumber		[12] SmsTpDestinationNumber OPTIONAL,

cAMELSMSInformation		[13] CAMELSMSInformation OPTIONAL,

systemType				[14] SystemType OPTIONAL,

locationExtension		[15] LocationCellExtension OPTIONAL

}

MTSMSRecord				::= SET

{

recordType				[0] RecordType,

serviceCentre			[1] AddressString,

servedIMSI				[2] IMSI,

servedIMEI				[3] IMEI OPTIONAL,

servedMSISDN				[4] MSISDN OPTIONAL,

msClassmark				[5] Classmark,

recordingEntity			[6] RecordingEntity,

location					[7] LocationAreaAndCell OPTIONAL,

deliveryTime				[8] TimeStamp,

smsResult				[9] SMSResult OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

systemType				[11] SystemType OPTIONAL,

cAMELSMSInformation		[12] CAMELSMSInformation OPTIONAL,

locationExtension		[13] LocationCellExtension OPTIONAL

}

MOSMSIWRecord			::= SET

{

recordType			[0] RecordType,

serviceCentre		[1] AddressString,

servedIMSI			[2] IMSI,

recordingEntity		[3] RecordingEntity,

eventTime			[4] TimeStamp,

smsResult			[5] SMSResult OPTIONAL,

recordExtensions	[6] ManagementExtensions OPTIONAL

}

MTSMSGWRecord			::= SET

{

recordType			[0] RecordType,

serviceCentre		[1] AddressString,

servedIMSI			[2] IMSI,

servedMSISDN			[3] MSISDN OPTIONAL,

recordingEntity		[4] RecordingEntity,

eventTime			[5] TimeStamp,

smsResult			[6] SMSResult OPTIONAL,

recordExtensions		[7] ManagementExtensions OPTIONAL

}

SSActionRecord			::= SET

{

recordType			[0] RecordType,

servedIMSI			[1] IMSI,

servedIMEI			[2] IMEI OPTIONAL,

servedMSISDN			[3] MSISDN OPTIONAL,

msClassmark			[4] Classmark,

recordingEntity		[5] RecordingEntity,

location				[6] LocationAreaAndCell OPTIONAL,

basicServices		[7] BasicServices OPTIONAL,

supplService			[8] SS-Code OPTIONAL,

ssAction				[9] SSActionType OPTIONAL,

ssActionTime			[10] TimeStamp,

ssParameters			[11] SSParameters OPTIONAL,

ssActionResult		[12] SSActionResult OPTIONAL,

callReference		[13] CallReferenceNumber,

recordExtensions		[14] ManagementExtensions OPTIONAL,

systemType			[15] SystemType OPTIONAL

}

HLRIntRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN,

recordingEntity			[3] RecordingEntity,

basicService				[4] BasicServiceCode OPTIONAL,

routingNumber			[5] RoutingNumber,

interrogationTime		[6] TimeStamp,

numberOfForwarding		[7] NumberOfForwarding OPTIONAL,

interrogationResult		[8] HLRIntResult OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

LocUpdateHLRRecord 		::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

recordingEntity			[2] RecordingEntity,

oldLocation				[3] Visited-Location-info OPTIONAL,

newLocation				[4] Visited-Location-info,

updateTime				[5] TimeStamp,

updateResult				[6] LocUpdResult OPTIONAL,

recordExtensions			[7] ManagementExtensions OPTIONAL

}

LocUpdateVLRRecord 		::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN OPTIONAL,

recordingEntity			[3] RecordingEntity,

oldLocation				[4] Location-info OPTIONAL,

newLocation				[5] Location-info,

msClassmark				[6] Classmark,

updateTime				[7] TimeStamp,

updateResult				[8] LocUpdResult OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL,

locationExtension		[10] LocationCellExtension OPTIONAL

}

CommonEquipRecord 		::= SET

{

recordType				[0] RecordType,

equipmentType			[1] EquipmentType,

equipmentId				[2] EquipmentId,

servedIMSI				[3] IMSI,

servedMSISDN				[4] MSISDN OPTIONAL,

recordingEntity			[5] RecordingEntity,

basicService				[6] BasicServiceCode OPTIONAL,

changeOfService			[7] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[8] SEQUENCE OF SuppServiceUsed OPTIONAL,

seizureTime				[9] TimeStamp,

releaseTime				[10] TimeStamp OPTIONAL,

callDuration				[11] CallDuration,

callReference			[12] CallReferenceNumber,

sequenceNumber			[13] INTEGER OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

systemType				[15] SystemType OPTIONAL,

rateIndication			[16] RateIndication OPTIONAL,

fnur						[17] Fnur OPTIONAL

}

--

--  OBSERVED IMEI TICKETS

--

ObservedIMEITicket		::= SET

{

servedIMEI			[0] IMEI,

imeiStatus			[1] IMEIStatus,

servedIMSI			[2] IMSI,

servedMSISDN			[3] MSISDN OPTIONAL,

recordingEntity		[4] RecordingEntity,

eventTime			[5] TimeStamp,

location				[6] LocationAreaAndCell	,

imeiCheckEvent		[7] IMEICheckEvent OPTIONAL,

callReference		[8] CallReferenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

--

--  CS LOCATION SERVICE RECORDS

--

MTLCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType,

lcsClientIdentity		[3] LCSClientIdentity,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lcsQos					[7] LCSQoSInfo OPTIONAL,

lcsPriority				[8] LCS-Priority OPTIONAL,

mlc-Number				[9] ISDN-AddressString,

eventTimeStamp			[10] TimeStamp,

measureDuration			[11] CallDuration OPTIONAL,

notificationToMSUser		[12] NotificationToMSUser OPTIONAL,

privacyOverride			[13] NULL OPTIONAL,

location					[14] LocationAreaAndCell OPTIONAL,

locationEstimate			[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause					[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

systemType				[19] SystemType OPTIONAL,

recordExtensions			[20] ManagementExtensions OPTIONAL,

causeForTerm				[21] CauseForTerm,

servedIMEI				[22] IMEI OPTIONAL

}

MOLCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI,

servedMSISDN				[5] MSISDN OPTIONAL,

molr-Type				[6] MOLR-Type,

lcsQos					[7] LCSQoSInfo OPTIONAL,

lcsPriority				[8] LCS-Priority OPTIONAL,

mlc-Number				[9] ISDN-AddressString OPTIONAL,

eventTimeStamp			[10] TimeStamp,

measureDuration			[11] CallDuration OPTIONAL,

location					[12] LocationAreaAndCell OPTIONAL,

locationEstimate			[13] Ext-GeographicalInformation OPTIONAL,

positioningData			[14] PositioningData OPTIONAL,

lcsCause					[15] LCSCause OPTIONAL,

diagnostics				[16] Diagnostics OPTIONAL,

systemType				[17] SystemType OPTIONAL,

recordExtensions			[18] ManagementExtensions OPTIONAL,

causeForTerm				[19] CauseForTerm

}

NILCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN				[5] MSISDN OPTIONAL,

servedIMEI				[6] IMEI OPTIONAL,

emsDigits				[7] ISDN-AddressString OPTIONAL,

emsKey					[8] ISDN-AddressString OPTIONAL,

lcsQos					[9] LCSQoSInfo OPTIONAL,

lcsPriority				[10] LCS-Priority OPTIONAL,

mlc-Number				[11] ISDN-AddressString OPTIONAL,

eventTimeStamp			[12] TimeStamp,

measureDuration			[13] CallDuration OPTIONAL,

location					[14] LocationAreaAndCell OPTIONAL,

locationEstimate			[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause					[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

systemType				[19] SystemType OPTIONAL,

recordExtensions			[20] ManagementExtensions OPTIONAL,

causeForTerm				[21] CauseForTerm

}

--

--  SRVCC RECORDS

--

MSCsRVCCRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI OPTIONAL,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

calledNumber					[5] CalledNumber,

recordingEntity				[9] RecordingEntity,

mscOutgoingTKGP				[11] TrunkGroup OPTIONAL,

location						[12] LocationAreaAndCell,

changeOfLocation				[13] SEQUENCE OF LocationChange OPTIONAL,

basicService					[14] BasicServiceCode,

supplServicesUsed			[17] SEQUENCE OF SuppServiceUsed OPTIONAL,

msClassmark					[20] Classmark OPTIONAL,

seizureTime					[22] TimeStamp OPTIONAL,

answerTime					[23] TimeStamp OPTIONAL,

releaseTime					[24] TimeStamp OPTIONAL,

callDuration					[25] CallDuration,

causeForTerm					[30] CauseForTerm,

diagnostics					[31] Diagnostics OPTIONAL,

callReference				[32] CallReferenceNumber,

sequenceNumber				[33] INTEGER OPTIONAL,

recordExtensions				[35] ManagementExtensions OPTIONAL,

partialRecordType			[69] PartialRecordType OPTIONAL,

iMS-Charging-Identifier		[75] IMS-Charging-Identifier OPTIONAL,

iCSI2ActiveFlag				[76] NULL OPTIONAL,

relatedICID					[77] IMS-Charging-Identifier OPTIONAL,

relatedICIDGenerationNode	[78] NodeAddress OPTIONAL

}

--

--  MTRF RECORD

--

MTRFRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] CalledNumber OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

roamingNumber			[5] RoamingNumber OPTIONAL,

recordingEntity			[6] RecordingEntity,

mscIncomingTKGP			[7] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[8] TrunkGroup OPTIONAL,

basicService				[9] BasicServiceCode OPTIONAL,

seizureTime				[10] TimeStamp OPTIONAL,

answerTime				[11] TimeStamp OPTIONAL,

releaseTime				[12] TimeStamp OPTIONAL,

callDuration				[13] CallDuration,

causeForTerm				[14] CauseForTerm,

diagnostics				[15] Diagnostics OPTIONAL,

callReference			[16] CallReferenceNumber,

sequenceNumber			[17] INTEGER OPTIONAL,

recordExtensions			[18] ManagementExtensions OPTIONAL,

partialRecordType		[19] PartialRecordType OPTIONAL

}

--

--  ICS RECORD

--

ICSregisterRecord ::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedMSISDN					[2] MSISDN,

privateUserID				[3] GraphicString OPTIONAL,

recordingEntity				[4] RecordingEntity,

newLocation					[5] Location-info,

locationExtension			[6] LocationCellExtension OPTIONAL,

updateTime					[7] TimeStamp OPTIONAL,

iMS-Charging-Identifier		[8] IMS-Charging-Identifier OPTIONAL,

interOperatorIdentifiers		[9] InterOperatorIdentifierList OPTIONAL,

transit-IOI-Lists			[10] TransitIOILists OPTIONAL,

updateResult					[11] LocUpdResult OPTIONAL,

recordExtensions				[12] ManagementExtensions OPTIONAL

}

--

--  NP Fields

--

LocationRoutingNumber	::= OCTET STRING (SIZE (5))

--

--  The format is selected to meet the existing standards for the wireline in Telcordia

--   Belcore GR-1100-CORE, BAF Module 720.

--

LocationRoutingNumberSourceIndicator	::=	INTEGER

{

lRN-NP-Database				(1),

switchingSystemData			(2),

incomingsignaling			(3),

unknown						(9)

}

LocationRoutingNumberQueryStatus 		::=	INTEGER

{

successfulQuery				(1),

noQueryResponseMsg			(2),

queryProtocolErr				(4),

queryResponseDataErr			(5),

queryRejected				(6),

queryNotPerformed			(9),

queryUnsuccessful			(99)

}

JurisdictionInformationParameter 	::= OCTET STRING (SIZE (5))

--

-- /* JIP Parameter */

--

JurisdictionInformationParameterSourceIndicator 	::=	INTEGER

--

-- Identical to LocationRoutingNumberSourceIndicator

--

{

lRN-NP-Database				(1),

switchingSystemData			(2),

incomingsignaling			(3),

unknown						(9)

}

JurisdictionInformationParameterQueryStatus 	::=	INTEGER

{

successfulQuery				(1),

noQueryResponseMsg			(2),

queryProtocolErr				(4),

queryResponseDataErr			(5),

queryRejected				(6),

queryNotPerformed			(9),

queryUnsuccessful			(99)

}

--

--  CS DATA TYPES

--

AdditionalChgInfo		::= SEQUENCE

{

chargeIndicator		[0] ChargeIndicator OPTIONAL,

chargeParameters	[1] OCTET STRING OPTIONAL

}

AiurRequested			::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

-- (note that value "4" is intentionally missing

-- because it is not used in TS 24.008 [208])

--

{

aiur09600BitsPerSecond		(1),

aiur14400BitsPerSecond		(2),

aiur19200BitsPerSecond		(3),

aiur28800BitsPerSecond		(5),

aiur38400BitsPerSecond		(6),

aiur43200BitsPerSecond		(7),

aiur57600BitsPerSecond		(8),

aiur38400BitsPerSecond1		(9),

aiur38400BitsPerSecond2		(10),

aiur38400BitsPerSecond3		(11),

aiur38400BitsPerSecond4		(12)

}

AOCParameters			::= SEQUENCE

--

-- See TS 22.024 [104].

--

{

e1					[1] EParameter OPTIONAL,

e2					[2] EParameter OPTIONAL,

e3					[3] EParameter OPTIONAL,

e4					[4] EParameter OPTIONAL,

e5					[5] EParameter OPTIONAL,

e6					[6] EParameter OPTIONAL,

e7					[7] EParameter OPTIONAL

}

AOCParmChange			::= SEQUENCE

{

changeTime			[0] TimeStamp,

newParameters		[1] AOCParameters

}

BasicServices			::= SET OF BasicServiceCode

CallingPartyCategory	::= Category

CallType				::= INTEGER

{

mobileOriginated	(0),

mobileTerminated	(1)

}

CallTypes 				::= SET OF CallType

CAMELDestinationNumber {PARAMETERS-BOUND : bound}	::= DestinationRoutingAddress {bound}

CAMELInformation {PARAMETERS-BOUND : bound}		::= SET

{

cAMELDestinationNumber		[1] CAMELDestinationNumber {bound} OPTIONAL,

connectedNumber				[2] ConnectedNumber OPTIONAL,

roamingNumber				[3] RoamingNumber OPTIONAL,

mscOutgoingTKGP				[4] TrunkGroup OPTIONAL,

seizureTime					[5] TimeStamp OPTIONAL,

answerTime					[6] TimeStamp OPTIONAL,

releaseTime					[7] TimeStamp OPTIONAL,

callDuration					[8] CallDuration OPTIONAL,

dataVolume					[9] DataVolume OPTIONAL,

cAMELInitCFIndicator			[10] CAMELInitCFIndicator OPTIONAL,

causeForTerm					[11] CauseForTerm OPTIONAL,

cAMELModification			[12] ChangedParameters OPTIONAL,

freeFormatData				[13] FreeFormatData OPTIONAL,

diagnostics					[14] Diagnostics OPTIONAL,

freeFormatDataAppend			[15] BOOLEAN OPTIONAL,

freeFormatData-2				[16] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[17] BOOLEAN OPTIONAL

}

CAMELInitCFIndicator	::= ENUMERATED

{

noCAMELCallForwarding		(0),

cAMELCallForwarding			(1)

}

CAMELModificationParameters		::= SET

--

-- The list contains only parameters changed due to CAMEL call handling.

--

{

callingPartyNumber			[0] CallingNumber OPTIONAL,

callingPartyCategory			[1] CallingPartyCategory OPTIONAL,

originalCalledPartyNumber	[2] OriginalCalledNumber OPTIONAL,

genericNumbers				[3] GenericNumbers OPTIONAL,

redirectingPartyNumber		[4] RedirectingNumber OPTIONAL,

redirectionCounter			[5] NumberOfForwarding OPTIONAL

}

CAMELSMSInformation		::= SET

{

gsm-SCFAddress						[1] Gsm-SCFAddress OPTIONAL,

serviceKey							[2] ServiceKey OPTIONAL,

defaultSMSHandling			 		[3] DefaultSMS-Handling OPTIONAL,

freeFormatData       				[4] FreeFormatData OPTIONAL,

callingPartyNumber 					[5] CallingNumber OPTIONAL,

destinationSubscriberNumber			[6] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[7] AddressString OPTIONAL,

smsReferenceNumber					[8] CallReferenceNumber OPTIONAL

}

Category		::= OCTET STRING (SIZE(1))

--

-- The internal structure is defined in Recommendation Q.763.

--

ChangedParameters		::= SET

{

changeFlags		[0] ChangeFlags,

changeList		[1] CAMELModificationParameters OPTIONAL

}

ChangeFlags				::= BIT STRING

{

callingPartyNumberModified			(0),

callingPartyCategoryModified			(1),

originalCalledPartyNumberModified	(2),

genericNumbersModified				(3),

redirectingPartyNumberModified		(4),

redirectionCounterModified			(5)

}

ChangeOfClassmark 		::= SEQUENCE

{

classmark			[0] Classmark,

changeTime			[1] TimeStamp

}

ChangeOfRadioChannel 	::= SEQUENCE

{

radioChannel		[0] TrafficChannel,

changeTime			[1] TimeStamp,

speechVersionUsed	[2] SpeechVersionIdentifier OPTIONAL

}

ChangeOfService 		::= SEQUENCE

{

basicService		[0] BasicServiceCode,

transparencyInd	[1] TransparencyInd OPTIONAL,

changeTime		[2] TimeStamp,

rateIndication	[3] RateIndication OPTIONAL,

fnur				[4] Fnur OPTIONAL

}

ChannelCoding				::= ENUMERATED

{

tchF4800				(1),

tchF9600				(2),

tchF14400			(3)

}

Classmark				::= OCTET STRING

--

-- See Mobile station classmark 2, Mobile station classmark 3, TS 24.008[208]

--

ConnectedNumber			::= BCDDirectoryNumber

DataVolume				::= INTEGER

--

-- The volume of data transferred in segments of 64 octets.

--

Day						::= INTEGER (1..31)

DayClass				::= ObjectInstance

DayClasses				::= SET OF DayClass

DayDefinition			::= SEQUENCE

{

day				[0] DayOfTheWeek,

dayClass			[1] ObjectInstance

}

DayDefinitions			::= SET OF DayDefinition

DateDefinition			::= SEQUENCE

{

month				[0] Month,

day					[1] Day,

dayClass				[2] ObjectInstance

}

DateDefinitions			::= SET OF DateDefinition

DayOfTheWeek			::= ENUMERATED

{

allDays				(0),

sunday				(1),

monday				(2),

tuesday				(3),

wednesday			(4),

thursday				(5),

friday				(6),

saturday				(7)

}

Destinations			::= SET OF AE-title

EmergencyCallIndEnable	::= BOOLEAN

EmergencyCallIndication	::= SEQUENCE

{

cellId			[0] CellId,

callerId			[1] IMSIorIMEI

}

EParameter ::= INTEGER

EquipmentId				::= INTEGER

EquipmentType			::= INTEGER

{

conferenceBridge	(0)

}

FileType				::= INTEGER

{

callRecords			(1),

traceRecords			(9),

observedIMEITicket	(14)

}

Fnur					::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

--

{

fnurNotApplicable			(0),

fnur9600-BitsPerSecond		(1),

fnur14400BitsPerSecond		(2),

fnur19200BitsPerSecond		(3),

fnur28800BitsPerSecond		(4),

fnur38400BitsPerSecond		(5),

fnur48000BitsPerSecond		(6),

fnur56000BitsPerSecond		(7),

fnur64000BitsPerSecond		(8),

fnur33600BitsPerSecond		(9),

fnur32000BitsPerSecond		(10),

fnur31200BitsPerSecond		(11)

}

ForwardToNumber 		::= AddressString

FreeFormatData			::= OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FCI message

-- See TS 29.078 [217]

--

GenericNumber			::= BCDDirectoryNumber

GenericNumbers			::= SET OF GenericNumber

Gsm-SCFAddress			::= ISDN-AddressString

--

-- See TS 29.002 [214]

--

GuaranteedBitRate ::= ENUMERATED

{

gbr14400BitsPerSecond (1),		-- BS20 non-transparent

gbr28800BitsPerSecond (2),		-- BS20 non-transparent and transparent,

-- BS30 transparent and multimedia

gbr32000BitsPerSecond (3),		-- BS30 multimedia

gbr33600BitsPerSecond (4),		-- BS30 multimedia

gbr56000BitsPerSecond (5),		-- BS30 transparent and multimedia

gbr57600BitsPerSecond (6),		-- BS20 non-transparent

gbr64000BitsPerSecond (7)		-- BS30 transparent and multimedia

}

HLRIntResult			::= Diagnostics

HSCSDParmsChange		::= SEQUENCE

{

changeTime				[0] TimeStamp,

hSCSDChanAllocated		[1] NumOfHSCSDChanAllocated,

initiatingParty			[2] InitiatingParty OPTIONAL,

aiurRequested			[3] AiurRequested OPTIONAL,

chanCodingUsed			[4] ChannelCoding,

hSCSDChanRequested		[5] NumOfHSCSDChanRequested OPTIONAL

}

IMEICheckEvent			::= INTEGER

{

mobileOriginatedCall	(0),

mobileTerminatedCall	(1),

smsMobileOriginating	(2),

smsMobileTerminating	(3),

ssAction				(4),

locationUpdate		(5)

}

IMEIStatus				::= ENUMERATED

{

trackListedMobileEquipment		(0),

blockListedMobileEquipment		(1),

nonAllowListedMobileEquipment	(2)

}

IMSIorIMEI				::= CHOICE

{

imsi				[0] IMSI,

imei				[1] IMEI

}

InitiatingParty			::= ENUMERATED

{

network				(0),

subscriber			(1)

}

LocationCellExtension	::= BIT STRING (SIZE (12))

LocationChange			::= SEQUENCE

{

location			[0] LocationAreaAndCell,

changeTime		[1] TimeStamp

}

Location-info			::= SEQUENCE

{

mscNumber			[1] MscNo OPTIONAL,

location-area		[2] LocationAreaCode,

cell-identification	[3] CellId OPTIONAL,

mCC-MNC				[4] MCC-MNC OPTIONAL

}

LocUpdResult			::= Diagnostics

MaximumBitRate ::= ENUMERATED

{

mbr14400BitsPerSecond	(1),		-- BS20 non-transparent

mbr28800BitsPerSecond	(2),		-- BS20 non-transparent and transparent,

-- BS30 transparent and multimedia

mbr32000BitsPerSecond	(3),		-- BS30 multimedia

mbr33600BitsPerSecond	(4),		-- BS30 multimedia

mbr56000BitsPerSecond	(5),		-- BS30 transparent and multimedia

mbr57600BitsPerSecond	(6)		-- BS20 non-transparent

}

Month		::= INTEGER (1..12)

MSPowerClasses	::= SET OF RFPowerCapability

NetworkCallReference	::= CallReferenceNumber

--

-- See TS 29.002 [214]

--

NetworkSpecificCode		::= INTEGER

--

-- To be defined by network operator

--

NetworkSpecificServices	::= SET OF NetworkSpecificCode

NumOfHSCSDChanRequested		::= INTEGER

NumOfHSCSDChanAllocated		::= INTEGER

ObservedIMEITicketEnable	::= BOOLEAN

OriginalCalledNumber	::= BCDDirectoryNumber

OriginDestCombinations	::= SET OF OriginDestCombination

OriginDestCombination	::= SEQUENCE

--

-- Note that these values correspond to the contents

-- of the attributes originId and destinationId

-- respectively. At least one of the two must be present.

--

{

origin					[0] INTEGER OPTIONAL,

destination				[1] INTEGER OPTIONAL

}

PartialRecordTimer	::= INTEGER

PartialRecordType	::= ENUMERATED

{

timeLimit					(0),

serviceChange				(1),

locationChange				(2),

classmarkChange				(3),

aocParmChange				(4),

radioChannelChange			(5),

hSCSDParmChange				(6),

changeOfCAMELDestination	(7)

}

PartialRecordTypes		::= SET OF PartialRecordType

RadioChannelsRequested	::= SET OF RadioChanRequested

RadioChanRequested	::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

--

{

halfRateChannel				(0),

fullRateChannel				(1),

dualHalfRatePreferred		(2),

dualFullRatePreferred		(3)

}

RateIndication ::= OCTET STRING(SIZE(1))

ReasonForServiceChange	::= ENUMERATED

{

msubInitiated			(0),

mscInitiated				(1),

callSetupFallBack		(2),

callSetupChangeOrder		(3)

}

RecordClassDestination	::= CHOICE

{

osApplication	[0] AE-title,

fileType			[1] FileType

}

RecordClassDestinations	::= SET OF RecordClassDestination

RecordingMethod	::= ENUMERATED

{

inCallRecord		(0),

inSSRecord		(1)

}

RedirectingNumber	::= BCDDirectoryNumber

RFPowerCapability	::= INTEGER

--

-- This field contains the RF power capability of the Mobile station

-- classmark 1 and 2 of TS 24.008 [208] expressed as an integer.

--

RoamingNumber			::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

RoutingNumber			::= CHOICE

{

roaming				[1] RoamingNumber,

forwarded			[2] ForwardToNumber

}

Service						::= CHOICE

{

teleservice					[1] TeleserviceCode,

bearerService				[2] BearerServiceCode,

supplementaryService			[3] SS-Code,

networkSpecificService		[4] NetworkSpecificCode

}

ServiceDistanceDependencies	::= SET OF ServiceDistanceDependency

ServiceDistanceDependency	::= SEQUENCE

--

-- Note that these values correspond to the contents

-- of the attributes aocServiceId and zoneId

-- respectively.

--

{

aocService			[0] INTEGER,

chargingZone			[1] INTEGER OPTIONAL

}

SimpleIntegerName			::= INTEGER

SimpleStringName			::= GraphicString

SpeechVersionIdentifier		::= OCTET STRING (SIZE(1))

--

--	see GSM 08.08[313]

--

--	000 0001	GSM speech full rate version 1

--	001 0001	GSM speech full rate version 2	used for enhanced full rate

--	010 0001	GSM speech full rate version 3	for future use

--	000 0101	GSM speech half rate version 1

--	001 0101	GSM speech half rate version 2 	for future use

--	010 0101	GSM speech half rate version 3	for future use

--

SSActionResult				::= Diagnostics

SSActionType				::= ENUMERATED

{

registration				(0),

erasure					(1),

activation				(2),

deactivation				(3),

interrogation			(4),

invocation				(5),

passwordRegistration		(6)

}

SSParameters				::= CHOICE

{

forwardedToNumber	[0] ForwardToNumber,

unstructuredData		[1] OCTET STRING

}

SupplServices				::= SET OF SS-Code

SuppServiceUsed				::= SEQUENCE

{

ssCode					[0] SS-Code,

ssTime					[1] TimeStamp OPTIONAL

}

SwitchoverTime				::= SEQUENCE

{

hour					INTEGER (0..23),

minute					INTEGER (0..59),

second					INTEGER (0..59)

}

TariffId					::= INTEGER

TariffPeriod				::= SEQUENCE

--

-- Note that the value of tariffId corresponds to the attribute tariffId.

--

{

switchoverTime			[0] SwitchoverTime,

tariffId				[1] INTEGER

}

TariffPeriods				::= SET OF TariffPeriod

TariffSystemStatus			::= ENUMERATED

{

available				(0),	-- available for modification

checked					(1),	-- "frozen" and checked

standby					(2),	-- "frozen" awaiting activation

active					(3)		-- "frozen" and active

}

TrafficChannel			::=	ENUMERATED

{

fullRate			(0),

halfRate			(1)

}

TranslatedNumber		::= 	BCDDirectoryNumber

TransparencyInd			::=	ENUMERATED

{

transparent			(0),

nonTransparent		(1)

}

TrunkGroup				::=	 CHOICE

{

tkgpNumber		[0] INTEGER,

tkgpName			[1] GraphicString

}

TSChangeover			::=	SEQUENCE

--

-- Note that if the changeover time is not

-- specified then the change is immediate.

--

{

newActiveTS			[0] INTEGER,

newStandbyTS			[1] INTEGER,

changeoverTime		[2] GeneralizedTime OPTIONAL,

authkey				[3] OCTET STRING OPTIONAL,

checksum				[4] OCTET STRING OPTIONAL,

versionNumber		[5] OCTET STRING OPTIONAL

}

TSCheckError			::=	SEQUENCE

{

errorId				[0] TSCheckErrorId,

fail					ANY DEFINED BY errorId OPTIONAL

}

TSCheckErrorId			::=	CHOICE

{

globalForm			[0] OBJECT IDENTIFIER,

localForm			[1] INTEGER

}

TSCheckResult			::=	CHOICE

{

success				[0] NULL,

fail					[1] SET OF TSCheckError

}

TSCopyTariffSystem		::=	SEQUENCE

{

oldTS				[0] INTEGER,

newTS				[1] INTEGER

}

TSNextChange			::=	CHOICE

{

noChangeover			[0] NULL,

tsChangeover			[1] TSChangeover

}

TypeOfSubscribers		::= ENUMERATED

{

home					(0),	-- HPLMN subscribers

visiting				(1),	-- roaming subscribers

all	(2)

}

TypeOfTransaction		::=	ENUMERATED

{

successful			(0),

unsuccessful			(1),

all					(2)

}

Visited-Location-info		::= SEQUENCE

{

mscNumber			[1] MscNo,

vlrNumber			[2] VlrNo

}

VlrNo		::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

.#END

Third change

#### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS and EPC CDR types defined in TS 32.251 [11].

.$GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AddressString,

CallDuration,

CallingNumber,

CauseForRecClosing,

CellId,

ChargingID,

CivicAddressInformation,

Diagnostics,

DiameterIdentity,

DynamicAddressFlag,

EnhancedDiagnostics,

GSNAddress,

InvolvedParty,

IPAddress,

LCSCause,

LCSClientIdentity,

LCSQoSInfo,

LevelOfCAMELService,

LocalSequenceNumber,

LocationAreaAndCell,

LocationAreaCode,

ManagementExtensions,

MBMSInformation,

MessageReference,

MSISDN,

MSTimeZone,

NodeID,

PDPAddress,

PLMN-Id,

PositioningData,

PSCellInformation,

RATType,

RecordingEntity,

RecordType,

RoutingAreaCode,

SCSASAddress,

ServiceSpecificInfo,

SMSResult,

SmsTpDestinationNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

DefaultGPRS-Handling,

DefaultSMS-Handling,

NotificationToMSUser,

ServiceKey

FROM MAP-MS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version20 (20)}

-- from TS 29.002 [214]

IMEI,

IMSI,

ISDN-AddressString,

RAIdentity

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CommonDataTypes (18) version20 (20)}

-- from TS 29.002 [214]

CallReferenceNumber

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CH-DataTypes (13) version20 (20)}

-- from TS 29.002 [214]

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25) version20 (20)}

-- from TS 29.002 [214]

LocationMethod

FROM SS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version16 (16)}

-- from TS 24.080 [209]

;

--

--  GPRS RECORDS

--

GPRSRecord	::= CHOICE

--

-- Record values 20, 22..27 are specific

-- Record values 76, 77, 86 are MBMS specific

-- Record values 78,79 and 92, 95, 96 are EPC specific

--

{

sgsnPDPRecord			[20] SGSNPDPRecord,

sgsnMMRecord			[22] SGSNMMRecord,

sgsnSMORecord			[23] SGSNSMORecord,

sgsnSMTRecord			[24] SGSNSMTRecord,

sgsnMTLCSRecord			[25] SGSNMTLCSRecord,

sgsnMOLCSRecord			[26] SGSNMOLCSRecord,

sgsnNILCSRecord			[27] SGSNNILCSRecord,

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord,

sGWRecord				[78] SGWRecord,

pGWRecord				[79] PGWRecord,

gwMBMSRecord			[86] GWMBMSRecord,

tDFRecord				[92] TDFRecord,

iPERecord				[95] IPERecord,

ePDGRecord				[96] EPDGRecord,

tWAGRecord				[97] TWAGRecord

}

SGWRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

s-GWAddress					[4] GSNAddress,

chargingID					[5] ChargingID,

servingNodeAddress			[6] SEQUENCE OF GSNAddress,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servingNodePLMNIdentifier	[27] PLMN-Id OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

mSTimeZone 					[31] MSTimeZone OPTIONAL,

userLocationInformation		[32] OCTET STRING OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

servingNodeType				[35] SEQUENCE OF ServingNodeType,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 	[41] NULL OPTIONAL,

userCSGInformation			[42] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

lowPriorityIndicator		[44] NULL OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

s-GWiPv6Address				[48] GSNAddress OPTIONAL,

servingNodeiPv6Address		[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

userLocationInfoTime		[52] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[53] CNOperatorSelectionEntity OPTIONAL,

presenceReportingAreaInfo	[54] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation	[55] OCTET STRING OPTIONAL,

lastMSTimeZone				[56] MSTimeZone OPTIONAL,

enhancedDiagnostics			[57] EnhancedDiagnostics OPTIONAL,

cPCIoTEPSOptimisationIndicator [59] CPCIoTEPSOptimisationIndicator OPTIONAL,

uNIPDUCPOnlyFlag			[60] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl		[61] ServingPLMNRateControl OPTIONAL,

pDPPDNTypeExtension			[62] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter		[63] MOExceptionDataCounter OPTIONAL,

listOfRANSecondaryRATUsageReports [64] SEQUENCE OF RANSecondaryRATUsageReport OPTIONAL,

pSCellInformation			[65] PSCellInformation OPTIONAL

}

PGWRecord 	::= SET

--

--	List of traffic volumes is only applicable when Charging per IP-CAN session is active and

--	IP-CAN bearer charging is being performed for the session.

--

--	EPC QoS Information is only applicable when Charging per IP-CAN session is active.

--

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

chargingID						[5] ChargingID,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext				[25] NULL OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

cAMELChargingInformation		[33] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 		[42] NULL OPTIONAL,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

lowPriorityIndicator				[46] NULL OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

userLocationInfoTime				[53] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		    	[54] CNOperatorSelectionEntity OPTIONAL,

ePCQoSInformation					[55] EPCQoSInformation OPTIONAL,

presenceReportingAreaInfo			[56] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation			[57] OCTET STRING OPTIONAL,

lastMSTimeZone						[58] MSTimeZone OPTIONAL,

enhancedDiagnostics					[59] EnhancedDiagnostics OPTIONAL,

nBIFOMMode							[60] NBIFOMMode OPTIONAL,

nBIFOMSupport						[61] NBIFOMSupport OPTIONAL,

uWANUserLocationInformation			[62] UWANUserLocationInfo OPTIONAL,

sGiPtPTunnellingMethod				[64] SGiPtPTunnellingMethod OPTIONAL,

uNIPDUCPOnlyFlag					[65] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl				[66] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[67] APNRateControl OPTIONAL,

pDPPDNTypeExtension					[68] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter				[69] MOExceptionDataCounter OPTIONAL,

chargingPerIPCANSessionIndicator    [70] ChargingPerIPCANSessionIndicator OPTIONAL,

threeGPPPSDataOffStatus				[71] ThreeGPPPSDataOffStatus OPTIONAL,

sCSASAddress						[72] SCSASAddress OPTIONAL,

listOfRANSecondaryRATUsageReports	[73] SEQUENCE OF RANSecondaryRATUsageReport OPTIONAL

}

TDFRecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

tDFAddress							[53] GSNAddress,

tDFiPv6AddressUsed					[54] GSNAddress OPTIONAL,

tDFPLMNIdentifier					[55] PLMN-Id OPTIONAL,

servedFixedSubsID					[56] FixedSubsID OPTIONAL,

accessLineIdentifier				[57] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation		[59] FixedUserLocationInformation OPTIONAL

}

IPERecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

iPEdgeAddress					[4] GSNAddress,

chargingID						[5] ChargingID,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

iPCANsessionType				[8] PDPType OPTIONAL,

servedIPCANsessionAddress		[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI		   				[29] IMEI OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servedMNNAI						[36] SubscriptionID OPTIONAL,

iPEdgeOperatorIdentifier		[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

servedIPCANsessionAddressExt 	[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt			[47] DynamicAddressFlag OPTIONAL,

iPEdgeiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission   				[52] NULL OPTIONAL,

servedFixedSubsID				[55] FixedSubsID OPTIONAL,

accessLineIdentifier			[56] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation	[57] FixedUserLocationInformation OPTIONAL

}

EPDGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

ePDGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

ePDGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

uWANUserLocationInformation	[53] UWANUserLocationInfo OPTIONAL,

userLocationInfoTime		[54] TimeStamp OPTIONAL,

iMSIunauthenticatedFlag 	[55] NULL OPTIONAL

}

TWAGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

tWAGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

tWAGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

tWANUserLocationInformation	[53] TWANUserLocationInfo OPTIONAL,

iMSIunauthenticatedFlag 	[54] NULL OPTIONAL

}

SGSNMMRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

sgsnAddress					[3] GSNAddress OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

routingArea					[5] RoutingAreaCode OPTIONAL,

locationAreaCode			[6] LocationAreaCode OPTIONAL,

cellIdentifier				[7] CellId OPTIONAL,

changeLocation				[8] SEQUENCE OF ChangeLocation OPTIONAL,

recordOpeningTime			[9] TimeStamp,

duration					[10] CallDuration OPTIONAL,

sgsnChange					[11] SGSNChange OPTIONAL,

causeForRecClosing			[12] CauseForRecClosing,

diagnostics					[13] Diagnostics OPTIONAL,

recordSequenceNumber		[14] INTEGER OPTIONAL,

nodeID						[15] NodeID OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

localSequenceNumber			[17] LocalSequenceNumber OPTIONAL,

servedMSISDN				[18] MSISDN OPTIONAL,

chargingCharacteristics		[19] ChargingCharacteristics,

cAMELInformationMM 			[20] CAMELInformationMM OPTIONAL,

rATType						[21] RATType OPTIONAL,

chChSelectionMode			[22] ChChSelectionMode OPTIONAL,

cellPLMNId					[23] PLMN-Id OPTIONAL,

servingNodePLMNIdentifier	[24] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[25] CNOperatorSelectionEntity OPTIONAL

}

SGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

sgsnAddress					[5] GSNAddress OPTIONAL,

msNetworkCapability			[6] MSNetworkCapability OPTIONAL,

routingArea					[7] RoutingAreaCode OPTIONAL,

locationAreaCode			[8] LocationAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

chargingID					[10] ChargingID,

ggsnAddressUsed				[11] GSNAddress,

accessPointNameNI			[12] AccessPointNameNI OPTIONAL,

pdpType						[13] PDPType OPTIONAL,

servedPDPAddress			[14] PDPAddress OPTIONAL,

listOfTrafficVolumes		[15] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[16] TimeStamp,

duration					[17] CallDuration,

sgsnChange					[18] SGSNChange OPTIONAL,

causeForRecClosing			[19] CauseForRecClosing,

diagnostics					[20] Diagnostics OPTIONAL,

recordSequenceNumber		[21] INTEGER OPTIONAL,

nodeID						[22] NodeID OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[25] APNSelectionMode OPTIONAL,

accessPointNameOI			[26] AccessPointNameOI OPTIONAL,

servedMSISDN				[27] MSISDN OPTIONAL,

chargingCharacteristics		[28] ChargingCharacteristics,

rATType						[29] RATType OPTIONAL,

cAMELInformationPDP  		[30] CAMELInformationPDP OPTIONAL,

rNCUnsentDownlinkVolume		[31] DataVolumeGPRS OPTIONAL,

chChSelectionMode			[32] ChChSelectionMode OPTIONAL,

dynamicAddressFlag			[33] DynamicAddressFlag OPTIONAL,

iMSIunauthenticatedFlag 	[34] NULL OPTIONAL,

userCSGInformation			[35] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[36] PDPAddress OPTIONAL,

lowPriorityIndicator		[37] NULL OPTIONAL,

servingNodePLMNIdentifier	[38] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[39] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMORecord	::= SET

--

--   also for MME UE originated SMS record

--

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

serviceCentre				[5] AddressString OPTIONAL,

recordingEntity				[6] RecordingEntity OPTIONAL,

locationArea				[7] LocationAreaCode OPTIONAL,

routingArea					[8] RoutingAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

messageReference			[10] MessageReference,

eventTimeStamp				[11] TimeStamp,

smsResult					[12] SMSResult OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

nodeID						[14] NodeID OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		[16] ChargingCharacteristics,

rATType						[17] RATType OPTIONAL,

destinationNumber			[18] SmsTpDestinationNumber OPTIONAL,

cAMELInformationSMS			[19] CAMELInformationSMS OPTIONAL,

chChSelectionMode			[20] ChChSelectionMode OPTIONAL,

servingNodeType				[21] ServingNodeType,

servingNodeAddress			[22] GSNAddress OPTIONAL,

servingNodeiPv6Address		[23] GSNAddress OPTIONAL,

mMEName						[24] DiameterIdentity OPTIONAL,

mMERealm					[25] DiameterIdentity OPTIONAL,

userLocationInformation		[26] OCTET STRING OPTIONAL,

retransmission				[27] NULL OPTIONAL,

servingNodePLMNIdentifier	[28] PLMN-Id OPTIONAL,

userLocationInfoTime		[29] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[30] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMTRecord	::= SET

--

--   also for MME UE terminated SMS record

--

{

recordType					 [0] RecordType,

servedIMSI					 [1] IMSI,

servedIMEI					 [2] IMEI OPTIONAL,

servedMSISDN				 [3] MSISDN OPTIONAL,

msNetworkCapability			 [4] MSNetworkCapability OPTIONAL,

serviceCentre				 [5] AddressString OPTIONAL,

recordingEntity				 [6] RecordingEntity OPTIONAL,

locationArea				 [7] LocationAreaCode OPTIONAL,

routingArea					 [8] RoutingAreaCode OPTIONAL,

cellIdentifier				 [9] CellId OPTIONAL,

eventTimeStamp				 [10] TimeStamp,

smsResult					 [11] SMSResult OPTIONAL,

recordExtensions			 [12] ManagementExtensions OPTIONAL,

nodeID						 [13] NodeID OPTIONAL,

localSequenceNumber			 [14] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [15] ChargingCharacteristics,

rATType						 [16] RATType OPTIONAL,

chChSelectionMode			 [17] ChChSelectionMode OPTIONAL,

cAMELInformationSMS			 [18] CAMELInformationSMS OPTIONAL,

originatingAddress			 [19] AddressString OPTIONAL,

servingNodeType				 [20] ServingNodeType,

servingNodeAddress			 [21] GSNAddress OPTIONAL,

servingNodeiPv6Address		 [22] GSNAddress OPTIONAL,

mMEName						 [23] DiameterIdentity OPTIONAL,

mMERealm					 [24] DiameterIdentity OPTIONAL,

userLocationInformation		 [25] OCTET STRING OPTIONAL,

retransmission				 [26] NULL OPTIONAL,

servingNodePLMNIdentifier	 [27] PLMN-Id OPTIONAL,

userLocationInfoTime		 [28] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMTLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType,

lcsClientIdentity			 [3] LCSClientIdentity,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationType				 [7] LocationType,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

notificationToMSUser		 [13] NotificationToMSUser OPTIONAL,

privacyOverride				 [14] NULL OPTIONAL,

location					 [15] LocationAreaAndCell OPTIONAL,

routingArea					 [16] RoutingAreaCode OPTIONAL,

locationEstimate			 [17] Ext-GeographicalInformation OPTIONAL,

positioningData				 [18] PositioningData OPTIONAL,

lcsCause					 [19] LCSCause OPTIONAL,

diagnostics					 [20] Diagnostics OPTIONAL,

nodeID						 [21] NodeID OPTIONAL,

localSequenceNumber			 [22] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [23] ChargingCharacteristics,

chChSelectionMode			 [24] ChChSelectionMode OPTIONAL,

rATType						 [25] RATType OPTIONAL,

recordExtensions			 [26] ManagementExtensions OPTIONAL,

causeForRecClosing			 [27] CauseForRecClosing,

servingNodePLMNIdentifier	 [28] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMOLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationMethod				 [7] LocationMethod,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNNILCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI OPTIONAL,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

servedIMEI					 [7] IMEI OPTIONAL,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofRAs					[3] SEQUENCE OF RAIdentity OPTIONAL,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

sgsnPLMNIdentifier			[15] PLMN-Id OPTIONAL,

numberofReceivingUE			[16] INTEGER OPTIONAL,

mbmsInformation				[17] MBMSInformation OPTIONAL

}

GGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

mbmsInformation				[15] MBMSInformation OPTIONAL

}

GWMBMSRecord	::= SET

{

recordType					[0] RecordType,

mbmsGWAddress				[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

pdpPDNType					[5] PDPType OPTIONAL,

servedPDPPDNAddress			[6] PDPAddress OPTIONAL,

listOfTrafficVolumes		[7] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[8] TimeStamp,

duration					[9] CallDuration,

causeForRecClosing			[10] CauseForRecClosing,

diagnostics					[11] Diagnostics OPTIONAL,

recordSequenceNumber		[12] INTEGER OPTIONAL,

nodeID						[13] NodeID OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

commonTeid   				[17] CTEID OPTIONAL,

iPMulticastSourceAddress	[18] PDPAddress OPTIONAL

}

--

--  PS DATA TYPES

--

AccessAvailabilityChangeReason		::= INTEGER (0..4294967295)

--

-- 0 (RAN rule indication)   : This value shall be used to indicate that the availability

--                             of an access is changed due to the RAN rule indication.

-- 1 (Access usable/unusable): This value shall be used to indicate that the availability

--                             of an access is changed due to the access is unusable or usable

--                             again.

--

AccessLineIdentifier	::= SEQUENCE

--

-- "Physical Access Id" includes a port identifier and the identity of the access node where the

--  port resides. "logical Access Id" contains a Circuit-ID. Both are defined ETSI TS 283 034 [314]

--

{

physicalAccessID	[0] UTF8String OPTIONAL,

logicalAccessID		[1] OCTET STRING OPTIONAL

}

AccessPointNameNI	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of APN in  dot representation.

-- For example, if the complete APN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- NI is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

AccessPointNameOI	::= IA5String (SIZE(1..37))

--

-- Operator Identifier part of APN in dot representation.

-- In the 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs' example, the OI portion is 'mnc022.mcc111.gprs'

-- and is presented in this form in the CDR.

--

ADCRuleBaseName		::= IA5String

--

-- identifier for the group of charging rules

-- see ADC-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

AdditionalExceptionReports		::= ENUMERATED

{

notAllowed		(0),

allowed			(1)

}

AFChargingIdentifier	::= OCTET STRING

--

-- see AF-Charging-Identifier AVP as defined in TS 29.214[221]

--

AFRecordInformation	::= SEQUENCE

{

aFChargingIdentifier		[1] AFChargingIdentifier,

flows						[2] Flows OPTIONAL

}

APNRateControl		::= SEQUENCE

--

-- See TS 24.008 [208] for more information

--

{

aPNRateControlUplink	[0] APNRateControlParameters OPTIONAL,

aPNRateControlDownlink	[1] APNRateControlParameters OPTIONAL

}

APNRateControlParameters		::= SEQUENCE

{

additionalExceptionReports	[0] AdditionalExceptionReports OPTIONAL,

rateControlTimeUnit			[1] RateControlTimeUnit OPTIONAL,

rateControlMaxRate			[2] INTEGER OPTIONAL,

rateControlMaxMessageSize	[3] DataVolumeGPRS OPTIONAL -- aPNRateControlDownlink only

}

APNSelectionMode	::= ENUMERATED

--

-- See Information Elements TS 29.060 [215], TS 29.274 [223] or TS 29.275 [224]

--

{

mSorNetworkProvidedSubscriptionVerified				(0),

mSProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

CalleePartyInformation	::= SEQUENCE

{

called-Party-Address				[0] InvolvedParty OPTIONAL,

requested-Party-Address				[1] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[2] SEQUENCE OF InvolvedParty OPTIONAL

}

CAMELAccessPointNameNI	::= AccessPointNameNI

CAMELAccessPointNameOI	::= AccessPointNameOI

CAMELInformationMM		::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

numberOfDPEncountered  			[4] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[5] LevelOfCAMELService OPTIONAL,

freeFormatData					[6] FreeFormatData OPTIONAL,

fFDAppendIndicator				[7] FFDAppendIndicator OPTIONAL

}

CAMELInformationPDP	::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

cAMELAccessPointNameNI			[4] CAMELAccessPointNameNI OPTIONAL,

cAMELAccessPointNameOI			[5] CAMELAccessPointNameOI OPTIONAL,

numberOfDPEncountered			[6] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[7] LevelOfCAMELService OPTIONAL,

freeFormatData					[8] FreeFormatData OPTIONAL,

fFDAppendIndicator				[9] FFDAppendIndicator OPTIONAL

}

CAMELInformationSMS		::= SET

{

sCFAddress								[1] SCFAddress OPTIONAL,

serviceKey								[2] ServiceKey OPTIONAL,

defaultSMSHandling						[3] DefaultSMS-Handling OPTIONAL,

cAMELCallingPartyNumber					[4] CallingNumber OPTIONAL,

cAMELDestinationSubscriberNumber		[5] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[6] AddressString OPTIONAL,

freeFormatData							[7] FreeFormatData OPTIONAL,

smsReferenceNumber						[8] CallReferenceNumber	OPTIONAL

}

ChangeCondition	::= ENUMERATED

{

qoSChange						(0),

tariffTime						(1),

recordClosure					(2),

cGI-SAICHange					(6),	-- bearer modification. "CGI-SAI Change"

rAIChange						(7),	-- bearer modification. "RAI Change"

dT-Establishment				(8),

dT-Removal						(9),

eCGIChange						(10),	-- bearer modification. "ECGI Change"

tAIChange						(11),	-- bearer modification. "TAI Change"

userLocationChange				(12),	-- bearer modification. "User Location Change"

userCSGInformationChange		(13), 	-- bearer modification. "User CSG info Change"

presenceInPRAChange				(14),	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

removalOfAccess					(15),	-- NBIFOM "Removal of Access"

unusabilityOfAccess				(16),	-- NBIFOM "Unusability of Access"

indirectChangeCondition			(17),	-- NBIFOM "Indirect Change Condition"

userPlaneToUEChange				(18),	-- bearer modification. "Change of user plane to UE"

servingPLMNRateControlChange	(19),

-- bearer modification “Serving PLMN Rate Control Change"

threeGPPPSDataOffStatusChange	(20),   -- "Change of 3GPP PS DataO ff Status"

aPNRateControlChange			(21)	-- bearer modification. "APN Rate ControlChange"

}

ChangeOfCharCondition	::= SEQUENCE

--

-- qosRequested and qosNegotiated are used in S-CDR only

-- ePCQoSInformation used in SGW-CDR,PGW-CDR, IPE-CDR, TWAG-CDR and ePDG-CDR only

-- userLocationInformation is used only in S-CDR, SGW-CDR and PGW-CDR

-- chargingID used in PGW-CDR only when Charging per IP-CAN session is active

-- accessAvailabilityChangeReason and relatedChangeOfCharCondition applicable only in PGW-CDR

-- cPCIoTOptimisationIndicator is used in SGW-CDR only

-- aPNRateControl is valid for PGW-CDR only

--

{

qosRequested					[1] QoSInformation OPTIONAL,

qosNegotiated							[2] QoSInformation OPTIONAL,

dataVolumeGPRSUplink					[3] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink					[4] DataVolumeGPRS OPTIONAL,

changeCondition							[5] ChangeCondition,

changeTime								[6] TimeStamp,

userLocationInformation					[8] OCTET STRING OPTIONAL,

ePCQoSInformation						[9] EPCQoSInformation OPTIONAL,

chargingID								[10] ChargingID OPTIONAL,

presenceReportingAreaStatus 			[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation						[12] UserCSGInformation OPTIONAL,

diagnostics								[13] Diagnostics OPTIONAL,

enhancedDiagnostics						[14] EnhancedDiagnostics OPTIONAL,

rATType									[15] RATType OPTIONAL,

accessAvailabilityChangeReason			[16] AccessAvailabilityChangeReason OPTIONAL,

uWANUserLocationInformation				[17] UWANUserLocationInfo OPTIONAL,

relatedChangeOfCharCondition			[18] RelatedChangeOfCharCondition OPTIONAL,

cPCIoTEPSOptimisationIndicator			[19] CPCIoTEPSOptimisationIndicator OPTIONAL,

servingPLMNRateControl					[20] ServingPLMNRateControl OPTIONAL,

threeGPPPSDataOffStatus					[21] ThreeGPPPSDataOffStatus OPTIONAL,

listOfPresenceReportingAreaInformation	[22] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL,

aPNRateControl							[23] APNRateControl OPTIONAL

}

ChangeOfMBMSCondition	::= SEQUENCE

--

-- Used in MBMS record

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeMBMSUplink		[3] DataVolumeMBMS OPTIONAL,

dataVolumeMBMSDownlink		[4] DataVolumeMBMS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL

}

ChangeOfServiceCondition	::= SEQUENCE

--

-- Used for Flow based Charging and Application based Charging service data container

-- presenceReportingAreaStatus is used in PGW-CDR Only

--

{

ratingGroup 						[1] RatingGroupId,

chargingRuleBaseName				[2] ChargingRuleBaseName OPTIONAL,

resultCode							[3] ResultCode OPTIONAL,

localSequenceNumber					[4] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage					[5] TimeStamp OPTIONAL,

timeOfLastUsage						[6] TimeStamp OPTIONAL,

timeUsage 							[7] CallDuration OPTIONAL,

serviceConditionChange				[8] ServiceConditionChange,

qoSInformationNeg					[9] EPCQoSInformation OPTIONAL,

servingNodeAddress 					[10] GSNAddress OPTIONAL,

datavolumeFBCUplink					[12] DataVolumeGPRS OPTIONAL,

datavolumeFBCDownlink				[13] DataVolumeGPRS OPTIONAL,

timeOfReport						[14] TimeStamp,

failureHandlingContinue				[16] FailureHandlingContinue OPTIONAL,

serviceIdentifier					[17] ServiceIdentifier OPTIONAL,

pSFurnishChargingInformation		[18] PSFurnishChargingInformation OPTIONAL,

aFRecordInformation					[19] SEQUENCE OF AFRecordInformation OPTIONAL,

userLocationInformation				[20] OCTET STRING OPTIONAL,

eventBasedChargingInformation		[21] EventBasedChargingInformation OPTIONAL,

timeQuotaMechanism					[22] TimeQuotaMechanism OPTIONAL,

serviceSpecificInfo					[23] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

sponsorIdentity						[25] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[26] OCTET STRING OPTIONAL,

aDCRuleBaseName						[27] ADCRuleBaseName OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedChangeOfServiceCondition		[33] RelatedChangeOfServiceCondition OPTIONAL,

servingPLMNRateControl				[35] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[36] APNRateControl OPTIONAL,

threeGPPPSDataOffStatus             [37] ThreeGPPPSDataOffStatus OPTIONAL,

trafficSteeringPolicyIDDownlink     [38] TrafficSteeringPolicyIDDownlink OPTIONAL,

trafficSteeringPolicyIDUplink       [39] TrafficSteeringPolicyIDUplink OPTIONAL,

tWANUserLocationInformation			[40] TWANUserLocationInfo OPTIONAL,

listOfPresenceReportingAreaInformation	[41] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL,

voLTEInformation						[42] VoLTEInformation OPTIONAL

}

ChangeLocation	::= SEQUENCE

--

-- used in SGSNMMRecord only

--

{

locationAreaCode		[0] LocationAreaCode,

routingAreaCode			[1] RoutingAreaCode,

cellId					[2] CellId OPTIONAL,

changeTime				[3] TimeStamp,

mCC-MNC					[4] PLMN-Id OPTIONAL

}

ChargingCharacteristics	::= OCTET STRING (SIZE(2))

ChargingPerIPCANSessionIndicator	::= ENUMERATED

{

inactive		(0),

active               (1)

}

ChargingRuleBaseName	::= IA5String

--

-- identifier for the group of charging rules

-- see Charging-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

ChChSelectionMode		::= ENUMERATED

{

servingNodeSupplied			(0),	-- For S-GW/P-GW

subscriptionSpecific		(1),	-- For SGSN only

aPNSpecific					(2),	-- For SGSN only

homeDefault					(3),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

roamingDefault				(4),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

visitingDefault				(5),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

fixedDefault				(6)		-- For TDF and IP-Edge

}

CNOperatorSelectionEntity	::= ENUMERATED

{

servCNSelectedbyUE		(0),

servCNSelectedbyNtw		(1)

}

CPCIoTEPSOptimisationIndicator ::= BOOLEAN

CSGAccessMode		::= ENUMERATED

{

closedMode  (0),

hybridMode  (1)

}

CSGId		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 23.003 [200]. Coded according to TS 29.060 [215] for GTP, and

-- in TS 29.274 [223] for eGTP.

--

CTEID		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 32.251[11] for MBMS-GW-CDR. Common Tunnel Endpoint Identifier

-- of MBMS GW for user plane, defined in TS 23.246 [207].

--

DataVolumeGPRS		::= INTEGER

--

-- The volume of data transferred in octets.

--

DataVolumeMBMS ::= INTEGER

--

-- The volume of data transferred in octets.

--

EPCQoSInformation	::= SEQUENCE

--

-- See TS 29.212 [220] for more information

--

{

qCI							[1] INTEGER,

maxRequestedBandwithUL		[2] INTEGER OPTIONAL,

maxRequestedBandwithDL		[3] INTEGER OPTIONAL,

guaranteedBitrateUL			[4] INTEGER OPTIONAL,

guaranteedBitrateDL			[5] INTEGER OPTIONAL,

aRP							[6] INTEGER OPTIONAL,

aPNAggregateMaxBitrateUL	[7] INTEGER OPTIONAL,

aPNAggregateMaxBitrateDL	[8] INTEGER OPTIONAL,

extendedMaxRequestedBWUL	[9] INTEGER OPTIONAL,

extendedMaxRequestedBWDL	[10] INTEGER OPTIONAL,

extendedGBRUL				[11] INTEGER OPTIONAL,

extendedGBRDL				[12] INTEGER OPTIONAL,

extendedAPNAMBRUL			[13] INTEGER OPTIONAL,

extendedAPNAMBRDL			[14] INTEGER OPTIONAL

}

EventBasedChargingInformation		::= SEQUENCE

{

numberOfEvents		[1] INTEGER,

eventTimeStamps  	[2] SEQUENCE OF TimeStamp OPTIONAL

}

FailureHandlingContinue		::= BOOLEAN

--

-- This parameter is included when the failure handling procedure has been executed and new

-- containers are opened. This parameter shall be included in the first and subsequent

-- containers opened after the failure handling execution.

--

FFDAppendIndicator	::= BOOLEAN

FixedSubsID	::= OCTET STRING

--

-- The fixed subscriber Id identifier is defined in Broadband Forum TR 134 [601].

--

FixedUserLocationInformation	::= SEQUENCE

--

-- See format in IEEE Std 802.11-2012 [408] for "SSID" and "BSSID".

--

{

sSID					[0] OCTET STRING OPTIONAL ,

bSSID					[1] OCTET STRING OPTIONAL,

accessLineIdentifier	[2] AccessLineIdentifier OPTIONAL

}

Flows		::= 	SEQUENCE

--

-- See Flows AVP as defined in TS 29.214 [221]

--

{

mediaComponentNumber	[1] INTEGER,

flowNumber  			[2] SEQUENCE OF INTEGER OPTIONAL

}

FreeFormatData		::=	OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FurnishChargingInformationGPRS

-- see TS 29.078 [217]

--

GSNAddress		::= IPAddress

MOExceptionDataCounter		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

counterValue		[0] INTEGER,

counterTimestamp	[1] TimeStamp

}

MSNetworkCapability	::= OCTET STRING (SIZE(1..8))

--

-- see TS 24.008 [208]

--

NBIFOMMode		::= ENUMERATED

{

uEINITIATED				(0),

nETWORKINITIATED		(1)

}

NBIFOMSupport 	::= ENUMERATED

{

nBIFOMNotSupported			(0),

nBIFOMSupported				(1)

}

NetworkInitiatedPDPContext		::= BOOLEAN

--

-- Set to true if PDP context was initiated from network side

--

NumberOfDPEncountered  ::= INTEGER

PDPType		::= OCTET STRING (SIZE(2))

--

-- OCTET 1: PDP Type Organization

-- OCTET 2: PDP/PDN Type Number

-- See TS 29.060 [215] for encoding details.

--

PDPPDNTypeExtension	::= INTEGER

--

-- This integer is 1:1 copy of the PDP type value as defined in TS 29.061 [215].

--

PresenceReportingAreaElementsList ::= OCTET STRING

--

-- For EPC see Presence-Reporting-Area-Elements-List AVP defined in TS 29.212 [220]

-- For 5GC see PresenceInfo defined in TS 29.571 [249] excluding praId and presenceState

--

PresenceReportingAreaInfo	::= SEQUENCE

{

presenceReportingAreaIdentifier	 [0] OCTET STRING,

presenceReportingAreaStatus		 [1] PresenceReportingAreaStatus OPTIONAL,

presenceReportingAreaElementsList[2] PresenceReportingAreaElementsList OPTIONAL,

presenceReportingAreaNode		 [3] PresenceReportingAreaNode OPTIONAL

}

PresenceReportingAreaNode ::= BIT STRING

{

oCS   		 (0),

pCRF  		 (1)

}

--

--	This bit mask has the same format as Presence-Reporting-Area-Node AVP in TS 29.212 [220]

--

PresenceReportingAreaStatus ::= ENUMERATED

{

insideArea   (0),

outsideArea  (1),

inactive 	 (2),

unknown      (3)

}

PSFurnishChargingInformation		::= SEQUENCE

{

pSFreeFormatData		[1] FreeFormatData,

pSFFDAppendIndicator	[2] FFDAppendIndicator OPTIONAL

}

QoSInformation	::= OCTET STRING (SIZE (4..255))

--

-- This  octet string

-- is a 1:1 copy of the contents (i.e. starting with octet 5) of the "Bearer Quality of

-- Service" information element specified in TS 29.274 [223].

--

RANSecondaryRATUsageReport	::= SEQUENCE

--

{

dataVolumeUplink				[1] DataVolumeGPRS,

dataVolumeDownlink				[2] DataVolumeGPRS,

rANStartTime					[3] TimeStamp,

rANEndTime						[4] TimeStamp,

secondaryRATType				[5] SecondaryRATType OPTIONAL,

chargingID						[6] ChargingID OPTIONAL

}

RateControlTimeUnit ::= INTEGER

{	unrestricted	(0),

minute			(1),

hour			(2),

day				(3),

week			(4)

}

RatingGroupId		::= INTEGER

--

-- IP service flow identity (DCCA), range of 4 byte (0... 4294967295)

-- see Rating-Group AVP as used in TS 32.299 [50]

--

RelatedChangeOfCharCondition	::= SEQUENCE

{

changeCondition					[5] ChangeCondition,

changeTime						[6] TimeStamp,

userLocationInformation			[8] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 	[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation				[12] UserCSGInformation OPTIONAL,

rATType							[15] RATType OPTIONAL,

uWANUserLocationInformation		[17] UWANUserLocationInfo OPTIONAL

}

RelatedChangeOfServiceCondition	::= SEQUENCE

{

userLocationInformation				[20] OCTET STRING OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedServiceConditionChange		[33] ServiceConditionChange OPTIONAL

}

ResultCode		::= INTEGER

--

-- charging protocol return value, range of 4 byte (0... 4294967295)

-- see Result-Code AVP as used in 32.299 [40]

--

SecondaryRATType	::= INTEGER

{

nR				(0)		-- New Radio 5G

}

ServiceConditionChange	::= BIT STRING

{

qoSChange 								 (0),	-- bearer modification

sGSNChange 								 (1),	-- bearer modification:

-- apply to Gn-SGSN /SGW Change

sGSNPLMNIDChange 						 (2),	-- bearer modification

tariffTimeSwitch 						 (3),	-- tariff time change

pDPContextRelease 						 (4),	-- bearer release

rATChange 								 (5),	-- bearer modification

serviceIdledOut 						 (6),	-- IP flow idle out, DCCA QHT expiry

reserved 								 (7),	-- old: QCTexpiry is no report event

configurationChange 					 (8),	-- configuration change

serviceStop 							 (9),	-- IP flow termination.From "Service Stop" in

-- Change-Condition AVP

dCCATimeThresholdReached 				(10),	-- DCCA quota reauthorization

dCCAVolumeThresholdReached 				(11),	-- DCCA quota reauthorization

dCCAServiceSpecificUnitThresholdReached	(12),	-- DCCA quota reauthorization

dCCATimeExhausted 						(13),	-- DCCA quota reauthorization

dCCAVolumeExhausted 					(14),	-- DCCA quota reauthorization

dCCAValidityTimeout 					(15),	-- DCCA quota validity time (QVT expiry)

reserved1								(16),	-- reserved due to no use case,

-- old: return Requested is covered by (17),(18)

dCCAReauthorisationRequest 				(17),	-- DCCA quota reauthorization request by OCS

dCCAContinueOngoingSession 				(18),	-- DCCA failure handling (CCFH),

-- continue IP flow

dCCARetryAndTerminateOngoingSession		(19),	-- DCCA failure handling (CCFH),

-- terminate IP flow after DCCA retry

dCCATerminateOngoingSession 			(20),	-- DCCA failure handling,

-- terminate IP flow

cGI-SAIChange							(21),	-- bearer modification. "CGI-SAI Change"

rAIChange								(22),	-- bearer modification. "RAI Change"

dCCAServiceSpecificUnitExhausted		(23),	-- DCCA quota reauthorization

recordClosure							(24),	-- PGW-CDR closure

timeLimit								(25),	-- intermediate recording. From "Service Data

--	Time Limit" Change-Condition AVP value

volumeLimit								(26),	-- intermediate recording.From "Service Data

--	Volume Limit" Change-Condition AVP value

serviceSpecificUnitLimit				(27),	-- intermediate recording

envelopeClosure 						(28),

eCGIChange								(29),	-- bearer modification. "ECGI Change"

tAIChange								(30),	-- bearer modification. "TAI Change"

userLocationChange						(31),	-- bearer modification. "User Location Change"

userCSGInformationChange				(32),	-- bearer modification. "User CSG info Change"

presenceInPRAChange						(33), 	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

accessChangeOfSDF						(34),	-- "access change of service data flow"

indirectServiceConditionChange			(35),	-- NBIFOM: "indirect service condition change"

servingPLMNRateControlChange			(36), 	-- bearer modification. "Serving PLMNRate

-- Control Change"

aPNRateControlChange					(37)	-- bearer modification. "APN Rate ControlChange

}

--

-- Trigger and cause values for IP flow level recording are defined for support of independent

-- online and offline charging and also for tight interworking between online and offline charging.

-- Unused bits will always be zero.

-- Some of the values are non-exclusive (e.g. bearer modification reasons).

--

SCFAddress	::= AddressString

--

-- See TS 29.002 [214]

--

ServiceIdentifier	::= INTEGER (0..4294967295)

--

-- The service identifier is used to identify the service or the service component

-- the service data flow relates to. See Service-Identifier AVP as defined in TS 29.212 [220]

--

ServingNodeType	::= ENUMERATED

{

sGSN		(0),

pMIPSGW		(1),

gTPSGW		(2),

ePDG		(3),

hSGW		(4),

mME			(5),

tWAN		(6)

}

ServingPLMNRateControl		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

sPLMNDLRateControlValue	[0] INTEGER,

sPLMNULRateControlValue	[1] INTEGER

}

SGiPtPTunnellingMethod		::= ENUMERATED

{

uDPIPbased		(0),

others			(1)

}

SGSNChange	::= BOOLEAN

--

-- present if first record after inter SGSN routing area update in new SGSN

--

SGWChange		::= BOOLEAN

--

-- present if first record after inter serving node change (SGW, ePDG, TWAG, HSGW)

--

TimeQuotaMechanism		::= SEQUENCE

{

timeQuotaType					[1] TimeQuotaType,

baseTimeInterval				[2] INTEGER

}

TimeQuotaType		::= ENUMERATED

{

dISCRETETIMEPERIOD			(0),

cONTINUOUSTIMEPERIOD		(1)

}

TrafficSteeringPolicyIDDownlink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-DL AVP as defined in TS 29.212[220]

TrafficSteeringPolicyIDUplink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-UL AVP as defined in TS 29.212[220]

TWANUserLocationInfo		::= SEQUENCE

{

sSID					 [0] OCTET STRING,			-- see format in IEEE Std 802.11-2012 [408]

bSSID					 [1] OCTET STRING OPTIONAL, 	-- see format in IEEE Std 802.11-2012 [408]

civicAddressInformation	[2] CivicAddressInformation OPTIONAL,

wLANOperatorId			[3] WLANOperatorId OPTIONAL,

logicalAccessID			[4] OCTET STRING OPTIONAL

}

UNIPDUCPOnlyFlag	::= BOOLEAN

UserCSGInformation		::= SEQUENCE

{

cSGId						[0] CSGId,

cSGAccessMode				[1] CSGAccessMode,

cSGMembershipIndication		[2] NULL OPTIONAL

}

UWANUserLocationInfo		::= SEQUENCE

{

uELocalIPAddress		 [0] IPAddress,

uDPSourcePort			 [1] OCTET STRING (SIZE(2)) OPTIONAL,

sSID					 [2] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

bSSID					 [3] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

tCPSourcePort			[4] OCTET STRING (SIZE(2)) OPTIONAL,

civicAddressInformation	[5] CivicAddressInformation OPTIONAL,

wLANOperatorId			[6] WLANOperatorId OPTIONAL,

logicalAccessID			[7] OCTET STRING OPTIONAL

}

VoLTEInformation ::= SEQUENCE

{

callerInformation	[0] SEQUENCE OF InvolvedParty OPTIONAL,

calleeInformation	[1] CalleePartyInformation OPTIONAL

}

WLANOperatorId ::= SEQUENCE

{

wLANOperatorName	[0] OCTET STRING,

wLANPLMNId			[1] PLMN-Id

}

.#END

Fourth change

#### 5.2.2.4	CP data transfer domain CDRs

This subclause contains the abstract syntax definitions that are specific to the CP data transfer CDR types defined in TS 32.253 [13].

.$CPDTChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) cpdtChargingDataTypes (13) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

ChargingID,

DiameterIdentity,

Diagnostics,

LocalSequenceNumber,

ManagementExtensions,

MSISDN,

NodeID,

PLMN-Id,

RATType,

RecordType,

ServiceContextID,

SubscriptionID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

IMEI,

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CommonDataTypes (18) version20 (20)}

-- from TS 29.002 [214]

AccessPointNameNI,

APNRateControl,

ChargingCharacteristics,

ChChSelectionMode,

DataVolumeGPRS,

ServingPLMNRateControl

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

;

--

#### -- CP data transfer RECORDS

--

CPDTRecord		::= CHOICE

--

-- Record values 105 to 106 are CP data transfer specific

--

{

cPDTSCERecord			[105] 	CPDTSCERecord,

cPDTSNNRecord			[106] 	CPDTSNNRecord

}

CPDTSCERecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

servedIMSI						[2] IMSI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

chargingID						[4] ChargingID,

serviceContextID				[5] ServiceContextID OPTIONAL,

nodeID							[6] NodeID OPTIONAL,

recordOpeningTime				[7] TimeStamp,

duration						[8] CallDuration,

accessPointNameNI				[9] AccessPointNameNI OPTIONAL,

sCEFID							[10] DiameterIdentity OPTIONAL,

chargingCharacteristics			[11] ChargingCharacteristics,

chChSelectionMode				[12] ChChSelectionMode OPTIONAL,

servingNodeIdentity				[13] DiameterIdentity OPTIONAL,

servingPLMNRateControl			[14] ServingPLMNRateControl OPTIONAL,

listOfNIDDsubmission			[15] SEQUENCE OF NIDDsubmission OPTIONAL,

causeForRecClosing				[16] CPCauseForRecClosing,

diagnostics						[17] Diagnostics OPTIONAL,

localSequenceNumber				[18] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[19] INTEGER OPTIONAL,

recordExtensions				[20] ManagementExtensions OPTIONAL,

externalIdentifier				[21] SubscriptionID OPTIONAL,

aPNRateControl					[22] APNRateControl OPTIONAL,

rATType							[23] RATType OPTIONAL,

servingNodePLMNIdentifier		[24] PLMN-Id OPTIONAL,

servedIMEI						[25] IMEI OPTIONAL

}

CPDTSNNRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

servedIMSI						[2] IMSI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

chargingID						[4] ChargingID,

serviceContextID				[5] ServiceContextID OPTIONAL,

nodeID							[6] NodeID OPTIONAL,

recordOpeningTime				[7] TimeStamp,

duration						[8] CallDuration,

accessPointNameNI				[9] AccessPointNameNI OPTIONAL,

sCEFID							[10] DiameterIdentity OPTIONAL,

chargingCharacteristics			[11] ChargingCharacteristics,

chChSelectionMode				[12] ChChSelectionMode OPTIONAL,

servingNodeIdentity				[13] DiameterIdentity OPTIONAL,

servingPLMNRateControl			[14] ServingPLMNRateControl OPTIONAL,

listOfNIDDsubmission			[15] SEQUENCE OF NIDDsubmission OPTIONAL,

causeForRecClosing				[16] CPCauseForRecClosing,

diagnostics						[17] Diagnostics OPTIONAL,

localSequenceNumber				[18] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[19] INTEGER OPTIONAL,

recordExtensions				[20] ManagementExtensions OPTIONAL,

externalIdentifier				[21] SubscriptionID OPTIONAL,

aPNRateControl					[22] APNRateControl OPTIONAL,

rATType							[23] RATType OPTIONAL,

servingNodePLMNIdentifier		[24] PLMN-Id OPTIONAL,

servedIMEI						[25] IMEI OPTIONAL

}

--

#### -- CP data transfer DATA TYPES

--

--

#### -- C

--

CPCauseForRecClosing	::= INTEGER

{

normalRelease					(0),

abnormalRelease					(1),

volumeLimit						(2),

timeLimit						(3),

maxNIDDsubmissions				(4),

servingNodeChange				(5),

pLMNChange						(6),

servingPLMNRateControlChange	(7),

aPNRateControlChange			(8),

rATTypeChange					(9),

managementIntervention			(10)

}

--

#### -- N

--

NIDDsubmission			::= SEQUENCE

{

submissionTimestamp			[0] TimeStamp OPTIONAL,

eventTimestamp				[1] TimeStamp OPTIONAL,

dataVolumeGPRSUplink		[2] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink		[3] DataVolumeGPRS OPTIONAL,

submissionResultCode		[4] SubmissionResultCode OPTIONAL,

serviceChangeCondition		[5] ServiceChangeCondition OPTIONAL

}

--

#### -- S

--

ServiceChangeCondition	::= BIT STRING

{

nIDDsubmissionResponseReceipt		(0),

nIDDsubmissionResponseSending	(1),

nIDDdeliveryToUE				(2),

nIDDdeliveryFromUEerror			(3),

nIDDsubmissionTimeout			(4)

}

SubmissionResultCode		::= INTEGER

--

-- Result-Code AVP and Experimental-Result AVP Values as specified in TS 29.128 [244]

-- for MO/MT data transfer

--

.#END

Fifth change

#### 5.2.4.2	LCS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.271 [31].

.$LCSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) lcsChargingDataTypes (6) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

IPAddress,

LCSClientIdentity,

LocalSequenceNumber,

ManagementExtensions,

MSISDN,

PLMN-Id,

PositioningData,

RecordingEntity,

RecordType,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25) version20 (20)}

-- from TS 29.002 [214]

AddressString,

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version20 (20)}

-- from TS 29.002 [214]

;

--

--  LCS RECORDS

--

LCSRecord	::= CHOICE

--

-- Record values 71..75 are LCS specific

--

{

lCSGMORecord		[71] LCSGMORecord,

lCSRGMTRecord		[72] LCSRGMTRecord,

lCSHGMTRecord		[73] LCSHGMTRecord,

lCSVGMTRecord		[74] LCSVGMTRecord,

lCSGNIRecord		[75] LCSGNIRecord

}

LCSGMORecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

servedIMSI					[4] IMSI,

servedMSISDN				[5] MSISDN OPTIONAL,

servingEntity				[6] ServingEntity OPTIONAL,

locationEstimate			[7] Ext-GeographicalInformation OPTIONAL,

positioningData				[8] PositioningData OPTIONAL,

userError					[9] UserError OPTIONAL,

providerError				[10] ProviderError OPTIONAL,

recordTimeStamp				[11] TimeStamp,

localSequenceNumber			[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

LCSRGMTRecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

targetIMSI					[4] IMSI,

targetMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lCSPriority					[7] LCS-Priority OPTIONAL,

resultCode					[8] ResultCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

homeGMLCIdentity			[12] IPAddress OPTIONAL

}

LCSHGMTRecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

targetIMSI					[4] IMSI,

targetMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lCSPriority					[7] LCS-Priority OPTIONAL,

resultCode					[8] ResultCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

requestingGMLCIdentity		[12] IPAddress OPTIONAL,

visitedGMLCIdentity			[13] IPAddress OPTIONAL,

servingNetworkIdentity		[14] PLMN-Id OPTIONAL

}

LCSVGMTRecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

targetIMSI					[4] IMSI,

targetMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lCSPriority					[7] LCS-Priority OPTIONAL,

resultCode					[8] ResultCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

homeGMLCIdentity			[12] IPAddress OPTIONAL

}

LCSGNIRecord	::= SET

{

recordType					[0] RecordType,

recordingEntity				[1] RecordingEntity,

lcsClientType				[2] LCSClientType OPTIONAL,

lcsClientIdentity			[3] LCSClientIdentity OPTIONAL,

servedIMSI					[4] IMSI,

servedMSISDN				[5] MSISDN OPTIONAL,

servingEntity				[6] ServingEntity OPTIONAL,

resultCode					[7] ResultCodeType OPTIONAL,

recordTimeStamp				[8] TimeStamp,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL

}

--

--  LCS DATA TYPES

--

ProviderError	::= INTEGER

--

-- see ITU-T Q.733 [307] for invoke problem codes

--

ResultCodeType			::= INTEGER (0..MAX)

--

-- Result codes as defined in OMA-MLP Specifications [311]

--

ServingEntity ::= AddressString

UserError	::= OCTET STRING (SIZE (1))

--

-- see TS 29.002 [214] for error code values

--

.#END

Seventh change

#### 5.2.4.1	MMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.270 [30].

.$MMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  mmsChargingDataTypes (5) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

ChargingID,

GSNAddress,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

MessageClass,

MscNo,

MSISDN,

MSTimeZone,

PLMN-Id,

PriorityType,

RATType,

RecordType,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

CallReferenceNumber

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CH-DataTypes (13) version20 (20)}

-- from TS 29.002 [214]

;

--

--  MMS RECORDS

--

MMSRecordType	::= CHOICE

--

-- Record values 30..62 are MMS specific

--

{

mMO1SRecord				[30] MMO1SRecord,

mMO4FRqRecord			[31] MMO4FRqRecord,

mMO4FRsRecord			[32] MMO4FRsRecord,

mMO4DRecord				[33] MMO4DRecord,

mMO1DRecord				[34] MMO1DRecord,

mMO4RRecord				[35] MMO4RRecord,

mMO1RRecord				[36] MMO1RRecord,

mMOMDRecord				[37] MMOMDRecord,

mMR4FRecord				[38] MMR4FRecord,

mMR1NRqRecord			[39] MMR1NRqRecord,

mMR1NRsRecord			[40] MMR1NRsRecord,

mMR1RtRqRecord			[41] MMR1RtRecord,

mMR1ARecord				[42] MMR1ARecord,

mMR4DRqRecord			[43] MMR4DRqRecord,

mMR4DRsRecord			[44] MMR4DRsRecord,

mMR1RRRecord			[45] MMR1RRRecord,

mMR4RRqRecord			[46] MMR4RRqRecord,

mMR4RRsRecord			[47] MMR4RRsRecord,

mMRMDRecord				[48] MMRMDRecord,

mMFRecord				[49] MMFRecord,

mMBx1SRecord			[50] MMBx1SRecord,

mMBx1VRecord			[51] MMBx1VRecord,

mMBx1URecord			[52] MMBx1URecord,

mMBx1DRecord			[53] MMBx1DRecord,

mM7SRecord				[54] MM7SRecord,

mM7DRqRecord			[55] MM7DRqRecord,

mM7DRsRecord			[56] MM7DRsRecord,

mM7CRecord				[57] MM7CRecord,

mM7RRecord				[58] MM7RRecord,

mM7DRRqRecord			[59] MM7DRRqRecord,

mM7DRRsRecord			[60] MM7DRRsRecord,

mM7RRqRecord			[61] MM7RRqRecord,

mM7RRsRecord			[62] MM7RRsRecord

}

MMO1SRecord	::= SET

{

recordType					[0] RecordType,

originatorMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

replyChargingID				[3] OCTET STRING OPTIONAL,

originatorAddress			[4] MMSAgentAddress,

recipientAddresses			[5] MMSAgentAddresses,

accessCorrelation			[6] AccessCorrelation OPTIONAL,

contentType					[7] ContentType,

mmComponentType				[8] MMComponentType OPTIONAL,

messageSize					[9] DataVolume,

messageClass				[10] MessageClass OPTIONAL,

chargeInformation			[11] ChargeInformation OPTIONAL,

submissionTime				[12] TimeStamp OPTIONAL,

timeOfExpiry				[13] WaitTime OPTIONAL,

earliestTimeOfDelivery		[14] WaitTime OPTIONAL,

durationOfTransmission		[15] INTEGER OPTIONAL,

requestStatusCode			[16] RequestStatusCodeType OPTIONAL,

deliveryReportRequested		[17] BOOLEAN OPTIONAL,

replyCharging				[18] BOOLEAN OPTIONAL,

replyDeadline				[19] WaitTime OPTIONAL,

replyChargingSize			[20] DataVolume OPTIONAL,

priority					[21] PriorityType OPTIONAL,

senderVisibility			[22] BOOLEAN OPTIONAL,

readReplyRequested			[23] BOOLEAN OPTIONAL,

statusText					[24] StatusTextType,

recordTimeStamp				[25] TimeStamp,

localSequenceNumber			[26] LocalSequenceNumber OPTIONAL,

recordExtensions			[27] ManagementExtensions OPTIONAL,

mMBoxstorageInformation		[28] MMBoxStorageInformation OPTIONAL,

mscfInformation				[29] MSCFInformation OPTIONAL,

sGSNPLMNIdentifier			[30] PLMN-Id OPTIONAL,

rATType						[31] RATType OPTIONAL,

mSTimeZone 					[32] MSTimeZone OPTIONAL

}

MMO4FRqRecord		::= SET

{

recordType					[0]  RecordType,

originatorMmsRSAddress		[1]  MMSRSAddress,

recipientMmsRSAddress		[2]  MMSRSAddress,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

originatorAddress			[5]  MMSAgentAddress,

recipientAddresses			[6]  MMSAgentAddresses,

contentType					[7]  ContentType,

mmComponentType				[8]  MMComponentType OPTIONAL,

messageSize					[9]  DataVolume,

messageClass				[10] MessageClass OPTIONAL,

submissionTime				[11] TimeStamp,

timeOfExpiry				[12] WaitTime OPTIONAL,

deliveryReportRequested		[13] BOOLEAN,

priority					[14] PriorityType OPTIONAL,

senderVisibility			[15] BOOLEAN,

readReplyRequested			[16] BOOLEAN,

acknowledgementRequest		[17] BOOLEAN,

forwardCounter				[18] INTEGER OPTIONAL,

forwardingAddress			[19] MMSAgentAddresses OPTIONAL,

recordTimeStamp				[20] TimeStamp,

localSequenceNumber			[21] LocalSequenceNumber OPTIONAL,

recordExtensions			[22] ManagementExtensions OPTIONAL

}

MMO4FRsRecord		::= SET

{

recordType					[0]  RecordType,

originatorMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

recipientMmsRSAddress		[2]  MMSRSAddress,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

requestStatusCode			[5] RequestStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMO4DRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

originatorMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

originatorAddress			[5]  MMSAgentAddress OPTIONAL,

recipientAddress			[6]  MMSAgentAddress,

mmDateAndTime				[7] TimeStamp,

acknowledgementRequest		[8] BOOLEAN,

mmStatusCode				[9] MMStatusCodeType,

statusText					[10] StatusTextType OPTIONAL,

recordTimeStamp				[11] TimeStamp OPTIONAL,

localSequenceNumber			[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

MMO1DRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

originatorMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

messageID					[4]  OCTET STRING,

mms3GPPVersion				[5]  OCTET STRING OPTIONAL,

originatorAddress			[6]  MMSAgentAddress OPTIONAL,

recipientAddress			[7]  MMSAgentAddress,

mmStatusCode				[8] MMStatusCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp OPTIONAL,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[12] PLMN-Id OPTIONAL,

rATType						[13] RATType OPTIONAL,

mSTimeZone 					[14] MSTimeZone OPTIONAL

}

MMO4RRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

originatorMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

originatorAddress			[5]  MMSAgentAddress OPTIONAL,

recipientAddresses			[6]  MMSAgentAddresses OPTIONAL,

mmDateAndTime				[7] TimeStamp OPTIONAL,

acknowledgementRequest		[8] BOOLEAN,

readStatus					[9] MMStatusCodeType OPTIONAL,

statusText					[10] StatusTextType OPTIONAL,

recordTimeStamp				[11] TimeStamp OPTIONAL,

localSequenceNumber		   	[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

MMO1RRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

originatorMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

messageID					[4]  OCTET STRING,

mms3GPPVersion				[5]  OCTET STRING OPTIONAL,

originatorAddress			[6]  MMSAgentAddress OPTIONAL,

recipientAddress			[7]  MMSAgentAddress OPTIONAL,

readStatus					[8] MMStatusCodeType OPTIONAL,

recordTimeStamp				[9] TimeStamp OPTIONAL,

localSequenceNumber			[10] LocalSequenceNumber OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[12] PLMN-Id OPTIONAL,

rATType						[13] RATType OPTIONAL,

mSTimeZone 					[14] MSTimeZone OPTIONAL

}

MMOMDRecord		::= SET

{

recordType					[0]  RecordType,

originatorMmsRSAddress		[1]  MMSRSAddress OPTIONAL,

recipientMmsRSAddress		[2]  MMSRSAddress OPTIONAL,

messageID					[3]  OCTET STRING,

messageSize					[4] DataVolume OPTIONAL,

mmStatusCode				[5] MMStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMR4FRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress,

originatorMmsRSAddress		[2]  MMSRSAddress,

messageID					[3]  OCTET STRING,

mms3GPPVersion				[4]  OCTET STRING OPTIONAL,

originatorAddress			[5]  MMSAgentAddress,

recipientAddresses			[6]  MMSAgentAddresses,

contentType					[7]  ContentType,

mmComponentType				[8]  MMComponentType OPTIONAL,

messageSize					[9]  DataVolume,

messageClass				[10] MessageClass OPTIONAL,

submissionTime				[11] TimeStamp,

timeOfExpiry				[12] WaitTime OPTIONAL,

deliveryReportRequested		[13] BOOLEAN,

priority					[14] PriorityType OPTIONAL,

senderVisibility			[15] BOOLEAN,

readReplyRequested			[16] BOOLEAN,

requestStatusCode			[17] RequestStatusCodeType,

statusText					[18] StatusTextType,

acknowledgementRequest		[19] BOOLEAN,

forwardCounter				[20] INTEGER OPTIONAL,

forwardingAddress			[21] MMSAgentAddresses OPTIONAL,

recordTimeStamp				[22] TimeStamp,

localSequenceNumber			[23] LocalSequenceNumber OPTIONAL,

recordExtensions			[24] ManagementExtensions OPTIONAL

}

MMR1NRqRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

replyChargingID				[3] OCTET STRING OPTIONAL,

senderAddress				[4] MMSAgentAddress,

recipientAddress			[5] MMSAgentAddress,

accessCorrelation			[6] AccessCorrelation OPTIONAL,

messageClass				[7] MessageClass OPTIONAL,

mmComponentType				[8] MMComponentType OPTIONAL,

messageSize					[9] DataVolume,

timeOfExpiry				[10] WaitTime OPTIONAL,

messageReference 			[11] OCTET STRING,

deliveryReportRequested		[12] BOOLEAN OPTIONAL,

replyCharging				[13] BOOLEAN OPTIONAL,

replyDeadline				[14] WaitTime OPTIONAL,

replyChargingSize			[15] DataVolume OPTIONAL,

mmStatusCode				[16] MMStatusCodeType OPTIONAL,

statusText					[17] StatusTextType OPTIONAL,

recordTimeStamp				[18] TimeStamp OPTIONAL,

localSequenceNumber			[19] LocalSequenceNumber OPTIONAL,

recordExtensions			[20] ManagementExtensions OPTIONAL,

mscfInformation				[21] MSCFInformation OPTIONAL,

vaspID						[22] OCTET STRING OPTIONAL,

vasID						[23] OCTET STRING OPTIONAL,

sGSNPLMNIdentifier			[24] PLMN-Id OPTIONAL,

rATType						[25] RATType OPTIONAL,

mSTimeZone 					[26] MSTimeZone OPTIONAL

}

MMR1NRsRecord		::= SET

{

recordType					[0]  RecordType,

recipientMmsRSAddress		[1]  MMSRSAddress,

messageID					[2]  OCTET STRING,

recipientAddress			[3] MMSAgentAddress,

accessCorrelation			[4] AccessCorrelation OPTIONAL,

reportAllowed				[5] BOOLEAN OPTIONAL,

mmStatusCode				[6] MMStatusCodeType OPTIONAL,

statusText					[7] StatusTextType OPTIONAL,

recordTimeStamp				[8] TimeStamp OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[11] PLMN-Id OPTIONAL,

rATType						[12] RATType OPTIONAL,

mSTimeZone 					[13] MSTimeZone OPTIONAL

}

MMR1RtRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

replyChargingID				[3] OCTET STRING OPTIONAL,

senderAddress				[4] MMSAgentAddress OPTIONAL,

recipientAddress			[5] MMSAgentAddress,

accessCorrelation			[6] AccessCorrelation OPTIONAL,

contentType					[7] ContentType,

mmComponentType				[8] MMComponentType OPTIONAL,

messageClass				[9] MessageClass OPTIONAL,

submissionTime				[10] TimeStamp,

messageSize					[11] DataVolume OPTIONAL,

deliveryReportRequested		[12] BOOLEAN OPTIONAL,

priority					[13] PriorityType OPTIONAL,

readReplyRequested			[14] BOOLEAN OPTIONAL,

mmStatusCode				[15] MMStatusCodeType OPTIONAL,

statusText					[16] StatusTextType OPTIONAL,

replyDeadline				[17] WaitTime OPTIONAL,

replyChargingSize			[18] DataVolume OPTIONAL,

durationOfTransmission		[19] INTEGER OPTIONAL,

timeOfExpiry				[20] WaitTime OPTIONAL,

recordTimeStamp				[21] TimeStamp OPTIONAL,

localSequenceNumber			[22] LocalSequenceNumber OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

messageReference 			[24] OCTET STRING,

vaspID						[25] OCTET STRING OPTIONAL,

vasID						[26] OCTET STRING OPTIONAL,

sGSNPLMNIdentifier			[27] PLMN-Id OPTIONAL,

rATType						[28] RATType OPTIONAL,

mSTimeZone 					[29] MSTimeZone OPTIONAL

}

MMR1ARecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

recipientAddress			[3] MMSAgentAddress,

accessCorrelation			[4] AccessCorrelation OPTIONAL,

reportAllowed				[5] BOOLEAN OPTIONAL,

mmStatusCode				[6] MMStatusCodeType OPTIONAL,

statusText					[7] StatusTextType OPTIONAL,

recordTimeStamp				[8] TimeStamp OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[11] PLMN-Id OPTIONAL,

rATType						[12] RATType OPTIONAL,

mSTimeZone 					[13] MSTimeZone OPTIONAL

}

MMR4DRqRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

originatorMmsRSAddress		[2] MMSRSAddress,

messageID					[3] OCTET STRING,

mms3GPPVersion				[4] OCTET STRING OPTIONAL,

originatorAddress			[5] MMSAgentAddress,

recipientAddress			[6] MMSAgentAddress,

mmDateAndTime				[7] TimeStamp OPTIONAL,

acknowledgementRequest		[8] BOOLEAN,

mmStatusCode				[9] MMStatusCodeType OPTIONAL,

statusText					[10] StatusTextType OPTIONAL,

recordTimeStamp				[11] TimeStamp OPTIONAL,

localSequenceNumber			[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

MMR4DRsRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

originatorMmsRSAddress		[2] MMSRSAddress,

messageID					[3] OCTET STRING,

mms3GPPVersion				[4] OCTET STRING OPTIONAL,

requestStatusCode			[5] RequestStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMR1RRRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

recipientAddress			[3] MMSAgentAddress,

originatorAddress			[4] MMSAgentAddress,

accessCorrelation			[5] AccessCorrelation OPTIONAL,

mmStatusCode				[6] MMStatusCodeType OPTIONAL,

statusText					[7] StatusTextType OPTIONAL,

recordTimeStamp				[8] TimeStamp OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[11] PLMN-Id OPTIONAL,

rATType						[12] RATType OPTIONAL,

mSTimeZone 					[13] MSTimeZone OPTIONAL

}

MMR4RRqRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

originatorMmsRSAddress		[2] MMSRSAddress,

messageID					[3] OCTET STRING,

mms3GPPVersion				[4] OCTET STRING OPTIONAL,

originatorAddress			[5] MMSAgentAddress,

recipientAddress			[6] MMSAgentAddress,

mmDateAndTime				[7] TimeStamp OPTIONAL,

acknowledgementRequest		[8] BOOLEAN,

mmStatusCode				[9] MMStatusCodeType OPTIONAL,

statusText					[10] StatusTextType OPTIONAL,

recordTimeStamp				[11] TimeStamp OPTIONAL,

localSequenceNumber			[12] LocalSequenceNumber OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL

}

MMR4RRsRecord		::= SET

{

recordType					[0] RecordType,

recipientMmsRSAddress		[1] MMSRSAddress,

originatorMmsRSAddress		[2] MMSRSAddress,

messageID					[3] OCTET STRING,

mms3GPPVersion				[4] OCTET STRING OPTIONAL,

requestStatusCode			[5] RequestStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMRMDRecord		::= SET

{

recordType					[0] RecordType,

originatorMmsRSAddress		[1] MMSRSAddress,

recipientMmsRSAddress		[2] MMSRSAddress OPTIONAL,

messageID					[3] OCTET STRING,

messageSize					[4] DataVolume,

mmStatusCode				[5] MMStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

recordTimeStamp				[7] TimeStamp OPTIONAL,

localSequenceNumber			[8] LocalSequenceNumber OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

MMFRecord		::= SET

{

recordType					[0] RecordType,

forwardingMmsRSAddress		[1] MMSRSAddress,

messageID					[2] OCTET STRING,

forwardingAddress			[3] MMSAgentAddress,

recipientAddresses			[4] MMSAgentAddresses,

chargeInformation			[5] ChargeInformation OPTIONAL,

timeOfExpiry				[6] WaitTime OPTIONAL,

earliestTimeOfDelivery		[7] WaitTime OPTIONAL,

deliveryReportRequested		[8] BOOLEAN OPTIONAL,

readReplyRequested			[9] BOOLEAN OPTIONAL,

messageReference 			[10] OCTET STRING,

mmStatusCode				[11] MMStatusCodeType OPTIONAL,

statusText					[12] StatusTextType OPTIONAL,

recordTimeStamp				[13] TimeStamp OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

recordExtensions			[15] ManagementExtensions OPTIONAL,

mMBoxstorageInformation		[16] MMBoxStorageInformation OPTIONAL

}

MMBx1SRecord	::= SET

{

recordType				[0]  RecordType,

mmsRelayAddress			[1]  IPAddress,

managingAddress			[2]  MMSAgentAddress,

accessCorrelation		[3]  AccessCorrelation OPTIONAL,

contentType				[4]  ContentType OPTIONAL,

messageSize				[5]  DataVolume OPTIONAL,

messageReference		[6]  OCTET STRING OPTIONAL,

mmState					[7]  OCTET STRING OPTIONAL,

mmFlags					[8]  OCTET STRING OPTIONAL,

storeStatus				[9]  StoreStatus OPTIONAL,

storeStatusText			[10] StatusTextType OPTIONAL,

sequenceNumber			[11] INTEGER OPTIONAL,

timeStamp				[12] TimeStamp OPTIONAL,

recordExtensions		[13] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier		[14] PLMN-Id OPTIONAL,

rATType					[15] RATType OPTIONAL,

mSTimeZone 				[16] MSTimeZone OPTIONAL

}

MMBx1VRecord	::= SET

{

recordType					[0] RecordType,

mmsRelayAddress				[1] IPAddress,

managingAddress				[2] MMSAgentAddress,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

attributesList				[4] AttributesList OPTIONAL,

messageSelection			[5] MessageSelection OPTIONAL,

start						[6] INTEGER OPTIONAL,

limit						[7] INTEGER OPTIONAL,

totalsRequested				[8] BOOLEAN OPTIONAL,

quotasRequested				[9] BOOLEAN OPTIONAL,

mmListing					[10] AttributesList OPTIONAL,

requestStatusCode			[11] RequestStatusCodeType OPTIONAL,

statusText					[12] StatusTextType OPTIONAL,

totals						[13] Totals OPTIONAL,

quotas						[14] Quotas OPTIONAL,

sequenceNumber				[15] INTEGER OPTIONAL,

timeStamp					[16] TimeStamp OPTIONAL,

recordExtensions			[17] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[18] PLMN-Id OPTIONAL,

rATType						[19] RATType OPTIONAL,

mSTimeZone 					[20] MSTimeZone OPTIONAL

}

MMBx1URecord	::= SET

{

recordType					[0] RecordType,

mmsRelayAddress				[1] IPAddress,

managingAddress				[2] MMSAgentAddress,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

recipientsAddressList		[4] MMSAgentAddresses,

messageClass				[5] MessageClass OPTIONAL,

uploadTime					[6] TimeStamp OPTIONAL,

timeOfExpiry				[7] WaitTime OPTIONAL,

earliestTimeOfDelivery		[8] WaitTime OPTIONAL,

priority					[9] PriorityType OPTIONAL,

mmState						[10] OCTET STRING OPTIONAL,

mmFlags						[11] OCTET STRING OPTIONAL,

contentType					[12] ContentType OPTIONAL,

messageSize					[13] DataVolume OPTIONAL,

messageReference			[14] OCTET STRING OPTIONAL,

requestStatusCode			[15] RequestStatusCodeType OPTIONAL,

statusText					[16] StatusTextType OPTIONAL,

sequenceNumber				[17] INTEGER OPTIONAL,

timeStamp					[18] TimeStamp OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[20] PLMN-Id OPTIONAL,

rATType						[21] RATType OPTIONAL,

mSTimeZone 					[22] MSTimeZone OPTIONAL

}

MMBx1DRecord	::= SET

{

recordType					[0] RecordType,

mmsRelayAddress				[1] IPAddress,

managingAddress				[2] MMSAgentAddress,

accessCorrelation			[3] AccessCorrelation OPTIONAL,

messageReference			[4] OCTET STRING OPTIONAL,

requestStatusCode			[5] RequestStatusCodeType OPTIONAL,

statusText					[6] StatusTextType OPTIONAL,

sequenceNumber				[7] INTEGER OPTIONAL,

timeStamp					[8] TimeStamp OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL,

sGSNPLMNIdentifier			[20] PLMN-Id OPTIONAL,

rATType						[21] RATType OPTIONAL,

mSTimeZone 					[22] MSTimeZone OPTIONAL

}

MM7SRecord	::= SET

{

recordType						[0] RecordType,

originatorMmsRSAddress			[1] MMSRSAddress,

linkedID						[2] OCTET STRING OPTIONAL,

vaspID							[3] OCTET STRING,

vasID							[4] OCTET STRING,

messageID						[5] OCTET STRING,

originatorAddress				[6] MMSAgentAddress,

recipientAddresses				[7] MMSAgentAddresses,

serviceCode						[8] OCTET STRING OPTIONAL,

contentType						[9] ContentType,

mmComponentType					[10] MMComponentType OPTIONAL,

messageSize						[11] DataVolume,

messageClass					[12] MessageClass OPTIONAL,

chargeInformation				[13] ChargeInformation OPTIONAL,

submissionTime					[14] TimeStamp OPTIONAL,

timeOfExpiry					[15] WaitTime OPTIONAL,

earliestTimeOfDelivery			[16] WaitTime OPTIONAL,

deliveryReportRequested			[17] BOOLEAN OPTIONAL,

readReplyRequested				[18] BOOLEAN OPTIONAL,

replyCharging					[19] BOOLEAN OPTIONAL,

replyDeadline					[20] WaitTime OPTIONAL,

replyChargingSize				[21] DataVolume OPTIONAL,

priority						[22] PriorityType OPTIONAL,

messageDistributionIndicator	[23] BOOLEAN OPTIONAL,

requestStatusCode				[24] RequestStatusCodeType OPTIONAL,

statusText						[25] StatusTextType OPTIONAL,

recordTimeStamp					[26] TimeStamp,

localSequenceNumber				[27] LocalSequenceNumber OPTIONAL,

recordExtensions				[28] ManagementExtensions OPTIONAL,

mscfInformation					[29] MSCFInformation OPTIONAL

}

MM7DRqRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress,

linkedID				[2] OCTET STRING OPTIONAL,

replyChargingID			[3] OCTET STRING OPTIONAL,

originatorAddress		[4] MMSAgentAddress,

recipientAddress		[5] MMSAgentAddress,

mmComponentType			[6] MMComponentType OPTIONAL,

messageSize				[7] DataVolume,

contentType				[8] ContentType,

priority				[9] PriorityType OPTIONAL,

recordTimeStamp			[10] TimeStamp OPTIONAL,

localSequenceNumber		[11] LocalSequenceNumber OPTIONAL,

recordExtensions		[12] ManagementExtensions OPTIONAL

}

MM7DRsRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress,

messageID				[2] OCTET STRING,

recipientAddress		[3] MMSAgentAddress,

serviceCode				[4] OCTET STRING OPTIONAL,

requestStatusCode		[5] RequestStatusCodeType OPTIONAL,

statusText				[6] StatusTextType OPTIONAL,

recordTimeStamp			[7] TimeStamp OPTIONAL,

localSequenceNumber		[8] LocalSequenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

MM7CRecord		::= SET

{

recordType				[0] RecordType,

originatorMmsRSAddress	[1] MMSRSAddress,

vaspID					[2] OCTET STRING,

vasID					[3] OCTET STRING,

messageID				[4] OCTET STRING,

originatorAddress		[5] MMSAgentAddress,

serviceCode				[6] OCTET STRING OPTIONAL,

requestStatusCode		[7] RequestStatusCodeType OPTIONAL,

statusText				[8] StatusTextType OPTIONAL,

recordTimeStamp			[9] TimeStamp OPTIONAL,

localSequenceNumber		[10] LocalSequenceNumber OPTIONAL,

recordExtensions		[11] ManagementExtensions OPTIONAL

}

MM7RRecord		::= SET

{

recordType				[0] RecordType,

originatorMmsRSAddress	[1] MMSRSAddress,

vaspID					[2] OCTET STRING,

vasID					[3] OCTET STRING,

messageID				[4] OCTET STRING,

originatorAddress		[5] MMSAgentAddress,

serviceCode				[6] OCTET STRING OPTIONAL,

contentType				[7] ContentType,

submissionTime			[8] TimeStamp OPTIONAL,

timeOfExpiry			[9] WaitTime OPTIONAL,

earliestTimeOfDelivery	[10] WaitTime OPTIONAL,

requestStatusCode		[11] RequestStatusCodeType OPTIONAL,

statusText				[12] StatusTextType OPTIONAL,

recordTimeStamp			[13] TimeStamp OPTIONAL,

localSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

recordExtensions		[15] ManagementExtensions OPTIONAL

}

MM7DRRqRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress OPTIONAL,

messageID				[2] OCTET STRING,

originatorAddress		[3] MMSAgentAddress OPTIONAL,

recipientAddress		[4] MMSAgentAddress,

mmDateAndTime			[5] TimeStamp OPTIONAL,

mmStatusCode			[6] MMStatusCodeType,

mmStatusText			[7] StatusTextType OPTIONAL,

recordTimeStamp			[8] TimeStamp OPTIONAL,

localSequenceNumber		[9] LocalSequenceNumber OPTIONAL,

recordExtensions		[10] ManagementExtensions OPTIONAL

}

MM7DRRsRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress OPTIONAL,

messageID				[2] OCTET STRING,

originatorAddress		[3] MMSAgentAddress OPTIONAL,

recipientAddress		[4] MMSAgentAddress,

requestStatusCode		[5] RequestStatusCodeType OPTIONAL,

statusText				[6] StatusTextType OPTIONAL,

recordTimeStamp			[7] TimeStamp OPTIONAL,

localSequenceNumber		[8] LocalSequenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

MM7RRqRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress OPTIONAL,

messageID				[2] OCTET STRING,

originatorAddress		[3] MMSAgentAddress OPTIONAL,

recipientAddress		[4] MMSAgentAddress,

mmDateAndTime			[5] TimeStamp OPTIONAL,

readStatus				[6] MMStatusCodeType,

mmStatusText			[7] StatusTextType OPTIONAL,

recordTimeStamp			[8] TimeStamp OPTIONAL,

localSequenceNumber		[9] LocalSequenceNumber OPTIONAL,

recordExtensions		[10] ManagementExtensions OPTIONAL

}

MM7RRsRecord		::= SET

{

recordType				[0] RecordType,

recipientMmsRSAddress	[1] MMSRSAddress OPTIONAL,

messageID				[2] OCTET STRING,

originatorAddress		[3] MMSAgentAddress OPTIONAL,

recipientAddress		[4] MMSAgentAddress,

requestStatusCode		[5] RequestStatusCodeType OPTIONAL,

statusText				[6] StatusTextType OPTIONAL,

recordTimeStamp			[7] TimeStamp OPTIONAL,

localSequenceNumber		[8] LocalSequenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

--

--  MMS DATA TYPES

--

AccessCorrelation ::= CHOICE

{

circuitSwitched			[0]	CircuitSwitchedAccess,

packetSwitched			[1]	PacketSwitchedAccess

}

AttributesList			::= SEQUENCE

--

-- Note: the values below are subject to WAP Forum ongoing standardization

--

{

messageID		[0]  OCTET STRING,

dateAndTime		[1]  TimeStamp,

senderAddress	[2]  MMSRSAddress,

subject			[3]  OCTET STRING,

messageSize		[4]  DataVolume ,

mmFlags			[5]  OCTET STRING,

mmState			[6]  MMState

}

ChargeInformation	::= SEQUENCE

--

-- one of the two following parameters must be present

--

{

chargedparty		[0]  ChargedParty OPTIONAL,

chargetype			[1]  ChargeType OPTIONAL

}

ChargedParty	::= ENUMERATED

{

sender				(0),

recipient			(1),

both				(2),

neither				(3),

notspecifiedbyVASP	(99)

}

ChargeType			::= ENUMERATED

{

postpaid				(0),

pre-paid				(1)

}

CircuitSwitchedAccess ::= SEQUENCE

{

mSCIdentifier			[0]  MscNo,

callReferenceNumber		[1]  CallReferenceNumber

}

ContentType			::= OCTET STRING

DataVolume			::= INTEGER

--

-- The volume of data transfered in octets.

--

DeltaSeconds		::= OCTET STRING (SIZE(8))

MediaComponent	::= SEQUENCE

{

mediaType		[0]  OCTET STRING,

mediaSize		[1]  DataVolume

}

MediaComponents	::= SET OF MediaComponent

MessageSelection ::= INTEGER

MMBoxStorageInformation   	::= SET

{

mmState					[0]  MMState,

mmFlag					[1]  OCTET STRING,

storeStatus				[2]  StoreStatus,

storeStatusText			[3]  StatusTextType,

storedMessageReference	[4]  OCTET STRING

}

MMComponentType		::= SEQUENCE

{

subject		[0]  SubjectComponent,

media		[1]  MediaComponents

}

MMSAgentAddress		::= SEQUENCE

--

-- mMSRecipeintType is only included when this datatype is used to identify recipients.

--

{

mMSAgentAddressData	[0]  MMSAgentAddressData,

mMSRecipientType	[1]  SEQUENCE OF MMSRecipientType OPTIONAL

}

MMSAgentAddresses	::= SET OF MMSAgentAddress

MMSAgentAddressData	::= CHOICE

{

eMail-address	[0]  OCTET STRING,

mSISDN			[1]  MSISDN,

shortCode		[2]  OCTET STRING

}

MMSRecipientType	::= ENUMERATED

{

tO				(0),

cC				(1),

bCC				(2)

}

MMSRSAddress		::= SEQUENCE

--

-- usage of SEQUENCE instead of CHOICE allows both address types to be present at the same time

--

{

domainName		[0]  OCTET STRING OPTIONAL,

iPAddress		[2]  IPAddress OPTIONAL

}

MMState		::= ENUMERATED

--

-- Note: the values below are subject to WAP Forum ongoing standardization

--

{

draft			(0),

sent			(1),

new				(2),

retrieved		(3),

forwarded		(4)

}

MMStatusCodeType		::= ENUMERATED

{

retrieved					(0),

forwarded					(1),

expired						(2),

rejected					(3),

deferred					(4),

unrecognised				(5),

read						(6),

deletedWithoutBeingRead 	(7)

}

MSCFInformation		::= SET

{

billingInformation		[0]  OCTET STRING OPTIONAL,

routeingAddressList		[1]  RouteingAddressList OPTIONAL

}

PacketSwitchedAccess ::= SEQUENCE

{

gSNAddress		[0]  GSNAddress,

chargingID		[1] ChargingID

}

Quotas	::= SEQUENCE

{

numberOfMessages	[0] INTEGER OPTIONAL,

numberOfOctets		[1] INTEGER OPTIONAL

}

RequestStatusCodeType	::= INTEGER

--

-- cause codes 0 to 15 are used as defined for 'CauseForTerm'

-- (cause for termination) and 16 to 20 are as defined for 'CauseForRecClosing'

--

{

normalRelease				(0),	-- ok

abnormalRelease				(4),	-- error unspecified

serviceDenied				(30),

messageFormatCorrupt		(31),

sendingAddressUnresolved	(32),

messageNotFound				(33),

networkProblem				(34),

contentNotAccepted		 	(35),

unsupportedMessage			(36)

}

RouteingAddress			::= SEQUENCE

--

-- usage of SEQUENCE instead of CHOICE allows several address types

-- to be present at the same time

--

{

eMail-address		[0] OCTET STRING,

mSISDN				[1] MSISDN OPTIONAL,

shortCode			[2] OCTET STRING OPTIONAL

}

RouteingAddressList	::= SET OF MMSAgentAddress

StatusTextType		::= OCTET STRING

StoreStatus	::= INTEGER

--

-- Note: the values below are subject to WAP Forum ongoing standardization

--

{

stored							(0),

errorTransientFailure			(1),

errorTransientMailboxFull		(2),

errorTransientNetworkProblems	(3),

errorPermanentFailure			(4),

errorPermanentPermissionDenied	(5),

errorPermanentMessageFormat		(6),

errorPermanentMessageNotFound	(7)

}

SubjectComponent	::= SEQUENCE

{

subjectType		[0] OCTET STRING,

subjectSize 	[1] DataVolume

}

Totals	::= SEQUENCE

{

numberOfMessages		[0] INTEGER OPTIONAL,

numberOfOctets			[1] INTEGER OPTIONAL

}

WaitTime		::= CHOICE

{

http-date		[0]	TimeStamp,

delta-seconds	[1]	DeltaSeconds

}

.#END

Sixth change

#### 5.2.4.4	MBMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.273 [33].

.$MBMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  mbmsChargingDataTypes (8) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

Diagnostics,

GSNAddress,

LocalSequenceNumber,

ManagementExtensions,

MBMSInformation,

MSISDN,

NodeID,

PDPAddress,

RecordType,

ServiceContextID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version20 (20)}

-- from TS 29.002 [214]

AccessPointNameNI,

ChangeOfMBMSCondition,

PDPType

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

Media-Components-List

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version2 (1)}

;

--

--  MBMS RECORDS

--

MBMSRecord	::= CHOICE

--

-- Record values 78..79 are MBMS specific

--

{

sUBBMSCRecord			[78] SUBBMSCRecord,

cONTENTBMSCRecord		[79] CONTENTBMSCRecord

}

SUBBMSCRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

ggsnAddress					[2] GSNAddress OPTIONAL,

accessPointNameNI			[3] AccessPointNameNI OPTIONAL,

servedPDPAddress			[4] PDPAddress OPTIONAL,

listOfTrafficVolumes		[5] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[6] TimeStamp,

duration					[7] CallDuration,

causeForRecClosing			[8] MBMSCauseForRecClosing,

diagnostics					[9] Diagnostics OPTIONAL,

recordSequenceNumber		[10] INTEGER OPTIONAL,

nodeID						[11] NodeID OPTIONAL,

recordExtensions			[12] ManagementExtensions OPTIONAL,

localSequenceNumber			[13] LocalSequenceNumber OPTIONAL,

servedMSISDN				[14] MSISDN OPTIONAL,

bearerServiceDescription	[15] Media-Components-List OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

serviceContextID			[17] ServiceContextID OPTIONAL

}

CONTENTBMSCRecord	::= SET

{

recordType					[0] RecordType,

contentProviderId			[1] GraphicString,

listofDownstreamNodes		[2] SEQUENCE OF GSNAddress,

accessPointNameNI			[3] AccessPointNameNI OPTIONAL,

servedPDPAddress			[4] PDPAddress OPTIONAL,

listOfTrafficVolumes		[5] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[6] TimeStamp,

duration					[7] CallDuration,

causeForRecClosing			[8] MBMSCauseForRecClosing,

diagnostics					[9] Diagnostics OPTIONAL,

recordSequenceNumber		[10] INTEGER OPTIONAL,

nodeID						[11] NodeID OPTIONAL,

recordExtensions			[12] ManagementExtensions OPTIONAL,

localSequenceNumber			[13] LocalSequenceNumber OPTIONAL,

recipientAddressList		[14] SEQUENCE OF MSISDN,

bearerServiceDescription	[15] Media-Components-List OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

serviceContextID			[17] ServiceContextID OPTIONAL,

servedpdpPDNType			[18] PDPType OPTIONAL

}

--

--  MBMS DATA TYPES

--

MBMSCauseForRecClosing	::= INTEGER

--

-- cause codes 0 to 15 are defined as used in 'CauseForTerm'

-- (cause for termination) and 16 to 20 are as defined for 'CauseForRecClosing'

-- (cause for record closing)

--

{

normalRelease					(0),

abnormalRelease					(4),

volumeLimit						(16),

timeLimit						(17),

maxChangeCond					(19),

managementIntervention			(20),

listofDownstreamNodeChange		(59)

}

.#END

Eighth change

#### 5.2.4.6	SMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.274 [34].

.$SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  smsChargingDataTypes (10) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

DataVolume

FROM CSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) csChargingDataTypes (1) asn1Module (0) version2 (1)}

DiameterIdentity,

LocalSequenceNumber,

ManagementExtensions,

MessageClass,

MessageReference,

MSISDN,

MSTimeZone,

NodeAddress,

PDPAddress,

PLMN-Id,

PriorityType,

RATType,

RecordType,

Session-Id,

SMSResult,

SubscriberEquipmentNumber,

SubscriptionID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

AddressString,

IMEI,

IMSI,

ISDN-AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version20 (20)}

-- from TS 29.002 [214]

CarrierSelectRouting,

NumberPortabilityRouting

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version2 (1)}

;

--

--  SMS RECORDS

--

SMSRecordType	::= CHOICE

--

-- Record values 93,94, 110 to 113 are SMS specific.

--

{

sCSMORecord				 [93] SCSMORecord,

sCSMTRecord				 [94] SCSMTRecord,

sCDVTT4Record			[110] SCDVTT4Record,

sCSMOT4Record			[111] SCSMTRecord,

iSMSMORecord			[112] ISMSMORecord,

iSMSMTRecord			[113] ISMSMTRecord

}

SCSMORecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

originatorInfo				[2] OriginatorInfo OPTIONAL,

recipientInfo				[3] SEQUENCE OF RecipientInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

eventtimestamp				[5]	TimeStamp,

messageReference			[6] MessageReference,

sMTotalNumber 				[7] INTEGER OPTIONAL,

sMSequenceNumber 			[8] INTEGER OPTIONAL,

messageSize					[9] DataVolume OPTIONAL,

messageClass				[10] MessageClass OPTIONAL,

sMdeliveryReportRequested	[11] BOOLEAN OPTIONAL,

sMDataCodingScheme			[12] INTEGER OPTIONAL,

sMMessageType				[13] SMMessageType OPTIONAL,

sMReplyPathRequested		[14] NULL OPTIONAL,

sMUserDataHeader			[15] OCTET STRING OPTIONAL,

userLocationInfo			[16] OCTET STRING OPTIONAL,

rATType						[17] RATType OPTIONAL,

uETimeZone					[18] MSTimeZone OPTIONAL,

sMSResult					[19] SMSResult OPTIONAL,

localSequenceNumber			[22] LocalSequenceNumber OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL

}

SCSMOT4Record	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

originatorInfo				[2] OriginatorInfo OPTIONAL,

recipientInfo				[3] RecipientInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

eventtimestamp				[5]	TimeStamp,

messageReference			[6] MessageReference,

messageSize					[9] DataVolume OPTIONAL,

messageClass				[10] MessageClass OPTIONAL,

sMdeliveryReportRequested	[11] BOOLEAN OPTIONAL,

sMDataCodingScheme			[12] INTEGER OPTIONAL,

sMMessageType				[13] SMMessageType OPTIONAL,

sMReplyPathRequested		[14] NULL OPTIONAL,

sMUserDataHeader			[15] OCTET STRING OPTIONAL,

userLocationInfo			[16] OCTET STRING OPTIONAL,

rATType						[17] RATType OPTIONAL,

uETimeZone					[18] MSTimeZone OPTIONAL,

sMSResult					[19] SMSResult OPTIONAL,

mTCIWFAddress				[20] NodeAddress OPTIONAL,

sMSApplicationPortID		[21] INTEGER OPTIONAL,

externalIdentifier			[22] SubscriptionID OPTIONAL,

localSequenceNumber			[23] LocalSequenceNumber OPTIONAL,

recordExtensions			[24] ManagementExtensions OPTIONAL

}

SCSMTRecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

recipientInfo				[2] RecipientInfo OPTIONAL,

originatorInfo				[3] OriginatorInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

submissionTime				[5]	TimeStamp OPTIONAL,

eventtimestamp				[6]	TimeStamp,

sMPriority					[7]	PriorityType OPTIONAL,

messageReference			[8] MessageReference OPTIONAL,

sMTotalNumber 				[9] INTEGER OPTIONAL,

sMSequenceNumber			[10] INTEGER OPTIONAL,

messageSize					[11] DataVolume OPTIONAL,

messageClass				[12] MessageClass OPTIONAL,

sMdeliveryReportRequested	[13] BOOLEAN OPTIONAL,

sMDataCodingScheme			[14] INTEGER OPTIONAL,

sMMessageType				[15] SMMessageType OPTIONAL,

sMReplyPathRequested		[16] NULL OPTIONAL,

sMUserDataHeader			[17] OCTET STRING OPTIONAL,

sMSStatus					[18] SMSStatus OPTIONAL,

sMDischargeTime				[19] TimeStamp OPTIONAL,

userLocationInfo			[20] OCTET STRING OPTIONAL,

rATType						[21] RATType OPTIONAL,

uETimeZone					[22] MSTimeZone OPTIONAL,

sMSResult					[23] SMSResult OPTIONAL,

sMDeviceTriggerInformation	[25] SMDeviceTriggerInformation OPTIONAL,

localSequenceNumber			[26] LocalSequenceNumber OPTIONAL,

recordExtensions			[27] ManagementExtensions OPTIONAL

}

SCDVTT4Record	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

eventtimestamp				[2]	TimeStamp,

originatorInfo				[3] OriginatorInfo OPTIONAL,

recipientInfo				[4] RecipientInfo OPTIONAL,

sMDeviceTriggerIndicator	[5] SMDeviceTriggerIndicator OPTIONAL,

sMDeviceTriggerInformation	[6] SMDeviceTriggerInformation OPTIONAL,

sMSResult					[7] SMSResult OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL

}

ISMSMORecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] NodeAddress,

originatorInfo				[2] OriginatorInfo OPTIONAL,

recipientInfo				[3] SEQUENCE OF RecipientInfo OPTIONAL,

subscriberEquipmentNumber	[4] SubscriberEquipmentNumber OPTIONAL,

eventtimestamp				[5]	TimeStamp,

messageReference			[6] MessageReference,

sMTotalNumber 				[7] INTEGER OPTIONAL,

sMSequenceNumber 			[8] INTEGER OPTIONAL,

messageSize					[9] DataVolume OPTIONAL,

messageClass				[10] MessageClass OPTIONAL,

sMdeliveryReportRequested	[11] BOOLEAN OPTIONAL,

sMDataCodingScheme			[12] INTEGER OPTIONAL,

sMMessageType				[13] SMMessageType OPTIONAL,

sMReplyPathRequested		[14] NULL OPTIONAL,

sMUserDataHeader			[15] OCTET STRING OPTIONAL,

sMSResult					[16] SMSResult OPTIONAL,

userLocationInfo			[17] OCTET STRING OPTIONAL,

rATType						[18] RATType OPTIONAL,

uETimeZone					[19] MSTimeZone OPTIONAL,

pDPAddress					[20] PDPAddress OPTIONAL,

session-Id					[21] Session-Id OPTIONAL,

numberPortabilityRouting	[22] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting		[23] CarrierSelectRouting OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

recordExtensions			[25] ManagementExtensions OPTIONAL

}

ISMSMTRecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] NodeAddress,

recipientInfo				[2] RecipientInfo OPTIONAL,

originatorInfo				[3] OriginatorInfo OPTIONAL,

subscriberEquipmentNumber	[4] SubscriberEquipmentNumber OPTIONAL,

submissionTime				[5]	TimeStamp OPTIONAL,

eventtimestamp				[6]	TimeStamp,

sMPriority					[7]	PriorityType OPTIONAL,

messageReference			[8] MessageReference,

sMTotalNumber 				[9] INTEGER OPTIONAL,

sMSequenceNumber 			[10] INTEGER OPTIONAL,

messageSize					[11] DataVolume OPTIONAL,

messageClass				[12] MessageClass OPTIONAL,

sMdeliveryReportRequested	[13] BOOLEAN OPTIONAL,

sMDataCodingScheme			[14] INTEGER OPTIONAL,

sMMessageType				[15] SMMessageType OPTIONAL,

sMReplyPathRequested		[16] NULL OPTIONAL,

sMUserDataHeader			[17] OCTET STRING OPTIONAL,

sMSStatus					[18] SMSStatus OPTIONAL,

sMDischargeTime				[19] TimeStamp OPTIONAL,

userLocationInfo			[20] OCTET STRING OPTIONAL,

rATType						[21] RATType OPTIONAL,

uETimeZone					[22] MSTimeZone OPTIONAL,

sMSResult					[23] SMSResult OPTIONAL,

pDPAddress					[24] PDPAddress OPTIONAL,

session-Id					[25] Session-Id OPTIONAL,

numberPortabilityRouting	[26] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting		[27] CarrierSelectRouting OPTIONAL,

localSequenceNumber			[28] LocalSequenceNumber OPTIONAL,

recordExtensions			[29] ManagementExtensions OPTIONAL

}

--

--  SMS DATA TYPES

--

OriginatorInfo	::= SEQUENCE

--

-- OriginatorInfo is used for information about Originator of a Short Message

--

{

originatorIMSI				[0] IMSI OPTIONAL,

originatorMSISDN			[1] MSISDN OPTIONAL,

originatorOtherAddress		[2] SMAddressInfo OPTIONAL,

-- included for backwards compatibility

originatorSCCPAddress		[3] AddressString OPTIONAL,

originatorReceivedAddress	[4] SMAddressInfo OPTIONAL,

sMOriginatorInterface		[5] SMInterface OPTIONAL,

sMOriginatorProtocolID		[6] OCTET STRING OPTIONAL,

originatorOtherAddresses		[7] SEQUENCE OF SMAddressInfo OPTIONAL

-- used if type different from IMSI and MSISDN

}

RecipientInfo	::= SEQUENCE

--

-- RecipientInfo is used for information about Recipient of a Short Message

--

{

recipientIMSI				[0] IMSI OPTIONAL,

recipientMSISDN				[1] MSISDN OPTIONAL,

recipientOtherAddress		[2] SMAddressInfo OPTIONAL,

-- included for backwards compatibility

recipientSCCPAddress		[3] AddressString OPTIONAL,

recipientReceivedAddress	[4] SMAddressInfo OPTIONAL,

sMDestinationInterface		[5] SMInterface OPTIONAL,

sMRecipientProtocolID		[6] OCTET STRING OPTIONAL,

recipientOtherAddresses		[7] SEQUENCE OF SMAddressInfo OPTIONAL

-- used if type different from IMSI and MSISDN

}

SMAddressDomain		::= SEQUENCE
{

sMDomainName			[0] GraphicString OPTIONAL,

threeGPPIMSI-MCC-MNC	[1] PLMN-Id OPTIONAL

}

SMAddressInfo		::= SEQUENCE
{

sMAddressType		[0] SMAddressType OPTIONAL,

sMAddressData		[1] GraphicString OPTIONAL,

sMAddressDomain		[2] SMAddressDomain OPTIONAL

}

SMAddressType	::= ENUMERATED

{

emailAddress			(0),

mSISDN					(1),

iPv4Address				(2),

iPv6Address				(3),

numericShortCode		(4),

alphanumericShortCode	(5),

other					(6),

iMSI					(7),

nAI						(8),

externalId				(9)

}

SMDeviceTriggerIndicator	::= ENUMERATED

{

notDeviceTrigger		(0),

deviceTriggerRequest	(1),

deviceTriggerReplace	(2),

deviceTriggerRecall		(3)

}

SMDeviceTriggerInformation	::= SEQUENCE

--

-- SMDeviceTriggerInformation is used for information on device triggering from T4

-- as specified in TS 29.337[231]

--

{

mTCIWFAddress			[0] NodeAddress OPTIONAL,

sMDTReferenceNumber		[1] INTEGER OPTIONAL,

sMServingNode			[2] SMServingNode OPTIONAL,

sMDTValidityPeriod		[3] INTEGER OPTIONAL,

sMDTPriorityIndication	[4] SMDTPriorityIndication OPTIONAL,

sMSApplicationPortID	[5] INTEGER OPTIONAL

}

SMDTPriorityIndication	::= ENUMERATED

{

nonpriority	(0),

priority	(1)

}

SMInterface		::= SEQUENCE
{

interfaceId			[0] GraphicString OPTIONAL,

interfaceText		[1] GraphicString OPTIONAL,

interfacePort		[2] GraphicString OPTIONAL,

interfaceType		[3] SMInterfaceType OPTIONAL

}

SMInterfaceType	::= ENUMERATED

{

unkown					(0),

mobileOriginating		(1),

mobileTerminating		(2),

applicationOriginating	(3),

applicationTerminating	(4),

deviceTrigger			(5)

}

SMMessageType	::= ENUMERATED

{

submission			(0),

deliveryReport		(1),

sMServiceRequest	(2),

delivery			(3),

t4DeviceTrigger		(4),

sMDeviceTrigger		(5)

}

SMServingNode		::= SEQUENCE
{

sGSNName			[0] DiameterIdentity OPTIONAL,

sGSNRealm			[1] DiameterIdentity OPTIONAL,

sGSNNumber			[2] AddressString OPTIONAL,

mMEName				[3] DiameterIdentity OPTIONAL,

mMERealm			[4] DiameterIdentity OPTIONAL,

mMENumberForMTSMS	[5] AddressString OPTIONAL,

mSCNumber			[6] AddressString OPTIONAL,

iPSMGWNumber		[7] AddressString OPTIONAL,

iPSMGWName			[8] DiameterIdentity OPTIONAL

}

SMSStatus		::= OCTET STRING (SIZE(1))

.#END

Ninth change

#### 5.2.4.7	ProSe CDRs

This subclause contains the abstract syntax definitions that are specific to the ProSe CDR types defined in TS 32.277 [36].

.$ProSeChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) proseChargingDataTypes (11) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

NodeID,

PLMN-Id,

RecordType,

ServiceContextID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version20 (20)}

-- from TS 29.002 [214]

ChargingCharacteristics,

ChChSelectionMode,

DataVolumeGPRS

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

;

--

#### -- ProSe RECORDS

--

ProSeRecordType		::= CHOICE

--

-- Record values 100..102 are ProSe specific

--

{

pFDDRecord			[100] PFDDRecord,

pFEDRecord			[101] PFEDRecord,

pFDCRecord			[102] PFDCRecord

}

PFDDRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

serviceContextID				[2] ServiceContextID OPTIONAL,

servedIMSI						[3] IMSI OPTIONAL,

proSeFunctionIPAddress			[4] IPAddress OPTIONAL,

chargingCharacteristics			[5] ChargingCharacteristics,

chChSelectionMode				[6] ChChSelectionMode OPTIONAL,

recordExtensions				[7] ManagementExtensions OPTIONAL,

proSeRequestTimestamp			[8] TimeStamp OPTIONAL,

roleofUE						[9] ProSeUERole OPTIONAL,

pCThreeControlProtocolCause		[10] INTEGER OPTIONAL,

roleofProSeFunction				[11] ProSeFunctionRole OPTIONAL,

proSeApplicationID				[12] UTF8String OPTIONAL,

proSeEventType					[13] ProSeEventType OPTIONAL,

nodeID							[14] NodeID OPTIONAL,

proseFunctionId					[15] UTF8String OPTIONAL,

announcingUEHPLMNIdentifier		[16] PLMN-Id OPTIONAL,

announcingUEVPLMNIdentifier		[17] PLMN-Id OPTIONAL,

monitoringUEHPLMNIdentifier	[18] PLMN-Id OPTIONAL,

monitoringUEVPLMNIdentifier		[19] PLMN-Id OPTIONAL,

monitoredPLMNIdentifier			[20] PLMN-Id OPTIONAL,

applicationID					[21] UTF8String OPTIONAL,

directDiscoveryModel			[22] UTF8String OPTIONAL,

validityPeriod					[23] INTEGER OPTIONAL,

monitoringUEIdentifier			[24] IMSI OPTIONAL,

discovererUEHPLMNIdentifier		[25] PLMN-Id OPTIONAL,

discovererUEVPLMNIdentifier		[26] PLMN-Id OPTIONAL,

discovereeUEHPLMNIdentifier		[27] PLMN-Id OPTIONAL,

discovereeUEVPLMNIdentifier		[28] PLMN-Id OPTIONAL,

announcingPLMNID				[29] PLMN-Id OPTIONAL,

pc5RadioTechnology				[30] PC5RadioTechnology OPTIONAL

}

PFEDRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

serviceContextID				[2] ServiceContextID OPTIONAL,

servedIMSI						[3] IMSI OPTIONAL,

proSeFunctionIPAddress			[4] IPAddress OPTIONAL,

chargingCharacteristics			[5] ChargingCharacteristics,

chChSelectionMode				[6] ChChSelectionMode OPTIONAL,

recordExtensions				[7] ManagementExtensions OPTIONAL,

proSeRequestTimestamp			[8] TimeStamp OPTIONAL,

roleofUE						[9] ProSeUERole OPTIONAL,

pCThreeEPCControlProtocolCause	[10] INTEGER OPTIONAL,

proseFunctionPLMNIdentifier		[11] PLMN-Id OPTIONAL,

proseFunctionId					[12] UTF8String OPTIONAL,

recordOpeningTime				[13] TimeStamp OPTIONAL,

recordClosureTime				[14] TimeStamp OPTIONAL,

applicationID					[15] UTF8String OPTIONAL,

requestorApplicationLayerUserID [16] UTF8String OPTIONAL,

wLANLinkLayerID					[17] UTF8String OPTIONAL,

requestorEPCProSeUserID			[18] UTF8String OPTIONAL,

requestedApplicationLayerUserID	[19] UTF8String OPTIONAL,

requestedPLMNIdentifier			[20] PLMN-Id OPTIONAL,

timeWindow						[21] INTEGER OPTIONAL,

rangeClass						[22] RangeClass OPTIONAL,

uELocation						[23] OCTET STRING OPTIONAL,

proximityAlertIndication		[24] ProximityAlertIndication OPTIONAL,

proximityAlertTimestamp			[25] TimeStamp OPTIONAL,

proximityCancellationTimestamp	[26] TimeStamp OPTIONAL,

reasonforCancellation			[27] ReasonforCancellation OPTIONAL,

causeForRecClosing				[28] ProSeCauseForRecClosing,

proximityRequestRenewalInfoBlockList	[29] SEQUENCE OF ProximityRequestRenewalInfoBlock OPTIONAL

}

PFDCRecord 	::= SET

{

-- General CDR information

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

serviceContextID				[2] ServiceContextID OPTIONAL,

servedIMSI						[3] IMSI OPTIONAL,

proSeFunctionIPAddress			[4] IPAddress OPTIONAL,

chargingCharacteristics			[5] ChargingCharacteristics,

chChSelectionMode				[6] ChChSelectionMode OPTIONAL,

recordExtensions				[7] ManagementExtensions OPTIONAL,

nodeID							[8] NodeID OPTIONAL,

proseFunctionPLMNIdentifier		[9] PLMN-Id OPTIONAL,

proseFunctionId					[10] UTF8String OPTIONAL,

recordOpeningTime				[11] TimeStamp OPTIONAL,

recordClosureTime				[12] TimeStamp OPTIONAL,

-- Common ProSe information. The same data is provided in all currently open group-level CDRs

listOfCoverageInfo				[13] SEQUENCE OF CoverageInfo OPTIONAL,

listOfRadioParameterSet			[14] SEQUENCE OF RadioParameterSetInfo OPTIONAL,

-- Group-specific information. This data could be different in each open group-level CDR

proSeUEID						[15] OCTET STRING OPTIONAL,

sourceIPaddress					[16] IPAddress OPTIONAL,

layerTwoGroupID				[17] OCTET STRING OPTIONAL,

proSeGroupIPmulticastaddress	[18] IPAddress OPTIONAL,

timeOfFirstTransmission			[19] TimeStamp OPTIONAL,

timeOfFirstReception			[20] TimeStamp OPTIONAL,

listOfTransmitters				[21] SEQUENCE OF TransmitterInfo OPTIONAL,

listOfTransmissionData			[22] SEQUENCE OF ChangeOfProSeCondition OPTIONAL,

listOfReceptionData				[23] SEQUENCE OF ChangeOfProSeCondition OPTIONAL,

causeForRecClosing				[24] ProSeCauseForRecClosing,

listOfAppSpecificData			[25] SEQUENCE OF AppSpecificData,

targetIPaddress					[26] IPAddress OPTIONAL,

relayIPaddress					[27] IPAddress OPTIONAL,

proSeUEtoNetworkRelayUEID		[28] OCTET STRING OPTIONAL,

proSeTargetLayerTwoID			[29] OCTET STRING OPTIONAL

}

--

#### -- ProSe DATA TYPES

--

--

#### -- A

--

AppSpecificData	::= OCTET STRING

--

#### -- C

--

ChangeOfProSeCondition			::= SEQUENCE

--

-- Used for transmitted and received data container

--

{

changeConditionTimestamp	[0] TimeStamp OPTIONAL,

coverageStatus					[1] CoverageStatus OPTIONAL,

uELocation						[2] OCTET STRING OPTIONAL,

dataVolume				[3] DataVolumeGPRS OPTIONAL,

serviceChangeCondition			[4] ServiceChangeCondition OPTIONAL,

localSequenceNumber				[5] LocalSequenceNumber OPTIONAL,

usageInformationReportSequenceNumber	[6] INTEGER OPTIONAL,

radioResourcesInd				[7] RadioResourcesIndicator OPTIONAL,

radiofrequency					[8] RadioFrequency OPTIONAL,

vPLMNIdentifier					[9] PLMN-Id OPTIONAL

}

CoverageInfo	::= SEQUENCE

{

coverageStatus	[0] CoverageStatus,

timeStamp		[1] TimeStamp OPTIONAL,

listOfLocation	[2] SEQUENCE OF LocationInfo OPTIONAL

}

CoverageStatus				::= ENUMERATED

{

outOfCoverage				(0),

inCoverage					(1)

}

--

#### -- L

--

LocationInfo	::= SEQUENCE

{

uELocation		[0] OCTET STRING OPTIONAL,

timeStamp		[1] TimeStamp OPTIONAL

}

--

#### -- P

--

PC5RadioTechnology			::= ENUMERATED

{

eUTRA					(0),

wLAN					(1),

bothEUTRAAndWLAN		(2)

}

ProSeCauseForRecClosing		::= ENUMERATED

{

proximityAlerted			(0),

timeExpiredWithNoRrenewal	(1),

requestorCancellation		(2),

timeLimited					(3),

maxNumberOfReports			(4),

abnormalRelease				(5)

}

ProSeEventType				::= ENUMERATED

{

openAnnouncing					(0),

openMonitoring					(1),

openMatchReport					(2),

restrictedAnnouncing 			(3),

restrictedMonitoring 			(4),

restrictedMatchReport			(5),

restrictedDiscoveryRequest 		(6),

restrictedDiscoveryReporting	(7)

}

ProseFunctionality		::= ENUMERATED

{

dIRECT-DISCOVERY		(0),

dIRECT-COMMUNICATION	(1)

}

ProSeFunctionRole			::= ENUMERATED

{

hPLMN			(0),

vPLMN			(1),

localPLMN		(2)

}

ProSeUERole					::= ENUMERATED

{

annoucingUE					(0),

monitoringUE				(1),

requestorUE					(2),

requestedUE					(3),

discovererUE				(4),

discovereeUE		 		(5)

}

ProximityAlertIndication	::= ENUMERATED

{

alerted						(0),

noAlert						(1)

}

ProximityRequestRenewalInfoBlock			::= SEQUENCE

--

-- Used for EPC-level discovery charging

--

{

proSeRequestTimestamp			[0] TimeStamp OPTIONAL,

timeWindow						[1] INTEGER OPTIONAL,

rangeClass						[2] RangeClass OPTIONAL,

uELocation						[3] OCTET STRING OPTIONAL

}

--

#### -- R

--

RadioFrequency	::= OCTET STRING

--

-- Format of the value is according to the carrierFreq-r12 ASN.1 data type described in TS

-- 36.331 [241].

--

RadioParameterSetInfo	::= SEQUENCE

--

-- Format of the params value is according to the ProsePreconfiguration-r12 ASN.1 data type

-- described in TS 36.331 [241].

--

{

timeStamp	[0] TimeStamp OPTIONAL,

params		[1] OCTET STRING

}

RadioResourcesIndicator	::= INTEGER

{

operatorProvided	(1),

configured			(2)

}

RangeClass					::= ENUMERATED

{

reserved					(0),

fiftyMeter					(1),

onehundredMeter				(2),

twohundredMeter				(3),

fivehundredMeter			(4),

onethousandMeter			(5)

}

ReasonforCancellation		::= ENUMERATED

{

proximityAlerted			(0),

timeExpiredWithNoRrenewal	(1),

requestorCancellation		(2)

}

--

#### -- S

--

ServiceChangeCondition	::= BIT STRING

{

pLMNchange 					(0),

coverageStatusChange		(1),

locationChange				(2)

}

--

#### -- T

--

TransmitterInfo		::= SEQUENCE

{

sourceIPaddress	[0] IPAddress,

proSeUEID		[1] OCTET STRING

}

.#END

Tenth change

#### 5.2.4.8	Monitoring Event CDRs

This subclause contains the abstract syntax definitions that are specific to the Monitoring Event CDR types defined in TS 32.278 [38].

.$MONTEChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) mONTEChargingDataType (12) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

DiameterIdentity,

LocalSequenceNumber,

ManagementExtensions,

NodeID,

RecordType,

ServiceContextID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version20 (20)}

-- from TS 29.002 [214]

UserCSGInformation

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

;

--

#### -- Monitoring Event RECORDS

--

MERecordType		::= CHOICE

--

-- Record values 103..104 are Monitoring Event specific

--

{

mECORecord			[103] MECORecord,

mERERecord			[104] MERERecord

}

MECORecord	::= SET

{

recordType								[0] RecordType,

retransmission							[1] NULL OPTIONAL,

serviceContextID						[2] ServiceContextID OPTIONAL,

nodeID									[3] NodeID OPTIONAL,

recordTimeStamp							[4] TimeStamp OPTIONAL,

eventTimestamp							[5] TimeStamp OPTIONAL,

monitoringEventConfigurationActivity	[6] MonitoringEventConfigurationActivity OPTIONAL,

sCEFReferenceID							[7] SCEFReferenceID OPTIONAL,

sCEFID									[8] DiameterIdentity OPTIONAL,

monitoringType							[9] MonitoringType OPTIONAL,

maximumNumberofReports					[10] INTEGER OPTIONAL,

monitoringDuration						[11] TimeStamp OPTIONAL,

chargeablePartyIdentifier				[12] UTF8String OPTIONAL,

monitoredUser							[13] IMSI OPTIONAL,

maximumDetectionTime					[14] INTEGER OPTIONAL,

localRecordSequenceNumber				[15] LocalSequenceNumber OPTIONAL,

reachabilityConfiguration				[16]	ReachabilityConfiguration OPTIONAL,

locationType							[17] LocationType OPTIONAL,

accuracy								[18] Accuracy OPTIONAL,

listofLocations							[19] SEQUENCE OF EPSLocationInfo OPTIONAL,

monitoringEventConfigStatus				[20] MonitoringEventConfigStatus OPTIONAL,

recordExtensions						[21] ManagementExtensions OPTIONAL

}

MERERecord	::= SET

{

recordType								[0] RecordType,

retransmission							[1] NULL OPTIONAL,

serviceContextID						[2] ServiceContextID OPTIONAL,

nodeID									[3] NodeID OPTIONAL,

recordTimeStamp							[4] TimeStamp OPTIONAL,

localRecordSequenceNumber				[5] LocalSequenceNumber OPTIONAL,

listofMonitoringEventReportData			[6] SEQUENCE OF MonitoringEventReportData OPTIONAL,	recordExtensions						[7] ManagementExtensions OPTIONAL

}

--

#### -- Monitoring Event DATA TYPES

--

--

#### -- A

--

Accuracy			::= ENUMERATED

--

-- Note: value "3" is not used in this specification: it is provided to reflect

-- the full list specified in TS 29.336 Accuracy AVP

--

{

cGIECGI				(0),

eNB					(1),

lATARA				(2),

pRA					(3)

}

CauseType		::= ENUMERATED

{

radioNetworkLayer		(0),

transportLayer			(1),

nAS						(2),

protocol					(3),

miscellaneous			(4)

}

--

#### -- C

--

CommunicationFailureInfo		::= SEQUENCE

{

causeType	[0] CauseType OPTIONAL,

s1APCause	[1] INTEGER OPTIONAL,

rANAPCause	[2] INTEGER OPTIONAL,

bSSGPCause	[3] INTEGER OPTIONAL,

gMMCause		[4] INTEGER OPTIONAL,

sMCause		[5] INTEGER OPTIONAL

}

CurrentLocationRetrieved		::= ENUMERATED

{

activeLocationRetrieval		(0)

}

--

#### -- E

--

EPSLocationInfo		::= SEQUENCE

--

-- Only one element is present.

--

{

mMELocationInformation	[0] MMELocationInformation OPTIONAL,

sGSNLocationInformation	[1] SGSNLocationInformation OPTIONAL

}

--

#### -- L

--

LocationType				::= ENUMERATED

{

currentLocation				(0),

lastKnownLocation			(1)

}

--

#### -- M

--

MMELocationInformation		::= SEQUENCE

{

eUTRANCellGlobalIdentity		[0] OCTET STRING OPTIONAL,

trackingAreaIdentity			[1] OCTET STRING OPTIONAL,

geographicalInformation		[2] OCTET STRING OPTIONAL,

geodeticInformation			[3] OCTET STRING OPTIONAL,

currentLocationRetrieved		[4] CurrentLocationRetrieved OPTIONAL,

ageOfLocationInformation		[5] INTEGER OPTIONAL,

userCSGInformation			[6] UserCSGInformation OPTIONAL,

eNodeBID						[7] OCTET STRING OPTIONAL

}

MonitoringEventConfigStatus		::= SEQUENCE

{

serviceResult	[0] ServiceResult OPTIONAL,

sCEFReferenceID	[1] SCEFReferenceID OPTIONAL,

sCEFID			[2] DiameterIdentity OPTIONAL

}

MonitoringEventConfigurationActivity				::= ENUMERATED

{

create				(0),

transfer			(1),

update				(2),

delete				(3)

}

MonitoringEventReportData		::= SEQUENCE

{

eventTimestamp							[0] TimeStamp OPTIONAL,

sCEFReferenceID							[1] SCEFReferenceID OPTIONAL,

sCEFID									[2] DiameterIdentity OPTIONAL,

monitoringEventReportNumber				[3] INTEGER OPTIONAL,

chargeablePartyIdentifier				[4] UTF8String OPTIONAL,

monitoredUser							[5] IMSI OPTIONAL,

monitoringType							[6] MonitoringType OPTIONAL,

reachabilityInformation					[7] ReachabilityType OPTIONAL,

reportedLocation							[8] EPSLocationInfo OPTIONAL,

communicationFailureInformation 			[9] SEQUENCE OF CommunicationFailureInfo OPTIONAL,

listOfNumberOfUEPerLocationReport		[10] SEQUENCE OF NumberOfUEPerLocationReport OPTIONAL

}

MonitoringType				::= ENUMERATED

--

-- Note: value "3" and "4" are not used in this specification: they are provided to reflect the full

-- list specified in TS 29.336 Monitoring-Type AVP.

--

{

lossOfConnectivity						(0),

ueReachability							(1),

locationReporting						(2),

changeOfIMSIIMEISVAssociation			(3),

roamingStatus							(4),

communicationFailure					(5),

availabilityAfterDDNFailure				(6),

numberOfUEPerLocation					(7)

}

--

#### -- N

--

NumberOfUEPerLocationReport		::= SEQUENCE

{

ePSLocationInformation	[0] EPSLocationInfo OPTIONAL,

uECount					[1] INTEGER OPTIONAL

}

--

#### -- R

--

ReachabilityConfiguration		::= SEQUENCE

{

reachabilityType			[0] ReachabilityType OPTIONAL,

maximumLatency				[1] INTEGER OPTIONAL,

maximumResponseTime			[2] INTEGER OPTIONAL

}

ReachabilityType			::= ENUMERATED

{

reachabilityforSMS				(0),

reachabilityforData				(1)

}

--

#### -- S

--

SGSNLocationInformation		::= SEQUENCE

{

cellGlobalIdentity			[0] OCTET STRING OPTIONAL,

locationAreaIdentity			[1] OCTET STRING OPTIONAL,

serviceAreaIdentity			[2] OCTET STRING OPTIONAL,

routingAreaIdentity			[3] OCTET STRING OPTIONAL,

geographicalInformation		[4] OCTET STRING OPTIONAL,

geodeticInformation			[5] OCTET STRING OPTIONAL,

currentLocationRetrieved		[6] CurrentLocationRetrieved OPTIONAL,

ageOfLocationInformation		[7] INTEGER OPTIONAL,

userCSGInformation			[8] UserCSGInformation OPTIONAL

}

SCEFReferenceID	::= INTEGER (0..4294967295)

--

-- 0..4294967295 is equivalent to 0..2**32-1

--

ServiceResult		::= SEQUENCE

{

vendorId			[0] INTEGER OPTIONAL,

serviceResultCode	[1] INTEGER OPTIONAL

}

.#END

End of changes