3GPP TSG-SA5 Meeting #154	S5-242067

# Changsha, China, 15 – 19 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0352 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.622 Fix description of MdtConfig <<dataType>> attributes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMDT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | S5-235196 (Rel-18 CR TS 28.622 Restructuring TraceJob) was agreed at SA5#150. This contribution proposed restructuring of TraceJob IOC, building on two main changes: - Allocating a large list of IOC attributes into two big dataTypes TraceConfig and MdtConfig.The allocation criterion was dependent on the semantics of these attributes (i.e. whether they qualified for either Trace or MDT). - Splitting MdtConfig into dataTypes InmmediateMdtConfig and LoggedMdtConfig. As part of change 1, TraceJob IOC has now a “mdtConfig” attribute, which is of MdtConfig dataType. This attribute has support qualifier CM, meaning it shall be supported if JobType includes MDT. If we have a look at the MdtConfig dataType, a number of issues can be noticed. The aim of this CR is to identify and fix them. Issue #1: the support qualifier for “anonymizationOfMdtData” (currently CM)” is wrong; it needs to be changed to M instead. Reason is as follows: - The condition for “areaScope” attribute notes that this attribute shall be supported when MDT is supported. However, this condition is always met, precisely because this attribute is defined within MdtConfig. In this regard, it makes sense to change CM to M for “areaScope” Issue #2: the support qualifier for “areaScope” (currently O) is wrong; it needs to be changed to M instead. Reason is as follows: - The condition for “anonymizationOfMdtData” attribute notes that this attribute shall be supported a) when MDT is supported and b) the areaScope attribute is present. Condition a) is always met, precisely because anonymizationOfMdtData is defined within MdtConfig. The fact that “areaScope” is mandatory makes condition b) true as well. As conditions a) and b) are always met, it makes sense to change O to M for “areaScope” Issue #3: conditions for “anonymizationOfMdtData” and “area Scope” are no longer needed. Reason is as follows: - By changing support qualifier of both attributes to M (see issues #1 and #2), conditions for both attributes no longer exist. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In MdtConfig dataType, for “areaScope” attribute change support qualifier from O to M, as reported in issue #1. - In MdtConfig dataType, for “anonymizationOfMdtData” attribute change support qualifier from CM to M, as reported in issue #2. - Remove the conditions of “areaScope” and “anonymizationOfMdtData”, as reported in issue #3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Attributes with wrong support qualifiers may compromise interoperability across vendor solutions, specially when these are mandatory, as is the case. Incorrect description of conditions may lead to wrong implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.58 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | S5-242067 is a revision of S5-241454 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 4.3.58	MdtConfig <<dataType>>

#### 4.3.58.1	Definition

This <<dataType>> defines the configuration parameters of IOC TraceJob which are specific for MDT.

The attribute anonymizationOfMdtData specifies the level of anonymization of MDT data.

The optional attribute areaScope allows to specify the area in terms of cells or Tracking Area/Routing Area/Location area where the MDT data collection shall take place. In case of RLF_REPORT_ONLY and RCEF_REPORT_ONLY the optional attribute areaScope allows to specify the eNB or list of eNBs or gNB or list of gNBs where the reports should be collected.

The optional attribute sensorInformation allows to specify  the sensor information to include.

Based on the value configured for attribute jobType in IOC TraceJob, the attributes immediateMdtConfig or loggedMdtConfig are available: In case of IMMEDIATE_MDT_ONLY or IMMEDIATE_MDT_AND_TRACE the attribute immediateMdtConfig is applicable. In case of LOGGED_MDT_ONLY or LOGGED_MBSFN_MDT the attribute loggedMdtConfig is applicable.

#### 4.3.58.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| anonymizationOfMdtData | M | T | T | F | T |
| areaScope | M | T | T | F | T |
| sensorInformation | CO | T | T | F | T |
| immediateMdtConfig | CM | T | T | F | T |
| loggedMdtConfig | CM | T | T | F | T |

#### 4.3.58.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sensorInformation (support qualifier) | This attribute shall be present only if NR MDT is supported. |
| immediateMdtConfig (support qualifier) | This attribute shall be present only if MDT is supported and the jobType attribute is set to Immediate MDT or combined Trace and Immediate MDT |
| loggedMdtConfig (support qualifier) | This attribute shall be present only if MDT is supported and the jobType attribute is set to Logged MDT or Logged MBSFN MDT. |

#### 4.3.58.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions.

End of change