3GPP TSG-SA5 Meeting #155 	S5-245291

Jeju, South Korea, 27 - 31 May 2024

# Source:	Ericsson

# Title:	Management of LTM

# Document for:	Discussion, Endorsement

Agenda Item:	6.4.27

# 1	Decision/action requested

SA5 to discuss and endorse this proposal.

# 2	References

[1]	S5-221400, Revised WID on Enhancement of Handover Optimisation

[2]	3GPP TS 28.554, Management and orchestration; 5G end to end Key Performance Indicators (KPI)

# 3	Rationale

The traditional layer 3 handover has in recent releases been supplemented with new handover mechanisms. In Rel-16, Conditional Handover (CHO) was introduced. During Rel-17, SA5 added management support for CHO. At that time, SA5 added requirements, use cases, and related NRM additions as well as PM measurements and an updated a KPI to its specifications, under the Work Item Revised WID on Enhancement of Handover Optimisation [1].

In Rel-18, the new handover mechanism Lower-layer Triggered Mobility (LTM) was added to NR. With the addition of LTM, we see a need to add management support. Similar to CHO, we propose to add requirements, use cases, NRM additions and related PM measurements and an updated a KPI for LTM.

When developing NRM support for CHO, the configuration SA5 concluded on was an on/off switch for CHO on cell relations. We believe that such a switch is needed for LTM as well.

Proposal 1: SA5 to add an on/off switch for LTM, on Neighbour Cell Relation level.

When introducing PM for CHO, extra effort was taken to separate measurements for legacy handover, and for CHO. One idea behind this effort was to provide a solution that could scale when new envisioned handover mechanisms were introduced. LTM is such a new envisioned handover mechanism. We therefore propose that PM for LTM shall be added in a way that follows the pattern SA5 created for adding PM for CHO.

Proposal 2: SA5 to add PM for LTM in a way that follows the pattern SA5 created for adding PM for CHO.

Furthermore, we suggest that no new KPIs shall be defined, and existing KPIs shall be extended to encompass LTM. For example, TS 28.554 [2] clause 6.6.1 defines NG-RAN handover success rate. We suggest that this KPI should cater for LTM, in addition to existing legacy HO and CHO.

Proposal 3: Existing KPIs shall be extended to cater for LTM.

# 4	Detailed proposal

Proposal 1: SA5 to add an on/off switch for LTM, on Neighbour Cell Relation level.

Proposal 2: SA5 to add PM for LTM in a way that follows the pattern SA5 created for adding PM for CHO.

Proposal 3: Existing KPIs shall be extended to cater for LTM.