3GPP TSG-SA5 Meeting #155 	S5-242659

Jeju, South Korea, 27 - 31 May 2024

# Source:	Huawei

# Title:	Add exposure roles and concepts

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

Group is asked to approve this proposal.

# 2	References

[1]	3GPP TS 28.879 Study on OAM for service management and exposure to external consumers

# 3	Rationale

There are multiple concepts and solutions focusing on exposure, the group needs to align the understanding on the following issues:

- Roles related to exposure of management services: means who may contribute and get involved into the exposure discussion.

- Concept of potential consumer: whether it’s necessary to differentiate the external consumer and internal consumer.

- What information/services to be exposed to serve management purpose: exposure with management services (i.e. by means of using component A/B/C) or abstraction information of management services with other interface format (e.g. openAPI etc.).

- Relation between SA5 exposure mechanism with the exposure solutions in other working groups: mainly SA6 and TM forum, need to have an integrated view on the relations with other groups.

# 4	Detailed proposal

It’s proposed to make the following change to TR 28.879.

# 4 Concepts and Background

## 4.x	Roles related to exposure of management services

The role concepts CSP, CSC, NOP, NEP, NISP, DCSP as defined in TS 28.530 clause 4.8 apply here in the context of exposure of management services.

## 4.y	External MnS Consumer concept

An external MnS consumer refers to the MnS Consumer which is outside of the entity who provides 3GPP services. The external MnS consumer could utilize the exposure capabilities provided by 3GPP management system and provide their own services to their customers. An External MnS consumer may not care about network details of 3GPP network information, but some high level abstraction information which help to build their services would be needed to be exposed. Verticals are typical example of external MnS consumer and they take the role of CSP.

## 4.1	Exposure of management services

### 4.1.0	Background of existing solutions

A MnS can be available (i.e., instantiated, and ready to be exposed) and hence discoverable by internal and external MnS consumers. An available MnS is described by the MnsInfo IOC (see clause 4.3.42 of TS 28.622 [3]). The available MnSs in the 3GPP management system can be found in the MnSRegistry IOC (see clause 4.3.41 of TS 28.622 [3]) and the use case procedures for discovery are described clause 5 in TS 28.537 [4]. In the study, we will focus on how the external MnS consumer can leverage external exposure frameworks (e.g., the CAPIF framework [5]) to access and consume management services with regards to all the stages involved: publishing, discovery and access control.

### 4.1.1	Overview

There are two types of management service concepts potentially could be exposed depends on the granularity of the details and purpose of usages:

1. Management service exposure: A management service (MnS) is identified by different component types, i.e., MnS component type A (management operations or notifications), MnS component type B (managed objects), and MnS component type C (performance and fault information) (as defined in clause 4.2 of TS 28.533[2]). MnS represents the network level management service and is always bound with network resource related managed object (e.g. gNBFunction, NWDAFFunction etc).

2. Abstraction of management exposure: the abstraction of management information represents the service level management service and is bound to service related managed object (e.g. network slice instance, area based services etc).

### 4.1.2	Exposure of management service

The present document studies a generic approach to expose management services to MnS consumers. A management service (MnS) is identified by different component types, i.e., MnS component type A (management operations or notifications), MnS component type B (managed objects), and MnS component type C (performance and fault information) (as defined in clause 4.2 of TS 28.533[2]).  In order for the MnS consumer to consume management services, the access rights have to be configured for the MnS consumer accordingly. The access rights determine what MnS managed objects and related information (i.e. component type B or C and their associated information) of the MnS they can access. In addition, the access rights also determine what operations (i.e. MnS component A) the MnS consumer can or cannot perform.

For example, considering Figure 4.1.1-1, the MnS A producer produces MnS A that is directly consumed by the internal MnS consumer and the external MnS consumers 1 and 2 respectively. Depending on the different access rights assigned to the MnS consumers, it’s possible that:

- The internal MnS Consumer and external MnS Consumer 1 and 2 can access the same or different managed objects under the management scope of MnS A producer.

- The operations (i.e., CRUD) or notifications that internal MnS Consumer and external MnS Consumer 1 and 2 can perform on the accessed managed objects are the same or different.

- The internal MnS Consumer and external MnS Consumer 1 and 2 can or cannot access the same or different performance and fault information associated with the accessible managed objects of MnS A.

Figure 4.1.1-1: Example of Exposing Management Services concept.

To provide a MnS to be consumed, it first needs to be published to the MnS discovery producer entity. After it’s published, the MnS will be available to be discovered by the MnS consumers. Following discovery, authentication and authorization mechanisms need to be applied to ensure that the MnS consumers only have access to the allowed MnS component type A, B, or C for the MnS.

### 4.1.3	Exposure of abstraction management service

Abstraction of management information could be done based on the capability of the abstraction layer and the requirements from the consumer. There are following potential solutions which are related to exposure of abstraction management services, study is needed to provide an integrated view on the management information exposure:

- Network exposure solution provided by SA5 MnS.

- NSCE and SEAL provided by SA6.

- TMF service management capability provided in TMF641.

The following diagram shows an example analysis of networks slicing exposure based on existing available solution from multiple working groups.