3GPP TSG-SA5 Meeting #155 	S5-242883

Jeju, South Korea, 27 - 31 May 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Add a new potential solution for logging the management service API invocations to the CCF

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

When exposing management services for consumption by external MnS consumers, it is crucial to log the service API invocations to the CCF. Accordingly, this pCR proposes to add a new potential solution for creating and logging the management service API invocations to the CCF to be added to clause 5.1 of TR 28.879 [1].

# 4	Detailed proposal

It is proposed that the following changes be made to clause 5.1 of TR 28.879 [1].

Begin Change

## 5.1 Exposure of management services

#### 5.1.W.3	Potential solutions

##### 5.1.W.3.X	Potential solution #X: Creation and logging of the management service API invocations

###### 5.1.W.3.X.1	Introduction

This potential solution describes how the MnS producer can create the management service API invocation log. Secondly, this solution describes how the MnS producer can send these logging data to the CCF for authorized consumers to consume.

###### 5.1.W.3.X.2	Description

To log the management service API invocations to the CCF, the MnS producer should create an invocation log (see clause 8.7 in TS 29.222[13]). Table 5.1.W.3X.2-1 maps the CAPIF InvocationLog data type and the MnSInfo IOC (see clause 4.3.42 of TS 28.622[3]) attributes.

Table 5.1.W.3X.2-1: Mapping between CAPIF InvocationLog data type and the MnSInfo IOC attributes (Table extract from Table 8.7.4.2.2-1 in clause 8.7.4.2.2 in TS 29.222[13])

| Attribute name | Data type | P | Cardinality | Description | Mapping to MnSInfo IOC attributes/Comments |
| --- | --- | --- | --- | --- | --- |
| aefId | string | M | 1 | Identity information of the API exposing function requesting logging of service API invFocations | MnS producer obtains this information during the API provider domain functionalities' registration process with the CCF |
| apiInvokerId | string | M | 1 | Identity of the API invoker which invoked the service API | Provided by the external MnS consumer when invoking the management service API |
| logs | array(Log) | M | 1..N | Service API invocation log | See Table 5.1.W.3X.2-2 |
| supportedFeatures | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |

Table 5.1.W.3X.2-2: Mapping between CAPIF Log data type and the MnSInfo IOC attributes (Table extract from Table 8.7.4.2.3-1 in clause 8.7.4.2.3 in TS 29.222[13]

| Attribute name | Data type | P | Cardinality | Description | Mapping to MnSInfo IOC attributes (see clause 4.3.42 of TS 28.622[3])/Comments |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | String identifying the API invoked. | The CCF assign this attribute during the publishing of MnS. So, the MnS producer (playing both the roles of the APF and AEF) would have this parameter for the published management service. |
| apiName | string | M | 1 | Name of the API which was invoked, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. | Same as mnsLabel |
| apiVersion | string | M | 1 | Version of the API which was invoked | Same as mnsVersion |
| resourceName | String | M | 1 | Name of the specific resource invoked | What would the value of this attribute be? The attribute could be the DNs of a subset or all of the MOIs in the mnsScope attribute of MnSInfo IOC. However, mnsScope attribute is of the "Dn" data type and not a string, and the cardinality is * |
| uri | Uri | 0 | 0..1 | Full URI of the API resource as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| protocol | Protocol | M | 1 | Protocol invoked. | HTTPS?? |
| operation | Operation | C | 0..1 | Operation that was invoked on the API, only applicable for HTTP protocol. | E.g., PUT, POST, GET, DELETE, PATCH etc |
| result | string | M | 1 | For HTTP protocol, it contains HTTP status code of the invocation | HTTP status codes |
| invocationTime | DateTime | O | 0..1 | Date on which it was invoked | Is this time when the MnS producer receives the service API invocation request from the external MnS consumer? Needs further clarification |
| invocationLatency | DurationMs | O | 0..1 | Latency for the API invocation. | Is this the duration of the service API invocation (i.e., from the time the external MnS consumer sends the invocation request to the MnS producer to the time when the MnS producer sends the response back to the external MnS consumer)? Needs further clarification |
| inputParameters | ANY TYPE (NOTE) | O | 0..1 | List of input parameters |  |
| OutputParameters | ANY TYPE (NOTE) | O | 0..1 | List of output parameters |  |
| srcInterface | InterfaceDescription | O | 0..1 | Interface description of the API invoker. |  |
| destInterface | InterfaceDescription | O | 0..1 | Interface description of the API invoked. |  |
| fwdInterface | string | O | 0..1 | It includes the node identifier (as defined in IETF RFC 7239 [20] of all forwarding entities between the API invoker and the AEF, concatenated with comma and space, e.g. 192.0.2.43:80, unknown:_OBFport, 203.0.113.60 |  |
| rootObjectInstances (clause 4.4.1 TS 28.622[3]) | Dn | O | * | List of object instances. Each object instance is identified by its DN and designates the root of a subtree that contains the root object and all descendant objects | New enhancement to the Log data type This could be the DNs of all the MOIs in the mnsScope of the management service. |
| objectInstances (clause 4.4.1 TS 28.622[3]) | Dn | O | * | List of managed object instances. Each object instance is identified by its DN. | New enhancement to the Log data type This could be the DNs of the subsets of MOIs in the mnsScope of the management service. |
| NOTE: Any basic data type defined in OpenAPI Specification [3] may be used. |  |  |  |  |  |

To monitor which MOIs the external MnS consumers accessed, it could be vital to enhance the Log data type with two new attributes, i.e., the rootObjectInstances or objectInstances, as indicated in Table 5.1.W.3X.2-2. 

After creating the invocation log, the MnS producer can log the data to the CCF via the CAPIF-3 interface.

#### 5.1.W.4	Evaluation of potential solutions

##### 5.1.W.4.1 Potential solution #X: Creation and logging of the management service API invocations

The proposed solution proposes to enhance the MnS producer with the capabilities to create the invocation log (indicated in Table 5.1.W.3X.2-1). Thus, the MnS producer logs these data to the CCF. However, the proposed solution still leaves a few open questions/gaps that need to be addressed:

- How does the MnS producer keep track of the mapping between the management service API that was invoked by the external MnS consumer and the ApiId attribute (which is an attribute assigned by the CCF during the publishing of MnS)? This capability would have to be enabled in the MnS producer.

- New requirement for eSBMA study

- What is the value of the mandatory resourceName attribute of the Log type (see Table 5.1.W.3X.2-2)?

- Clarifications on what invocationTime and invocationLatency attributes of the Log data type (see Table 5.1.W.3X.2-2) represent? The descriptions are not clear.

- Some clarifications are to be sent to 3GPP SA6/CT3.

- When exposing management services to external MnS consumers, there is a need for a mechanism to track which MOIs were accessed and what operations were performed. The MnS producer would have this information (i.e., either the rootObjectInstances or objectInstances that were affected as part of the invocation); accordingly, the MnS producer should be enabled to provide this information as part of the invocation log.

- New requirement for eSBMA study

- There should be enhancements to the log data type to support the rootObjectInstances and objectInstances attributes (as shown in Table 5.1.W.3X.2-2).

End Change