3GPP TSG-SA5 Meeting #155 	S5-242986

Jeju, South Korea, 27 - 31 May 2024

# Source:	China Unicom

# Title:	Add new use case and requirements on S-RAN management of Indirect Network Sharing

# Document for:	Approval

Agenda Item:	6.19.19

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	S5-242035 Revised SID on Management of Network Sharing Phase 3.

[2]	S2-2405669, CR 23.501, Indirect network sharing.

[3]	3GPP TS 32.130: " Network sharing; Concepts and requirements".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

3	Rationale

As agreed in S5-242035[1], WT-3 is described as "Investigate potential enhancements for Indirect Network Sharing scenarios". Since S2-2405669[2] has been approved in SA2, this contribution is to add new use case and requirements on management supporting for Indirect Network Sharing.

In TS 32.130[3], 3GPP management system has already supported S-RAN management in two types of active RAN sharing scenarios, MOCN and GWCN. As a novel type of active RAN sharing defined in TS 22.261[4], S-RAN of Indirect Network Sharing is supposed to be managed by 3GPP management system.

4	Detailed proposal

1st Change

5 	Use Cases

5.X	Use Case#X: S-RAN management of Indirect Network Sharing

5.X.1	Description

According to TS 22.261[2], Indirect Network Sharing is defined as follow:

" a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator’s core network is routed through the Hosting NG-RAN Operator’s core network ".

Therefore, Indirect Network Sharing is proposed to share NG-RAN indirectly through core network. Regarding 5G MOCN, NG-RAN is shared directly by the connection to each POP’s core network. According to the definition of  S-RAN in TS 32.130[4], shared NG-RAN in the Indirect Network Sharing scenario is S-RAN. As a result, each POP has requirements to manage S-RAN similar with MOCN and GWCN scenarios. Similarly, the hosting operator, as MOP in the Indirect Network Sharing scenario, should have capabilities to authorize consumers to manage S-RAN.

5.X.2	Potential requirements

REQ-NetShare_Req-1: The 3GPP management system of the MOP should support a capability allowing its authorized consumer to manage S-RAN in the Indirect Network Sharing scenario.

End of Change