# 3GPP TSG-SA5 Meeting #156	S5-243550

# 19 – 23 August 2024, Maastricht, NL

# Source:	Nokia

# Title:	pCR 28.915 NDT data drift

# Document for:	Approval

# Agenda Item:	6.19.5

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 	3GPP TR 28.915-010 “Study on management aspect of Network Digital Twin”.

# 3	Rationale

Network digital twins are supposed to provide modelling capabilities that may be used to accomplish network related automation operations. To provide meaningful results, the Network digital twin needs to emulate the behavior of the 3GPP network, so that the result of the operations on the virtual replica are a good approximation to similar operations on the actual network. The automation function may be able to rely on the results from network digital twin only in the case that the approximation between the virtual replica and the actual network is sufficiently good. This pCR extends the use case of NDT support to network automation enabling the definition of target values of approximation or drift  between the virtual replica and the actual network.

# 4	Detailed proposal

Start of change

# 5	Use cases, potential requirements and solutions

## 5.5 Use case 5: NDT support to network automation

### 5.5.1	Description

NDTs may be used to support many automation use cases. An NDT may be integrated into a network automation function, or it may be external to the network automation function. In the case where the NDT is external to the network automation function, it should be possible for the network automation function to define and configure into the NDT the scenario that should be modelled and simulated by the NDT. Then the NDT should implement the defined scenario, simulate it, and subsequently provide an output representing the statues of different network metrics for the simulated scenario.

The NDT can be realized by synchronizing the data from the mobile network to the emulated network using different data granularities as well as time-spans, e.g. historical data, real-time data, or any combination thereof. Thus, the measure on how good is the approximation between actual mobile network and its virtual replica NDT can largely differentiate based on the data used to synchronize the NDT with the actual network, the synchronization frequency, etc. According to such approximation level the automation functions can rely to different extent on the results from network digital twin. Therefore, MnS consumers need to be enabled to provide the target values for measure of approximation or drift between actual mobile network and its NDT. Such measure may include the following:

• Input data drift: Determining the drift in input data used to synchronize the NDT compared to the actual mobile network data.

• Output data drift: Determining the drift between the data obtained as a result of execution of NDT environment and the corresponding data observed in the actual mobile network.

The MnS consumer may use such measurements as input to decide when and how to perform the synchronization between the actual mobile network and the NDT.

### 5.5.2	Potential Requirements

REQ-NDT-1: The NDT should support a capability to model the behavior of the network and provide the outcomes of such modelling to consumers.

REQ-NDT-2: The NDT should support a capability enabling an Mns consumer to define the network scenario that should be modelled and simulated.

REQ-NDT-3: The NDT should support a capability to provide an output representing the statues of different network metrics for the simulated scenario.

REQ-NDT-4:  The NDT should support a capability enabling MnS consumers to define the target values for measure of drift between actual mobile network and its NDT

### 5.5.3	Potential Solutions

- introduce an information object class representing an NDT, say called NetworkDigitalTwin

- introduce a data type representing the network scenario to be modeled and simulated, say called nDTSimulationScope

- introduce a data type representing the output of modelling and simulating a specific network scenario. The datatype may be called nDTSimulationOutput

  - the NDT may have 1 or more nDTSimulationOutput objects wit

- introduce a data type representing the performance data and/or KPI that are computed by the NDT for the simulated scenario.

- introduce on the NetworkDigitalTwin IOC an attribute representing the measure of drift between actual mobile network and its NDT. The drift, say called nDTDataDrift, may for example be a percentage value indicating how much the NDT has drifted for the actual network data

### 5.5.4	Evaluation of solutions

TBD

End of modifications