# 3GPP TSG SA WG5 Meeting #156	S5-243667

# Maastricht, The Netherlands 19 - 23 August 2024

# Source:	China Mobile

# Title:	TR 28.915 Add a general NDT framework

# Document for:	Approval

Agenda Item:	6.19.5

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP draft TR 28.915: “Management and orchestration; Study on management aspects of Network Digital Twin v0.1.0”.

3	Rationale

This contribution proposes to add a general NDT framework in TR 28.915, to show the NDT role in the management loop, between the NDT producer, potential NDT consumers, and other network/management functions/services.

4	Detailed proposal

First Change

4.x.1	General framework

Figure 4.x.1 illustrates the general framework of NDT. Depending on the scenario, the MnS consumer could request the NDT to model a mobile network or part of one, evaluate the corresponding impact, and receive the report of the simulated impact generated by the NDT. The NDT may synchronize data from the managed network for modeling by acting as an MnS consumer. The MnS produce here is the the entity who provides the NDT for network simulation. Therefore, in the context of 3GPP network management, the major components of the system are:

- NDT

- Managed network

- Management service producers

- Management service consumers

Managed network

Management service producers

NDT

Management service consumers

- Figure 4.x.1: Major components of 3GPP network management

4.x.2	Illustrate the life-cycle management of an NDT

- When the NDT received a request to create an NDT from MnS consumer, the MnS producer who provides the NDT may create an instance to fulfil or satisfy the specific scenarios. Figure 4.x.2 shows the following management capabilities to support an NDT instance life-cycle management:

- Figure 4.x.2: Life-cycle phases of an NDT instance

An NDT instance includes the following management capabilities:

-	NDT Creation, a MnS Consumer request MnS producer to create a new NDT instance.

-	NDT Activation, MnS Consumer request MnS producer to activate an NDT instance when the  resources and models to fulfil the NDT instance is ready.

-	NDT Commissioning, NDT instance executes the required actions.

-	NDT De-activation, MnS consumer request MnS producer to de-activate an NDT instance for a temporary suspension.

-	NDT Termination, MnS Consumer request MnS producer to remove an NDT instance.

NDT

End of Changes