3GPP TSG-SA5 Meeting #156 	S5-243882

Maastricht, NL, 19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.558 | CR | 0013 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Measured UE Identifier in the measurement template |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UE level measurements defined in TS 28.558 are collected by Trace/MDT mechanisms as defined in TS 32.422. For signaling based activation, the identifier of the Trace Target (UE) has been already defined in TS 32.422. For management based activation for the 5GC UE level measurements collection, the Trace Target in TS 32.422 refers to the “Measured UE Identifier” defined in TS 28.558. For management based activation of MDT to collect NG-RAN UE level measurements collection, the Trace Target has already been defined in TS 32.622. Therefore the “Measured UE Identifier” is only used for management based activation of the 5GC UE level measurement collection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the “Measured UE Identifier” is only used for management based activation for 5GC UE level measurements collection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unclear definition would lead to wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modified section

Annex A (normative):
Template for definitions of UE level measurements

Following is the template for defining the UE level measurements for 5G system.

Clause title: descriptive measurement name

This is a descriptive name of the measurement type that is specified as clause X of the present document.

a)	Description

This subclause contains an explanation of the measurement.

b)	Collection Method

This n contains the form in which this measurement data is obtained:

-	CC (Cumulative Counter);

-	GAUGE (dynamic variable), used when data being measured can vary up or down during the period of measurement;

-	DER (Discrete Event Registration), when data related to a particular event are captured every nth event is registered, where n can be 1 or larger;

-	SI (Status Inspection);

-	TF (Transparent Forwarding);

-	OM (Object Mapping).

c)	Condition

This subclause contains the condition which causes the measurement result data to be updated;
This will be defined by identifying protocol related trigger events for starting and stopping measurement processes, or updating the current measurement result value. Where it is not possible to give a precise condition, then the conditional circumstances leading to the update are stated.

If a measurement is related to “external” technologies, this subclause shall give a brief reference to other standard bodies.

d)	Measurement Result (measured value(s), Units)

This subclause contains a description of expected result value(s) (e.g. a single integer value). If a measurement is related to “external” technologies, this subclause shall also give a brief reference to other standard bodies.

The definition applies for each measurement result.

e)	Measurement Type

This subclause contains a short form of the measurement name specified in the header, which is used to identify the measurement type in the result files.

The measurement names are dotted sequences of items. The sequence of elements identifying a measurement is organised from the general to the particular:

-	The first item identifies the measurement family (e.g. RRC, DRB, HO). Note that this family may also be used for measurement administration purpose.

-	The second item identifies the name of the measurement itself, for which the following rules shall apply:

- The second item of the measurement name can be divided into <Operation>, <Reason/Result> and <Direction> (and in that order)

- Examples of Operation can be Establishment, Release, and Modification

- Examples of Reason/Result can be Attempt, Failure, Success, Throughput and Volume

- Examples of Direction can be Incoming, Outgoing, Uplink and Downlink

-	Depending on the measurement type, additional items may be present to specify subcounters (failure causes, traffic classes, min, max, avg, G, U, ...). In case of multiple additional items, they are also represented as a dotted sequence of items. When available, the template will describe to which standard it is referring to for these additional items (e.g. cause, traffic class). Otherwise, the additional item semantics need to be described in detail in the present document.

Standardised causes will be a number. This number shall be derived according to which of the following rules applies:

-	For the standardised causes with numeric values explicitly specified in the reference specification, the subcounter name will be the number assigned to this cause in this reference specification.

-	For the standardised causes without numeric values explicitly specified in the reference specification, but where the causes are identified, the subcounter name shall be a number from 1 to n mapped in an incremental sequence to each of the specified causes following top-down sequence in the order they are identified in the reference specification (e.g. RRC.ConnEstabAtt.1, 1 identifies the establishment cause "emergency", see TS 38.331 [6]).

-	For the standardised causes without numeric values explicitly specified in the reference specification and the causes identified and the causes have been divided into different cause groups, the subcounter should be defined as the form of ‘Cause group’.Cause, where:

-	the subcounter name of ‘Cause group’ shall be an incremental number from 1 to n to identify each cause group specified in a top-down sequence following the order they are identified in the reference specification;

-	the subcounter name of cause within this cause group shall be an incremental number from 1 to n to identify each cause within the group specified in a top-down sequence following the order they are identified within the cause group in the reference specification;

the non standardised causes should be a string (e.g. RRC.ConnEstab.NoReply).

f)	Measured Object Class

This subclause describes the measured object class (e.g., NRCellCU, NRCellDU, GNBCUCPFunction, UPFFunction). The object class used for this purpose shall be in accordance with NRMs defined in any NRM IRPs, such as those defined in 3GPP TS 28.541 [3].

For object classes currently not defined according to the above, the present document defines its own nomenclature.

It is possible to use the same measurement name for a standardized measurement type implemented at a different object class level than the one defined in the Standard. The same measurement type can apply to one or more measurements for which all fields of the measurement template are the same except the item f) "Measurement Object Class". For instance, a measurement which uses the same template as a given measurement type but relates to another or different object classes shall have the same name.

g)	Measured UE Identifier

This element only indicates the type of the UE identifier that are used as Trace Target for management activation of 5GC UE level measurement collection (see TS 32.422 [13]).

The type of the UE identifier could be IMSI, IMEI, SUPI, or N4 Session ID, etc., subject to the specific measurement.

h)	Purpose

This optional clause aims at providing additional information on the interest of the measurement but is purely indicative.

End of modified sections