3GPP TSG SA WG5 Meeting #156	S5-243993

# Maastricht, The Netherlands 19 - 23 August 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0231 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.290 Correction on the non-blocking mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to 3GPP TS 23.503 clause 6.3.1 and 3GPP TS 29.512 clause 4.1.4.4.4, the PCC rule for charging includes a dedicated attribute “Service Data flow handling while requesting credit”, which indicates whether the service data flow is allowed to start while the SMF is waiting for the response to the credit request. This attribute is only applicable for charging method online and with values blocking or non-blocking. However, the charging principle for non-blocking mode is ambiguous. For example, when service delivery started in the non-blocking mode, it is not clear how to charge for the data volume that has been used before SMF receives the response from CHF for not granting quota to this service delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the non-blocking charging procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to charge for data volume in non-blocking mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.290 … CR 0232 TS 32.255 … CR 0544 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.3.2.3	Session based charging

For Converged Session based Charging, the following cases are supported:

-	SCUR

-	ECUR

Figure 5.3.2.3.1 shows a blocking mode scenario for Session based charging (SCUR) with: Unit Reservation, Decentralized and Centralized Unit Determination, Centralized Rating configuration, user’s account deduction, where the NF (CTF) invokes a converged charging service towards the CHF.

Figure 5.3.2.3.1: SCUR - Session based charging with Decentralized and Centralized Unit Determination, Centralized Rating

1)	Request for service delivery: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by the CHF to start.

2)	Units Determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging Data Request [Initial, Quota Requested]: The NF (CTF) sends the request to the CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Account, Rating, Reservation Control: the CHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the CHF performs the corresponding reservations.

5)	Open CDR: based on policies, the CHF opens a CDR related to the service.

6)	Charging Data Response [Initial, Quota Granted]: The CHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

7)	Granted Units Supervision: the NF (CTF) monitors the consumption of the granted units.

8)	Start of service delivery: the NF (CTF) starts to deliver the content/service based on the reserved number of units.

9)	Usage Reporting Trigger: the NF (CTF) generates charging data related to the service delivered that is not under quota management, based on a trigger for usage reporting is met.

10)	Charging Data Request [Update, Unit Used]: the NF (CTF) sends the request for reporting the related charging data, including the used units, to the CHF.

11)	Account, Rating Control: The CHF performs the reported usage process involving rating entity and user's account balance.

12)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

13)	Charging Data Response [Update]: The CHF informs the NF (CTF) on the result of the request.

14)	Quota management Trigger: A Trigger associated to Quota management is met. Units determination is performed when applicable.

15)	Charging Data Request [Update, Unit Used, Quota Requested]: the NF (CTF) sends the request to the CHF, for more units to be granted for the service to continue, and reporting the used units.

16)	Account, Rating, Reservation Control: The CHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

17)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

18)	Charging Data Response [Update, Quota Granted]: The CHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

19)	Content/Service Delivery: the NF (CTF) delivers the content/service based on the granted quota.

20)	Session released: the session is released.

21)	Charging Data Request [Termination, Unit Used]: the NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final consumed units.

22)	Account, Rating Control: The CHF performs the service termination process involving rating entity and user's account balance.

23)	 Close CDR: based on policies, the CHF closes the CDR with charging data related to the service termination and the last reported units.

24)	Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

Figure 5.3.2.3.2 shows a Non-blocking mode scenario for Session based charging (SCUR) with: Unit Reservation, Decentralized and Centralized Unit Determination, Centralized Rating configuration, user’s account deduction, where the NF (CTF) invokes a converged charging service towards the CHF.

NF (CTF) may use blocking mode instead when risk of quota overdraft is more important than latency.

Figure 5.3.2.3.2: SCUR - Session based charging with Decentralized and Centralized Unit Determination, Centralized Rating, immediate start of service delivery (Non-blocking mode)

1)	Request for service delivery and start of service delivery: A request for service session establishment is received in the NF (CTF). The NF (CTF) is configured to allow the service to be delivered.

2)	Units Determination: the NF (CTF) determines the number of units depending on the service requested, in "Decentralized Units determination" scenario.

3)	Charging Data Request [Initial, Unit Used, Quota Requested]: the NF (CTF) sends the request to the CHF to reserve the number of units for a rating group of the service if determined in step 2, it also report the used units for this rating group which indicate the non-blocking mode is used.

4)	Account, Rating, Reservation Control: the CHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the CHF performs the corresponding reservation.

5)	 Open CDR: based on policies, the CHF opens a CDR related to the service.

6)	Charging Data Response [Initial, Quota Granted]: the CHF grants the reserved number of units to NF (CTF).

7)	Granted Units Supervision: The NF (CTF) monitors the consumption of the granted units.

8)	Service delivery ongoing: the NF (CTF) continues to deliver the service.

9)	Usage reporting trigger: the NF (CTF) generates charging data related to a service delivered that is not under quota management, based on that a trigger for service usage reporting is met.

10)		Charging Data Request [Update, Unit Used]: the NF (CTF) reports the charging data related to service delivered, including the used units, to the CHF.

11)		Account, Rating Control: the CHF uses the reported charging data to rate the usage and deduct the funds corresponding to the usage on the account balance.

12)		Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

13)		Charging Data Response [Update]: The CHF informs the NF (CTF) on the result of the request.

14	)	Quota management Trigger: A Trigger associated to Quota management is met. Units determination is performed when applicable.

15)		Charging Data Request [Update, Unit Used, Quota Requested]: the NF (CTF) sends the request to the CHF, for more units to be granted for the service to continue, and reporting the used units.

16)		Account, Rating, Reservation Control: same as step 4, with the option to also deduct the funds corresponding to the usage on the account balance.

17)		Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

18)		Charging Data Response [Update, Quota Granted]: The CHF grants quota to NF (CTF) for the service, with the reserved number of units.

19)		Service delivery ongoing: the NF (CTF) continues to deliver the service.

20)		Service release: the NF (CTF) is requested to end the service delivery and does this.

21)		Charging Data Request [Termination, Unit Used]: the NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final consumed units.

22)		Account, Rating Control: the CHF performs the service termination process which involve using the reported charging data to rate the usage and deduct the funds corresponding to the usage on the account balance.

23)		Close CDR: based on policies, the CHF closes the CDR with charging data related to the service termination and the last reported units.

24)		Charging Data Response [Termination]: The CHF informs the NF (CTF)on the result of the request.

During the ongoing charging session in Figure 5.3.2.3.2, the first quota request for a new rating group that is using the non-blocking mode may also occur during step 8 to 19, which repeats from step 2 and replaces step 3 with Charging Data Request [Update, Unit Used, Quota Requested].

Figure 5.3.2.3.3 shows a scenario for Session based charging (ECUR) in Decentralized and Centralized Unit Determination, Centralized Rating configuration, where the NF (CTF) invokes a converged charging service towards the CHF, prior to service delivery if needed.

Figure 5.3.2.3.3: ECUR - Session based charging with - Decentralized and Centralized Unit Determination, Centralized Rating.

1)	Request for resource usage: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by the CHF to start.

2)	Units Determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging Data Request [Initial, Quota Requested]: The NF (CTF) sends the request to the CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Account, Rating, Reservation Control: the CHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the CHF performs the corresponding reservation.

5)	 Open CDR: based on policies, the CHF opens a CDR related to the service.

6)	Charging Data Response [Initial, Quota Granted]: The CHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

7)	 Granted Units Supervision: The service starts and the NF (CTF) monitors the consumption of the granted units.

8)	Content/Service Delivery: the NF (CTF) delivers the content/service based on the reserved number of units.

9)	Charging Data Request [Termination, Unit Used]: the NF (CTF) sends the request to the CHF, for charging data related to the delivered service with the consumed units.

10)		Account, Rating Control: The CHF performs the process for the delivered service involving rating entity and user's account balance.

11)		Close CDR: based on policies, the CHF closes the CDR with charging data related to the delivered service.

12)		Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

End of change