3GPP TSG-SA5 Meeting #156 	S5-244062

# Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	China Unicom

# Title:	pCR TR 28.874 Add potential solution for MBS broadcast service management

# Document for:	Approval

Agenda Item:	6.19.15

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	3GPP draft TR 28.874: " Study on management aspects of NTN phase 2 v0.2.0".

[2] 			SP-231733: " New SID on management aspects of NTN phase 2".

3	Rationale

This contribution proposes to add use case of the draft TR 28.874 based on SP-231733[2].

4	Detailed proposal

It proposes to make the following changes to the draft TR 28.874[1]

1st Change

5.2	Management for mobility coordination

5.2.3.3	Potential solutions

Propose a new IOC or the addition of new attributes to existing IOCs  to provision an Area of Interest  within NTN cell coverage for MBS services. It would enable the configuration of specific geolocation areas, ensuring that the gNB can understand the geolocation for broadcasting the MBS service. The proposed IOC or attributes may include the following:

- Coordinates: Latitude and longitude data that define the central point or boundaries of the area.

- Radius: The coverage radius around the central coordinates that defines the extent of the area to be covered by the MBS services.

- Polygonal Boundaries: the area can be defined by a set of latitude and longitude points that form a polygonal boundary

End of Change