3GPP TSG-SA5 Meeting #156 	S5-244116

Maastricht, NL, 19 - 23 Aug 2024

# Source:	China Unicom

# Title:	pCR TR 28.876 Add potential solution for Use case #6 Performance evaluation on resource load of RedCap network service

# Document for:	Approval

Agenda Item:	6.19.17

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.876: “Management aspects of RedCap feature”

[2]	3GPP TS 28.552: “Management and orchestration;5G performance measurements”

[3]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)"

3	Rationale

It was approved in SP-231734 to study the management of aspects of RedCap features. One of the working taks is to investigate the measurements and KPIs to evaluate the performance of NR networks delivering communication services for RedCap UEs. A use case and requirement on performance evaluation on resource load of RedCap network service was approved in the last meeting. In order to satisfy the requirement mentioned above, a potential solution is proposed in this contribution.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

## 5.6	Use case #6:  Performance evaluation on resource load of RedCap network service

### 5.6.1	Description

The radio resource utilization measurements could provide operators the load information of the radio network during the measurement time period. Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) relative to eMBB (enhanced mobile broadband) devices due to their reduced capabilities and lower data rate requirements. PRBs related measurements may include total PRB usage, PDSCH PRB Usage, PUSCH PRB Usage etc.

In TS 28.552 [10], radio resource utilization metrics are used to evaluate the performance of the entire network, but the current approach combines the resource utilization of RedCap UEs and non-RedCap UEs, which may not accurately reflect the performance of the RedCap service.

RedCap UEs typically have reduced capabilities and different performance characteristics compared to standard UEs.

Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) related to eMBB (enhanced mobile broadband) devices. By distinguishing between the two types of UEs, network operators can better identify and troubleshoot specific network resource issues related to RedCap UEs and non-RedCap UEs, thus can allocate resources more efficiently and ensure that RedCap UEs receive adequate resource.

Therefore, it is essential to investigate an approach for separately calculating the resource load of RedCap UEs and defining the resource load performance of networks that deliver RedCap service. This will ensure a more precise evaluation of the user experience for RedCap UEs, thus make the operator to be aware of whether a cell has ever experienced high load or not in the monitoring period, and is a key input to network capacity planning and load balancing for RecCap service.

Similarly, to distinguish RedCap UEs and non-RedCap UEs for metric of the number of the active UEs in each cell is helpful for operators to know how many DRBs are running with buffered data per cell for different kinds of UEs.

Through the metrics of radio resource utilization and active users of RedCap UEs and non-RedCap UEs, operators can better evaluate the overall consumption of network resources of RedCap UEs to facilitate adjustment of related network policies.

### 5.6.2	Potential requirements

REQ-RedCap-Perf-Resource: The 3GPP management system should have capability to provide measurements related to resource load of RedCap service.

5.6.3	Potential Solutions

5.6.3.1	Potential solution #1

This solution proposes to reuse and enhance the exsiting measurements related to resource load defined in TS28.552 [10] for this use case.

Current PRBs related measurements are specified in clause 5.1.1.2 in TS 28.552 [10]. In order to achieve the requirement in clause 5.6.2, most aspects of the existing measurments can be reused and some enhancements also need to be introduced.

In radio access procedure in 5G system, UEs (no matter legacy UEs or RedCap UEs) need to report to gNB about the NR UE Radio Access Capability Parameter using UE-NR-Capability IE, which is specified in clause 6.3.3 in TS 38.331 [2]. RedCapParameters is part of UE-NR-Capability IE which can be deliverd by the UE capability inquiry process.

Based on the procedures above, gNB can be aware of whether the UE is RedCap or not when a UE tries to access to NG-RAN. Consequestly, the measurements that performed after the inquiry of UE capability can be seperated for different types of UEs.

When there is more than one type of UEs (e.g. RedCap UEs, eMBB UEs) covered by a cell, the filter needs to be enhanced to optionally separate the measurement into subcounters to represent the RedCap PRB number.

Take the DL Total PRB Usage as an example:

- Introduce a new filter which can be named as NewFilter, then RRU.PrbDl can be optionally separated as RRU.PrbDl.NewFilter to represent RedCap PRB number when the value of NewFilter is RedCap.

The subcounters filtered by the new filter with the value of RedCap can be used as metrics of RedCap PRB number.

End of changes