3GPP TSG-SA5 Meeting #156 	S5-244132

Maastricht, Netherlands, 19 - 23 August 2024

# Source:CSCN

# Title: pCR TR28.846 Charging between satellite operator and satellite MVNO

# Document for:	Approval, Information, Discussion

Agenda Item:	7.5.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]		3GPP TS 22.101 V19.0.0 Service aspects; Service principles

[2]		3GPP TS 22.261 V19.7.0 Service requirements for the 5G system; Stage 1

[3]		3GPP TS 23.501 V19.0.0 System architecture for the 5G System (5GS)

[4]		3GPP TR 28.827 V18.0.0 Study on 5G charging for additional roaming scenarios and actors

[5]		3GPP TS 32.255 V18.4.0 5G data connectivity domain charging; stage 2

[6]		3GPP TS 32.256 V18.3.0 5G connection and mobility domain charging; stage 2

[7]		3GPP TS 32.254 V18.3.0 Application Program Interfaces (APIs) charging

3	Rationale

3.1Business scenario for NG-RAN sharing network is shared between multiple operators.

MVNO (Mobile Virtual Network Operator) is the acronym for mobile virtual network operators. These operators do not own their own wireless network infrastructure but instead lease network capacity from traditional mobile network operators (MNOs) and then provide services to end users under their own brand.

Satellite communication networks, as an advanced communication technology, offer advantages such as wide coverage and fast transmission. However, they also face significant challenges in infrastructure investment. For most ordinary operators, the cost of independently building and maintaining satellite communication infrastructure networks is prohibitively high. In this situation, the Mobile Virtual Network Operator (MVNO) model has become an important avenue for ordinary operators to enter the satellite communication service sector.

Through the MVNO model, ordinary operators can lease existing satellite communication infrastructure without bearing enormous initial investments, enabling them to provide satellite communication services to users. This model not only lowers the entry barrier for operators but also promotes the effective utilization of satellite communication resources. Additionally, the MVNO model can help operators rapidly expand their business scope and improve market competitiveness.

3.2Potential charging requirements

-	charging in the satellite MNO’s network for the purpose of wholesale charging towards an additional charging model actor e.g. Satellite MVNO.

4	Detailed proposal

This document proposes the following changes in TR 28.846.

1st Change

5. X	Business scenario for satellite MVNO

Business scenario#1a: MVNO (Mobile Virtual Network Operator) is the acronym for mobile virtual network operators. These operators do not own their own wireless network infrastructure but instead lease network capacity from Satellite mobile network operators (Satellite MNOs) and then provide services to end users under their own brand.

Satellite MVNO could be charged by Satellite MNO based on the total data volume, 5G connection and mobility, SMS transferred via the satellite, or based on resource usage.

Next Change

6	Charging scenarios and key issues

6. X	Topic 1 Charging in satellite operator for the purpose of wholesale charging satellite MVNO

6. X.1	Use cases

#### 6. X.1.1	Use case #1a: satellite operator does wholesale charging for 5G data connectivity towards additional actors

This use case focuses on satellite operator and satellite MVNO business roles.

A subscriber has a UE and a subscription with the MVNO which allows usage of 5G data connectivity while in the home satellite operator.

The satellite operator to MVNO charging could be based on data volume that the MVNO UE has used in the satellite operator’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

#### 6. X.1.2	Use case #1b: satellite operator does wholesale charging for 5G connection and mobility towards additional actors

This use case focuses on satellite operator and MVNO (additional actor) business roles.

A CSC (UE) has a subscription with the additional actor which allows usage of 5G data connectivity connection and mobility.

The home satellite operator to additional actor charging is based number of home satellite operator’s UEs (i.e. unique SUPI) connected to the visited satellite operator’s network over a specific period of time.

#### 6. X.1.3	Use case #1c: satellite operator does wholesale charging for 5G SMS towards additional actors

This use case focuses on satellite operator and MVNO (additional actor) business roles.

A CCS (UE) has a subscription with the MVNO which allows usage of SMS over NAS in 5GS.

The home satellite operator to additional actor charging could be based on number of SMS sent or received by the additional actor’s UEs served by the home satellite operator’s network considering of the following aspects:

-	RAT type;

-	Location

6. X.2	Potential charging requirements

REQ-CH_SMVNO-01: The charging mechanism in satellite operator should support collecting charging information related to 5G data connectivity usage for each MVNO.

REQ-CH_SMVNO-02: The charging mechanism in satellite operator should support collecting charging information related to 5G connection and mobility usage for each MVNO.

REQ-CH_SMVNO-03: The charging mechanism in satellite operator should support collecting charging information related to SMS usage for each MVNO.

6. X.3	Key issues

The following key issues are identified:

-	Key Issue #1a: Collection of charging information in satellite operator about the 5G data connectivity usage per MVNO.

-	Key Issue #1b: Collection of charging information in satellite operator about the 5G connection and mobility usage per MVNO.

-	Key Issue #1c: Collection of charging information in satellite operator about the SMS usage per MVNO.

6. X.4	Possible solutions

Following solution references clause 7.3.4.1 in TR28.827.

#### 6. X.4.1	Solution #1.1: CDR in satellite operator for wholesale of non-roaming 5G of additional actor without CHF

##### 6. X.4.1.1	General

A possible solution for key issue #3a, #3b, and #3c covering requirements REQ-CH_MNOMVNO-01, REQ-CH_MNOMVNO-02, and REQ-CH_MNOMVNO-03, charging for 5G data connectivity, connection and mobility, SMS, provided to the additional actor by the satellite operator, covering non-roaming case.

##### 6. X.4.1.2	Reference architecture

The AMF, SMF, and SMSF with CTF interact with CHF using Nchf interface, without any requirement of quota management. The CCS produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of data per additional actor, it could be done based on SUPI, IMSI series, SNSSAI(s), DNN(s) etc., the wholesale part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 6. X.4.1.3	Message flows

The flows would be the same as the non-roaming cases in clause 5.2.2 of TS 32.255 [4], clause 5.2.2 of TS 32.256 [5] and clause 5.4.2 of TS 32.274 [8].

6. X.5	Evaluation

Solutions #1.1 solves Key issue#1a, #1b, #1c.

6. X.6	Conclusion

End of change