3GPP TSG-SA5 Meeting #156	S5-244134

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Ericsson LM

# Title:	Use of CHF Group ID for CHF selection

# Document for:	Endorsement

Agenda Item:	7.4.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.840 Study on CHF Segmentation

[2]	3GPP TS 23.501 System architecture for the 5G System (5GS); Stage 2

[3]	3GPP TS 23.502 Procedures for the 5G System (5GS); Stage 2

[4]	3GPP TS 29.504 5G System; Unified Data Repository Services; Stage 3

[5]	3GPP TS 29.510 5G System; Network Function Repository Services; Stage 3

## 3.1	Background

In TR 28.840 [1] there are three solutions using the CHF Group ID:  #1.1 CHF Selection and Discovery for NF Service Consumers Solution based on UDM, #1.6 NRF inferred CHF Group ID, and #1.7 SCP retrieved CHF Group ID. The #1.6 and #1.7 follows the recommendations in TS 23.501 [2] clause 6.3.11 and 6.2.19 while the #1.1 requires a new type of interaction between CHF and UDM/UDR.

There are five Group IDs defined: UDM, UDR, AUSF, PCF, CHF, and HSS. According to TS 23.501 [2] the CHF Group ID works similarly to the Group ID for: UDM, AUSF, and PCF (i.e. like for solutions #1.6 and #1.7). All the Group IDs are described for the NF Profile in NRF, TS 23.501 [2] clause 6.2.6.2, except for CHF which is missing.

The NF Group ID can, according to TS 23.501 [2] table 7.2.10-1, be retrieved from the UDR using the Nudr_GroupIDmap service defined in TS 23.502 [3] clause 5.2.12.9, the Nudr_GroupIDmap is however defined in TS 23.502 [3] clause 5.2.12.3.

There are only two NF service consumers mentioned of the Nudr_GroupIDmap service in TS 29.504 [4] clause 5.3.1.1 and they are: SCP and NRF.

This means that if an NF Consumer would like to find a NF provider for the Nchf_ConvergedService, which is missing in TS 29.510 [5] table 6.1.6.3.11-1, the flows could look as follows.

Figure 3.1-1: Message Flow for CHF Selection based on NRF and CHF Group ID

1.	An NF Consumer that would like to find a NF Provider that can provide the Nchf_ConvergedService service will do a discovery towards the NRF and may include service name and SUPI that it would like to find the NF provider for.

2.	NRF decides that it needs to find the CHF Group ID to be able to find the correct CHF, and therefore query the UDR for the NF of type CHF and may include the SUPI.

3.	UDR responds with the CHF Group ID applicable for the SUPI provided.

4.	NRF responds with the CHF instances applicable and may include the CHF Group ID.

5.	NF Consumer initiates a converged charging service with a CHF instance provided by the NRF, it may use the CHF Group ID provided if a reselection is required.

If an SCP with delegated discovery is used between the NF Consumer and the CHF, then the SCP may do the query for CHF Group ID towards the UDR instead of the NRF. There is also a possibility for NRF and SCP to subscribe to changes of the mapping between user ID and NF Group ID

## 3.2 Evaluation

The CHF Group ID stored in the UDR can be used for determining CHF instance to use, it is however the NRF and SCP that decides wherever to use CHF Group ID, not the NF Consumer.

There are some issues with the specifications in relating to Group ID:

-	CHF Group IDs is missing in the description of NF Profile in NRF, TS 23.501 [2] clause 6.2.6.2.

-	Nudr_GroupIDmap in TS 23.501 [2] table 7.2.10-1 should refer to TS 23.502 [3] clause 5.2.12.3 (instead of 5.2.12.9).

-	Nchf_ConvergedService is missing in TS 29.510 [5] table 6.1.6.3.11-1.

## 3.3 Conclusion

No further specification is required to be able to use the CHF Group ID for CHF instance selection.

# 4	Detailed proposal

Add description that where to use CHF Group ID for CHF instance selection is done by NRF or SCP.

Annex A (informative):
Plant UML source code

# A.1	Message Flow for CHF Selection based on NRF and CHF Group ID

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

participant "NF Consumer" as SMF

participant "NRF" as NRF

participant "CHF" as CHF

SMF -> NRF : 1. Nrf_NFDiscovery_Request

note over SMF, NRF

target-nf-type = CHF

supi = "supi"

end note

NRF -> UDR : 2. Nudr_GroupIDmap Query

note over NRF, UDR

nf-type = CHF

subscriber-id = "supi"

end note

UDR -> NRF : 3. Nudr_GroupIDmap response

note over UDR, NRF

chfGroupID: "CHF Group ID"

end note

NRF -> SMF : 4. Nrf_NFDiscovery_Request response

note over NRF, SMF

nfInstanceId

chfInfo

groupId: "CHF Group ID"

end note

SMF -> CHF : 5. Nchf_ConvergedCharging request

note over NRF, SMF

nfInstanceId

chfInfo

groupId: "CHF Group ID"

end note

@enduml