3GPP TSG-SA5 Meeting #156	S5-244135

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Ericsson LM

# Title:	PCF interaction when N107 is used

# Document for:	Discussion

Agenda Item:	7.4.1

# 1	Decision/action requested

The group is asked to discuss the proposal.

# 2	References

[1]	3GPP TS 32.255 Charging management; 5G data connectivity domain charging; Stage 2

[2]	3GPP TS 32.256 Charging management; 5G connection and mobility domain charging; Stage 2

[3]	3GPP TS 23.501 System architecture for the 5G System (5GS); Stage 2

[4]	3GPP TS 23.502 Procedures for the 5G System (5GS); Stage 2

[5]	3GPP TS 23.503 Policy and charging control framework for the 5G System (5GS); Stage 2

## 3.1	Background

In TS 32.255 [1] and TS 32.256 [2] both describes an architecture where a CHF interacts with another CHF using N107, what is not fully documented is how the interaction between the PCF and the CHF.

There are basically four possible solutions:

1.	PCF interacts directly with both CHFs

2.	PCF interact via SCP with one CHF

3.	PCF interacts via another PCF for one CHF

4.	PCF interacts via one CHF for the other CHF

### 3.1.1	PCF interacts directly with both CHFs

This solution is based on TS 23.501 [3], and in this solution the PCF will have knowledge of both CHFs and interact with the CHF that have knowledge of the subscriber’s policy counters. The PCF may use the same way to find the A-CHF as the CHF would to find the A-CHF e.g., NRF or local configurations.

Figure 3.1.1-1: PCF interacts directly with both CHFs

### 3.1.2	PCF interact via SCP with one CHF

This solution is based on TS 23.501 [3], and in this solution the PCF use an SCP with delegated discover for at least the A-CHF (it may use it for both CHFs), to find the A-CHF that have knowledge of the subscriber’s policy counters. The PCF and SCP may use the same way to find the A-CHF as the CHF would to find the A-CHF e.g., NRF or local configurations.

Figure 3.1.2-1: PCF interact via SCP with one CHF

### 3.1.3	PCF interacts via another PCF for one CHF

This solution is based on TS 23.503 [5], and in this solution the V-PCF may retrieve UE policy information from the H-PCF over N24 and the H-PCF could interact with the H-CHF. Since the V-CHF wouldn’t have any subscriber related policy counters there would be no reason for the V-PCF to interact with the V-CHF for inbound roaming UEs.

Figure 3.1.3-1: PCF interacts via another PCF for one CHF

### 3.1.4	PCF interacts via one CHF for the other CHF

This solution is unspecified, and in this solution the PCF use the CHF for interaction with the A-CHF, if it is the A-CHF that have knowledge of the subscriber’s policy counters. How the CHF could find the A-CHF will be deployment dependent e.g., NRF or local configurations.

Figure 3.1.4-1: PCF interacts via one CHF for the other CHF

### 3.1.5	PCF interacts with both CHFs when SMF using N47

This solution is based on TS 23.501 [3] where the SMF uses the N47 to communicate with the A-CHF, and in this solution the PCF use the same way to find the CHFs as the SMF.

Figure 3.1.4-1: PCF interacts with both CHFs when SMF using N47

## 3.2 Evaluation

All solution except 4 are based on current specifications and are independent of if N47 or N107 is used.

Solutions 1 and 2 are the most straight forward solutions, where the NRF can be configured to give different CHF instances based on the service requested and/or the requester NF type.

Solution 3 is specified for local breakout roaming and could be used for the A-CHF scenario but would require a separate PCF for the A-CHF as well as complicating the case where both a A-CHF and a V-CHF could be involved.

Solution 4 would be similar to the solution 2 where the CHF would act like an SCP.

All the solutions can be considered outside the scope of 3GPP SA5.

## 3.3 Conclusion

The solutions 1, 2, and 3 could possibly be described in an informative document.

# 4	Detailed proposal

An informative annex could be added describing the deployment options based on 1,2 and 3 could be added.

Annex A (informative):
Plant UML source code

## A.1	PCF interacts directly with both CHFs

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

agent "A-CHF" as ACHF

agent "CHF" as CHF

agent "PCF" as PCF

agent "SMF" as SMF

agent "UPF" as UPF

CHF -- ACHF : N107

PCF -- ACHF : N28

SMF -- CHF : N40

PCF -- CHF : N28

PCF -- SMF : N7

UPF -- SMF : N4

@enduml

## A.2	PCF interact via SCP with one CHF

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

agent "A-CHF" as ACHF

agent "CHF" as CHF

agent "PCF" as PCF

agent "SCP" as SCP

agent "SMF" as SMF

agent "UPF" as UPF

CHF -- ACHF : N107

PCF -- SCP : N28

SCP -- ACHF : N28

SMF -- CHF : N40

PCF -- CHF : N28

PCF -- SMF : N7

UPF -- SMF : N4

@enduml

## A.3	PCF interacts via another PCF for one CHF

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

rectangle VPLMN {

agent "V-CHF" as ACHF

agent "V-PCF" as VPCF

agent "SMF" as SMF

agent "UPF" as UPF

}

rectangle HPLMN {

agent "H-CHF" as CHF

agent "H-PCF" as PCF

}

CHF -- ACHF : N107

PCF -- VPCF : N24

VPCF .. ACHF : N28

SMF -- ACHF : N40

PCF -- CHF : N28

VPCF -- SMF : N7

UPF -- SMF : N4

@enduml

## A.4	PCF interacts via one CHF for the other CHF

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

agent "A-CHF" as ACHF

agent "CHF" as CHF

agent "PCF" as PCF

agent "SMF" as SMF

agent "UPF" as UPF

CHF -- ACHF : N107

SMF -- CHF : N40

PCF -- CHF : N28

PCF -- SMF : N7

UPF -- SMF : N4

@enduml

## A.5	PCF interacts with both CHFs when SMF using N47

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

agent "A-CHF" as ACHF

agent "CHF" as CHF

agent "PCF" as PCF

agent "SMF" as SMF

agent "UPF" as UPF

SMF -- ACHF : N47

PCF -- ACHF : N28

SMF -- CHF : N40

PCF -- CHF : N28

PCF -- SMF : N7

UPF -- SMF : N4

@enduml