3GPP TSG-SA5 Meeting #156 	S5-244226

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Discussion paper on signaling monitoring activation

# Title:	Vodafone

# Document for:	Endorsement

Agenda Item:	6.1

# 1	Decision/action requested

The group to discuss and endorse the proposal.

# 2	References

[1]	3GPP TS 28.532 Management and orchestration; Generic management services

# 3	Rationale

The current 5G system prevents continuation of the network surveillance capabilities carried out by MNOs. This is due to the introduction of encryption of the signalling exchanged between network functions and the different implementations of VNFs/CNFs from vendors in the 5G network. For network operators, standard monitoring capabilities are thus essential to continue performing health checks and troubleshooting of networks without additional integration costs and without the need to standardise, in 3GPP, the internals of VNFs/CNFs.

This work was triggered by the GSMA which expressed their concerns about the monitoring of encrypted signalling traffic looking for a mechanism for a copy of this traffic to be sent to a monitoring system. This concern was reflected in a Liaison Statement sent from the GSMA to 3GPP (WGs SA2, SA3 and SA5) explaining that the current encryption mechanism sending the information with (m)TLS prevents the operators of doing proper troubleshooting for operation and management procedures.

SA WG1 has agreed the feature level requirements needed for monitoring of signalling traffic in a secure way.

- One of the SA1 requirements is:

  - The 5G network shall enable the MNO to configure network monitoring, e.g., switching on/off per network element, selecting what type of elements and what type of signalling from these elements is the target for monitoring.

SA5 should be the WG enabling this mechanism.

The requirement from SA1 implies that the external monitoring system that will receive copies of the signalling traffic needs to be able to interact with the monitored NEs to enable the signalling monitoring. For doing so a mechanism to instruct the delivery of the monitored traffic needs to be put in place.  A new service needs to be created completely dedicated to this purpose. That service would be simpler from specification and implementation perspective, only used for this purpose and including only the needed operations.

A new Service called Signalling monitoring service could be created with the following operations:

- establishSignallingMonitoring operation

- terminateSignallingMonitoring operation

This new service allows the external Monitoring System that will receive the flow of Signalling Packets to instruct the Network Element to send the copies of the signalling traffic. It will comply with the requirement of having a switching on/off per network element.

These operations would need to be defined as subscribe-notify, with a notify operation in addition that should carry the copy of the signalling traffic as a file with the proper format.

NOTE: The security requirements for authentication, authorisation, integrity protection, application level protection, transport protection, etc are within the scope of SA3.

# 4	Detailed proposal

The contributors ask SA5 to endorse the creation of a new service, as described above, to accommodate the stage 2 requirements responding to MonStra work.