3GPP TSG-SA5 Meeting #156 	S5-244246

# Maastricht, The Netherlands, 19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0600 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Measurements for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of performance measurements for LTM is needed, in the same way as it was for legacy handover, CHO and DAPS handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurements for Inter-gNB LTM Cell Switches added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperable observability of LTM performance will not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.x (new), 5.1.1.6.y (new), 5.1.1.6.y.1 (new), 5.1.1.6.y.2 (new), 5.1.1.6.y.3 (new), 5.1.3.7.1.z (new), 5.1.3.7.1.w (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.1.6.x	Inter-gNB LTM Cell Switches

Editor's note: Placeholder for measurements for Inter-gNB Cell Switches.

##### 5.1.1.6.y	Intra-gNB LTM Cell Switches

###### 5.1.1.6.y.1	Number of configured LTM Cell Switch candidates

a)	This measurement provides the number of outgoing intra-gNB LTM Cell Switch candidates requested by the source NRCellCU.

b)	CC.

c)	On transmission of RRCReconfiguration message (TS 38.331 [20] clause 5.3.5), where the message denotes a LTM Cell Switch configuration, to the UE configuring an intra-gNB LTM Cell Switch from the source NRCellCU to the target NRCellCU. The counter on NRCellCU is incremented by the number of candidates configured in the LTM-Candidate IE. The counter on NRCellRelation is incremented by 1 for each relation that is present in the LTM-Candidate IE.

d)	A single integer value.

e)	MM.ConfigIntraReqLTMCellSwitch.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.y.2	Number of UEs configured with LTM Cell Switch

a)	This intra-gNB handover measurement provides the number of UEs that has been configured with LTM Cell Switch by the source cell.

b)	CC.

c)	On reception of RRC ReconfigurationComplete message (see TS 38.331 [20] clause 5.3.5) from the UE to the target NRCellCU indicating a successful intra-gNB LTM Cell Switch, the counter is stepped by 1.

d)	A single integer value.

e)	MM.ConfigIntraReqLTMCellSwitchUes.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.y.3	Number of successful LTM Cell Switches

a)	This measurement provides the number of successful intra-gNB LTM Cell Switch executions received by the source NRCellCU.

b)	CC.

c)	On reception of RRCReconfigurationComplete message (see TS 38.331 [20] clause 5.3.5) from the UE to the target NRCellCU indicating a successful intra-gNB LTM Cell Switch, the counter is stepped by 1.

d)	A single integer value.

e)	MM.LTMCellSwitchExeIntraSucc.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

Next change

###### 5.1.3.7.1.z	Number of UEs for which LTM Cell Switch preparations are requested

a)	This measurement provides the number of UEs for which outgoing intra-gNB LTM Cell Switch preparations are requested by the source NRCellCU for a split gNB deployment.

b)	CC.

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP is sending a UE CONTEXT MODIFICATION REQUEST message (see TS 38.473 [6] clause 8.3.4) to gNB-DU or UE CONTEXT SETUP REQUEST message (see TS 38.473 [6] clause 8.3.1) to target gNB-DU to request resources for an intra-gNB LTM Cell Switch. The counter is incremented by 1 for each UE, even if UE CONTEXT MODIFICATION REQUEST or UE CONTEXT SETUP REQUEST messages were sent for several cells.

d)	A single integer value.

e)	MM.LTMCellSwitchPrepIntraReqUes.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.w	Number of UEs for which LTM Cell Switch preparations are successful

a)	This measurement provides the number of UEs for which intra-gNB LTM Cell Switch preparations received by the source NRCellCU are successful, for a split gNB deployment.

b)	CC

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP receives a UE CONTEXT MODIFICATION RESPONSE message (see TS 38.473 [6] clause 8.3.4) from gNB-DU or UE CONTEXT SETUP RESPONSE message (see TS 38.473 [6] clause 8.3.1) from target gNB-DU to initiate a successful intra-gNB LTM Cell Switch. The counter is incremented by 1 for each UE, even if UE CONTEXT MODIFICATION RESPONSE or UE CONTEXT SETUP RESPONSE messages were received for several. cells.

d)	A single integer value.

e)	MM.LTMCellSwitchPrepIntraSuccUes.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

End of changes