3GPP TSG-SA5 Meeting #156 	S5-244308

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Evaluation analysis for authorization of the external MnS consumer to access the management service API use case

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes to add an evaluation analysis for the authorization of the external MnS consumer to access the management service API use case in clause 5.1.4 of TR 28.879 [1].

# 4	Detailed proposal

It is proposed that the following changes be made in clause 5.1.4 of TR 28.879 [1].

Begin Change

#### 5.1.4.3	Potential solutions

###### 5.1.4.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

###### 5.1.4.3.i.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

#### 5.1.4.4	Evaluation of potential solutions

The proposed solution covers the requirement PREQ-FS_MExpo-Auth-01 of the use case. However, the proposed solution has the following limitations:

- The proposed solution expresses the access rule(s) by using Jex expressions to select the resources (i.e., the dataNodeSelector attribute of the accessRule class). The CCF authorization server can not process these Jex expressions.

- Given that the access token is carried in the HTTP authorization header, size limitations are introduced on how big the access token could be. In the 3GPP management system, the access control for each MnS consumer is at the granularity of the resources (i.e., MOIs, IOCs, or their corresponding operations), the operations that can be performed on these resources, and the actions that are allowed on the permitted operations. This implies that as the number of resources the external MnS consumer is authorized to access increases, so does the size of the access token since the access rule(s) are stored in the "scope" attribute of the access token.

- At the MnS producer, depending on the size/complexity of the access rules within the access token,  there is a computational cost or delay associated with processing the access token provided by the external MnS consumer.

End of Change